3GPP TSG-RAN WG4 Meeting #112-bis	R4-2416137

Hefei, China, October 14th – 18th 2024

Agenda Item:	6.20.2.3

Source:	Ericsson

Title:	Impact on SBFD BS RF RX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF RX requirements for SBFD slots. For convenience, the content in Clause 10.1 in TR 38.858 (V18.0.0) [1] was copied to the tables under each requirement for the purpose of making observations and proposals.

Discussion on existing BS RF TX requirements for SBFD slots is presented in R4-2416136 [2], potentially new requirements for SBFD slot is presented in R4-2416135 [3], and SBFD general aspects is presented in R4-2416134 [4].

# 2	Co-existence simulation for in-band blocking

2.1	Simulation assumption

## 1.8. In-band blocking Agreement: - On contributing source for in-band blocking requirement, it is only the DL adjacent channel interference. - RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements. - For co-existence study, - The reference point for the power level which should be before array gain WF: - FFS on the followings: - RAN4 needs to discuss on how in-band blocking requirement are derived: - Option 1: MCL assumption for BS2BS CLI interference - Option 2: Co-existence study - Option 3: Both - For co-existence study, RAN4 need firstly agree on the open parameters for simulation assumption: - FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - 99% of the UL SBFD wideband received power CDF

In RAN4#112 meeting, several agreements were reached concerning the additional co-existence simulation for in-band blocking requirement derivation. However, there are still some open issues that require further study.

Regarding the requirement derivation, majority companies have done the co-existence simulations, and the results are largely similar. Therefore, we think the requirement could be derived based on the simulation results.

Derive in-band blocking requirement based on co-existence simulation results.

About other options that are FFS we consider the following assumptions:

- About different grid-shift values, we believe the low grid-shift is a reality and thus the lowest grid-shift scenario should be considered.

The worst-case scenario is a grid-shift as small as possible, to account for uncoordinated deployments. We propose to consider 5%. As comparison, the results for 10% and 100% are also provided.

- About how to use the power level probability distributions to derive the in-band blocking levels, we support option 1:

Find the X% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios. We propose X to be 99.

In the following sections, our simulation results for FR1, as well as the discussion and proposals based on the results, are provided.

2.2	Discussion of simulation results for FR1

In this section we focus on discussing simulation results that have been obtained following the procedure described in section 2, and with focus on FR1 scenarios: scenarios 1 (urban macro), 3 (indoor hotspot), 5 (Micro). The simulation assumptions strictly follow those agreed for the coexistence simulation study conducted during the SI phase and captured in TR 38.858, and are also summarized in the Annex.

2.2.1 Urban Macro

We consider a coexistence scenario where urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%. The grid-shift examples are depicted in Figure 2.2.1-1.

Figure 2.2.1-1: Grid-shift 10% and 100% between two UMa network deployments

Two different values of BS TRP are considered 1) 49 dBm, which is the default value in the assumptions of the SI; 2) 53 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.1-2.

Figure 2.2.1-2: FR1 UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.1-2 it can be observed that the in-band blocking requirement is higher as the TRP increases or the grid-shift decreases. In addition, it can be observed that when considering TRP 49 dBm and grid-shift 100% (best case assumptions), we are already around 20 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS.

- Considering TRP 53 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is 4.6 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -0.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -20.4 dBm

- Considering TRP 49 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is 1.3 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -4.7 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -23.4 dBm.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network. Note that 53 dBm TRP is quite common.

- The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

- For FR1 urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Define in-band blocking interferer signal power to 4.6 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

2.2.2 Urban Micro

We consider a coexistence scenario where urban micro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 5% and 10%. The grid-shift examples are depicted in Figure 2.2.2-1.

Figure 2.2.2-1: Grid-shift 10% and 100% between two UMi network deployments

Two different values of BS TRP are considered 1) 46 dBm, which is the default value in the assumptions of the SI; 2) 38 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.2-2.

Figure 2.2.2-2: FR1 UMi: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.2-2 it can be observed that the in-band blocking requirement is higher as the TRP increases or the grid-shift decreases.

Current in-band blocking level for Medium Range BS is -38 dBm, so we can observe that when we consider lower power levels and coordinated deployments (100% grid-shift), there would not be need for higher in-band blocking requirements. However, when the transmission power is higher or the grid-shift is reduced, more stringent requirements would be needed.

- Considering TRP 46 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is -1.7 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -7.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -31.5 dBm.

- Considering TRP 38 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is -9.3 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -15.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -39.4 dBm.

- In FR1 UMi deployments, with mid-power BSs, the need for more stringent in-band blocking requirements is deployment dependent. Coordinated deployments operating in lower power level may not need an update in requirements, but uncoordinated deployments may need it.

Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

2.2.3 Indoor office

We consider a coexistence scenario where indoor office deployments of two different operators are deployed in the same building and in adjacent channels, as shown in Figure 2.2.3-1. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Figure 2.2.3-1: InH deployment with two coordinated operators

The indoor scenario facilitates coordination of the two operators, compared to outdoor deployments, so that it is inherently coordinated with ISD= 20 m. The BS transmission power is fixed at 24 dBm. The maximum UE power is set to 23 dBm, so that there is not a big difference between UEs and BSs in terms of potential in-band blocking level. This results in that the current requirement is still valid. Figure 2.2.3-2 shows the UL received wideband power from the neighbor operator. For comparison we consider the case where the victim is the UL of SBFD, and the in-band blocking level is represented by the TDD BSs of the neighbor operator transmitting DL, and the synchronized case where the victim is a TDD deployment so that the blockers are the UEs. It can be observed that both the curves are below the current in-band blocking level of -35 dBm for Local Area BSs. This means there is no need to modify the in-band blocking level for this BS class for SBFD networks.

Figure 2.2.3-2: FR1 InH: Received UL wideband power from DL of TDD network 2

- In FR1 InH deployments, with low-power BSs, the power level of the BSs (24 dBm) is comparable to the maximum power level of the UEs (23 dBm), so that the fact that the BSs are blockers for an SBFD deployment, does not introduce a need for more stringent requirements for this BS class.

- FR1 InH deployments are easier to coordinate, so that certain distance between BSs of different networks can be assumed, and this also facilitates that there is no need for more stringent requirements.

Re-use the existing in-band blocking requirement for FR1 Local Area BSs in SBFD networks for both DUD and DU/UD configurations.

# 3	Impact on BS RX requirements

Reference sensitivity level and OTA sensitivity

## 1.5. OTA sensitivity degradation Agreement: - OTA sensitivity is measured with only self-interference. FFS on the OTA sensitivity degradation within [0.5~1.0] dB.

It has been agreed that an OTA sensitivity requirement, based on sensitivity in the RX sub-band with the transmitter turned on in the TX sub-bands is needed for SBFD. Some degradation compared to the sensitivity requirement in non-SBFD slots should be allowed for, where it is important to consider that all sources of interference, i.e., self-interference, inter-sector interference, inter-site interference and interference from other operators, will cause the sensitivity of the receiver to degrade. An open issue for discussion is the value of degradation. The limiting factor can be IM3 in receiver, especially for wide area deployment, the degradation of the receiver is due to IM3 can get many more dBs rapidly.

For example, if the gNB self-interference degradation is 1 dB (1.25 in linear with the corresponding self-interference relative to noise being 0.25) and the network can be designed such that the sensitivity degradation from other sector and site interference (without self-interference) would also be 1 dB (1.25 in linear with the corresponding other interference relative to noise being 0.25), then the total sensitivity degradation for a real network experiencing both self-interference and other-sector interference would be 1.8 dB (1.5 in linear with the corresponding total interference relative to noise being 0.5) if the interferences would just be additive and uncorrelated. If the receiver is IM3 limited, considering the interference at the input to the LNA is doubled, the interference at the output increases by 2 ^ 3, i.e. 8 times, then the total interference becomes 8*0.25 = 2. Hence, the ratio of (interference + noise) / noise becomes (2+1)/1 = 3 in linear, or 4.8dB. These two numbers represent lower and upper boundaries, so the real total sensitivity degradation would be somewhere between 1.8 dB and 4.8 dB for such a deployment.

If the self-interference degradation is 0.5 dB and the sites can be designed such that the degradation from other sectors or sites without self-interference would also be 0.5 dB then the desensitization in a real network with both sources present would be between 0.95 dB and 3.2 dB.

Thus, if the desensitization due to self-interference alone is 1 dB then in a real deployment, even with highly isolated sites, it would not be possible to keep the desensitization less than 2 - 4 dB.

- OTA sensitivity should be defined considering in real life the receiver is further desensitized by other sources of interference including inter-site interference and inter-sector interference.

- RAN4 requirements should be conservative enough that the SBFD BS can be expected to perform well in real deployments considering inter-site interference and inter-sector interference.

- Since the receiver performance is limited by receiver non-linearity, the sensitivity degradation is much greater than just the sum of the sensitivity degradation from each source alone.

For SBFD-capable BS OTA sensitivity requirement, select 0.5 dB as the maximum degradation value.

Dynamic range

## 1.6. Dynamic range WF: - FFS on the followings: - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work. - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations. - Option 3: for receiver dynamic requirement, both uplink signals and BS2BS CLI signal should be considered for IoT levels.

In normal uplink, the IoT depends on the expected interference from other UEs. While for SBFD, it actually depends on the interference from other base stations. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

- RX dynamic range requirement is applicable for SBFD-capable BS. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

In-band selectivity and blocking

Other operators BSs may be co-located with the SBFD BS or may be at some distance. There is currently no requirement for supporting co-location of non-synchronized operators in the same band. The nearest requirement is the FDD protection of own receiver requirement. A co-location blocking requirement could be created for SBFD and declaration based; declaring compliance to the requirement would demonstrate sufficient RX performance to enable co-location. However, the requirement would not necessarily ensure that emissions from the co-located BS would not desensitize the SBFD receiver.

- The BS co-location requirements can be applied as they are, with a declaration whether the SBFD BS meets the co-location requirements.

For non-co-located blocking, dedicated co-existence simulations were conducted to determine the expected blocker levels due to other operator site BS during SBFD slots. Results and discussion are presented in Section 2.

Moreover, it worth noting that, the definition of in-band blocking contains an offset, while the simulation results do not take the offset into account. However, the offset cannot be applied outside of both DUD and DU/UD configurations, which means that the proposed interferer level will be the ACS interferer level and needs to be applied directly outside the carrier supporting SBFD, as illustrated in Figure 3-1 and Figure 3-2.

Figure 3-1 Interferer applied outside of SBFD DU configuration

Figure 3-2 Interferer applied outside of SBFD DUD configuration

Out-of-band blocking

Regarding Out-of-band blocking requirement, the existing OOBB requirement is still applicable for SBFD-capable BS except for OTA sensitivity degradation with the power level of wanted signal taken into account.

- The OOB blocking requirement is the same in SBFD slots as for normal TDD.

Receiver spurious emissions

Regarding the receiver spurious emission requirement, apart from existing requirements for normal reception on UL symbols/slots, it’s not necessary to specify additional receiver spurious emissions requirement for SBFD operation in SBFD symbols/slots.

- The receiver spurious emissions requirement is the same for both SBFD and non-SBFD slots.

- The receiver spurious emission is only measurable with conducted testing and OTA testing with transmitter deactivated in SBFD slots.

Receiver intermodulation

The RX intermodulation requirement for conventional TDD consists of a CW signal and a modulated signal that are located in frequency such that their intermodulation produce falls within the receiver frequency range. The RX intermodulation sets a minimum requirement on receiver linearity.

For SBFD, it is possible that a single signal can intermodulate in the receiver with the TX sub-band TX signal. It is necessary to supress such intermodulation sufficiently that the receiver is not desensitized. In this case, transmission in DL sub-band needs to be activated when testing RX IM, and the offsets are chosen in a way that intermodulation product between DL sub-band and interferer falls in the UL sub-band.

The scenarios for 2 input signals and 1 input signal + TX sub-band may need different levels of receiver linearity to be met, depending on the level of the TX sub-band signal in the receiver. Thus, in our view, a requirement based on a single input signal causing IM with the TX sub-band could also be introduced to ensure additional robustness, or such a requirement may anyhow be covered by the RX blocking requirement for SBFD slots.

Investigate whether an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness, and whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

Observation 1	Considering TRP 53 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is 4.6 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -0.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -20.4 dBm

Observation 2	Considering TRP 49 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is 1.3 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -4.7 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -23.4 dBm.

Observation 3	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network. Note that 53 dBm TRP is quite common.

Observation 4	The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

Observation 5	For FR1 urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Observation 6	Considering TRP 46 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is -1.7 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -7.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -31.5 dBm.

Observation 7	Considering TRP 38 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is -9.3 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -15.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -39.4 dBm.

Observation 8	In FR1 UMi deployments, with mid-power BSs, the need for more stringent in-band blocking requirements is deployment dependent. Coordinated deployments operating in lower power level may not need an update in requirements, but uncoordinated deployments may need it.

Observation 9	In FR1 InH deployments, with low-power BSs, the power level of the BSs (24 dBm) is comparable to the maximum power level of the UEs (23 dBm), so that the fact that the BSs are blockers for an SBFD deployment, does not introduce a need for more stringent requirements for this BS class.

Observation 10	FR1 InH deployments are easier to coordinate, so that certain distance between BSs of different networks can be assumed, and this also facilitates that there is no need for more stringent requirements.

Observation 11	OTA sensitivity should be defined considering in real life the receiver is further desensitized by other sources of interference including inter-site interference and inter-sector interference.

Observation 12	RAN4 requirements should be conservative enough that the SBFD BS can be expected to perform well in real deployments considering inter-site interference and inter-sector interference.

Observation 13	Since the receiver performance is limited by receiver non-linearity, the sensitivity degradation is much greater than just the sum of the sensitivity degradation from each source alone.

Observation 14	RX dynamic range requirement is applicable for SBFD-capable BS. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

Observation 15	The BS co-location requirements can be applied as they are, with a declaration whether the SBFD BS meets the co-location requirements.

Observation 16	The OOB blocking requirement is the same in SBFD slots as for normal TDD.

Observation 17	The receiver spurious emissions requirement is the same for both SBFD and non-SBFD slots.

Observation 18	The receiver spurious emission is only measurable with conducted testing and OTA testing with transmitter deactivated in SBFD slots.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Derive in-band blocking requirement based on co-existence simulation results.

Proposal 2	Define in-band blocking interferer signal power to 4.6 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

Proposal 3	Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

Proposal 4	Re-use the existing in-band blocking requirement for FR1 Local Area BSs in SBFD networks for both DUD and DU/UD configurations.

Proposal 5	For SBFD-capable BS OTA sensitivity requirement, select 0.5 dB as the maximum degradation value.

Proposal 6	Investigate whether an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness, and whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.

# References

TR 38.858, Study on evolution of NR duplex operation

R4-2416136, Impact on SBFD BS RF TX requirements, Ericsson

R4-2416135, Potentially new SBFD BS RF requirements, Ericsson

R4-2416134, Discussion on SBFD general aspects, Ericsson