3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416182

Hefei, Anhui, China, October 14 - 18, 2024

Source: 	Huawei, HiSilicon

Title: 	On 6Rx SRS antenna switching requirements

Agenda Item:	6.1.1.3.3

Document for:	Approval

Introduction

During RAN4#112, FR1 6Rx UE RF requirements have been discussed. Following conclusion regarding ΔTRxSRS can be found in [1].

- Topic 2: SRS antenna switching and ΔTRxSRS - Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Way forward: Further discuss the following options. Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Issue 2-1-2: Whether to consider separate ∆TRxSRS values when the device is power class 2 in the band and ΔPPowerClass = 0dB and not indicating Tx diversity capability Agreement: Have additional 3dB for power class 2 when ΔPPowerClass = 0dB and not indicating Tx diversity capability. - Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers. - Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n41, n77, n78; 4.0; 3.5; 4.5. - Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79, n104; 5.4; 4.7; 5.9. - Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79; 5.4; 4.7; 5.9. Band n104; 5.7; 5.0; 6.3.

In this contribution, we would like to share our views regarding this topic.

Discussion

While RAN1 will consider introducing 3T6R SRS for antenna switching in Rel-19 as requested in the latest WID [2], we provide our preference on the prioritized 1T6R and 2T6R ΔTRxSRS requirements.

From our understanding, at least following aspects should be carefully considered:

- Due to form factor limitation, physical antenna layout in order to accommodate increased Rx number could lead to compromised performance on:

  - RF isolation, and

  - Antenna efficiency

- Availability on RF resource could be different per operating band

And it was shared by some companies that the aforementioned implementation difficulties could be more serious for higher frequency like n104.

Observation 1: Additional breakpoint for bands whose FUL_high is higher than the FUL_low of band n104 may be needed.

As captured in the WID [2], the 6Rx SRS insertion loss requirement is targeting at both handheld and FWA UE. As for FWA, there seems to be less controversial given that the 8Rx requirements have already been introduced. When it comes to handheld UE, it could be more complicated because RF component layout and/or trace loss could vary depending on phablet or foldable phone. For instance, as one implementation challenge for foldable phone, the insertion loss would increase for a diversity branch lays through the hinge connecting different screens.

Observation 2: The derivation on 6Rx ∆TRxSRS requirement needs to consider real implementation in both phablet and foldable phone.

- For a foldable phone, increasing insertion loss can be expected for a diversity branch that lays through the hinge connecting different screens.

Hence our preference is to follow the way in Option 11, which is to specify different requirements for n79 and n104. Regarding the value for different AS-SRS capability, we think a 1dB margin on top of the values proposed for band n79 can be enough for band n104.

Proposal 1: Using the following values as starting point.

|  | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79 | 5.4 | 4.7 | 5.9 |
| Band n104 | [6.4] | [5.7] | [6.9] |

Conclusion

In this contribution we discussed on the 6Rx SRS antenna switching requirements. We have the following observations and proposal:

Observation 1: Additional breakpoint for bands whose FUL_high is higher than the FUL_low of band n104 may be needed.

Observation 2: The derivation on 6Rx ∆TRxSRS requirement needs to consider real implementation in both phablet and foldable phone.

- For a foldable phone, increasing insertion loss can be expected for a diversity branch that lays through the hinge connecting different screens.

Proposal 1: Using the following values as starting point.

|  | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79 | 5.4 | 4.7 | 5.9 |
| Band n104 | [6.4] | [5.7] | [6.9] |

- References

- R4-2414285, “WF on requirements for 6Rx UE requirements”, AT&T, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024.

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, HiSilicon, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17-20, 2024.