3GPP TSG-RAN WG4 Meeting #112bis	R4-2416378

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	LP-WUS ACS

Source:	OPPO

Agenda item:	6.23.2.2

Document for:	Approval

- Introduction

This contribution provides our simulation results and findings on LP-WUS ACS requirement based on the simulation assumptions agreed in RAN4 #111 and #112 meetings.

- Discussion

Based on the simulation parameters for LLS simulation agreed in the WF [1] of RAN4 #111 meeting, we simulate the MDR performance versus ACS values. Some configurations are compared to quantify the impact on ACS values.

The basic simulation parameters are listed as below.

- Center frequency: 900MHz

- SCS: 30kHz

- UE channel bandwidth: 20MHz (51 RB)

- WUS position within channel: edge of 20MHz channel

- Channel model: AWGN

- Performance metric: 1% MDR/BLER

Besides the above simulation parameters, the following comparisons are performed based on simulation results.

- Wave form: OOK-4(M=1) vs OOK-4(M=2)

- Filter order: Lowpass Butterworth 3th order vs 5th order

- ADC bit width: 4 bits vs 8 bits

- Guard RB: 1 RB vs 2 RB vs 3 RB vs 4 RB

  - OOK-4 (M=1) vs OOK-4 (M=2), with 1/2/3/4 of guard RB

The simulation results of OOK-4 (M=1) vs OOK-4 (M=2) are illustrated in the below figure.

For OOK-4 (M=1), the ACS values are in the range of 33dB to 34dB with 1~4 guard RB. For OOK-4 (M=2), the ACS values are in the range of 25dB to 30dB when the guard RB is configured with 1/2/3/4 RB respectively. OOK-4 (M=1) can withstand higher NR signal level compared with OOK-4 (M=2). And OOK-4 (M=1) has less than 1dB deviation when guard RB is configured with 1~4 RB, while OOK-4 (M=2) has nearly 5dB spread with guard RB configured 1~4 RB.

Observation 1: OOK-4 (M=1) has 4-8dB higher ACS values than OOK-4 (M=2).

  - Lowpass Butterworth 3th order vs 5th order, with 1/2/3/4 of guard RB

The simulation results of Lowpass Butterworth 3th order vs 5th order are illustrated in the below figure.

For 3th order Lowpass Butterworth filter, the ACS values are in the range of 26dB to 30dB with 1/2/3/4 guard RB. For 5th order Lowpass Butterworth filter, the ACS values are the same as OOK-4 (M=1) in the 2.1 section. 5th order filter can bring 3~7dB increase in ACS, with the cost of the filter complexity raise.

Observation 2: 5th order filter can bring 3~7dB increase in ACS.

  - ADC 4 bits vs 8bits, with 1/2/3/4 of guard RB

The simulation results of 4 bits ADC vs 8 bits ADC are illustrated in the below figure.

For each number of guard RB, the ACS values for ADC 4 bits and ADC 8bits are almost the same, i.e. the gap between 4 bits and 8 bits is less than 1dB. Therefore, more ADC bits can not benefit to improve ACS performance. To keep the simplicity of the LP-WUR system, it is proposed to specify the ACS requirement based on ADC bit width equals 4 bits.

Observation 3: More ADC bits can not benefit to improve ACS performance.

Proposal: It is proposed to specify the ACS requirement based on ADC bit width equals 4 bits from LP-WUR system simplicity perspective.

- Conclusions

This contribution discusses our simulation results and findings on LP-WUS ACS requirement. And provide the following proposal.

Observation 1: OOK-4 (M=1) has 4-8dB higher ACS values than OOK-4 (M=2).

Observation 2: 5th order filter can bring 3~7dB increase in ACS.

Observation 3: More ADC bits can not benefit to improve ACS performance.

Proposal: It is proposed to specify the ACS requirement based on ADC bit width equals 4 bits from LP-WUR system simplicity perspective.

- References

[1] R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo