# 3GPP TSG-RAN WG4 Meeting # 112      													R4-2414273

# Maastricht, Netherlands, 19th-23th, Aug, 2024

Title: 	WF on introduction of NR band n68

Agenda item:	7.16.1

Source:	Ericsson

Document for:	Approval

# 1	Background

This contribution  is a WF on band n68.

# 2 	Discussion

## 2.1	System parameters

The following system parameters for n68 are agreed.

- Band definition:

Table 2.1-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n68 | 698 MHz – 728 MHz | 753 MHz – 783 MHz | FDD |

- UL MIMO :

Table 2.1-2: NR operating bands for UL MIMO in FR1

NR operating band

n68

- Channel BWs:

Table 2.1-3 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n68 | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Channel raster:

Table 2.1-4: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 100 | 139600 – <20> – 145600 | 150600 – <20> – 156600 |

- Enhanced channel raster:

Table 2.1-5: Applicable NR-ARFCN per operating band for enhanced channel raster

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 10 | 139600 – <2> – 145600 | 150600 – <2> – 156600 |

- Sync raster:

Table 2,1-6: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 15 kHz | Case A | 1888 – <1> – 1951 |

- TX-RX frequency separation:

Table 2,1-7: UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n68 | 55 MHz |

## 2.2	UE RF requirements

### 2.2.1	Changes overview

Agreement on the following UE RF expected changes:

Table 2.2.1-1: Expected UE RF changes to TS 38.101-1 due to the introduction of band n68

| Requirement | n68 proposal |
| --- | --- |
| UE maximum output power | PC3 |
| A-MPR | NS_26 and NS_36 to be updated |
| Spurious emissions for UE co-existence | LTE requirement is a baseline |
| REFSENS power level | Re-evaluate NR REFSENS (10 and 15 MHz CBW) as NRB are not the same for LTE and NR |
| REFSENS UL allocation | Re-use LTE specification |
| IBB | Add n68 into Table 7.6.2-2 |
| OBB | Add n68 into Table 7.6.3-2 |
| NBB | Add n68 into Table 7.6.4-1 |

### 2.2.2	A-MPR evaluation

Agreement: Re-evaluate NS_26 and NS_36 A-MPR for 10 and 15 MHz channel BW with the following assumptions:

  - PA model calibration

    - DFT-s-OFDM QPSK 20MHz

    - 100 RBs

    - 4dB post PA loss

    - 1dB MPR

  - Carrier Leakage: 28dBc

  - IQ Image: 28dBc

  - CIM3: 60dBc

  - EVM: 17.5%

  - SCS: 15kHz, 30kHz

  - Waveform type: DFT-s-OFDM, CPOFDM

  - ACLR: 30dB for PC3

## 2.3	BS requirements

Agreement on the following BS RF expected changes:

Table 2.3-1: Expected BS RF changes to TS 38.104 due to the introduction of band n68

| Requirement | n68 proposal |
| --- | --- |
| OBUE | Add band n68 to cat A and cat B limits for bands below 1GHz, in clauses 6.6.4.2.1 and 6.6.4.2.2: |
| Spurious - coexistence | Add band n68 and corresponding exceptions |
| Spurious - colocation | Add band n68 |

## 2.4	AOB

### 2.4.1	Work split

| Specification | Responsible company for draft and formal CRs |
| --- | --- |
| 38.101-1 | Ericsson |
| 38.133 | CATT |
| 38.106 | ZTE |
| 38.115-1 | ZTE |
| 38.174 | ZTE |
| 38.176-1 | ZTE |
| 38.176-2 | ZTE |
| 36.104 | Ericsson |
| 36.141 | Nokia |
| 37.104 | Nokia |
| 37.141 | Nokia |
| 38.104 | Ericsson |
| 38.141-1 | Nokia |
| 38.141-2 | Huawei |
| 38.307 | Nokia |
| 37.105 | Huawei |
| 37.145-1 | Huawei |
| 37.145-2 | Huawei |
| 38.101-5 | Ericsson |

# 3	Conclusion

This contribution captures the agreements for introduction of band n68 made in RAN4#112.

# 4	References

- [1] RP-241664, New WID on introduction of NR band n68, Ericsson