3GPP TSG-RAN WG4 Meeting #112												        R4-2414309

# Maastricht, Netherlands, August 19th – August 23rd, 2024

Agenda item:			8.22.5

Source:	vivo

Title:	WF on LP-WUS UE RF requirements

Document for:	Approval

Introduction

This WF capture the agreements for Rel-19 LP-WUS UE RF.

Topic #1: General and system parameters

Sub-topic 1-1 General

Issue 1-1-1: New RAN4 TR to capture simulation and analysis of LP-WUS receiver and requirements

Agreement:

- RAN4 suggest having a new RAN4 TR to capture the simulations and analysis for LP-WUS RF.

Issue 1-1-2: Performance metric for Rx RF requirements

Agreement:

- 1% MDR for evaluation [and core requirements]

- FFS on the testability issue.

Issue 1-1-3: Performance metric for Demodulation requirements

Agreement:

- [1% or 0.1%] FAR as assumption for MDR result calibration.

Issue 1-1-6: Specify FR2 LP-WUS RF requirements

Agreements:

- Wait to RAN1 progress on FR2 LP-WUS work.

Sub-topic 1-2 System parameters

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

  - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS.

  - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful.

Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS

Agreements:

  - Leave RAN1 to make decision.

  - RAN4 can discuss assumption of RBs under 15kHz SCS for simulation

Issue 1-2-5: Whether need to define CBW for LP-WUR?

Agreements:

  - No need to define BW for LP-WUR

Topic #2: REFSENS, ASCS and ACS requirements

Sub-topic 2-1 SNR simulation and values

Issue 2-1-1: Channel model to specify LP-WUS RF requirements

Agreement:

  - Follow typical RF requirements approach, AWGN should be selected.

Issue 2-1-2: Target SNR simulation condition

Agreement:

  - Follow similar approach of MR NR, no repetition should be used when simulate target SNR.

Issue 2-1-3: Whether RAN4 should conclude a target SNR first

Agreement:

- Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first.

Issue 2-1-4: Target SNR value

Agreements:

  - RAN4 further discuss target SNR value for LP-WUS

  - FFS single SNR for OOK-based and OFDM-based receiver

Sub-topic 2-2 NF and REFSENS requirements

Issue 2-2-2: Baseline architecture for OOK-based LP-WUS

Agreement:

  - use zero-IF receiver as a baseline architecture for envelop based LP_WUR

Issue 2-2-3: Baseline architecture for OFDM-based LP-WUS

Agreements:

  - Wait for RAN1’s conclusion on the OFDM-based LP-WUR.

Issue 2-2-4: whether different NF for OOK-based and OFDM-based

Agreement:

  - Differentiate NF for OOK-based and OFDM-based receiver.

Issue 2-2-9: MDR value for REFSENS test case

Agreements:

  - FFS the MDR for conformance testing should be aligned with the value for RF requirement.

Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-1: ASCS requirements value

Agreements:

  - RAN4 further discuss how to specify ASCS requirement based on simulation outcome.

Issue 2-3-2: Required number of guard RB for ASCS

Agreements:

  - RAN4 further discuss the required number of guard RBs for ASCS requirement based on simulation outcome.

Issue 2-3-3: Test parameters for ASCS

Agreements:

  - RAN4 further discuss the test parameters for ASCS requirement.

Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-2: ACS requirements value

Agreements:

  - RAN4 further discuss how to specify ACS requirements based on simulation outcome.

    - FFS same as MR, or further relaxed

Issue 2-4-3: Required guard RB for ACS requirements

Agreements:

  - RAN4 further discuss the number of guard RBs for ACS requirement based on simulation outcome.

Issue 2-4-4: Test parameters for LP-WUR ACS case

Agreements:

  - RAN4 further discuss the test parameters for ACS requirement.

Topic #4: Testability issues

Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: General framework on LP-WUS testing

Agreements:

  - RAN4 further discuss the high-level approach on how to achieve LP-WUS performance verification.