3GPP TSG-RAN WG4 Meeting #112bis	R4-2417125

Hefei, Anhui, China, October 14th ‒ 18th, 2024

Title: 	WF on UL CA_n5-n8

Agenda Item:	5.3.3

Source: 	CATT, China Telecom, Huawei, HiSilicon

Document for:	Approval

# Support of CA_n5-n8 in Rel-19

For band combination CA_n5-n8, two implementation architectures are identified:

1) Full band n5 and n8 RF filters implementation with option 1 and option2:

- Option 1: Only support 1UL/2DL CA. Single UL in n5 (already in 38.101)

- Option 2: Support 2UL/2DL CA using non-concurrent n5 DL and n8 UL

Note: Potential impacts on RAN2 are observed, i.e., an FDD-FDD non-simultaneous Rx/Tx capability due to the frequency overlapping between DL frequency range of band n5 and UL frequency range of band n8, and the capability should support the indication of the band pair (n5, n8) in a higher order band combination consisting of the band pair.

2) Dedicated RF filters implementation with partial frequency range

- Option 3: Support 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

<Way forward/Agreement>

- RAN4 agrees to specify the support of the following implementation architecture in Rel-19 for CA_n5-n8 (i.e., Option 2)

  - 2UL/2DL CA is supported using non-concurrent n5 DL and n8 UL

  - Send LS to RAN2 to complete necessary signalling designs

- Stop discussion on Option 3 in Rel-19. If option 3 is discussed in Rel-20 or later, new band numbers are considered.

# Reference

[1] R4-2415103	Discussion on how to handle the RF requirements for 2U/2D CA_n5-n8	CATT

[2] R4-2415877	Draft CR to 38.101-1: UE RF requirements for uplink CA_n5-n8	China Telecom

[3] R4-2416038	Draft LS on enabling RRC configuration UL CA_n5-n8 with DL CA_n5-n8	Huawei, HiSilicon

[4] R4-2416039	Discussion on UL CA_n5-n8	Huawei, HiSilicon