3GPP TSG-RAN WG1 Meeting #115												R4-2414323

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.17.5

Source: 	Qualcomm Incorporated

Title: 	WF on AI/ML

Document for:	Approval

# Introduction

This WF captures the agreements for the discussion carried out on AI/ML under the [112][129]NR_AIML_air thread.

# Agreements

## General Issues

### Agreements in main session:

Issue 1-2: Channel modeling

Agreement:

- Use synthetic channels as the default assumption for conformance requirements/test.

  - RAN4 may conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data.

    - The field data can be considered for the analysis.

Issue 1-3: Testing goals

Agreement:

- The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

  - FFS on whether and how many different test configurations/parameters are used for tests.

  - LCM would be tested

## Testability and interoperability issues for beam management

### Agreements in main session:

Issue 2-3: Channel models

Agreement: Further discuss and down-select the following options considering TE feasibility

- Option 2: TDL

- Option 5: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Issue 2-4: Test setup needs

Issues listed below to be further investigated in order to determine the capabilities of the test equipment:

- set B Tx beams; set A Tx beams

      - number of beams transmitted simultaneously

      - overall number of beams transmitted during the test

- AoA

- AoD

- UE rotation during the test

Other aspects can also be considered

### Agreements in ad-hoc session (R4-2414446)

Issue 2-6:	Measurement error impact

Agreement:

RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study.

impact on different prediction accuracy metrics should be taken into account

studies already performance by other groups(RAN1) could also be taken into account

Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Issue 2-7:	UE reporting for network side models

Agreement:

postpone discussion until other WGs define any reporting mechanisms

## Testability and interoperability issues for positioning accuracy enhancement

### Agreements in ad-hoc session (R4-24014446)

Issue 3-2: Requirements for case 2a

Agreement:

RAN4 should not define any positioning accuracy requirements because positioning is LMF based

Issue 3-3: Reported metrics for case 2a

Agreement:

- RAN4 will take existing requirements for existing metrics as baseline

  - FFS whether to revisit these requirements

- RAN4 will study how to define requirement for any new reported metrics

  - FFS how to define requirements for LOS/NLOS

Issue 3-5: Reported metrics for case 2b

Agreement:

postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported.

Issue 3-6: Reported metrics for case 3a/3b

Agreement:

postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

## Testability and interoperability issues for CSI compression and CSI prediction

### Agreements in ad-hoc session (R4-2414446)

Issue 4-1: Option 3 next steps

- Check on performance alignment -> see simulation results from contributing companies

  - repeat simulations until good alignment is achieved

  - move to next step after alignment

- Share models(encoder or decoder or both)/datasets (training/testing/inference)

  - framework to share data among companies(dataset/model) to be discussed

- Select one or more decoder for further analysis

  - selection criteria to be discussed

- Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - FFS on what the feasibility criteria

Note: some steps could be reused for Option 4, FFS on what can be reused

Issue 4-2: Simulation results and alignment

- perform a round of alignment until RAN4#112-Bis

- refined simulation parameters can be found in R4-2414447

Issue 4-4: Options for Option 4

Option 4 sub-options:

- 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder)

- 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

FFS whether training dataset (channel information) needs to be specified

- 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Issue 4-5: CSI prediction accuracy metric

Agreement:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to