3GPP TSG-RAN WG4 Meeting #112	R4-2413528

- Maastricht, Netherlands,19 – 23 Aug, 2024

Title: 	WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF

Source: 	Xiaomi

Agenda item:	8.8.5

Document for:	Approval

# WF

## Agreement

Issue 1-1-1: Channel bandwidth

- Agreement

  - Only introduce 3MHz channel bandwidth with 15kHz SCS

Issue 1-1-2: Operating bands for 3MHz channel bandwidth

- Agreement

  - Applicable NTN bands: band n256, n255 and n254

Issue 1-1-3: Maximum transmission bandwidth configuration(NRB)

- Agreement

  - Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN.

Issue 1-1-4: Minimum guardband

- Agreement

  - Reuse 142.5kHz minimum guardband for 3MHz channel bandwidth with 15kHz SCS for NR NTN.

Issue 1-1-5: Asymmetric channel bandwidth

- Agreement

  - Introduce 3 MHz channel bandwidth for UL in n254 in Table 5.3.6-1 as 3MHz UL + X MHz DL

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n254 | 3 | 10,15 | 1 |
|  | 3 | 15 | 1 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

Issue 1-2-1: Channel spacing

- Agreement

  - Reuse the existing channel spacing formula in TS 38.101-1

Issue 1-2-2: Channel raster

- Agreement

  - Adopt 100kHz and 10kHz channel raster for 3MHz channel bandwidth.

Issue 1-2-3: Synchronization raster

- Agreement

  - Reuse TN sync raster for 3MHz channel bandwidth and extend to 3000MHz for NTN FR1 3MHz channel bandwidth as below table:

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |

  - FFS on the note1:

      - NOTE 1:	Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.

## Open issues

System parameters

Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

- FFS

Issue 1-2-4: Additional sync raster

- FFS

Issue 1-2-5: SS raster entry

- FFS

Issue 1-2-6: Signalling of SS raster

- FFS

Issue 1-3-1: Whether 3MHz channel bandwidth applies to Redcap/eRedcap NTN

- FFS

Tx requirements

Issue 2-1-1: UE maximum output power

- FFS on how to define the additional requirements for transmit power density for 3MHz channel bandwidth in n254

Issue 2-1-2: MPR

- FFS

Issue 2-1-3: A-MPR

- Further check

  - Neither 3MHz channel bandwidth or A-MPR are needed define in Table 6.2.3.1-1 for N_100

  - Need introduce 3MHz channel bandwidth in Table 6.2.3.1-1, but no A-MPR need define for below NS values:

      - NS_01, NS_02N

  - Need introduce 3MHz channel bandwidth in Table 6.2.3.1-1 and define A-MPR for below NS values:

      - NS_03N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.2-1/Table 6.2.3.2-1

      - NS_04N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.3-1/ Table 6.2.3.3-2

      - NS_05N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.4-1/ Table 6.2.3.4-2

      - NS_24: A-MPR need define for 3MHz channel bandwidth for NTN n256 in a new clause 6.2.3.X in NTN spec 38.101-5.

- Companies are encouraged to provide the related A-MPR values

Issue 2-1-4: Minimum output power/ Transmit OFF power

- Further check

  - Whether reuse the 3MHz channel bandwidth requirements of NR TN FR1 UE

    - Minimum output power: -40dBm with the measurement bandwidth 2.715 MHz

    - Transmit OFF power: -50dBm with the measurement bandwidth 2.715 MHz

Issue 2-1-5: Occupied bandwidth

- Further check

  - Addition of 3 MHz into Table 6.5.1-1 in TS 38.101-5

Issue 2-1-6: Spectrum emissions mask

- Further check

  - Whether reuse the 3MHz channel bandwidth requirements of NR TN FR1 UE

Issue 2-1-7: Additional spectrum emissions mask

- FFS

Issue 2-1-8: NR ACLR

- Further check

  - Addition of 3 MHz into Table 6.5.2.4.1-1 in TS 38.101-5

Issue 2-1-9: General spurious emission

- Further check

  - Whether reuse the TN UE approach for 3 MHz channel bandwidth correspondingly FOOB (6 MHz)

Issue 2-1-10: Additional spurious emissions

- FFS

Issue 2-1-11: Spurious emissions for UE co-existence

- FFS

Issue 2-1-12: Other Tx requirements

- Further check

  - Whether there are no specification impact for below Tx requirements.

6.2.4 Configured transmitted power

6.3.3 Transmit ON/OFF time mask

6.3.4 Power control

6.4.1 Frequency error

6.4.2 Transmit modulation quality

6.5.2.4.2 UTRA ACLR

6.5.4 Transmit intermodulation

Rx requirements

Issue 2-2-1: Reference sensitivity

- FFS

Issue 2-2-2: Maximum input level

- FFS

Issue 2-2-3: ACS

- FFS

Issue 2-2-4: In-band blocking

- FFS

Issue 2-2-5: Out-of-band blocking

- Further check

  - Define out of band blocking requirement similar to TN for 3MHz  (REFSENS + 6.0 dB)

Issue 2-2-6: Narrow band blocking

- Further check

  - Define narrow band blocking requirement similar to TN for 3MHz  (-18dB Pw)

Issue 2-2-7: Spurious response

- Further check

  - 3 MHz shall be added in table 7.7-1, same column as for 5 and 10 MHz channel bandwidth (REFSENS + 6.0 dB)

Issue 2-2-8: Intermodulation characteristics

- Further check

  - There is no specification impact.

Issue 2-2-9: Spurious emissions

- Further check

  - There is no specification impact.

# Reference

[1] R4-2413412, Topic summary for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Moderator (Xiaomi)