Question1: What’s the different between the PDCCH in step6 and Step16 in TTCN except the v_TimingDL? Does TTCN need PDCCH in step16 to trigger PDSCH CRC FAIL? Why SS could only send CRC FAILURE PDSCH in SPS activated scenario.

We have tried with two SS device to delete step16, all of them can trigger PDSCH CRC FAILURE, although we do not know the detail how they trigger.

| 6 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘P’, NDI=0; (Where Y+X<P<Y+2X) | <-- | (DL SPS Grant) | - | - |
| --- | --- | --- | --- | --- | --- |
| 7 | The SS transmits in Slot ‘P’, a DL MAC PDU containing a RLC PDU (DL-SQN=2) on UM DRB. | <-- | MAC PDU | - | - |

//@siclog "Step 6-7" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2);

v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2);

// PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B)

v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@

v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) };

v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) };

f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS);

DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL));

| 16 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘P+3X’, NDI=0. | <-- | (DL SPS Grant) | - | - |
| --- | --- | --- | --- | --- | --- |
| 17 | The SS transmits in Slot ‘P+3X’, a DL MAC PDU containing 1 RLC PDU’s (DL-SQN=5) on UM DRB; CRC is calculated in such a way will result in CRC error in UE. | <-- | MAC PDU | - | - |

// @siclog "Step 16-17" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2);

f_NR_SS_L1_TestModeConfig(nr_Cell1,cas_NR_L1_TestModeCRC_CSRNTI_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL), Error1AndNormal));

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2);

v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period);

// PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B)

v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@

v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) };

v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) };

f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS);

DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id)));

template (value) NR_DL_SCH_CRC_Type cs_NR_DL_SCH_CRC_CSRNTI(template (value) NR_MAC_Test_DL_SCH_CRC_Mode_Type p_NR_MAC_Test_DL_SCH_CRC_Mode) := { CS_RNTI := p_NR_MAC_Test_DL_SCH_CRC_Mode }; /* @status    APPROVED (ENDC, NR5GC) */

template (value) NR_SYSTEM_CTRL_REQ cas_NR_L1_TestModeCRC_CSRNTI_REQ(NR_CellId_Type p_NR_CellId,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template (value) NR_MAC_Test_DL_SCH_CRC_Mode_Type p_NR_MAC_Test_DL_SCH_CRC_Mode := Normal) :=

{ /* @status    APPROVED (ENDC, NR5GC) */

Common := cs_NR_ReqAspCommonPart(p_NR_CellId, -, -, p_TimingInfo),

Request := {

L1_TestMode := {

DL_SCH_CRC := cs_NR_DL_SCH_CRC_CSRNTI(p_NR_MAC_Test_DL_SCH_CRC_Mode)

}

}

};

Question2: In 7.1.1.6.1, step21 the test spec allow 3 HARQ NACK but TTCN allow 0 HARQ NACK? However, in 7.1.1.2.1, both test spec and TTCN allow 3 HARQ NACK.

According to 38.321, if the new transmission data (SS create with PDSCH CRC FAIL), then UE receive a retransmission data(SS create with PDSCH CRC PASS), UE will combine them and then decode it. So may be the second decode also CRC FAIL. We understanding is that TTCN need to follow the Spec.

| - | EXCEPTION: Step 19 and 20 shall be repeated until HARQ retransmission count = 3 is reached for MAC PDU at step 17.(Note 3) | - | - | - | - |
| --- | --- | --- | --- | --- | --- |
| 19 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘Z’, NDI=1; Where (P+3X < Z <P+4X); The DL HARQ process is same as in step 18. | <-- | (DL SPS Grant) | - | - |
| 20 | The SS re-transmits in Slot ‘Z’, a DL MAC PDU containing a RLC PDU (DL-SQN=5) on UM DRB. | <-- | MAC PDU | - | - |
| - | EXCEPTION: Up to 3 HARQ NACK from the UE should be allowed at step 21(Note 3). | - | - | - | - |
| 21 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 3 | P |

//@siclog "Step 19-20" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2);

v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period/2);

f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS,true);//NDI=1

DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id)));

//@siclog "Step 9" siclog@

// MAC PDU retransmision automatically performed by SS

//@siclog "Step 10" siclog@

alt {

[] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

t_Watchdog.stop;

}

[v_Nack_Count<3] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack))

{

v_Nack_Count := v_Nack_Count +1;

repeat;

}

[] t_Watchdog.timeout

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: UE did not send HARQ ACK");

}

}

}

[38.321]

The MAC entity then shall:

1>	if this is a new transmission:

2>	attempt to decode the received data.

1>	else if this is a retransmission:

2>	if the data for this TB has not yet been successfully decoded:

3>	instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.