3GPP TSG-SA WG2#164	S2-240xxxx

19-23 August 2024, Maastricht, Netherlands	(was S2-240xxxx)

Source:	Apple (Rapporteur)

Title:	Summary of NWM feedback on FS_MASSS DualSteer conclusions

Document for:	Information

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution provides a summary of the pre-SA2#164 NWM discussion on the FS_MASSS study conclusions on DualSteer Key Issues.

# 1.	Discussion

Details of the NWM discussion on FS_MASSS study conclusions were captured at:

https://nwm-trial.etsi.org/#/documents/8911

A PDF version of the feedback forms and summary are available in the attached file.

# 2.	Proposals for Conclusions

## 2.1 	Proposals for Conclusions related to KI#1.1

-	The two SUPIs for DualSteer should be correlated in the UDM via subscription information, each subscription should contain information on the other SUPI.

-	The HPLMN can provide correlation information to the DualSteer Device during two SUPI’s registration procedure.

-	Either of the SUPIs can be the first one to register with the network. The first SUPI that is registered is considered as the primary SUPI and the second one that is registered is considered as the secondary SUPI.

-	The way how each of the SUPI is used will be determined by network policies defined by the operator.

Open issues for further discussions:

-	It's unclear if network needs to ensure that the two SUPIs of a DualSteer subscription are used by the same DualSteer Device. Perhaps an LS to SA3 could clarify if that is needed.

## 2.2	Proposals for Conclusions related to KI#1.2

-	The UE shall include in the Registration request whether it is DualSteer capable.

- 	If the DualSteer Device sends DualSteer capabilities as part of Registration Request, the serving network shall send that information to the UDM.

-	If the DualSteer Device sends DualSteer capabilities as part of Registration Request, HPLMN will perform DualSteer authorization based on that.

-	The serving PLMN needs to support DualSteer feature, the impacts are on SMF/PCF selection and UE capabilities handling.

Open issues for further discussions:

-	It's unclear whether there is a need for registration-related policies and whether network needs to enforce registration-related policy constraints.

-	It's unclear whether DualSteer Device needs to apply registration-related policies for second access. If SA1 comes back with new requirements to the SA2 LS, SA2 should re-discuss this topic.

## 2.3	Proposals for Conclusions related to KI#1.3

-	The UE uses URSP rules to link the service to PDU session and uses DualSteer rules (ATSSS-like) received during the DualSteer PDU Session establishment to determine which access is used (i.e. how to steer) for the service.

-	Steering of services for LBO sessions is not in the scope for DualSteer.

- 	The steering decision is based on the DualSteer UE rules provided to the UE during PDU Session establishment, in the same way of ATSSS. The PCF can take into account UE's PLMN, RAT, or Access Type to generate PCC rules that is used by SMF to generate the DualSteer UE rule.

-	The DualSteer PDU session can be established even if only a single access is available. Once the second access becomes available, the DualSteer Device can initiate the PDU Session Establishment procedure to add the second access to the existing DualSteer PDU Session based on the URSP rules associated with that SUPI.

-	When the PDU Session Establishment Request is received by the network for the second leg of the DualSteer PDU session, a session correlation information stored in the UDM ensures the same SMF is selected.

-	SMF manages the DualSteer PDU Session using single PCF association that provides unified policies and single PFCP association/N4 rules towards UPF.

-	Access Network resources only need to be set up for one leg of the DualSteer PDU Session at the time of PDU Session Establishment based on the DualSteer steering/switching policy.

-	DualSteer Devices supporting simultaneous transmission can establish a DualSteer PDU Session including different services/applications with different steering/switching conditions.

-	Different services/applications are subject to DualSteer UE/UPF Rules that indicate to the UE/UPF how to handle the related traffic (same behaviour as ATSSS rules / N4 rules). For a non-simultaneous transmission scenario, PCF needs to adapt the policy, i.e. only Active-Standby steering mode, active access is the same for all services.

-	The DualSteer Device evaluates the switching conditions based on DualSteer UE rules. In case only non-simultaneous transmission is allowed, Active-Standby steering mode can be used with setting the same access type as active access for all services. In case simultaneous transmission is allowed, per service switching is possible.

- 	By only allowing Active-Standby steering mode in DualSteer UE Rules sent to the UE can ensure that only one access is used for a service (both DL and UL).

- 	The PDU Sessions are pre-established on both 3GPP accesses. When the UE decides to switch the traffic to the second access the UE only activates the PDU Session to start sending/receiving UL/DL PDUs on the second access.

-	The switching decision is based on the DualSteer UE rules provided to the UE during PDU Session establishment, in the same way of ATSSS.

-	For a DualSteer Device that is only capable for non-simultaneous transmission across all PDU sessions, the DualSteer UE Rules sent to this device only allow the Active-Standby steering mode, active access is the same for all services.

-	Rel-19 DualSteer Device can be enabled to support simultaneous transmission to transfer different application traffic via different access.

-	Whether MPQUIC steering functionality is needed if the DualSteer Device supports simultaneous transmission needs more discussion / SoH. MPTCP steering functionality can potentially be de-prioritized

## 2.4	Proposals for Conclusions related to KI#1.4

-	Unless required by SA1 requirements, SUPIs register independently, network does not provide policies to control the DualSteer Device's registration to the second access.

-	DualSteer steering and switching rules are defined with flow-level granularity to enable more efficient and flexible control.

-	The DualSteer rules identify the UE/SUPI for steering and switching decision by using Access Descriptor in the rules.

-	The same URSP rules are provisioned for the two SUPIs of the DualSteer Device.

-	DualSteer UE Rules (similar to ATSSS rules) are provided during the DualSteer PDU Session establishment and defined to control the steering of services in the DualSteer Device.

-	The DualSteer UE Rules indicate which SUPI to use for steering. Either SUPI can be used to establish DualSteer PDU Session if both SUPIs are registered to 5GC.

-	DualSteer UE Rules (similar to ATSSS rules) are provided during the DualSteer PDU Session establishment and defined to control the switching of services in the DualSteer Device.

-	Traffic switching is controlled by DualSteer UE rules and conditions are defined in a similar way as for ATSSS rules.

-	PCF, based on PCRT, can update the PCC rules and that will trigger the SMF to update the DualSteer UE rules.

-	A Dualsteer PDU session always has an Anchor SMF and an anchor UPF. DualSteer UE Rules are provided to the UE that establishes the DualSteer PDU Session.