3GPP TSG-WG2 Meeting #164	S2-24-xxxx

19th – 23rd August 2024, Maastricht, Netherlands

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | <CR#> | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Handling for multi hop U2N without N3IWF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Principles for QoS handling of multi-hop UE-to-Network Relays are agreed in the conclusions of the TR 23.700-03 as following: - End-to-end QoS management for multi-hop U2N Relays is done similarly to the end-to-end QoS management for single hop L3 U2N Relay as defined in TS 23.304 [4], with enhancement to handle QoS split over multiple legs of the PC5 interface. - The QoS flows setup can be initiated by the network or the 5G ProSe Layer-3 Remote UE during Layer-2 link establishment or link modification procedure. There are two main options of QoS split being studied in the TR for the case of 5G ProSe Multi-hop Layer-3 UE-to-Network Relay without N3IWF. One is that E2E QoS is provided to all the nodes in the path, meanwhile QoS split per hop is done by Intermediate U2N based on the e2e QoS info and hop info (e.g. hop count or path info). The other is that the remaing QoS is provided to next hop node(s) in the path, and QoS split per hop is done by Intermediate U2N based on the received remaining QoS info from the earlier hop and hop info (e.g. hop count or path info). Then the Intermediate U2N updates and sends the updated remaining QoS (of remaining hops) to its next hop node. If the QoS flow is initiated by the Remote UE, the U2N performs QoS mapping of 5QI and PQI, determines Uu QoS parameters based on the received QoS info, following the same logic of QoS handling for single hop L3 U2N Relay. However in case of the second option, the U2N is not aware of the end-to-end QoS requirement which is originally provided by the Remote UE for the intended ProSe service, but only the remaining QoS which is received from the earlier hop (Intermediate U2N). Hence the U2N would not find the proper mapping entry of 5QI for Uu QoS control based on the pre-provisioned QoS mapping. It is proposed that the first Option (e.g. Solution #1 of TR 23.700-03) is used for normative work and E2E QoS is provided to the Multi-hop U2N relay for QoS split. So that common and unified QoS handling is achived for both cases of the QoS flow being initiated by the Remote UE or by the Network. Also the existing QoS mapping provisioned in the U2N as defined in Rel-18 can be reused. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause for QoS handling for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay without N3IWF is included. Similiar end-to-end QoS management as single hop L3 U2N Relay is adapted, with enhancement to handle QoS split over multiple legs of the PC5 interface and with the e2e QoS info being included in the QoS flow setup procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS handling for Multi-hop Layer-3 UE-to-Network Relay without N3IWF to support conclusions defined in the TR 23.700-03 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 5.6.2.x	QoS handling for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay without N3IWF

End-to-end QoS management for Multi-hop UE-to-Network Relays is done similarly as QoS management for single hop Layer-3 UE-to-Network Relay as defined in clause 5.6.2.1, with enhancement to handle QoS split over multiple legs of PC5 interface.

As shown in figure 5.6.2.x-1 below, the end-to-end QoS can be met only when the QoS requirements are properly translated and satisfied over the multiple legs respectively.

Figure 5.6.2.x-1: End-to-End QoS translation for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay operation

To achieve this, the QoS mapping can be pre-configured or provided to the 5G ProSe Layer-3 UE-to-Network Relay by the PCF using Prose Policy as specified in clause 5.1.4.1. The QoS mapping information for the 5G ProSe Layer-3 UE-to-Network Relay contains the 5QI and PQI mapping as specified in clause 5.6.2.1.

Additionally, the 5G ProSe Layer-3 UE-to-Network Relay and the intermediate UE-to-Network Relay, based on its implementation, can locally configure one or more hop adjustment factors to determine PC5 QoS to be used over each adjacent hop. The hop adjustment factor is used by the 5G ProSe Layer-3 UE-to-Network Relay and the intermediate UE-to-Network Relays to derive the packet delay budget that applies to the PC5 link over each adjacent hop from end-to-end QoS info, e.g. 1/5 of the standardized PDB value for hop count =5.

5.6.2.x.1	Network initiated QoS Flows

If the QoS Flows setup are initiated by network, the SMF can base on the PCC rules or its local configuration to generates the QoS rules and QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR) and signal to the 5G ProSe Layer-3 UE-to-Network Relay using PDU Session Establishment/Modification procedure. For the PDU sessions used for relaying, the SMF always provides the QoS Flow level QoS parameters to the 5G ProSe Layer-3 UE-to-Network Relay when establishes a QoS Flow.

Based on the received QoS parameters from SMF, the 5G ProSe Layer-3 UE-to-Network Relay decides the end-to-end PC5 QoS parameters for the corresponding PC5 QoS Flow based on the QoS mapping as defined in clause 5.6.2.1. The end-to-end PC5 QoS parameters are interpreted as the end-to-end QoS requirements for the traffic transmission between 5G ProSe Layer-3 Remote UE and the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay also decides the next hop PC5 QoS parameters, based on the end-to-end PC5 QoS parameters, hop adjustment factor and hop info (e.g. hop count or path info). The 5G ProSe Layer-3 UE-to-Network Relay provides the end-to-end PC5 QoS info and the generated next hop PC5 QoS info to its connected next hop intermediate UE-to-Network Relay.

The Intermediate UE-to-Network Relay(s) accepts the received next hop PC5 QoS info and uses it with its upstream PC5 QoS Flow. The Intermediate UE-to-Network Relay(s), based on its implementation, determines the PC5 QoS parameters for the corresponding downstream next hop PC5 link, based the received end-to-end PC5 QoS info, hop adjustment factor and hop info (e.g. hop count or path info). The GFBR and MFBR values for the next hop PC5 GBR QoS Flow are set equal to the GFBR and MFBR values of the received PC5 QoS info respectively. Then the Intermediate UE-to-Network Relay(s) provides the received end-to-end PC5 QoS info and the determined next hop PC5 QoS info to its next hop Intermediate UE-to-Network Relay or the 5G ProSe Remote UE.

5.6.2.x.2	Remote UE initiated QoS Flows

If the 5G ProSe Layer-3 Remote UE initiates PC5 QoS Flows setup or modification during the Layer-2 link establishment or modification procedure, the 5G ProSe Layer-3 Remote UE provides the QoS Info as described in clause 6.4.3.6 to its connected Intermediate UE-to-Network Relay. The PC5 QoS parameters of the QoS Info (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.) sent by the 5G ProSe Layer-3 Remote UE are interpreted as the end-to-end QoS requirements by the Intermediate UE-to-Network Relay(s) for the traffic transmission between 5G ProSe Layer-3 Remote UE and UPF. The intermediate UE-to-Network Relay(s) forwards the received QoS info to the 5G ProSe Layer-3 UE-to-Network Relay.

Based on the received end-to-end QoS parameters from the 5G ProSe Layer-3 Remote UE via intermediate UE-to-Network Relay(s), the 5G ProSe Layer-3 UE-to-Network Relay provides decides the 5QI for the Uu QoS control and the PQI for end-to-end PC5 QoS control based on the QoS mapping as defined in clause 5.6.2.1. The 5G ProSe Layer-3 UE-to-Network Relay also decides the next hop PC5 QoS parameters of the PC5 link with its connected Intermediate UE-to-Network Relay, as described in clause 5.6.2.x.1. The 5G ProSe Layer-3 UE-to-Network Relay provides the determined end-to-end PC5 QoS info and the next hop PC5 QoS info to its connected intermediate UE-to-Network Relay as part of the Accept message to the 5G ProSe Layer-3 Remote UE.

The Intermediate UE-to-Network Relay(s) accepts the received next hop PC5 QoS info as part of the Accept message from the 5G ProSe Layer-3 UE-to-Network Relay or its upstream Intermediate UE-to-Network Relay and uses it with the upstream PC5 QoS Flow. The Intermediate UE-to-Network Relay(s) decides the PC5 QoS parameters for the corresponding next hop PC5 QoS Flow as described in clause 5.6.2.x.1 and provides the end-to-end PC5 QoS info and the generated next hop PC5 QoS info to its connected next hop intermediate UE-to-Network Relay or the 5G ProSe Layer-3 Remote UE as part of the Accept message to the 5G ProSe Layer-3 Remote UE.

* * * End of Changes * * *