3GPP TSG-SA WG6 Meeting #62	S6-243028

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	China Mobile

Title:	Sol for KI#4: Application enablement layer capabilities usage to support the user group level QoS guarantee

Spec:	3GPP TR 23.700-01

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution propose to address the following open issues in KI#4,

-	Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

-	Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

2. Reason for Change

It is proposed to provide user group level QoS guarantee, based on the interaction with NSCE server and SEALDD server.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

7.x	Solution #x: Application enablement layer capabilities usage to support the user group level QoS guarantee

7.x.1	Architecture Impacts

No architecture impact identified.

7.x.2	Solution description

Figure 7.x.2-1 illustrates the procedure to support the user group level QoS guarantee. The user group level QoS guarantee provides different QoS for different users in single XR service.

Pre-conditions:

1.	The SEALDD server acts as NSaaS provider, and is authorized to get network slice related capabilities from NSCE server.

Figure 7.x.2-1: procedure of supporting the user group level QoS guarantee

- The VAL server sends a user group level QoS guarantee request to SEALDD server. The request includes Service Description, QoS guarantee requirement list, and corresponding different target UE(s). When the All other VAL UEs Indication is indicated, the corresponding requirements apply to all other VAL UEs of the application identified by VAL server ID and/or VAL service ID, except for those already indicated (if any). The QoS guarantee requirement list could include the Delay requirement, Delay Variation requirement, Bandwidth requirement, Number of UE requirement, Coverage Area requirement, Other requirement(s), Delay Variation requirement.

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Target UE(s) | M | Identifier of the VAL UE(s) |
| >VAL UE identity | O (See NOTE) | Identifier of the VAL UE, e.g. UE ID |
| >VAL UE group ID | O (See NOTE) | Identifier of a specific VAL UE group, as defined in clause 7.5 of 3GPP TS 23.434 [4] |
| >VAL UE identity list | O (See NOTE) | Identifies a list of VAL UEs, e.g. the list of UE ID |
| >All other VAL UEs Indication | O (See NOTE) | Indicates all other VAL UEs of the application identified by VAL server ID and/or VAL service ID, except for those already indicated (if any) |
| QoS guarantee requirement list | M | The QoS guarantee requirement information |
| > Delay requirement | O | Maximum delay accepted by the target UE(s) |
| > Delay Variation requirement | O | Maximum Delay Variation accepted by the target UE(s) |
| > Bandwidth requirement | O | Minimum bandwidth accepted by the target UE(s) |
| > Number of UE requirement | O | Number of UE requirement |
| > Coverage Area requirement | O | Coverage Area requirement |
| > Other requirement(s) | O | In addition to the requirements above, other service requirements, e.g., isolation requirement, PDU Set QoS Parameters, etc.. |
| > Valid time | O | The valid time for the requirement |
| NOTE: One of them shall be present as the measurement target UE. |  |  |

2a.	The SEALDD server determines whether and which slice is needed based on the received QoS guarantee requirement list. The SEALDD server may initiate the slice related communication service lifecycle management as defined in TS 23.435, clause 9.12.2, if the network slice could meet the requirement.

Before triggering the network slice management service, to determine the target UE of network slice(s), the SEALDD server acting as AF, may receive a UE location report or a monitoring event report from 5GC (assuming that SEALDD server has subscribed to consume 5GC services like LCS or NEF monitoring events related to UE actual location, or UE mobility analytics from NWDAF) or Location service from SEAL.

2b.	The SEALDD server determines whether it is needed to request a specific session. If needed(e.g., multiple user groups have different QoS requirements), the SEALDD server translate the QoS guarantee requirement into individual QoS parameters, and initiates AF session with required QoS as defined in TS 23.502, clause 4.15.6.6a, or Multi-member AF session with required QoS as defined in TS 23.502, clause 4.16.5.3.

2c.	The SEALDD server determines whether the SEALDD enabled bandwidth control is needed. If needed(e.g., multiple user groups have different bandwidth requirements), the SEALDD server may initiate the SEALDD enabled bandwidth control for different VAL users as defined in TS23.433, clause 9.8.2.

3.	The SEALDD server sends the user group level QoS guarantee response to the VAL server indicating the result of the request.

7.x.3	Solution evaluation

This solution addresses KI#4, the user group level QoS guarantee is provided based on the interaction with NSCE server and 5GC.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>