3GPP TSG-SA6 Meeting #62	S6-243035

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0303 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure and information flows of Obtaining Geofencing UE(s) Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in 3GPP TR 23.700-72, it is concluded that the Sol#2 (Application enabled Geofencing) can be considered in the normative work to support Geofencing based location services. Based on this solution, this CR proposes to add the procedure, information flows and an API for obtaining UE(s) location information using Geofencing criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the Obtaining Geofencing UE(s) information procedure in clause 9.3.x. - Add the Get Geofencing UE(s) request and response information flows in clause 9.3.2.x and clause 9.3.2.y. - Add the SS_GeofencingSupport API description in clause 9.4.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Geofencing based location services may not be fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.x, 9.3.2.x, 9.3.2,y, 9.4.x (new sub-clauses) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.X	Application enabled Geofencing Procedure

9.3.x.1 	General

The VAL server can request UE(s) location information by explicitly indicating a specific location area defined in various representation formats, such as geographical code and various types of shapes.

9.3.x.2	Obtaining Geofencing UE(s) Information

The figure 9.3.x-1 describes the procedure for obtaining Geofencing UE(s) information.

Figure 9.3.x-1: Obtaining Geofencing UE(s) information

- The VAL Server sends Get Geofencing UE(s) information request to the LM Server to obtain location data of UE(s) within a range of location area. The request includes Geofencing criteria containing geographical area information (e.g., location area represented by geocode, polygon or circle) defined by the VAL Server.

- The LM Server checks whether the VAL Server is authorized to initiate the Geofencing location service request.

- If the VAL Server is authorized in step 2, the LM Server may obtain the list of UE(s) within a location area from the 3GPP core network by interacting with SCEF or NEF.

- The VAL Server obtains location data of UE(s) using the Location information procedure as defined in clause 9.3.10 of this specification.

- After collecting location data of UE(s) in step 4, the LM Server applies the Geofencing criteria requested by the VAL server to select UE(s) that match the Geofencing criteria.

- The LM Server sends Get Geofencing UE(s) information response to the VAL server with the location data of UE(s) selected in step 5.

* * * Second Change * * * *

9.3.2.x	Get Geofencing UE(s) information request

Table 9.3.2.x-1 describes the information flow for a VAL server to get Geofencing UE(s) information from the LM Server.

Table 9.3.2.x-1: Get Geofencing UE(s) information request

| Information element | Status | Description |
| --- | --- | --- |
| VAL Server ID | M | Identity of VAL Server requesting Geofencing UE(s) location information. |
| Geofencing filters | M | Geofencing filter specified by the vertical application. |
| >Application defined area information (See NOTE) | M | Area as a geofencing criteria |
| >>Geographical area | O | Information representing a geographical area (e.g., geocode) for which the vertical application is requesting UE(s) location data |
| >>Shape type | O | Geographical area represented using various shapes (e.g., circle, polygon) |
| >>> Area Coordinate of shape | O | Geographical coordinate of shape |
| NOTE: At least one of Geographical area or Shape type should be specified. |  |  |

9.3.2.y	Get Geofencing UE(s) information response

Table 9.3.2.y-1 describes the information flow for a LM Server to respond with UE(s) location data to the VAL Server.

Table 9.3.2.y-1: Get Geofencing UE(s) information response

| Information element | Status | Description |
| --- | --- | --- |
| List of UE(s) information | O | Identities of UE(s) that were detected in a range of area requested by the application. The list can be empty. |
| >UE ID | M | Identity of VAL UE. |
| >Location information | M | Location information of UE within the requested area. |

* * * Third Change * * * *

9.4.x	SS_GeofencingSupport API

9.4.x.1	General

API description: This API enables the VAL server to obtain UE(s) information that satisfies Geofencing criteria from the LM Server over the LM-S reference point.

9.4.x.2	Obtain_Geofencing_UEs_Info operation

API operation name: Obtain_Geofencing_UEs_Info

Description: Retrieve UE(s) location information that satisfies geofencing criteria specified by the application.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.x

Outputs: Refer subclause 9.3.2.y

See subclause 9.3.x.2 for the details of usage of this API operation.

* * * End of Changes * * * *