3GPP TSG-SA WG6 Meeting #62	S6-243143

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Samsung

Title:	Pseudo-CR on update to solution#8

Spec:	3GPP TR 23.700-21 (v1.1.0)

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR updates solution#8.

2. Reason for Change

Based on SA1 use case description, it is possible that to produce spatial map, the enabler server may need to have sensor data inpurs over time. The spatial map may not be available for localization service immediately.

- 3GPP TR 22.856. Clause 5.5.1, Quote:

“Once the spatial mapping service has sufficient information, it has to map all the stationary and moving objects related to UE’s environment. This information can be provided to the UE, service providers and surrounding subscribed users as well [17,19,24]”

- 3GPP TR 22.856. Clause 5.5.4, Quote:

“For Spatial Mapping

The spatial mapping enabler re-structures the data provided by mobile capture devices to create a spatial map. This information could be combined with other information available, e.g. floor plans of buildings, survey data, satellite images, etc. Over time sufficient information will be captured to allow the spatial mapping enabler to distinguish between static and dynamic objects in the environment.”

It is required to capture this in SA6 solution.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 (v1.1.0).

* * * First Change * * * *

#### 7.8.3.1	Producing spatial map

Figure 7.8.3.1-1 depicts the procedure for producing a spatial map. For the request from the spatial map requestor (VAL server or SEAL LM client), the service is provided by SEAL LM server.

Figure 7.8.3.1-1: Producing spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to produce the spatial map. The request includes requestor ID, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map.

NOTE 1:	The information to be included in the spatial map, in the request message, will be further defined in the normative work phase.

The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request.

2)	The SEAL LM server produces a requested spatial map using the processed sensor data required to estimate the number of objects, type of the object, position, direction and speed of the object, etc.

NOTE 2:	In this study, for current release, the processed sensor data is stored at VAL layer (e.g. in database at VAL layer) and the SEAL LM server gets access to such database to produce spatial map via application specific way which is out of SA6.

3)	The SEAL LM server sends response message to the requestor with produced spatial map ID and information which includes three-dimensional space defined by the spatial map, stationary or moving objects with attributes of them. If response indicates "in-progress", the VAL server (or SEAL LM client) subscribes to receive notification when spatial map is ready for the localization service.

NOTE 3:	The spatial map information in the response message will be further defined in the normative work phase. As an example, a list of spatial anchors which belong to the spatial map could be included.

NOTE 4:	The necessary of supporting third party triggered spatial map producing, in addition to spatial map service provider self-determined of map producing requires further discussion in normative phase.

* * * Next Change * * * *

#### 7.8.4.1	Produce spatial map

Table 7.8.4.1-1 shows the produce spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to produce a spatial map.

Table 7.8.4.1-1: Produce spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Requested information | O | Information to be included in the spatial map (e.g. access control rules) |

NOTE 1:	The requested information will be further defined during the normative work phase.

Table 7.8.4.1-2 shows the produce spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the produce spatial map request.

Table 7.8.4.1-2: Produce spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result (NOTE 1) | M | Result of the producing spatial map (e.g., success, in-progress or fail), including cause of failure if needed |
| Spatial map ID (NOTE 2) | O | ID of the produced spatial map |
| Spatial map information (NOTE 2) | O | If Spatial map ID is valid, a collection of information is included. The information corresponds to the produced spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them |
| NOTE 1: The result IE contains in-progress when the enabler server does not have sufficient sensor data to produce spatial map. NOTE 2: This IE is added when result IE indicates success. |  |  |

NOTE 2:	Details about Spatial map information is FFS and will be considered during the normative work phase.

* * * End of Change * * * *