3GPP TSG-SA WG6 Meeting #63	S6-244073

Hyderabad, India, 14th – 18th October 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on Support for spatial mapping information providers

Spec:	3GPP TS 23.437 v0.2.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution defines registration and discovery of spatial mapping information providers based on Solution #17.

2. Reason for Change

The following has been concluded in TR 23.700-21.

…

Solution #17 resolves open issue 1) by providing registration and discovery of spatial mapping information (e.g. sensor data) provider entities. The solution enables a VAL server to discover sensor devicews which can provide spatial mapping information to maintain spatial map. Solution #17 is based on architecture option #1 and can be adapted to other proposed architecture options. The solution can be considered for the normative work.

Solution #8, Solution #16 and Solution #17 are complementary and can be combined in the normative phase.

…

In this release, VAL server can consume the spatial mapping enabler service, can discover UE(s) per Solution #17, and also act for providing sensor data to spatial map enabler service.

Key issue #4 conclusions:

iv.	for Key issue #4 (Spatial mapping): Solution #8 , Solution #15, Solution #16 and Solution #17 can be considered for the normative work by defining a new SEAL server for spatial map management with necessary changes to adapt to the new SEAL server.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[r23273]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * First Change * * * *

### 9.3.x	Discover spatial mapping information provider

#### 9.3.x.1	General

A UE consenting and capable of providing spatial mapping information for the purpose of maintaining a spatial map can register to the SM server as a spatial mapping information provider.

A VAL server with spatial mapping capabilities can discover spatial mapping information providers registered with the SM server for the purpose obtaining spatial mapping information needed to maintain spatial maps.

NOTE 1:	For the current release, spatial mapping information is stored at the VAL layer.

NOTE 2:	How the VAL server uses the obtained spatial mapping information is out of scope.

The following procedures are supported for discovery of a spatial mapping information provider:

-	SM information provider registration request-response procedure;

-	SM information provider registration update request-response procedure;

-	SM information provider deregistration request-response procedure;

-	SM information provider discovery request-response procedure.

#### 9.3.x.2	Procedures

##### 9.3.x.2.1	SM information provider registration

Figure 9.3.x.2.1-1 depicts the procedure for an authorized VAL client, via a SM client, to register as a SM information provider with the SM server.

Pre-conditions:

1.	The SM client has received information (e.g. URI, IP address) related to the SM server;

2.	The SM client has received security credentials authorizing it to communicate with the SM server.

3.	The SM server is authorized to use Nnef Event Exposure API for UE location, based on service level agreement with MNO.

Figure 9.3.x.2.1-1: SM information provider registration procedure

1.	The SM client sends a SM information provider registration request to the SM server. The request includes a requestor ID, security credentials, UE identifier, VAL client identifier, SM client notification endpoint and a SM information provider profile including SM information details (e.g., provider identifier, type, position, state).

2.	Upon receiving the request, the SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SM server assigns a unique identifier for the registration and stores the SM information provider profile provided in the request.

The SM server may obtain UE location information from the 3GPP core network by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [r23273] and 3GPP TS 23.502 [5], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4].

3.	The SM server sends a SM information provider registration response to the SM client. If the SM server successfully stored the SM information provider profile, the response includes an indication of success, the registration identifier and may include a proposed expiration time for the registration. Otherwise, the response includes an indication of failure and may include a reason for failure.

##### 9.3.x.2.2	SM information provider registration update

Figure 9.3.x.2.2-1 depicts the procedure for an authorized VAL client, via a SM client, to update a registered SM information provider information.

Pre-conditions:

1.	The SM client has successfully registered the SM information provider with the SM server.

Figure 9.3.x.2.2-1: SM information provider registration update procedure

1.	The SM client sends a SM information provider registration update request to the SM server. The request includes a requestor ID, security credentials, a registration ID, and may include an updated SM information provider profile and a proposed expiration time for the updated registration.

2.	Upon receiving the request, the SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SM server updates the SM information provider profile based on the request information.

3.	The SM server sends the SM information provider registration update response to the SM client. If the SM server successfully updated the SM information provider profile, the response includes an indication of success. Otherwise, the response includes an indication of failure and may include a reason for failure.

##### 9.3.x.2.3	SM information provider deregistration

Figure 9.3.x.2.3-1 depicts the procedure for an authorized SM client to deregister a SM information provider.

Pre-conditions:

1.	The SM client has already registered the SM information provider with the SM server.

Figure 9.3.x.2.3-1: SM information provider deregistration procedure

1.	The SM client sends a SM information provider deregistration request to the SM server. The request includes a requestor ID, security credentials, and a registration ID.

2.	Upon receiving the request, the SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SM server ends the SM information provider registration and removes the stored SM information provider profile.

3.	The SM server sends the SM information provider deregistration response to the SM client. If the SM server successfully deregistered the SM client, the response includes an indication of success. Otherwise, the response includes an indication of failure and may include a reason for failure.

##### 9.3.x.2.4	SM information provider discovery

Figure 9.3.x.2.4-1 depicts the procedure for an authorized VAL server to discover SM information provider(s).

Pre-conditions:

1.	The VAL server has received information (e.g. URI, IP address) related to the SM server;

2.	The VAL server has received security credentials authorizing it to communicate with the SM server.

Figure 9.3.x.2.4-1: SM information provider discovery procedure

1.	The VAL server sends a SM information provider discovery request to the SM server. The request includes a requestor ID, security credentials, discovery filters and endpoint information for receiving spatial mapping information from discovered SM information providers. Discovery filters inlcude the area of interest (e.g., three-dimensional area, localization information) and requirements on spatial mapping information as defined in clause 9.3.x.3.7.

NOTE 1:	How the VAL server identifies the area of interest for SM information provider discovery is not in the scope of this document.

2.	The SM server authorizes the VAL server and validates the request. If the request is authorized, the SM server determines one or more registered SM information provider(s) according to the discovery filters.

The SM server evaluates if the current UE location is in accordance with the area of interest included in the discovery request when determining the SM information provider(s). The SM server may obtain UE location information from the 3GPP core network by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [r23273] and 3GPP TS 23.502 [5], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4].

3.	The SM server sends a SM information provider notification to the determined SM client(s). The notification includes details about the requested spatial mapping information (e.g., spatial mapping information type and position, reporting frequency, etc.) and endpoint information for sending the spatial mapping information to the VAL server. The SM client provides the information included in the notification to the VAL client.

4.	The SM server sends a SM information provider discovery response to the requestor. The response includes information about SM information provider(s) when at least one SM information provider has been determined. If no SM information provided was determined, the SM server sends a failure indication with a reason for failure.

#### 9.3.x.3	Information flows

##### 9.3.x.3.1	SM information provider registration request

Table 9.3.x.3.1-1 describes information elements in the SM information provider registration request from the SM client to the SM server.

Table 9.3.x.3.1-1: SM information provider registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., SM client) |
| Requestor security credentials | M | The security credentials of the requestor |
| Proposed expiration time | O | Proposed expiration time for the registration |
| UE ID | O | The identifier of the hosting UE (i.e., GPSI) |
| VAL client ID | O | The identifier of the VAL client |
| Notification endpoint | M | The endpoint for notifying the SM information provider about a SM information request. |
| SM information provider profile | M | The SM information provider profile of the registering SM information provider as described in Table 9.3.x.3.1-2. |

Table 9.3.x.3.1-2 describes information elements in the SM information provider profile.

Table 9.3.x.3.1-2: SM information provider profile

| Information element | Status | Description |
| --- | --- | --- |
| SM information provider ID | M | The identifier of the SM information provider |
| SM information details | M | The SM information details available at the SM information provider; each element includes the information described below. |
| > SM data identifier | M | The identifier of SM information |
| > SM data type | M | The type of SM information (e.g., LiDAR camera, RGB-D camera, high-resolution cameras, etc.) |
| > SM data format | O | The format of a SM information (e.g., raw, processed, etc.) |

##### 9.3.x.3.2	SM information provider registration response

Table 9.3.x.3.2-1 describes information elements in the SM information provider registration response from the SM server to the SM client.

Table 9.3.x.3.2-1: SM information provider registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful |
| > Registration ID | M | Identifier of the registration |
| > Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O | Indicates that the registration request failed |
| > Cause | O | Indicates the cause of registration request failure |

##### 9.3.x.3.3	SM information provider registration update request

Table 9.3.x.3.3-1 describes information elements in the SM information provider registration update request from the SM client to the SM server.

Table 9.3.x.3.3-1: SM information provider registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., SM client) |
| Requestor security credentials | M | The security credentials of the requestor |
| Registration ID | M | Identifier of the registration |
| Proposed expiration time (NOTE) | O | Proposed expiration time for the registration |
| Updated SM information provider profile (NOTE) | O | The SM information provider profile, as described in Table 9.3.x.3.1-2, with updated SM information details. Included only if there is an update in SM information details. |
| NOTE: At least one of the IEs is included. |  |  |

##### 9.3.x.3.4	SM information provider registration update response

Table 9.3.x.3.4-1 describes information elements in the SM information provider registration update response from the SM server to the SM client.

Table 9.3.x.3.4-1: SM information provider registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration update request was successful |
| > Expiration time | O | Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

##### 9.3.x.3.5	SM information provider deregistration request

Table 9.3.x.3.5-1 describes information elements in the SM information provider deregistration request from the SM client to the SM server.

Table 9.3.x.3.5-1: SM information provider deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., VAL server) |
| Requestor security credentials | M | The security credentials of the requestor |
| Registration ID | M | Identifier of the registration |

##### 9.3.x.3.6	SM information provider deregistration response

Table 9.3.x.3.6-1 describes information elements in the SM information provider deregistration response from the SM server to the SM client.

Table 9.3.x.3.6-1: SM information provider deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the deregistration request was successful |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

##### 9.3.x.3.7	SM information provider discovery request

Table 9.3.x.3.7-1 describes information elements in the SM information provider discovery request from the VAL server to the SM server.

Table 9.3.x.3.7-1: SM information provider discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., VAL server) |
| Requestor security credentials | M | The security credentials of the requestor |
| Area of interest | M | Three-dimensional area of interest or localization information to discover SM information provider(s) in the area |
| SM information reporting endpoint | O | The endpoint information for receiving spatial mapping information from discovered SM information providers |
| Discovery filters | O | Set of characteristics to determine SM information provider(s) in the area of interest |
| > SM information details | O | The SM information details to match |
| >> SM data identifier | O | The identifier of SM information |
| >> SM data type | O | The type of SM information (e.g., LiDAR camera, RGB-D camera, high-resolution cameras, etc.) |
| >> SM data format | O | The format of a SM information (e.g., raw, processed, etc.) |

##### 9.3.x.3.8	SM information provider discovery response

Table 9.3.x.3.8-1 describes information elements in the SM information provider discovery response from the SM server to the VAL server.

Table 9.3.x.3.8-1: SM information provider discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the discovery request was successful |
| > SM information provider profiles | O | The list of SM information provider profiles as described in Table 9.3.x.3.1-2. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

##### 9.3.x.3.9	SM information provider notification

Table 9.3.x.3.9-1 shows the notification sent by a SM server to a SM client for triggering a spatial mapping information session from a SM information provider.

Table 9.3.x.3.9-1: SM information provider notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., SM server, VAL server). |
| Requestor security credentials | M | The security credentials of the requestor. |
| SM information provider ID | M | The identifier of the SM information provider. |
| SM information reporting session | M | The characteristics of SM information reporting session. |
| > SM data identifier | M | The identifier of spatial SM data to report. |
| > SM data type | O | The type of SM information data to report. (e.g., LiDAR camera, RGB-D camera, high-resolution cameras, etc.) data to report. |
| > SM data format | O | The format of a SM information data to report. (e.g., raw, processed, etc.). |
| > SM duration | O | The duration for which the SM data is reported. |
| > SM frequency | O | The frequency at which the SM data is reported. |
| > SM endpoint | M | The endpoint where to report the SM data. |

* * * End of Changes * * * *