3GPP TSG-SA WG6 Meeting #63	S6-244074

Hyderabad, India, 14th – 18th October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0058 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UAE service management for a UAV under NTZ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Observations: - a UAV UE decision to enforce NTZ may be decided at the UAV UE (based on application configuration/logic). - a UAV UE while enforcing NTZ may not be able to communicate with network (e.g., UAE server, USS). Based on these observations, the UAE server/USS may be unaware of a UAV UE decision to enforce NTZ which may disrupt functionality (e.g., DAA, C2, Location tracking etc.) at the UAE server, when UAV stops all transmissions (e.g., measurements, messages). Therefore, it is needed that a UAV UE informs the UAE server/USS about NTZ enforcement decisions to ensure proper behavior at the UAE server/USS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR propose support for a new NTZ indication procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality at the UAE server and services offered to a UAV UE and USS are broken when a UAV UE enforces NTZ rules. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.9 (new), 4.9.1 (new), 4.9.2 (new), 7.1a.3.1, 7.1a.3.5, 7.xx (new), 7.xx.2 (new), 7.xx.2.1 (new), 7.xx.3 (new), 7.xx.3.1 (new), 7.xx.3.2 (new), 7.xx.3.3 (new) ,8.2.1, 8.2.xx (new) 8.2.xx.1 (new), 8.2.xx.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

DAA application policy: The configuration provided to the UAS application specific client by a UAS application specific server for handling of detect and avoid.

DAA assist capability: The functionality for the UAE layer to assist the UAV application specific layer with handling of detect and avoid during flight.

LDGS capability: A UAE client with LDGS capability offers the ground based DAA support for the UAVs in the monitoring range of the LDGS. The Local DAA Ground-Station (LDGS) can be operated by a USS and can serve one or more USSes.

Multi-USS capability: The functionality for the UAE layer to assist at change of USS during flight.

NOTE:	A UAV with Multi-USS capability can be controlled by more than one USS during a flight, but at any given time, the UAV is under the control of only one USS.

Multi-USS policy: The configuration provided by a UAS application specific server to assist at change of USS.

Remote Identification (Remote ID) of UAS: The ability of a UAS to provide identification and tracking information that can be received by other parties, to facilitate advanced operations for the UAS (such as Beyond Visual Line of Sight operations as well as operations over people), assist regulatory agencies, air traffic management agencies, law enforcement, and security agencies when a UAS appears to be flying in an unsafe manner or where the UAS is not allowed to fly.

UAS Service Supplier (USS): An entity that provides services to support the safe and efficient use of airspace by providing services to the operator / pilot of a UAS in meeting UTM operational requirements. A USS can provide any subset of functionality to meet the provider's business objectives (e.g., UTM, Remote Identification). In the scope of this specification, the term USS refers to both USS and USS/UTM.

UAV: The Uncrewed Aerial Vehicle (also called remotely piloted aircraft or drone) of a UAS.

No-transmit zone: A geographical area where UAVs are not allowed to transmit in a certain frequency band.

For the purposes of the present document, the following terms given in 3GPP TS 22.125 [2] apply

Command and Control (C2) Communication

Uncrewed Aerial System (UAS)

Uncrewed Aerial System Traffic Management (UTM)

UAV controller

For the purposes of the present document, the following terms given in clause 4.2 of 3GPP TS 22.125 [2] apply

Direct C2 Communication

Network-Assisted C2 communication

UTM-Navigated C2 communication

* * * Next Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BVLOS	Beyond Visual Line Of Sight

C2	Command and Control

CAPIF	Common API Framework for northbound APIs

DAA	Detect And Avoid

LDGS	Local DAA Ground Station

NTZ	No-Transmit Zone

QoS	Quality of Service

SEAL	Service Enabler Architecture Layer

UAE	UAS Application Enabler

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

UAV-C	Uncrewed Aerial Vehicle-Controller

USS	UAS Service Supplier

UTM	UAS Traffic Management

* * * Next Change * * * *

## 4.9	Support for NTZ indication

### 4.9.1	Description

This clause specifies the requirements related to support for indication of UAV NTZ enforcement.

### 4.9.2	Requirements

[AR-4.9.2-a] The UAE Server shall provide a mechanism for the UAE Client to report its NTZ indication capability to the UAE Server.

[AR-4.9.2-b] The UAE Server shall provide a mechanism for the UAE Client to report NTZ indications to the UAE Server.

[AR-4.9.2-c] The UAE layer shall provide a mechanism for a UAS application specific server to obtain NTZ indication events for a UAV.

[AR-4.9.2-d] The UAS application enabler layer shall provide a mechanism for a UAS application specific client to provide NTZ indications events for a UAV.

* * * Next Change * * * *

#### 7.1a.3.1	Registration request

Table 7.1a.3.1-1 describes the information flow for a UAE client to register with the UAE server.

Table 7.1a.3.1-1: Registration request

| Information element | Status | Description |
| --- | --- | --- |
| UAV ID | M | The identifier of the UAV/UAV-C/LDGS (e.g., 3GPP UE ID or CAA level UAV ID) which initiates the registration request. |
| UAS UE information | O | UAS UE information like IP address, Multi-USS capability, DAA assist capability, Dual Network-Assisted C2 communication link capability, real time flight path monitoring assistance capability, LDGS capability, NTZ indication capability, etc. |
| Proposed registration lifetime (NOTE) | O | Proposed registration lifetime. |
| NOTE: If Proposed registration lifetime IE is not included, then the registration lifetime is valid until explicit deregistration is performed. |  |  |

* * * Next Change * * * *

#### 7.1a.3.5	Registration update request

Table 7.1a.3.5-1 describes the information flow for a UAE client to update registration with the UAE server.

Table 7.1a.3.5-1: Registration update request

| Information element | Status | Description |
| --- | --- | --- |
| UAV ID | M | The identifier of the UAV/UAV-C/LDGS (e.g., 3GPP UE ID or CAA level UAV ID) which initiates the registration request |
| UAS UE information | M | UAS UE information like IP address, Multi-USS capability, DAA assist capability, Dual Network-Assisted C2 communication link capability, real time flight path monitoring assistance capability, LDGS capability, NTZ indication capability, etc. |
| Proposed registration lifetime | O | Proposed registration lifetime. |

* * * Next Change * * * *

## 7.xx	NTZ indication procedure

### 7.xx.1	General

This feature introduces the UAS application enablement services for support of NTZ indications from a UAV about NTZ enforcement at the UAV. In particular, the UAE layer provides support for the following operations as described in clause 7.xx.2.1:

-	Support for informing the UAE server about NTZ status at the UAV.

-	Support for informing the UAS application enablement services about NTZ status at the UAV.

NOTE:	The NTZ status at the UAV can be "NTZ imminent entry" which is indicated prior to enforcing NTZ rules at the UAV or "NTZ exit" which is indicated once NTZ enforcement is completed at the UAV.

### 7.xx.2	Procedures

#### 7.xx.2.1	NTZ indication procedure

This procedure informs the UAE server and the UAS application specific server about the NTZ status at the UAV.

Figure 7.xx.2.1-1 describes the NTZ indication procedure.

Pre-conditions:

-	The UAV has performed the UAS UE registration procedure.

-	The UAV has received NTZ assistance information and detects a NTZ status change for imminent entry into a NTZ or for exit from a NTZ.

-	The UAE client has received an indication about an NTZ enforcement from the application layer.

Figure 7.xx.2.1-1: NTZ indication procedure

1.	The UAE client sends an NTZ indication request towards the UAE server. The request is sent to indicates a detected NTZ status change at the UAE client. The request contains the UAV identifier, the UASS identifier, the NTZ status change indication, 5D information and may contain NTZ enforcement times as described in clause 7.xx.3.1.

2.	Upon receiving the NTZ indication request, the UAE server updates the UAE client context based on the received NTZ information and informs the relevant UAV application enabling services about the NTZ status of the UAE client.

3.	The UAE server sends NTZ indication notification message to the UAS application specific server about the NTZ indication received from the UAE client, indicating that the UAE client may not be reachable for a period of time or became reachable again. The UAE server may include NTZ related information as received from the UAE client in step 1.

4.	The UAE server sends the NTZ indication response to the UAE client. If the NTZ status indicated by the UAE client is "NTZ imminent entry", the response may indicate whether the UAE server shall be notified when the NTZ condition is over.

### 7.xx.3	Information flows

#### 7.xx.3.1	NTZ indication request

Table 7.xx.3.1-1 describes the information flow for NTZ indication request sent from the UAE client to the UAE server.

Table 7.xx.3.1-1: NTZ indication request

| Information element | Status | Description |
| --- | --- | --- |
| UAV ID | M | The identifier of the UAV (e.g. 3GPP UE ID or CAA level UAV ID). |
| UASS ID | M | The Identity of the UAS application specific server. This ID can be the USS identifier, when the UAS application specific server is the USS. |
| NTZ status | M | The status of the NTZ enforcement; the status can be "NTZ imminent entry" or "NTZ exit". |
| NTZ 5D information | M | The information about the NTZ condition at the UAV. |
| > Area | M | The 3D geographical area of the NTZ. |
| > Time period | M | The time period when the NTZ is valid. |
| > Frequencies | M | The frequencies applicable to the NTZ. |
| NTZ enforcement time | O | The information about the NTZ enforcement time. |
| > estimated start time (NOTE 1) | O | The time the NTZ enforcement is expected to start. |
| > estimated completion time (NOTE 1) | O | The time the NTZ enforcement is expected to end. |
| > actual completion time (NOTE 2) | O | The time the NTZ enforcement completed. |
| NOTE 1: Mandatory if the NTZ status is for "NTZ imminent entry". NOTE 2: Mandatory if the NTZ status is for "NTZ exit". |  |  |

#### 7.xx.3.2	NTZ indication response

Table 7.xx.3.2-1 describes the information flow for NTZ indication response sent from the UAE server to the UAE client.

Table 7.xx.3.2-1: NTZ indication response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the UAE server in response to NTZ indication request indicating success or failure. |
| NTZ exit indication (NOTE) | O | The indication informs about the need to send a NTZ indication request with NTZ status of "NTZ exit" when the NTZ enforcement is over. |
| NOTE: Present only if the response is for an "NTZ imminent entry" request. |  |  |

#### 7.xx.3.3	NTZ indication notification

Table 7.xx.3.3-1 describes the information flow for NTZ indication notification sent from the UAE server to the UAS application specific server.

Table 7.xx.3.3-1: NTZ indication notification

| Information element | Status | Description |
| --- | --- | --- |
| UAV ID | M | The identifier of the UAV (e.g. 3GPP UE ID or CAA level UAV ID). |
| NTZ status | M | The status of the NTZ enforcement; the status can be "NTZ imminent entry" or "NTZ exit". |
| NTZ 5D information | M | The information about the NTZ that the condition applies to. |
| > Area | M | The 3D area of the NTZ. |
| > Time period | M | The time period of the NTZ. |
| > Transmission frequencies | M | The transmission frequencies of the NTZ |
| NTZ enforcement time | O | The information about the NTZ enforcement time. |
| > estimated start time (NOTE 1) | O | The time the NTZ enforcement is expected to start. |
| > estimated completion time (NOTE 1) | O | The time the NTZ enforcement is expected to end. |
| > actual completion time (NOTE 2) | O | The time the NTZ enforcement completed. |
| NOTE 1: Mandatory if the NTZ status is for "NTZ imminent entry". NOTE 2: Mandatory if the NTZ status is for "NTZ exit". |  |  |

* * * Next Change * * * *

### 8.2.1	General

Table 8.2.1-1 illustrates the UAE server APIs.

Table 8.2.1-1: List of UAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| UAE_C2OperationModeManagement API | Manage_C2OperationMode | UAS application specific server | Request/ Response |
|  | Notify_SelectedC2Mode (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2ModeSwitching (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2OperationModeManagementComplete (NOTE) | UAS application specific server | Subscribe/notify |
| UAE_RealtimeUAVStatus API | Subscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Notify_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
| UAE_ChangeUSSManagement API | Manage_USSManagement | UAS application specific server | Request/ Response |
|  | Notify_USSManagementComplete | UAS application specific server | Subscribe/notify |
|  | Manage_USSChange | UAS application specific server | Request/ Response |
|  | Notify_USSChange | UAS application specific server | Subscribe/notify |
|  | Notify_USSChangeTrigger | UAS application specific server | Subscribe/notify |
| UAE_DAASupport API | Manage_DAAManagement | UAS application specific server | Request/ Response |
|  | Notify_DAAManagementComplete | UAS application specific server | Subscribe/notify |
|  | Notify_DAAClientSupportEvent | UAS application specific server | Subscribe/notify |
|  | Manage_DAAServerSupportEvent | UAS application specific server | Request/ Response |
| UAE_UAVDynamicInfo API | Subscribe_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
|  | Notify_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
| UAE_FlightPathMonitoring API | Manage_FlightPathReportingConfiguration | UAS application specific server | Request/ Response |
|  | Notify_FlightPathReportingConfigurationComplete | UAS application specific server | Subscribe/notify |
|  | Notify_FlightPathEventReport | UAS application specific server | Subscribe/notify |
| UAE_FlightRouteSupport API | Manage_QoSFlightRoutePlan | UAS application specific server, UAE client | Request/ Response |
| UAE_NTZIndication API | Manage_NTZIndication | UAE client | Request/ Response |
|  | Notify_NTZIndication | UAS application specific server | Subscribe/notify |
| NOTE: The subscribe operation for Notify_C2OperationModeManagementComplete, Notify_SelectedC2Mode and Notify_C2ModeSwitching is part of Manage_C2OperationMode |  |  |  |

* * * Next Change * * * *

### 8.2.xx	UAE_NTZIndication API

#### 8.2.xx.1	General

API description: This API enables the UAE client to communicate with the UAE server to send information related to NTZ status at the UAS UEs (i.e., UAV) and enables the UAS application specific server to receive notifications informing about the UAS UEs (i.e., UAV) NTZ status.

#### 8.2.xx.2	Manage_NTZIndication operation

API operation name: Manage_NTZIndication

Description: Request indicating the UAS UE (i.e., UAV) NTZ status.

Known Consumers: UAE client.

Inputs: Refer clause 7.xx.3.1.

Outputs: Refer clause 7.xx.3.2.

See clause 7.xx.2.1 for the details of usage of this API operation.

#### 8.2.xx.2	Notify_NTZIndication operation

API operation name: Notify_NTZIndication

Description: Notification of the UAS UE (i.e., UAV) NTZ status.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.xx.3.3.

Outputs: None.

See clause 7.xx.2.1 for the details of usage of this API operation.

* * * End of Changes * * * *