3GPP TSG-SA WG6 Meeting #63	S6-244098

Hyderabad, India, 19th – 23rd August 2024	(revision of S6-24xxxx)

Source:	Ericsson

Title:	Pseudo-CR on clause 5

Spec:	3GPP TR 23.700-09 v 0.2.0

Agenda item:	8.8

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR address KI#1 with the focus on clause 5.

2. Reason for Change

This pCR looks into having clause 5 of TS 23.180 as a generic solution.

3. Conclusions

This pCR updates clause 5.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-09 v 0.2.0

* * * First Change * * * *

## 6.x	Solution #X: Clause 5 (Architecture requirement) of TS 23.180

### 6.x.1	General

This clause is related to KI#1 addresses clause 5 of 3GPP TS 23.180. It describes which changes are needed to make it applicable for all supported 3GPP access methods.

### 6.x.2	Solution

# 5	Architectural requirements

## 5.1	General requirements

### 5.1.1	Description

This clause specifies the general requirements for the support of MC services in the IOPS mode of operation.

### 5.1.2	Requirements

[AR-5.1.2-a] For the case of a backhaul failure between the radio access network and the macro 3GPP core network, the IOPS MC system shall provide support for MC services in the IOPS mode of operation until the failure is recovered.

[AR-5.1.2-b] MC service UEs within the coverage of the IOPS local core network shall be able to provide MC services over the IOPS MC system.

[AR-5.1.2-c] The IOPS MC system shall provide mechanisms to verify the availability of MC service UEs within the IOPS MC system.

## 5.2	MC service requirements

### 5.2.1	Description

This clause specifies the MC service requirements in the IOPS mode of operation.

### 5.2.2	Requirements

[AR-5.2.2-a] The following MCPTT service features shall be supported in the IOPS mode of operation

-	MCPTT group call

-	MCPTT emergency group call

-	MCPTT private call

-	MCPTT emergency private call

[AR-5.2.2-b] The following MCData service features shall be supported in the IOPS mode of operation

-	MCData short data service

## 5.3	Network bearer or session requirements

### 5.3.1	Description

This clause specifies the network bearer requirements in the IOPS mode of operation.

### 5.3.2	Requirements

[AR-5.3.2-a] The IOPS MC system shall provide support for unicast bearer or session establishment and modification to support MC services via unicast transmissions in the IOPS mode of operation.

[AR-5.3.2-b] The IOPS MC system shall provide support for multicast bearer or MBS session establishment and modification to support MC services via multicast or broadcast transmissions in the IOPS mode of operation.

[AR-5.3.2-c] The IOPS MC system shall support announcement of multicast bearers or MBS sessions to the MC service UEs.

## 5.4	IP connectivity functionality

### 5.4.1	Description

The support of MC services in the IOPS mode of operation can be based on the support of the IP connectivity functionality. The IP connectivity functionality enables that MC services are provided by the MC service clients via the IOPS MC connectivity function.

An IOPS MC connectivity function supporting the IP connectivity functionality does not provide MC services. Instead, the IOPS MC connectivity function enables discovering MC service users and provides IP connectivity for the communication among the discovered MC service users, i.e. it distributes the related IP traffic to the MC service UEs over IP unicast and/or multicast transmissions. A MC service UE supporting the IP connectivity functionality in the IOPS mode of operation enables transmitting the IP packets related to a MC service communication over the IOPS MC connectivity function.

This clause specifies requirements for the support of the IP connectivity functionality in the IOPS mode of operation.

### 5.4.2	General requirements

[AR-5.4.2-a] The IOPS MC connectivity function supporting the IP connectivity functionality shall support discovering MC service users within the coverage of the IOPS local core network.

[AR-5.4.2-b] The IOPS MC connectivity function supporting the IP connectivity functionality shall support announcement, subscription and notification mechanisms for discovered MC service users supporting the IP connectivity functionality.

[AR-5.4.2-c] The IOPS MC connectivity function supporting the IP connectivity functionality shall provide IP connectivity for communication among MC service users requesting the support of the IP connectivity functionality.

[AR-5.4.2-d] The MC service UEs supporting the IP connectivity functionality in the IOPS mode of operation shall enable transmitting the IP packets related to a MC service communication over the IOPS MC connectivity function.

### 5.4.3	Bearer management requirements

[AR-5.4.3-a] The MC service UEs shall use specific APNs or DNNs for the support of the IP connectivity functionality, as follows:

-	An APN/DNN for the IOPS related application signalling related to the IP connectivity functionality support, e.g. for IOPS discovery, subscription and notification.

-	An MC service APN/DNN for the communication with another MC user or group via the IOPS MC connectivity function based on the IP connectivity functionality, e.g. for the signalling and media.

[AR-5.4.3-b] The APNs or DNNs for the IP connectivity functionality support shall be made available to the MC service UEs either via UE (pre)configuration or via initial UE configuration, as described in 3GPP TS 23.280 [3], and 3GPP TS 23.289 [23.289].

### 6.x.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

#### 6.x.3.1	Clause 2

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services ".

[4]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[5]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.002: "Network Architecture".

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[10]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[11]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[23.289]	3GPP TS 23.289: "Mission Critical Services over 5G System; Stage 2".

#### 6.x.3.2	Clause 3

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: As described in 3GPP TS 23.401 Annex K [2].

IOPS local core: IOPS local core is a group of 3GPP network core entities that are either co-sited with or co-located in an IOPS capable base station.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

APN	Access Point Name

DNN	Data Network Name

IOPS	Isolated Operation for Public Safety

EPC	Evolved Packet Core

EPS	Evolved Packet System

MBMS	Multimedia Broadcast and Multicast Service

MC	Mission Critical

PLMN	Public Land Mobile Network

### 6.x.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 into an access generic IOPS solution.

* * * End of Changes * * * *