3GPP TSG-SA WG6 Meeting #63	S6-244139

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0092 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support UE-to-UE communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SEALDD (as XR app enabler) can support the coordination between network-based communication (on-network communication) and direct communication (off-network communication) between UEs, as concluded in TR 23.700-23 (KI#3, sol#14) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SEALDD-PC5 reference point in architecture. Add procedure description in clause 9.x. The corresponding IE table and API impacts are also added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting new feature in Rel19 XR work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: 7.4.X, 7.5.3.X 9.12.2.x, 9.12.2.x.1, 9.12.2.x.2 9.12.3.x1, 9.12.3.x2, 9.12.3.x3, 9.12.3.x4 9.12.4.x, 9.12.4.y Existing clauses: 2, 7.2, 9.10.3.1, 9.12.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[8]	3GPP TS 23.548: "5G System Enhancements for Edge Computing ".

[9]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[11]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[13]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[14]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[15]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[TS23303]	3GPP TS 23.303: "Proximity-based Services (ProSe); Stage 2".

[TS23304]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[P12033]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

* * * Next Change * * * *

## 7.2	Architecture

This clause describes the architecture for enabling SEAL Data Delivery applications in the following representations:

-	A service-based representation as specified in 3GPP TS 23.434 [4], where the SEAL Data Delivery Enabler Layer functions (e.g. SEALDD server) enable other authorized Vertical Application Layer functions (e.g. VAL server) to access their services.

-	A service-based representation as specified in 3GPP TS 23.501 [5], where the Network Functions (e.g. NEF) enable authorized SEAL Data Delivery Layer functions (e.g. SEALDD server) i.e. Application Functions, to access their services;

-	A service-based representation, where the Core Network Northbound APIs as specified in 3GPP TS 23.501 [5] and 3GPP TS 23.502 [6], are utilized by authorized SEAL Data Delivery Enabler Layer functions via CAPIF core function specified in 3GPP TS 23.222 [3]; and

-	A reference point representation, where existing interactions between any two functions (e.g. SEALDD client and SEALDD server) is shown by an appropriate point-to-point reference point (e.g. SEALDD-UU).

SEAL Data Delivery Enabler Layer functions shown in the service-based representation of the SEAL Data Delivery architecture shall only use service-based interfaces for their interactions.

The service based representation of SEAL Data Delivery function in the overall SEAL service-based representation is specified in clause 15 of 3GPP TS 23.434 [4]. The SEALDD function exhibits service-based interfaces which are used for providing and consuming SEALDD services. The service-based interface for SEALDD function is representation as Sdd.

Figure 7.2-1 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [5].

Figure 7.2-1: Utilization of 5GS network services based on the 5GS SBA – service based representation

The SEALDD server acts as AF for consuming network services from the 3GPP 5G Core Network entities over the Service Based Architecture specified in 3GPP TS 23.501 [5].

Figure 7.2-2 illustrates the service-based representation for utilization of the Core Network (5GC, EPC) northbound APIs via CAPIF.

Figure 7.2-2: Utilization of Core Network Northbound APIs via CAPIF – service based representation

The SEALDD server acts as authorized API invoker to consume services from the Core Network (5GC, EPC) northbound API entities like SCEF, NEF, SCEF+NEF which act as API Exposing Function as specified in 3GPP TS 23.222 [3].

The mechanism for northbound APIs discovery using the service-based interfaces depicted in figure 7.2-3 is as specified in 3GPP TS 23.222 [3].

Figure 7.2-3 illustrates the architecture for SEAL Data Delivery enabler service.

Figure 7.2-3 Architecture for SEAL Data Delivery Service

The SEALDD server can communicate with the control plane of 3GPP core network via N33/N5 interface with the SEALDD control plane functionality. The SEALDD server may consume other SEAL (e.g. NRM) services.

For uplink traffic, VAL client sends application data traffic to SEALDD client for SEALDD service over SEALDD-C. After data plane packet processing by SEALDD client, the application data traffic is converted to SEALDD data traffic and transferred to SEALDD server over SEALDD-UU. The SEALDD server restores the application data traffic and sends it to VAL server over SEALDD-S. For downlink traffic, VAL server sends application data traffic to SEALDD server for SEALDD service over SEALDD-S. After data plane packet processing by SEALDD server, the application data traffic is converted to SEALDD data traffic and transferred to SEALDD client over SEALDD-UU. The SEALDD client restores the application data traffic and sends it to VAL client over SEALDD-C. Optionally, VAL deployments may choose to route application signalling traffic and application data traffic for some or all functions it offers using SEALDD service and figure 7.2-4 illustrates the architecture for achieving this. In this case the VAL client and VAL server may choose not to maintain application connection by themselves and transfer all the application traffic over SEALDD connections for those functions. The data storage functionality may be provided by SEALDD server or provided by other storage functions in VAL server, or other cloud platform.

NOTE 1:	It is up to the implementation of VAL server about which storage entity (e.g. VAL server, SEALDD server, or other cloud platform) is selected and used.

NOTE 2:	SEALDD capabilities are provided as APIs to the VAL Layer, it is up to VAL layer to decide which traffic to be transferred (e.g. application signalling, application data).

Figure 7.2-4: Architecture for application traffic transfer

The SEAL Data Delivery client interacts with the SEAL data delivery server to establish application layer data transport path.

Through this path, the SEALDD server and client provides data transport service capabilities such as data plane packet processing (e.g. packet duplication, elimination or transport coordination), data forwarding, data caching, background data transfer, etc. to support the VAL server and VAL client. Annex C describes a typical lifecycle of SEALDD to establish the SEALDD connection for the VAL client and VAL server.

Figure 7.2-5 illustrates the architecture for SEAL Data Delivery enabler service used to support UE-to-UE communication.

Figure 7.2-5: SEALDD architecture with on-network and off-network

VAL UE 1 and VAL UE 2 have on-network connectivity, and both VAL UEs can directly communication with each other via off-network connectivity (i.e. SEALDD-PC5 reference point) or via on-nework connectivity (i.e. SEALDD-UU reference point).

* * * Next Change * * * *

### 7.4.X	SEALDD-PC5

SEALDD-PC5 reference point is between the SEAL clients of two VAL UEs. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [TS23303] or 3GPP TS 23.304 [TS23304]. It supports the user plane functionality of transfering data content and the signalling plane functionality of exchanging information for SEALDD service provisioning, control, reporting etc.

* * * Next Change * * * *

#### 7.5.3.X	SEALDD-PC5 (Between SEAL clients of two VAL UEs)

The following cardinality rules apply for the reference of SEALDD-PC5:

a)	One SEALDD client may communicate with one or more SEALDD client(s) concurrently.

* * * Next Change * * * *

#### 9.12.2.x	SEALDD enabled UE-to-UE communication based on policy

##### 9.12.2.x.1	General

When two VAL UEs need to communicate with each other for exchanging gaming/interactive data in XR service, they can use servers in the DN as data relay or communicate with each other directly.

NOTE 1:	This solution is only applicable for the XR transmission scenario without involvement of VAL server. As an example, XR gaming application client can support LAN-type of multi-player game mode without involvement of XR gaming server to process run-time game data.

In order to improve service experience (esp. for reducing service latency), SEALDD server monitors distance of UEs involved in the XR communication. When SEALDD server finds that the two UEs are within direct communication range, the SEALDD server instructs the SEALDD clients to switch to direct communication mode. Such mode switch decision needs to take E2E communication performance into consideration.

If the UE-to-UE application performance analytics result from ADAE shows the quality in off-network mode will be degraded and the quality in on-network mode is estimated to be good, or vice versa, the mode switch is done in SEALDD layer.

NOTE 2:	For the same XR application service, all XR traffic flows are transmitted via either off-network or on-network between UEs.

##### 9.12.2.x.2	SEALDD enabled UE-to-UE communication

The SEALDD servers has Data Delivery (DD) policy being provisioned for UE-to-UE communication. The DD policy is enforced by the SEALDD server to switch the SEALDD connection for UE-to-UE communication (either direct or indirect).

Pre-conditions:

- The SEALDD server has DD policies available.

- The SEALDD clients in UE1 and UE2 has discovered the same SEALDD server.

Figure 9.12.2.x.2-1: Policy enforced by SEALDD server for connectivity between two UEs

1a.	The UE-to-UE communication is established between UE 1 and UE 2 via SEALDD client.

1b.	The SEALDD client in UE1/2 informs the SEALDD server about the establishment of direct SEALDD communication.

2.	Based on SEALDD UE-to-UE policy, the SEALDD server requests 3GPP CN (NWDAF/NEF) for UE relative proximity analytics for UE1 and UE2, requests 3GPP CN (GMLC/NEF) for SL/Ranging service exposure about the relative locations or distances and directions related to the UE1 and UE2, and requests ADAES for UE-to-UE application performance analytics as described in 3GPP TS 23.436 [13] clause 8.4, requests NWDAF/NEF for QoS Sustainability Analytics and Service Experience Analytics for UEs and/or requests PCF/NEF for QoS monitoring.

Editor's note:	Whether SA2 provided services can support UE-to-UE communication analytics is FFS. Whether ADAES supports off-network communication analytics is FFS.

Editor's note:  For the switch between direct mode and indirect mode, it is FFS whether UE can trigger such an action.

3.	According to the SEALDD UE-to-UE policy, if relative proximity analytics result shows that both UEs are not in vicinity to each other (distance between UEs > proximity threshold in UE-to-UE policy), and/or the QoS/QoE for UE-to-UE direct communication analytics result is no so good (e.g. PLR in analytics report > PLR threshold in UE-to-UE policy), the SEALDD continues with step 4 for indirect communication preparation; otherwise, the SEALDD keeps monitoring (as described in step 2) for the two UEs.

4.	After establishment of SEALDD connections for UE1 and UE2, the SEALDD traffic carrying XR flows are processed in the SEALDD server. The SEALDD server forwards traffic between UE1 and UE2, and may buffer data burst from originating UE and send it to destination UE smoothly.

5.	If later on, both UEs are in proximity which is capable of direct communication and the QoS/QoE analytics result from ADAES for UE-to-UE direct communication shows good quality, the SEALDD server helps to establish direct SEALDD communication by informing the SEALDD client of any of the two UEs with the information of its peer SEALDD client (of UE1 or UE2). Then the SEALDD client 1 or 2 establishes SEALDD connection towards the peer SEALDD client via SEALDD-PC5 and the SEALDD traffic carrying XR flows are exchanged directly between UE1 and UE2 via SEALDD-PC5.

Based on the monitoring and analytics result (as requested in step 2), and SEALDD UE-to-UE policy:

-	if the current UE1-UE2 communication mode is direct and SEALDD server decides to switch to indirect communication mode, the SEALDD server establishes SEALDD connections with SEALDD client 1 and SEALDD client 2 as described in step 4, then triggers direct SEALDD communication release towards the SEALDD client which received establishment request in step 5 and the SEALDD client further releases the direct SEALDD connection towards its peer SEALDD client via SEALDD-PC5.

-	if the current UE1-UE2 communication mode is indirect and SEALDD server decides to switch to direct communication mode, the SEALDD server triggers direct SEALDD communication setup as described in step 5 and releases SEALDD connections towards SEALDD client 1 and SEALDD client 2.

* * * Next Change * * * *

#### 9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| -Identities | O | Identifier of the VAL UEs or VAL users for which SEALDD policy applies |
| SEALDD policy | O (See NOTE 4) | The SEALDD policy associated with application traffic identifiers, VAL UE identity |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the event (e.g. measurement threshold) to be measured for the quality guarantee |
| > Quality optimization policy | O (See NOTE 1) | Indicates the data transmission adjustment policy (e.g., adjustment need) to be performed in SEALDD layer. |
| > Bandwidth control policy | O (See NOTE 2) | Indicate the bandwidth control preference, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL |
| > Geofence policy | O (See NOTE 3) | Indicates the geofence policy per VAL service |
| >> Geofence location | O (See NOTE 3) | Indicates the geofence location information for the mentioned VAL service |
| >>Geofence policy action | O (See NOTE 3) | Indicates the policy action like allowed or blocked |
| > temporal policy | O (See NOTE 3) | Indicates the time period for which the sealdd traffic is allowed |
| > policy expiration time | O | Indicates the validity of the policy |
| Multi-modal SEALDD policy | O (See NOTE 4) | Multi-modal SEALDD policy associated with set of individual SEALDD flows. |
| > Synchronization policy | O | Indicates the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2] |
| > Policy expiration time | O | Indicates the validity period of the policy |
| > UE-to-UE policy | O | Specifies UE-to-UE direct communication policy including proximity threshold, QoS threshold (e.g. bitrate, latency and jitter, PLR, PER) or QoE threshold (e.g. MOS as specified in ITU-T P.1203.3 [P12033]) for entering direct communication mode, and proximity threshold, QoS threshold or QoE threshold for leaving direct communication mode. |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. NOTE 3: This IE is used for the SEALDD connection establishment and data delivery, as specified in clause 9.2 NOTE 4: At least one of these IEs shall be present. |  |  |

* * * Next Change * * * *

#### 9.12.3.x1	Inform XR transmission request

Table 9.12.3.x1-1 describes the information flow from the SEALDD client to the SEALDD server to inform XR transmission connection status between two UEs.

Table 9.12.3.x1-1: Inform XR transmission request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (i.e. SEALDD client). |
| VAL service ID | O | Identity of the VAL service |
| SEALDD-PC5 flow ID | M | Identity of the SEALDD-PC5 flow. |
| Identities | M | Identifier of UEs or VAL users involved in the UE-to-UE direct communication. |
| Status | M | Identifies the UE-to-UE direct communication status (e.g. establishment, release). |

#### 9.12.3.x2	Inform XR transmission response

Table 9.12.3.x2-1 describes the information flow from the SEALDD server to the SEALDD client to acknowledge XR transmission connection status between two UEs.

Table 9.12.3.x2-1: Inform XR transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the operation. |

#### 9.12.3.x3	Trigger XR transmission request

Table 9.12.3.x3-1 describes the information flow from the SEALDD server to the SEALDD client to trigger XR transmission connection operation between two UEs.

Table 9.12.3.x3-1: Trigger XR transmission request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (i.e. SEALDD server). |
| VAL service ID | O | Identity of the VAL service |
| Identities | M | Identifier of UEs or VAL users involved in the UE-to-UE direct communication. |
| Operation | M | Identifies the action for UE-to-UE direct communication (e.g. establishment, release). |

#### 9.12.3.x4	Trigger XR transmission response

Table 9.12.3.x4-1 describes the information flow from the SEALDD client to the SEALDD server to acknowledge XR transmission connection operation between two UEs.

Table 9.12.3.x4-1: Trigger XR transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the operation. |

* * * Next Change * * * *

##### 9.12.4.1	General

Table 9.12.4.1-1 illustrates the APIs exposed by SEALDD server for XR transmission.

Table 9.12.4.1-1: List of SEALDD server APIs for XR transmission

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_XRTransmission | Request | Request/Response | VAL server |
| Sdd_XRTransmissionConnnection | Establish | Request/Response | SEALDD client |
|  | Trigger (NOTE) | Request/Response | SEALDD server |
|  | Inform | Request/Response | SEALDD client |
| NOTE: The Sdd_XRTransmissionConnection_Trigger API operation is exposed by the SEALDD client. |  |  |  |

* * * Next Change * * * *

##### 9.12.4.x	Sdd_XRTransmissionConnection_Trigger operation

API operation name: Sdd_XRTransmissionConnection_Trigger

Description: The consumer requests to trigger direct multi-modal transmission connection establishment between two UEs.

Inputs: See clause 9.12.3.x1.

Outputs: See clause 9.12.3.x2.

See clause 9.12.2.x for details of usage of this operation.

##### 9.12.4.y	Sdd_XRTransmissionConnection_Inform operation

API operation name: Sdd_XRTransmissionConnection_Inform

Description: The consumer informs direct multi-modal transmission connection establishment between two UEs.

Inputs: See clause 9.12.3.x3.

Outputs: See clause 9.12.3.x4.

See clause 9.12.2.x for details of usage of this operation.

* * * END of Change * * * *