3GPP TSG-SA WG6 Meeting #63	S6-244153

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0233 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving EN in Late entry pre-arranged group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedure on Late entry pre-arranged group call (7.1.2.3.1.1.4) contains the following EN: Editor's Note: Checking of functional alias of MCVideo client 1 by the MCVideo server is FFS. Independent whether the reason for (re)-joining the group call due to loss of coverage or due to newly affiliated, the MCVideo is required to keep the dynamic data in conjunction with functional aliases always up-to date: See TS 23.280 and the tables below: Table 10.1.5.5.1-1: Dynamic data associated with a group Parameter description. Status i.e. indication of potential emergency or in-peril status of the group, together with the identification of the user who has performed the last modification of this status.. Affiliation status of each MC service ID of the group corresponding to the MC service and the Contact URI(s) from which the user affiliated.. Contact URIs used for designation of the group e.g. aliases of group broadcast, group regroup group URIs.. Media description for group media, including transport and multiplexing information.. Group call ongoing.. Regroup status of group where regrouping using a preconfigured group is used, e.g. whether group is regrouped, MC service group identity of regroup group.. Functional alias binding information for each user within the group.. Table 10.1.7.3-1: Dynamic data associated with a functional alias Parameter description. Activation status of each functional alias and the corresponding MC service ID of the user who activated that functional alias (subscription and notification procedures for functional alias are defined in clause 10.13.10). The MCVideo server needs to know which functional alias are used in the group and it needs to know the association between an activated functional alias and the related MCVideo ID. So the answer to the question in the editor’s note is YES and steps 3 and 4 within the pre-conditions part of the procedure describes all what is needed. The EN can be deleted. |
| Summary of change: |  | The EN in the procedure is deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved editor’s note in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.1.2.3.1.1.4	Late entry pre-arranged group call

Procedures in figure 7.1.2.3.1.1.4-1 are the signalling control plane procedures for the MCVideo server requesting a newly affiliated member or a member coming back from out of coverage to join an ongoing MCVideo group call.

Pre-conditions:

1.	MCVideo group is previously defined on the group management server with MCVideo users affiliated to that group. All members of the group belong to the same MC system.

2.	It is assumed that MCVideo users on MCVideo client 2 to MCVideo client n are on an ongoing call.

3.	Optionally, the MCVideo client 1 may have activated functional alias to be used.

4.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.1.2.3.1.1.4-1: Late entry pre-arranged group call

1.	MCVideo server determines that MCVideo client 1 which is newly affiliated or coming back from out of coverage has to be invited to join an ongoing group call (late entry).

2.	MCVideo server generates group call request including the information such as MCVideo service identifier (possible for the SIP core to route the request to the MCVideo server), MC service group ID of the group invited to join, offer one or more media types and sends towards the MCVideo client 1 via SIP core.

3.	MCVideo user at MCVideo client 1 is notified about the incoming group call.

4.	Upon MCVideo user at MCVideo client 1 accepting the incoming group call request, MCVideo client 1 sends the group call response including the selected media types to the MCVideo server through the signalling path. If the incoming group call request is rejected by the MCVideo client 1, the MCVideo server should not resend the group call request

5.	MCVideo client 1 is successfully added to the ongoing group call and MCVideo users at MCVideo client 1 to MCVideo client n may be notified about the MCVIDEO client 1 joining the group call.

6.	A notification with the information of media transmissions in the group call is sent to MCVideo client1.