3GPP TSG-SA WG6 Meeting #63	S6-244169

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	ETRI, Convida Wireless LLC

Title:	Spatial map procedure updates and Information Flows

Spec:	3GPP TS 23.437 v0.2.0

Agenda item:	9.11

Document for:	Approval

Contact:	Byung Jun AHN (bjahn@etri.re.kr)

1. Introduction

Spatial map procedures are updated to reflect “In-progress” create spatial map response and spatial map layering information. And information flow IEs and SEAL APIs are provided.

2. Reason for Change

“In-progress” response for create spatial map procedure and spatial map layering information has been added to the latest TR 23.700-21, which are not reflected to TS 23.437 v0.2.0 yet. Also, information flow IEs and SEAL APIs for spatial map procedures are required to be provided.

3. Conclusions

Updated procedure text, information flow IEs and SEAL APIs for spatial map procedures are provided.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437.

* * * First Change * * * *

9	Spatial Map (SM) management

9.1	General

The Spatial Map (SM) management is a SEAL service that offers the spatial map related capabilities to vertical application(s).

* * * Next Change * * * *

9.3	Spatial map management procedures

9.3.1	Create spatial map

Figure 9.3.1-1 depicts the procedure for creating a spatial map. For the request from VAL server or SEAL SM client, as a spatial map consumer, SEAL SM server creates a spatial map. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. space, objects).

Figure 9.3.1-1: Create spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map and spatial map layering information parameters (e.g., layer 1-basic space, layer 2-objects, and relationship between layers).

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. The SEAL SM server produces a requested spatial map using the processed sensor data obtained from various data sources (e.g. VAL server, VAL database or VAL UE) mapped to one or more layers.

NOTE:	For current release, some of processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of SA6 scope.

3)	The SEAL SM server sends response message to the requestor with a indication of success, in-progress or fail. If the requested spatial map has been created sussesfully, the response message includes assigned spatial map ID and information which includes three-dimensional space defined by the spatial map, stationary or moving objects with attributes of them and/or spatial map layering related information (e.g.  a list of spatial map layers and its corresponding layer IDs). If the response indicates "in-progress", an assigned spatial map ID is included in the message, then the requestor subscribes spatial map event to receive notification when the spatial map is ready for the utilization (e.g. for localization service). Otherwise, the response includes an indication of failure and may include a reason for the failure.

Editor’s Note: For the “in-progress” response case, as an alternative way, it is FFS whether and how the create spatial map procedure implicitly subscribes the spatial map event to receive notification on the behalf of the requestor.

9.3.1.1	Information flows for Create spatial map

Table 9.3.1.1-1 descibes the information elements from VAL server (or SEAL SM client) to SEAL SM server for create spatial map request.

Table 9.3.1.1-1: Create spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information on requesting VAL application service entity |
| > Application service ID | M | ID of the requesting VAL application service |
| > Service endpoint | M | Endpoint information of the application service to communicate with (e.g., URI, FQDN, IP address) |
| > Service connection information | O | Information for establishing connectivity to the application service (e.g., DNN, DNAI(s)) |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Information to include | O | Information to be included in the spatial map |
| > Access control rules | O | Access control rules defining which entities are permitted to discover and access the spatial map |
| > Spatial map layering information | O | Layering information for multi-layered spatial map |
| >> List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >>> Spatial map layer information | O | A container to specify the corresponding spatial map layer specific information |
| >> Relationship between layers | O | A container to specify the relationship between layers |
| > Spatial map specific information | O | A container to specify the requesting spatial map specific information |

NOTE 1:	Further elements of the information to include can be defined.

Table 9.3.1.1-2 descibes the information elements from SEAL SM server to VAL server (or SEAL SM client) for create spatial map response.

Table 9.3.1.1-2: Create spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Indicates successful create |
| > Spatial map ID | M | ID of the created spatial map |
| > Spatial map coverage space | O | Three-dimensional space coordinates information which defines the spatial map |
| > Access control rules | O | Access control rules defining which entities are permitted to discover and access the spatial map |
| > Spatial map layering information | O | Layering information for multi-layered spatial map |
| >> List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >>> Spatial map layer information | O | A container to carry the corresponding spatial map layer specific information |
| >> Relationship between layers | O | A container to carry the relationship between layers |
| > Spatial map specific information | O | A container to carry the corresponding spatial map specific information |
| > List of objects | O | List of object information. Each element includes the information below. |
| >> Object ID | M | ID of the corresponding object |
| >> Object type | M | Type of object (stationary or moving) |
| >> Object specific information | O | A container to carry the corresponding object specific attribute information |
| In-progress response | O (NOTE) | Indicates spatial map creation process is not yet completed, but “in-progress” |
| > Spatial map ID | M | Assigned spatial map ID |
| Failure response | O (NOTE) | Indicates failed create spatial map |
| > Cause | O | Cause of the failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE 2:	Further elements of the spatial map information can be defined.

9.3.2	Discover spatial map

Figure 9.3.2-1 depicts the procedure for an authorized spatial map consumer (VAL server or SEAL SM client) to discover spatial map(s). The service is provided by SEAL SM server.

Figure 9.3.2-1: Discover spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to discover spatial map(s). The request includes requestor ID, security credentials, three-dimensional area of interest or localization information to discover spatial maps in the area, optionally discovery filters, i.e., a set of characteristics (e.g., a specific list of spatial map layers with corresponding layer IDs) to search the matching spatial maps when multiple spatial maps are allowed in the area of interest.

NOTE:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this document.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then the server finds spatial map(s) which matches requested discovery requirements.

3)	The SEAL SM server sends the discover spatial map response message to the requestor with spatial map ID(s) when matching spatial map is found, otherwise it sends failure indication with reason.

9.3.2.1	Information flows for Discover spatial map

Table 9.3.2.1-1 descibes the information elements from VAL server (or SEAL SM client) to SEAL SM server for discover spatial map request.

Table 9.3.2.1-1: Discover spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information on requesting VAL application service entity |
| > Application service ID | M | ID of the requesting VAL application service |
| > Service endpoint | M | Endpoint information of the application service to communicate with (e.g., URI, FQDN, IP address) |
| > Service connection information | O | Information for establishing connectivity to the application service (e.g., DNN, DNAI(s)) |
| Filtering criteria | M | Criteria to get spatial map(s) |
| > Area of interest | O (NOTE) | Three-dimensional area information to discover spatial maps |
| > Localization information | O (NOTE) | A known location to discover spatial maps in the three-dimensional space, including an orientation |
| > Optional discovery filters | O | A set of characteristics to search the matching spatial maps |
| >> List of spatial map layers | O | A list of spatial map layers with corresponding layer IDs to discover spatial map(s) with matching layers. Each element includes the information described below. |
| >>> Spatial map layer ID | O | ID of specific spatial map layer |
| >> Relationship between layers | O | A container to carry the relationship between layers |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE:	Further optional discovery filters can be defined.

Table 9.3.2.1-2 descibes the information elements from SEAL SM server to VAL server (or SEAL SM client) for Discover spatial map response.

Table 9.3.2.1-2: Discover spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Indicates successful discovery |
| > List of discovered spatial maps | M | List of discovered spatial anchor(s). Each element includes the information described below |
| >> Spatial map ID | M | ID of a spatial map discovered |
| Failure response | O (NOTE) | Indicates failed discovery |
| > Cause | O | Cause of the failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

9.3.3	Get spatial map information

Figure 9.3.3-1 depicts the procedure for an authorized spatial map consumer (VAL server or SEAL SM client) to get the spatial map. The service is provided by SEAL SM server.

Figure 9.3.2-1: Get spatial map information procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to get the spatial map information. The request includes requestor ID, security credentials, spatial map ID to get the information for it.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then the server collects the requested spatial map information.

3)	The SEAL SM server sends the get spatial map information response message to the requestor with requested spatial map information when successful, otherwise it sends failure indication with reason.

9.3.3.1	Information flows for Get spatial map information

Table 9.3.3.1-1 descibes the information elements from the VAL server (or SEAL SM client) to the SEAL SM server for get spatial map request.

Table 9.3.3.1-1: Get spatial map information request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | ID of the spatial map to get information of it |
| VAL service information | M | Information on requesting VAL application service entity |
| > Application service ID | M | ID of the requesting VAL application service |
| > Service endpoint | M | Endpoint information of the application service to communicate with (e.g., URI, FQDN, IP address) |
| > Service connection information | O | Information for establishing connectivity to the application service (e.g., DNN, DNAI(s)) |

Table 9.3.3.1-2 descibes the information elements from the SEAL SM server to the VAL server (or SEAL SM client) for get spatial map response.

Table 9.3.3.1-2: Get spatial map information response

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the requested spatial map to get information |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Indicates successful get information |
| > Spatial map information | O | Information on the requested spatial map |
| >> Spatial map coverage space | O | Three-dimensional space coordinates information which defines the spatial map |
| >> Access control rules | O | Access control rules defining which entities are permitted to discover and access the spatial map |
| >> Spatial map layering information | O | Layering information for multi-layered spatial map |
| >>> List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >>>> Spatial map layer information | O | A container to specify the corresponding spatial map layer specific information |
| >>> Relationship between layers | O | A container to specify the relationship between layers |
| >> Spatial map specific information | O | A container to carry the corresponding spatial map specific information |
| > List of objects | O | List of object information. Each element includes the information below. |
| >> Object ID | M | ID of the corresponding object |
| >> Object type | M | Type of object (stationary or moving) |
| >> Object specific information | O | A container to carry the corresponding object specific attribute information |
| Failure response | O (NOTE) | Indicates failed get information |
| > Cause | O | Cause of the failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE:	Further elements of the spatial map information can be defined.

9.3.4	Update spatial map

Figure 9.3.4-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server updates the spatial map as requested.

Figure 9.3.4-1: Update spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to update a spatial map. The request includes requestor ID, security credentials, spatial map ID, information on what to update such as updated spatial map layering information parameters, access control rules and updated spatial map coverage area, etc.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client) and validates the request. The SEAL SM server updates the spatial map as requested.

3)	The SEAL SM server sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID and a list of spatial map layers and its corresponding layer IDs.

9.3.4.1	Information flows for Update spatial map

Table 9.3.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SEAL SM server for update spatial map request.

Table 9.3.4.1-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information on requesting VAL application service entity |
| > Application service ID | M | ID of the requesting VAL application service |
| > Service endpoint | M | Endpoint information of the application service to communicate with (e.g., URI, FQDN, IP address) |
| > Service connection information | O | Information for establishing connectivity to the application service (e.g., DNN, DNAI(s)) |
| Spatial map ID | M | ID of the spatial map to update |
| Requested update information | M | Information on what to update, e.g., access control rules or spatial map coverage area, etc. |
| > Spatial map coverage space | O (NOTE) | Three-dimensional space coordinates information which defines the spatial map |
| > Access control rules | O (NOTE) | Access control rules defining which entities are permitted to discover and access the spatial map |
| > Spatial map layering information | O (NOTE) | Layering information for multi-layered spatial map |
| >> List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >>> spatial map layer information | O | A container to specify the corresponding spatial map layer specific information. |
| >> relationship between layers | O | A container to specify the relationship between layers |
| > Spatial map specific information | O (NOTE) | A container to specify the corresponding spatial map specific information |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE:	Further elements of the requested update information can be defined.

Table 9.3.4.1-2 descibes the information elements from the SEAL SM server to the VAL server (or SEAL SM client) for update spatial map response.

Table 9.3.4.1-2: Update spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the requested spatial map to get information |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Indicates successful update |
| > Spatial map information | O | Updated spatial map information |
| >> Spatial map coverage space | O | Updated three-dimensional space coordinates information which defines the spatial map |
| >> Access control rules | O | Updated access control rules defining which entities are permitted to discover and access the spatial map |
| >> Spatial map layering information | O | Updated layering information for multi-layered spatial map |
| >>> List of spatial map layers | O | Updated list of spatial map layer information. Each element includes the information described below. |
| >>>> spatial map layer information | O | A container to carry the updated corresponding spatial map layer specific information |
| >>> relationship between layers | O | A container to specify the updated relationship between layers |
| >> Spatial map specific information | O | A container to carry the updated corresponding spatial map specific information |
| Failure response | O (NOTE) | Indicates failed update |
| > Cause | O | Cause of failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE:	Further elements of the spatial map information can be defined.

9.3.5	Delete spatial map

Figure 9.3.5-1 depicts the procedure to delete the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server deletes the spatial map as requested.

Figure 9.3.5-1: Delete spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to delete the spatial map. The request includes requestor ID, security credentials, a spatial map ID to delete and optionally the reason to delete.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client) and validates the request. The SEAL SM server deletes the spatial map as requested.

3)	The SEAL SM server sends response message to the requestor with the result of success or failure.

9.3.5.1	Information flows for Delete spatial map

Table 9.3.5.1-1 descibes the information elements from VAL server (or SEAL SM client) to SEAL SM server for delete spatial map request.

Table 9.3.5.1-1: Delete spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | Spatial map ID to delete |
| Reason to delete | O | Reason to delete |

Table 9.3.5.1-2 descibes the information elements from SEAL SM server to VAL server (or SEAL SM client) for delete spatial map response.

Table 9.3.5.1-2: Delete spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates successful delete |
| > Spatial map ID | O | Deleted spatial map ID |
| Failure response | O (NOTE) | Indicates failed delete |
| > Cause | O | Cause of failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

9.3.6	Subscribe spatial map event

Figure 9.3.6-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to subscribe the spatial map event. The service is provided by SEAL SM server. Spatial map consumer can get notification information when the the SM server detects subscribed event.

Figure 9.3.6-1: Subscribe spatial map event procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to subscribe spatial map event. The request includes requestor ID, security credentials, a notification endpoint, a spatial map ID, event to subscribe which triggers the notification. The request message may also include a list of spatial map layers with its corresponding layer IDs to subscribe spatial map layering information related events. The requestor (VAL server or SEAL SM client) may subscribe to "changes of objects" events to indicate the SM server to notify the requestor when changes of the objects are detected. The notification triggering criteria for "changes of objects" event may include added or removed objects, position or direction changes of the spatial map objects. The requestor which received “in-progress” indication for the create spatial map response, can subscribe to “completion of creating spatial map” event with assigned spatial map ID received.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then server add the requestor into the subscriber list of the event. Then SM server starts monitoring the events.

3)	The SEAL SM server sends response message with the result. For the successful subscribe request, a subscription ID is included in the message. Otherwise, the response message includes an indication of failure and may include a reason for failure.

9.3.6.1	Information flows for Subscribe spatial map event

Table 9.3.6.1-1 descibes the information elements from the VAL server (or SEAL SM client) to the SEAL SM server for subscribe spatial map event request.

Table 9.3.6.1-1: Subscribe spatial map event request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information on requesting VAL application service entity |
| > Application service ID | M | ID of the requesting VAL application service |
| > Notification target endpoint | M | Endpoint information of the application service for notification (e.g., URI, FQDN, IP address) |
| > Notification connection information | O | Information for establishing connectivity to the application service for notification (e.g., DNN, DNAI(s)) |
| Expiration time | O | Requested expiration time for the subscription |
| Spatial map ID | M | ID of the Spatial map to subscribe |
| Event type | M | Type of event to subscribe. Supported event types includes any changes in: - “Objects” - “Access control rules” - “Spatial map layering information” - “Spatial map status” |
| Triggering criteria | M | Specifies the condition to trigger the notification |
| > Changes in objects | O (NOTE) | Triggers notification if any or specified (or specified type of) objects are added, removed, or changed their position or direction, etc. |
| > Access control rules | O (NOTE) | Triggers notification if any or specified access control rules changed is detected |
| > Spatial map layering information | O (NOTE) | Triggers notification if any or specified spatial map layering information change is detected |
| > Spatial map status | O (NOTE) | Triggers notification if any or specified spatial map status change is detected. This notification triggering criteria can be used to notify the entity which received the “in-progress” create spatial map response and waiting for the completion. |
| > Notification intervals | O | Specifies when to notify the detected event |
| >> promptly | O | Indicates prompt notification |
| >> periodically | O | Indicates periodic notification with intervals specified below. |
| >>> intervals | O | minimum time between consecutive notifications |
| NOTE: At least one of the sub-elements shall be provided. |  |  |

NOTE:	Further elements of event type and triggering criteria can be defined.

Table 9.3.6.1-2 descibes the information elements from the SEAL SM server to the VAL server (or SEAL SM client) for subscribe spatial map response.

Table 9.3.6.1-2: Subscribe spatial map event response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates successful subscription |
| > Subscription ID | O | Allocated ID for the successful subscription to the event |
| Failure response | O (NOTE) | Indicates failed subscription |
| > Cause | O | Cause of failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

9.3.7	Notify spatial map event

Figure 9.3.7-1 depicts the notify spatial map event operation between the authorized spatial map consumer (VAL server or SEAL LM client) and SEAL SM server.

Figure 9.3.7-1: Notify spatial map event

1)	The SEAL SM server detects an event that triggers the notification to the subscribers for the corresponding event. When SM server detected an event, it determines whether the detected event matchs triggering criteria and notifies subscribers accordingly.

2)	The SEAL SM server notifies the VAL servers (or SEAL SM clients) which subscribed the corresponding event. The notification includes a subscription ID and event-specific notification information

9.3.7.1	Information flows for Notify spatial map event

Table 9.3.7.1-1 describes the information flow form SEAL SM server to VAL server (or SEAL SM client) for  spatial map event notification  when a subscribed triggering event is detected.

Table 9.3.7.1-1: Spatial map event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |
| > Spatial map ID | M | ID of the spatial map that detected the triggering event |
| > Event type | O | Type of event which triggered the notification |
| > Triggering condition information | M | Information on what triggered the notification |
| >> Event ID | M | ID of the event detected and trigged the notification |
| >> Changed object | O (NOTE) | Information on the object which triggered the notification |
| >>> Object ID | O | ID of the changed object |
| >>> Type of change | O | Specifies the detected type of change for the object (e.g., added, removed, or changed position or direction) |
| >>> Changed object information | O | A container to carry the changed object specific information |
| >> Access control rule information | O (NOTE) | A container to carry the changed access control rules which triggered the notification |
| >> Spatial map layering information | O (NOTE) | A container to carry the changed spatial map layering information which triggers the notification |
| >> Spatial map status information | O (NOTE) | A container to carry the information on changed spatial map status which triggers the notification |
| NOTE: At least one of the sub-elements shall be provided. |  |  |

9.3.8	Unsubscribe spatial map

Figure 9.3.8-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to unsubscribe from the spatial map event. The service is provided by SEAL SM server.

Figure 9.3.8-1: Unsubscribe spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to unsubscribe from spatial map event. The request includes requestor ID, security credentials, spatial map ID and the subscription ID for the event.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then SM server removes the requestor’s subscription ID from the subscriber list for the spatial map event.

3)	The SEAL SM server sends unsubscribe spatial map event response message with the result. If successful, the response includes an indication of success. Otherwise, the response includes an indication of failure and may include a reason for failure.

9.3.8.1	Information flows for Unsubscribe spatial map

Table 9.3.8.1-1 descibes the information elements from VAL server (or SEAL SM client) to SEAL SM server for unsubscribe spatial map request.

Table 9.3.8.1-1: Unsubscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information on requesting VAL application service entity |
| > Application service ID | M | ID of the requesting VAL application service |
| > Service endpoint | M | Endpoint information of the application service to communicate with (e.g., URI, FQDN, IP address) |
| > Service connection information | O | Information for establishing connectivity to the application service (e.g., DNN, DNAI(s)) |
| Subscription ID | M | Subscription ID to unsubscribe, which was allocated by the SEAL SM server when the requestor subscribed the event |

Table 9.3.8.1-2 descibes the information elements from the SEAL SM server to the VAL server (or SEAL SM client) for unsubscribe spatial map response.

Table 9.3.8.1-2: Unsubscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates successful unsubscription |
| > Subscription ID | O | Unsubscribed subscription ID |
| Failure response | O (NOTE) | Indicates failed unsubscription |
| > Cause | O | Cause of failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

9.3.x	<Procedure Name>

Editor’s Note: This clause provides procedure for Spatial Map (SM) management.

9.3.x.1	Information flows for <Procedure Name>

Editor’s Note: This clause provides information flows for Spatial Map (SM) management.

9.3.x.2	Services

Editor’s Note: This clause provides information flows for Spatial Map (SM) management.

* * * Next Change * * * *

9.x	SEAL APIs for spatial map management

9.x.1	General

Table 9.x.1-1 illustrates the SEAL APIs for spatial map management.

Table 9.x.1-1: List of SEAL APIs for spatial map management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SmManagement | Create | VAL server, SEAL SM client | Request/Response |
|  | Discover | VAL server, SEAL SM client | Request/Response |
|  | Get | VAL server, SEAL SM client | Request/Response |
|  | Update | VAL server, SEAL SM client | Request/Response |
|  | Delete | VAL server, SEAL SM client | Request/Response |
|  | Subscribe | VAL server, SEAL SM client | Request/Response |
|  | Notify | VAL server, SEAL SM client | Notify |
|  | Unsubscribe | VAL server, SEAL SM client | Request/Response |

9.x.1	SS_SmManagement API

9.x.1.1	General

Service description: This API enables the VAL server or SEAL SM client to manage spatial maps on SM server.

9.x.1.2	Create operation

API operation name: SS_SmManagement_Create

Description: Create a spatial map in the given three-dimensional area of interest.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer Table 9.3.1.1-1

Outputs: Refer Table 9.3.1.1-2

See subclause 9.3.1 for the details of usage of this API operation.

9.x.1.3	Discover operation

API operation name: SS_SmManagement_Discover

Description: Discover a spatial map in the given three-dimensional area of interest.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer Table 9.3.2.1-1

Outputs: Refer Table 9.3.1.2-2

See subclause 9.3.2 for the details of usage of this API operation.

9.x.1.4	Get operation

API operation name: SS_SmManagement_Get

Description: Provide requested spatial map information to the authorized consumers.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer Table 9.3.3.1-1

Outputs: Refer Table 9.3.3.1-2

See subclause 9.3.3 for the details of usage of this API operation.

9.x.1.5	Update operation

API operation name: SS_SmManagement_Update

Description: Update the information on a spatial map as requested.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer Table 9.3.4.1-1

Outputs: Refer subclause 9.3.4.1-2

See subclause 9.3.4 for the details of usage of this API operation.

9.x.1.6	Delete operation

API operation name: SS_SmManagement_Delete

Description: Delete spatial map.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer Table 9.3.5.1-1

Outputs: Refer Table 9.3.5.1

See subclause 9.3.5 for the details of usage of this API operation.

9.x.1.7	Subscribe operation

API operation name: SS_SmManagement_Subscribe

Description: Process the authorized consumer’s subscription to the spatial map events.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer Table 9.3.6.1-1

Outputs: Refer Table 9.3.6.1-2

See subclause 9.3.6 for the details of usage of this API operation.

9.x.1.8	Notify operation

API operation name: SS_SmManagement_Notify

Description: Notify the information on the detected triggering events to which the authorized consumers are subscribed.

Known Consumers: VAL server, SEAL SM client.

Inputs: None

Outputs: Refer Table 9.3.7.1-1

See subclause 9.3.7 for the details of usage of this API operation.

9.x.1.9	Unsubscribe operation

API operation name: SS_SmManagement_Unsubscribe

Description: Revoke the authorized consumer’s subscription to the spatial map events.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer Table 9.3.8.1-1

Outputs: Refer Table 9.3.8.1

See subclause 9.3.8 for the details of usage of this API operation.

* * * End of Change * * * *