3GPP TSG-SA WG6 Meeting #63	S6-244178

Hyderabad, India, 14th – 18th Oct 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0329 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location services for multiple UEs that sharing the same location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-72 v1.2.0, the KI#6 for Supporting of location services for multiple USIMs/UEs sharing the same location has been concluded as follows: Solution #10 addressing the KI#6 can be considered in the normative work. The related function and procedure will be specified in 3GPP TS 23.434 to support the solution. And the detailed APIs and information flows can be discussed in the normative phase. So it’s needed to specify the functions, procedures and information flows for the KI#6 and Sol#10 in TR 23.434 for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add functions of LM server identifying multiple UEs that sharing the same location. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the solution can’t be performed for multiple UEs, the energy and power consumption for these UEs will increase greatly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.15,9.3.x.1(new),9.3.x.2(new),9.3.x.3(new),9.3.2.23, 9.3.2.36, 9.3.2.a(new), 9.3.2.b(new), 9.3.2.c(new), 9.3.2.d(new), 9.3.2.e(new), 9.3.2.f(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.15	Location service registration procedure

Before the Location Management Server requesting the location information for the target UE, the Location Management Client may register the available location services to the LM Server to report the UE's location capabilities.

Figure 9.3.15-1 illustrates the procedure of client-triggered location service registration.

Figure 9.3.15-1: Location service registration procedure

1.	The LM Client of a VAL UE sends location service registration request to the LM Server, with the identifier of the UE (e.g. GPSI), UE-based location capabilities, the associated ID with other UEs, the type of UE (e.g. IoT), the sharing location indicator, etc. The sharing location indicator indicates the associated UEs are allowed to share the same location.

2.	The LM Server checks authorization for the VAL UE's registration request.

3.	After successful authorization, LM Server sends location service registration response to the LM Client and stores the UE’s information received in step 1.

* * * Next Change * * * *

9.3.x	Optimization location service for multiple UEs that sharing the same location

9.3.x.1	General

If multiple VAL UEs share the same location area with each other, when the VAL server requests the location information for each UE among them, the LM Server will select one or several of them to obtain the location data instead of triggering all of UE's positioning procedure, which could reduce the signaling messages, save energy and power consumption.

9.3.x.2	Procedure of LM Server identifying if the UEs sharing the same location

Figure 9.3.x.2-1 describes the procedure that how LM server identifies the UEs that sharing the same location.

Pre-condition:

-	The VAL UE1 and VAL UE2 are associated and may belong to the same user or different users.

-	The LM Client 1(in VAL UE1) and LM Client 2(in VAL UE2) have reported the location capabilities (e.g. associated ID, the sharing location indicator) for VAL UE1 and VAL UE2 to LM server respectively as defined in clause 9.3.15.

-	The LM Client 1 (in VAL UE1) can connect to the LM Client 2 (in VAL UE2) to receive the offline location reports.

NOTE 1:	Both VAL UE1 and VAL UE2 are connected to the access network and able to send individual location report to LM Server. The offline reports is used here to determine whether two UEs are within proximity of each other or not.

NOTE 2:	The associated ID for VAL UE1 and VAL UE2 is same.

Figure 9.3.x.2-1: Procedure of LM Server identifying the UEs that sharing the same location

- The LM Server identifies the registered LM Client1 and LM Client 2 are associated and may are allowed to share the same location with each other based on the received registration information (e.g. same associated ID and the sharing location indicator as defined in clause 9.3.15). Then it verifies if they are sharing the same location at present. There are two options can be performed as following:

-	Option 1: the LM Client reports the enable/disable location reuse confirmation to the LM Server as specified in option A1(steps 2 to 5) and option A2(steps 9 to 12)

-	Option 2: the LM Server triggers the location verification request to the LM Client directly as specified in option B(steps 6 to 8).

NOTE 1:	It is up to the SEAL LM service provider configuration to decide which mechanism is required for option A and option B.

2.	The LM Client 1 receives off-network location report from the LM Client 2. Based on the offline location report, if UE2 is within certain range of UE1, the LM Client 1 determines that the UE2 is within allowed proximity range of the UE1 (i.e. UE1 and UE2 are close enough and UE1's location can be reused by LM Server instead of obtaining UE2's location).

3.	The LM Client 1 sends request message to the LM Server to enable reuse of location of UE1 for UE2. The request message includes user’s identity for UE1 and UE2, and the latest location report of UE1.

4.	The LM Server authorizes the LM Client 1 request. If authorized, the LM Server enables reuse of location of UE1 for UE2.

5.	The LM Server sends enable location reuse response to the LM Client 1.

6.  The LM Server initiates the verify location sharing request to the LM Client 1 with the user ID of LM Client 2 to verify if they are sharing the same location.

NOTE 2:	The step 6 may occur after the LM Server receives the location request to the LM Client 2.

7.  The LM Client 1 checks if the LM Client 2 is close enough and share the same location via the following ways.

-	The LM Client 1 may trigger the location reporting to the LM Client 2 via some positioning capabilities (e.g., Prose) and receive off-network location report from the LM Client 2 and determines that the VAL UE2 is within allowed proximity range of the VAL UE1.

-	The LM Client 1 may ask the VAL user to check if VAL UE1 and VAL UE2 are in the same location. If the answer is yes, the VAL user may inform the LM Client 1 that the VAL UE1 and VAL UE2 are in the same location area.

8.  The LM Client 1 sends the verify location sharing response to the LM Server with the confirmation of sharing same location with LM Client 2, and the validity timer is also included to indicate the valid duration for the confirmation .

If the response is negative (e.g. the LM Client 1 confirms the LM Client 2 is not close with LM Client 1 or the LM Client 1 can’t confirm whether both of UEs are closing enough), the LM Server will not reuse the stored UE1’s location for UE2.

9.	The LM Client 1 receives off-network location report from the LM Client 2. Based on location report, if UE2 is out of certain range of UE1, the LM Client 1 determines that the UE2 is outside allowed proximity range of the UE1 (i.e. UE1 and UE2 are not close enough and UE1's location cannot be used by LM Server for UE-2's location).

10.	The LM Client 1 sends request message to LM Server to disable reuse of location of UE1 for UE2. The request message includes user's identity for UE1 and UE-2, and the latest location report of UE1.

11.	The LM Server authorizes the LM Client 1’s request. If authorized, the LM Server disables reuse of location of UE1 for UE2.

12.	The LM Server sends disable location reuse response to the LM Client 1.

9.3.x.3	Procedure of LM Server reusing the existing UE’s location for other UE’s location requests

Figure 9.3.x.3-1 describes the procedure that LM server reuses the location of UE1 for UE2 if they are sharing the same location.

Pre-condition:

-	The LM Server has confirmed the VAL UE1 and VAL UE2 are associated and sharing the same location as specified in clause 9.3.x.2.

Figure 9.3.x.3-1: Procedure of LM Server reusing the existing UE’s location for other UE’s location requests

1.  The VAL server 1 sends a location information request to the LM Server to request the location data for the UE1 as defined in clause of 9.3.4 of 3GPP TS 23.434 [9]. The enhancement is the request includes the associated ID with UE2.

2.	The LM Server sends a location information request to the LM Client 1.

3.	The LM Client 1 responds to the LM Server with a location report of UE1.

4.	The LM Server stores or updates the received location report of UE1.

5.	The LM Server sends the VAL UE1's location report to the VAL Server 1.

6.	Same with Step 1, the VAL server 2 sends a location information request to the LM Server to request the location data for the UE2 including the UE2 ID and the associated ID with UE1.

NOTE 1:	The VAL server 1 and the VAL server 2 may be the same.

7.	As the LM Server has identified that LM Client 1 and LM Client 2 are associated and share the same location as specified in clause of 9.3.x.2, it determines to reuse of location of UE1 for UE2 instead of obtaining the UE 2's location data directly. But if the LM Server doesn’t store the UE1's location data or the stored location data is invalid (e.g. the valid timer for the confirmation of sharing location information is expired, the value of sharing location indicator is false, or the verify location sharing response is negative), the LM Server will ask the UE2 to report the UE location data.

8.	The LM Server sends the VAL UE1's location report to the VAL Server 2.

* * * Next Change * * * *

9.3.2.23	Location service registration request

Table 9.3.2.23-1 describes the information flow from the location management client to the location management server for registration of the location service.

Table 9.3.2.23-1: Location service registration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Location capability | O | The information of location capability of VAL UE for which the location service is registered. |
| > Location access type (NOTE 2) | O (NOTE 1) | Identifies the available location access type of the VAL UE. E.g. as described in TS 23.273[50] and TS 29.572[51]. |
| >Positioning method | O (NOTE 1) | Identifies the available positioning methods of the VAL UE. E.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Associated ID | O | Indicates the ID that VAL UEs who are associated with others. |
| Type of UE | O | Indicates the type of VAL UEs (e.g. IoT). |
| Sharing location indicator | O | Indicates the associated UEs are allowed to share the same location. |
| NOTE 1: At least one of these rows shall be present. NOTE 2: The non-3GPP access as defined in TS 23.273[50] and TS 29.572 [51] is out of scope of the present specification. |  |  |

* * * Next Change * * * *

9.3.2.36	Location service registration update request

Table 9.3.2.36-1 describes the information flow from the location management client to the location management server for update the registered location service.

Table 9.3.2.36-1: Location service registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location service is updated. |
| Location capability | O | The information of location capability of VAL UE for which the location service is updated. |
| >Location access type (NOTE 2) | O (NOTE 1) | Identifies the updated location access type of the VAL UE. E.g. as described in TS 23.273[50] and TS 29.572[51]. |
| >Positioning method | O (NOTE 1) | Identifies the updated positioning methods of the VAL UE. E.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Associated ID | O | Indicates the ID that VAL UEs who are associated with others. |
| Type of UE | O | Indicates the type of VAL UEs (e.g. IoT). |
| Sharing location indicator | O | Indicates the associated UEs are allowed to share the same location. |
| NOTE 1: At least one of these rows shall be present. NOTE 2: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. |  |  |

* * * Next Change * * * *

9.3.2.3	Location information request

Table 9.3.2.3-1 describes the information flow from the VAL server to the location management server and/or from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |
| Requested location information | O | Identifies what location information is requested |
| Requested location access type (NOTE 4) | O (NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Requested positioning method | O (NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Location QoS | O (NOTE 3) | Definition of the location Quality of Service for which the location information is requested (see NOTE 2). |
| Associated ID | O | Indicates the ID that VAL UEs who are associated with others. |
| NOTE 1: This e element is only applicable for request sent from the LM server to the LM client. NOTE 2: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 3: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 4: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. |  |  |

Editor's Note: It's FFS the security aspects for LM-Uu and LM-S in relation to the VAL service ID that need to be coordinated with SA3.

* * * Next Change * * * *

9.3.2.a	Enable location reuse request

Table 9.3.2.a-1 describes the information flow from the location management client to the location management server for enabling to reuse the location information.

Table 9.3.2.a-1: Enable location reuse request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Identity of sharing location | O | Indicates the list of identities of VAL UEs whose location could reuse the requesting VAL UE’s as they are sharing the same location. |
| Location Information | M | Location Information of the requesting VAL UE which could be reused for other UEs. |

* * * Next Change * * * *

9.3.2.b	Enable location reuse response

Table 9.3.2.b-1 describes the information flow from the location management server to the location management client for enabling to reuse the location information response.

Table 9.3.2.b-1: Enable location reuse response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| Request status | M | Indicates the request result. |

* * * Next Change * * * *

9.3.2.c	Disable location reuse request

Table 9.3.2.c-1 describes the information flow from the location management client to the location management server for disabling to reuse the location information.

Table 9.3.2.c-1: Disable location reuse request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Identity of sharing location | O | Indicates the list of identities of VAL UEs whose location cannot reuse the requesting VAL UE’s as they are not sharing the same location. |
| Location Information | M | Location Information of the requesting VAL UE. |

* * * Next Change * * * *

9.3.2.d	Disable location reuse response

Table 9.3.2.d-1 describes the information flow from the location management client to the location management server for disabling to reuse the location information response.

Table 9.3.2.d-1: Disable location reuse response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| Request status | M | Indicates the request result. |

* * * Next Change * * * *

9.3.2.e	Verify location sharing request

Table 9.3.2.e-1 describes the information flow from the location management server to the location management client to verify if it is sharing the same location with other UEs.

Table 9.3.2.e-1: Verify location sharing request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requested VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Identity of sharing location | O | Indicates the list of identities of VAL UEs who share the same location with the requested VAL UE. |

* * * Next Change * * * *

9.3.2.f	Verify location sharing response

Table 9.3.2.f-1 describes the information flow from the location management client to the location management server to respond if it is sharing the same location with other UEs.

Table 9.3.2.f-1: Verify location sharing response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| Request status | M | Indicates the request result. |

* * * End of Changes * * * *