3GPP TSG-SA WG6 Meeting #63	S6-244182

Hyderabad, India, 14th – 18th Oct 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0093 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support the satellite S&F operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-01 v2.0.0, the KI#7 of Usage of S&F events information for the application enablement has been concluded as follows: The study concludes with following solution considerations for the normative work: - Solution #AE4 and - Solution #AE8. So it’s needed to specify the functions, procedures and information flows for the KI#7 and this CR focuses on the Sol#AE4 and implements it in TS 23.433 for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedures and information flows for SEALDD server to perform the satellite S&F operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IoT service will be impacted when it is using the satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x.1(new) , 9.x.2(new)，9.x.3(new)，9.x.3.1(new), 9.x.3.2(new),9.x.4(new), 9.x.4.1(new), 9.x.4.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.x	IoT service with SEALDD over Satellite S&F operation

9.x.1	General

This following sub-clauses shows the procedure and information flow for SEALDD server performing the satellite S&F operations to minimize the service impact when there is DL traffic from the IoT application server.

9.x.2	SEALDD Server performing the satellite S&F operations procedure

Pre-condition:

-	The SEALDD server and the IoT VAL server all are deployed on the ground.

-	The PGW-U in the 3GPP CN is deployed on the satellite.

-	SEALDD enabled regular data transmission connection has been established.

Figure 9.x.2-1: SEALDD Server performing the satellite S&F operations procedure

1.	If the IoT application server doesn’t subscribe the satellite S&F events or doesn’t support to adjust the application behaviours as described in clause 14.3.x.2 of 3GPP TS 23.434 [4], the IoT application server may send the DL traffic data to the SEALDD server even when e.g., the feeder link is not available after the SEALDD enabled regular data transmission connection has been established as defined in clause 9.2.2.2.

2.	The SEALDD server interacts with the 3GPP CN to obtain the satellite S&F events information as specified in clause 8.2 of 3GPP TR 23.700-29 [x] and further decides to takes the following corresponding policies and actions based on the different status of events.

3.	The SEALDD server notifies the DL traffic data transmission policies to the VAL server based on the obtained satellite S&F events in step 2.

3a.	The SEAL server may send “Pending DL data” request to the VAL server when e.g. the UE is in S&F mode, with a proper reason, and the pending timer indicating the DL data pending duration may also be included.

If the Feeder Link is available, the SEALDD server transmits the DL data towards the 3GPP network. At the same time, the SEALDD server starts an estimated delivery timer based on the exposed estimated delivery time from the 3GPP core network as specified in clause 8.2 of 3GPP TR 23.700-29 [x]. The estimated delivery timer is greater than or equal to the estimated delivery time.

3b.	The SEALDD server may send “Reject DL data” request to the VAL server with the failure reason (e.g. the UE is in S&F mode).

3c.	The SEALDD server may send “Forward DL data” request to the VAL server with the forwarding priories (e.g. when the DL data volume exceeds the maximum S&F data storage quota, only higher priorities services will be forwarded, such as the emergency service).

3d.	The SEAL server may send “Adjust DL data volume” request to the VAL server with the adjusted traffic volume which is smaller than the maximum S&F data storage quota (e.g. when the DL data volume exceeds the maximum S&F data storage quota, the smaller traffic volume will be suggested). If the estimated delivery timer expires, the SEALDD server calculates the new DL data volume (e.g., increasing the volume by the data size delivered with the previous estimated delivery time), and informs the VAL server the adjusted DL data volume.

* * * Next Change * * * *

9.x.3	Information flows

9.x.3.1	Downlink traffic data transmission policy for satellite S&F operation request

Table 9.x.3.1-1 describes the information flow from the SEALDD server to the VAL server for notifying the downlink data traffic transmission policy for satellite S&F operation.

Table 9.x.3.1-1: Downlink traffic data transmission policy for satellite S&F operation request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| Identity | M | Identifier of the VAL UE or VAL user for which SEALDD policy applies |
| Downlink traffic data transmission policy | O (See NOTE 1) | The SEALDD transmission policy associated with the downlink application traffic over satellite S&F operation. |
| > Pending DL data | O | Indicates to pend the DL traffic. |
| >> Pending reason | O | Indicates the pending reason for the DL traffic. |
| >> Pending timer | O | Indicates the pending timer for the DL traffic. |
| > Reject DL data | O | Indicates to reject the DL traffic. |
| >>Reject reason | O | Indicates the failure reason for rejecting the DL traffic. |
| > Forward DL data | O | Indicates to forward the DL traffic data with the priority. |
| >> Forwarding priority | O | Indicates the forwarding priority for the DL traffic when forwarding the DL traffic. |
| > Adjust DL data volume | O | Indicates to increase or reduce the DL traffic. |
| >> Increase DL data volume | O | Indicates to increase the DL volume e.g. by the data size delivered with the previous estimated delivery time. |
| >> Reduce DL data volume | O | Indicates to reduce the DL traffic data volume e.g. smaller than the maximum S&F data storage quota. |
| NOTE 1: This IE is only used when the UE is accessing the satellite and starts to perform the satellite S&F operation. |  |  |

9.x.3.2	Downlink traffic data transmission policy for satellite S&F operation response

Table 9.x.3.2-1 describes the information flow from the VAL server to the SEALDD server for responding the downlink data traffic transmission policies.

Table 9.x.3.2-1: Downlink traffic data transmission policy for satellite S&F operation response

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| Result | M | Indicates the request result. |

9.x.4	APIs

9.x.4.1	General

Table 9.x.4.1-1 illustrates the APIs exposed by SEALDD server for SEALDD DL data transmission policy.

Table 9.x.4.1-1: List of SEALDD server APIs for DL data transmission policy configuration

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SEALDD DL transmission policy notify | Notify | Request/Response | VAL server |

9.x.4.2	SEALDD DL transmission policy notify operation

API operation name: Sdd_DL transmission policy_notify

Description: The consumer requests for one time for SEALDD DL transmission policy notification.

Inputs: See clause 9.x.3.1.

Outputs: See clause 9.x.3.2.

See clause 9.x.2 for details of usage of this operation.

* * * End of Changes * * * *