3GPP TSG-SA WG6 Meeting #63	S6-244207

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on DC application and profile update

Spec:	3GPP TS 23.392 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the DC application and profile update procedure.

2. Reason for Change

With the conclusion of FS_eMMTelAPP for KI#7 and KI#2, the MMTel Enabler layer is responsible for the DC application management, the corresponding normative work need to be implementd.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.1.0.

* * * First Change * * * *

### 8.2.2	DC application and profile update

#### 8.2.2.1	General

DC application and profile update enables a Controlling Application Server to update the DC application and profile with the MMTel Enabler Server.

#### 8.2.2.2	Procedure

Figure 8.2.2.2-1 illustrates the DC application and profile update procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.2.2-1: DC application and profile update

1.	The Controlling Application Server sends a DC Application and Profile update request to the MMTel Enabler Server. The request message includes information elements as specified in Clause 8.2.2.3.1.

2.	Upon receiving the request, the MMTel Enabler Server initiates authentication procedures with the Requester Identity and authorizes the Controlling Application Server if needed. If successful, the MMTel Enabler Server generates a new DC Application Profile based on information provided in the request, then delete the old version of the DC Application or Profile in DCAR and send the new DC Application or Profile to DCAR. The northbound interfaces for DC Application management on DCAR is implementation-based.

NOTE:	There is a one-to-one relationship between DC application and DC application profile. DCAR stores the latest version of DC application and DC application profile.

3.	The MMTel Enabler Server sends a DC Application and Profile configuration response to the Controlling Application Server. The response message includes information elements as specified in Clause 8.2.2.3.2.

#### 8.2.2.3	Information flows

##### 8.2.2.3.1	DC Application and profile update request

The elements of the DC Application and profile update request from the Controlling Application Server to the MMTel Enabler server are the same as Clause 8.2.1.3.1.

##### 8.2.2.3.2	DC Application and profile update response

Table 8.2.2.3.2-1 describes information elements for the DC Application and profile update response from the MMTel Enabler Server.

Table 8.2.2.3.2-1: Information elements for DC Application and Profile update response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application and profile update response list | M | A list of Application and profile update response. Each element in this list contains a response for a DC Application and Profile. The detailed information is listed in Table 8.2.2.3.2-2. |

Table 8.2.2.3.2-2: Information elements in DC Application and profile update response list

| Information element | Status | Description |
| --- | --- | --- |
| update result | M | Indicates the success or failure of the update of this DC Application and Profile. |
| Reason | O | Indicates the reason for failure. |

* * * End of Changes * * * *