3GPP TSG-SA WG6 Meeting #63	S6-244262

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0105 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEALDD enabled Bandwidth guarantee transmission connection establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the data transmission connection establishment, in order to provide friendly API to the VAL server, the paper propose to split the data transmission connection establishment procedure into Bandwidth guarantee data transmission connection establishment, Reliability guarantee data transmission connection establishment, Critical frame guarantee data transmission connection establishment. In this paper, the data transmission connection establishment is modified into Bandwidth guarantee data transmission connection establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add clarification on three types of data transmission connection establishment - Change the data transmission connection establishment into Bandwidth guarantee signalling transmission connection establishmen - Make adaptively adjustment on message and information flow |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is difficult for VAL server to invoke SEALDD functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.2, 8.2.11, 8.8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.2.1	General

The following clauses specify procedures, information flow and APIs for establishing an SEALDD enabled end-to-end connection between VAL client and VAL server. The end-to-end connection (also termed SEALDD-UU flow) is uniquely identified in the SEALDD layer by the SEALDD-UU flow ID. The specific procedures detailed in the subsequent clauses are for cases in which the SEALDD regular connection is used respectively for application signalling, application data delivery initiated by VAL server, and application data delivery initiated based on DD policy.

As for the data transmission connection establishment, the SEALDD enabler layer can determine to establish different data transmission connection (i.e. Bandwidth guarantee, Reliability guarantee, Critical frame guarantee) for the VAL server based on the data transmission requirement.

NOTE:	SEALDD server and VAL server may have different behaviour when establishing the connection for signalling transmission and regular data transmission. For signalling transmission, the VAL server may allocate the same address and port to send/receive the signalling traffic of all the users. For data transmission, the VAL server may allocate different addresses and ports to send/receive the data traffic of different users. And SEALDD server may need to identify the data traffic by checking the SEALDD connection establishment request since different SEALDD clients’ application data traffic should be mapped to their specific SEALDD-S connection.

9.2.2.2	SEALDD enabled Bandwidth guarantee data transmission connection establishment procedure

Figure 9.2.2.2-1 illustrate the procedure for establishing Bandwidth guarantee SEALDD data transmission connection.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 9.2.2.2-1: SEALDD enabled Bandwidth guarantee data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for application traffic transfer and allocates address as SEALDD-S connection information for receiving the data packets from SEALDD server. The VAL server sends SEALDD enabled Bandwidth guarantee data transmission connection establishment request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S connection information of the VAL server side, and optionally, the application data transmission requirement for the application traffic.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address of the SEALDD server to receive the packets from the VAL server for application data transfer as SEALDD-S connection information of the SEALDD server side. The SEALDD server allocates a specific address used for SEALDD-S application data traffic transfer with the specific UE for the VAL server and responds with a SEALDD enabled Bandwidth guarantee data transmission connection response (including SEALDD-S connection information of the SEALDD server side). The VAL server and SEALDD server can use SEALDD-S connection information to establish the Bandwidth guarante data transmission connection between VAL server and SEALDD server for application data transfer.

The SEALDD server determines the QoS requirement based on the application data transmission requirement provided by VAL server. Then, the

SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the address or ports allocated by SEALDD server, and the QoS information for application traffic. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4].

NOTE 1:	The SEALDD-S data transmission connection information of the SEALDD server side is optional to respond to the VAL server, if the SEALDD server uses the downlink pull mode to obtain the data/content from the address provided by the VAL server in step 1, and uses the uplink push mode to send the data/content to the address provided by VAL server.

3.	Data transmission session information is provisioned to the VAL client by the VAL server via application signalling.

NOTE 2:	The application signalling may be transmitted via direct application layer connection or via the SEALDD layer.

4.	The VAL client sends a SEALDD service request to SEALDD client. The VAL client receives a SEALDD service response to the SEALDD client. The response indicates that whether the SEALDD service request is successful or not.

5.	The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client can get the SEALDD server's address.

6.	The SEALDD client allocates SEALDD-UU flow ID(s) mapping to the identifiers of the application traffic. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD-UU flow ID(s), the SEALDD traffic descriptor of the SEALDD client side (the address/port of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID. The request message also contains the selected VAL server endpoint information and UE ID. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the data connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the data connection establishment, otherwise SEALDD server returns a failed result e.g. performs connection reject.

NOTE 3: The SEALDD server can use or update the association between SEALDD-UU connection and SEALDD-S connection that associated with UE ID, VAL service ID, VAL server endpoint, which is used to correlate the SEALDD traffic and the VAL application traffic.

NOTE 4:	The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic of the same SEALDD client. The SEALDD-UU flow ID may be same with the identifiers of the application traffic or new simplified IDs allocated by SEALDD.

7.	The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. address, transport layer protocol) mapping to the application traffic.

8.	If the connection between VAL server and SEALDD server is not established in step 2, the SEALDD server establishes connection with VAL server for the VAL client to transmit application traffic mapping to the SEALDD-S traffic according to the SEALDD-S information negotiated in step 1-2.

9.	The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD connection establishment.

After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the UE's address/port and SEALDD server's address/port for the SEALDD traffic transmission).

After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between application traffic and SEALDD-UU flow ID. The SEALDD server gets the mapping information between the SEALDD-UU flow ID and the SEALDD-S connection. Upon receiving application traffic from VAL client, the SEALDD maps it to SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 6 and step 7. The SEALDD traffic is sent to the SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID and SEALDD-UU flow ID. The SEALDD server sends the recovered application traffic to the address provided by VAL server in step 1, via the connection established in step 2 or 8 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly.

The SEALDD server receives any UE location change notification using SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12, then the SEALDD server performs the data delivery in alignment with the geofence policy. If the UE is in the forbidden location or not allowed for the given VAL service to send/receive data as per the Geofence policy, then the SEALDD server performs action like releases the connection and informs the VAL server that UE is not reachable because in a forbidden location using connection event status procedure. If UE enters the allowed location area then the SEALDD server initiates the connection establishment using the procedure defined in clause 9.2.2.3.

9.2.3.X	SEALDD enabled Bandwidth guarantee data transmission connection establishment request

Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service.

Table 9.2.3.1-1: SEALDD enabled Bandwidth guarantee data transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| UEID/ VAL user ID | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g. IP address and/or port, URL) of VAL server to receive the signalling traffic from the VAL server |
| Data transmission requirement | M | The data transmission requirement |
| > VAL server’s total bandwidth limit | M (See NOTE) | The total bandwidth limit of VAL server, including UL/DL |
| > VAL users’ bandwidth limit | M (See NOTE) | The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL |
| > Requested Priority | O | The priorigy requested by the VAL server |
| > Maximum Burst Size | O | The Maximum Burst Size provided by the VAL server |
| > Requested 5GS Delay | O | The Requested 5GS Delay provided by the VAL server |
| > Requested Maximum Bitrate | O | The Requested Maximum Bitrate provided by the VAL server |
| > Requested Guaranteed Bitrate | O | The Requested Guaranteed Bitrate provided by the VAL server |
| > Requested Packet Error Rate | O | The Requested Packet Error Rate provided by the VAL server |
| NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users. |  |  |

9.2.3.Y	SEALDD enabled Bandwidth guarantee data transmission connection establishment response

Table 9.2.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to Bandwidth guarantee application data traffic transmission.

Table 9.2.3.2-1: SEALDD enabled Bandwidth guarantee data transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicate the bandwidth guarantee data transmission connection establish success or failure. |
| SEALDD-S connection information | O | Address information (e.g. IP address and/or port, URL) of SEALDD server to receive the signalling traffic from the SEALDD server |
| Cause | O See NOTE | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

9.2.3.A1	SEALDD data transmission connection establishment initiated by SEALDD client request

Table 9.2.3.A2-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the data transmission connection establishment.

Table 9.2.3.A1-1: SEALDD data transmission connection establishment initiated by SEALDD client request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request). |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime, applicable |
| Capability for BAT and periodicity adaptation | O (See NOTE 2) | Indicates BAT and periodicity adaptation capability for SEALDD client. |
| Transmission assistance info | O (See NOTE 2) | Indicates transmission assistance information for uplink SEALDD traffic. It includes BAT, BAT window, periodicity, and periodicity range. |
| L4S feedback capability | O (See NOTE 3) | Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback). |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: If provided, only one of these IEs shall be present in the message. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2. |  |  |

9.2.3.A2	SEALDD data transmission connection establishment initiated by SEALDD client response

Table 9.2.3.A2-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the data SEALDD connection establishment.

Table 9.2.3.4-1: SEALDD regular transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side |
| Pending timer | O (See NOTE 1) | The pending timer to trigger the re-connection from SEALDD server when bandwidth limit check is failed. |
| Suggested traffic transmission bandwidth | O (See NOTE 1) | The suggested traffic transmission bandwidth used by SEALDD server to perform bandwidth control for VAL users, including UL/DL. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users. NOTE 2: This IE is only present if the Result is failure. |  |  |

* * * END of Change * * * *