3GPP TSG-SA WG6 Meeting #63	S6-244268

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0697 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on bundle EAS feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, discovery direct bundle T-EAS(s) procedure is not clear, necessary description on T-EES determination and direct bundle T-EAS determination is not clear in the spefication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification on T-EES determination and direct bundle T-EAS determination |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The discovery direct bundle T-EAS(s) procedure will remain in-consistency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.2, 8.8.3.3, 8.8.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.8.3.2	Discover T-EAS

Figure 8.8.3.2-1 illustrates the procedure for fetching T-EAS information. This procedure may be utilized by a S-EAS, which undertakes the transfer of application context information to a T-EAS directly, or can be invoked by the S-EES itself on deciding to execute ACR.

T-EAS discovery procedure also supports EAS retrieval which enables a S-EAS to obtain T-EAS(s) serving the application group so that the S-EAS can start communication with obtained EAS(s) for EAS content synchronization.

Pre-conditions:

1.	Information related to the EES is available with the S-EAS, if the procedure is triggered by the S-EAS.

Figure 8.8.3.2-1: Discover T-EAS

1a.	The S-EAS sends the EAS discovery request to the S-EES or the S-EES decides to execute the ACR. The EAS discovery request from the S-EAS includes the requestor identifier [EASID] along with the security credentials and includes EAS discovery filter matching its EAS profile. If target DNAI is available at the S-EAS via User Plane Path change event, the S-EAS provides the S-EES with the target DNAI. If target tunnel information is available at the S-EAS via User Plane Path change event, the S-EAS provides the S-EES with the target tunnel information. The S-EAS also includes an EAS service continuity support indicator indicating that the S-EAS decided ACR according to clause 8.8.2.4 is to be used for the ACR. The S-EAS includes the bundle ID and bundle type indicating the proxy bundle case to which the S-EAS belongs to. The request may include prediction expiration time.

The EAS may send EAS discovery request with EAS ID, Application Group ID and EAS content synchronization support, which indicates the request to obtain EAS(s) currently serving the Application Group ID with the requested EAS ID in order to perform EAS content synchronization.

The Application group ID is included in the EAS discovery request if the S-EAS wants to discover a common T-EAS for UE in the group due to UE's movement.

If the EAS discovery request is triggered by S-EAS due to overload or maintenance reason for the Application Group, the S-EAS also includes list of UE IDs in the EAS discovery request.

NOTE 1:	The trigger condition to invoke the Discover T-EAS API is up to application service logic, which is out of scope of this specification.

1b.	The S-EES either receive the target DNAI and/or target tunnel information for T-EES discovery from the step 1a or by the user plane management event notification from the core network.

2.	If the request is received from the S-EAS, the S-EES checks whether the requesting EAS is authorized to perform the discovery operation.

If Application Group ID and EAS content synchronization support in EAS characteristics are received and the ECS-ER is available, the S-EES checks with the ECS-ER with the received EAS ID and Application Group ID and obtains a list of EAS(s) supporting EAS content synchronization and serving the application group for the desired application service identified by the EAS ID as described in clause 8.20. Step 2 to step 4 are skipped.

If the UE location is not known to the S-EES or provided by the S-EAS request, then the S-EES may interact with 3GPP core network to retrieve the UE location. If the S-EES decided to execute the ACR or when the requesting EAS is authorized, the S-EES checks if there exists a T-EAS information (registered or cached) that can satisfy the requesting EAS information, target DNAI, target tunnel information, additional query filters and the Expected AC Service KPIs and the Minimum required AC Service KPIs if received from the EEC during the EAS discovery or from the S-EAS in step 1. In this case, the S-EES may collect Edge load performances from ADAES or OAM to find T-EAS(s) that satisfies the Expected AC service KPIs or the Minimum required AC Service KPIs. The S-EES may determine the use of statistics or prediction for evaluating KPIs based on the situation of the T-EAS discovery. If target tunnel information is received, the S-EES additionally takes the target tunnel information into consideration in identifying T-EAS(s). For instance, the IP address(es) of identified T-EAS(s) needs to be topologically close to the IP address of the target tunnel server. The S-EES also considers resource consumption needs for the list of UEs (e.g. received in step 1a) when identifying T-EAS(s). If Application Group ID is not received and if the S-EES finds the T-EAS(s) in the cached or registered information, the flow either continues with step 5 for the S-EAS triggered discovery or stops for the S-EES decided ACR execution, else the S-EES retrieves the T-EES address from the ECS as specified in clause 8.8.3.3 and continues with step 3.

If UE ID list is available in the S-EES (e.g. received in step 1a or known by the S-EES via other means):

-	for UE(s) not in the overlapping EDN area (e.g. UE is not in the common S-EAS service area), the S-EES tries to identify a T-EAS within locally registered EAS(s) based on UE location, EAS discovery filters and application group profile.

-	for UE(s) in the overlapping EDN area, the S-EES checks if there is available common S-EAS in each EDN which has overlapping EDN area with the current UE related EDN by using procedure described in clause 8.20.2.2 when common EAS information is available at S-EES or ECS-ER. If any common EAS is found, the S-EES decides whether to select an available common EAS or locally registered T-EAS as common EAS considering UE location, EAS discovery filters, application group profile, and E2E response time and continues with step 5; otherwise, the S-EES tries to identify a T-EAS within locally registered EAS(s).

Updating common EAS with registered T-EAS in the ECS-ER is needed as described in clause 8.20.2.5 when locally registered T-EAS is selected by the S-EES. If no registered T-EAS can be found, the S-EES continues with step 5.

If Application group ID is available in the S-EES (e.g. received in step 1a or ACR management event subscription), the S-EES retrieves T-EES information and corresponding EDN connection information from the ECS as described in clause 8.8.3.3.

If Prediction expiration time is provided then the EES may determine whether to identify the instantiable but not instantiated EAS as T-EAS based on Prediction expiration time and the predicted EAS deployment time information obtained from ADAES as specified in clause 8.11 of TS 23.436 [28] or from local configured maximum EAS deployment time.

3.	The S-EES invokes the EAS discovery request on the T-EES retrieved from the ECS. The EAS discovery request includes the requestor identifier [EESID] along with the security credentials and includes EAS discovery filter. In the EAS discovery filter, the S-EES may include prediction expiration time, the Expected AC Service KPIs and the Minimum required AC Service KPIs if received from the EEC during the EAS discovery or from the S-EAS in step 1. If target tunnel information is available at the S-EES (e.g. received from S-EAS in step 1a or by the user plane management event notification from the core network), the S-EES provides the T-EES with the target tunnel information.

The S-EES also includes the EEC service continuity support indicator received from the EEC during EAS discovery. If in step 1 the S-EES received an EAS service continuity support indicator from the S-EAS, then the S-EES includes this EAS service continuity support indicator and its own EES service continuity support indicator indicating the ACR scenarios supported by the EES. If in step 1 the S-EES decided to execute the ACR, the S-EES includes the EAS service continuity support indicator received from the S-EAS during EAS registration and includes an EES service continuity support indicator indicating that the S-EES executed ACR according to clause 8.8.2.5 is to be used for the ACR.

Upon receiving the request, the T-EES may trigger the ECSP management system to instantiate the T-EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12.

4.	The T-EES discovers the T-EAS(s) and responds with the discovered T-EAS information to the S-EES. To filter T-EAS(s), the T-EES utilizes the discovery filters (e.g. Expected AC Service KPIs and the Minimum required AC Service KPIs) and the indications which ACR scenarios are supported by the AC, the EEC, the T-EES and the S-EAS. If T-EES gets the Expected AC service KPIs or the Minimum required AC Service KPIs, the T-EES may collect edge load analytics from ADAES (as specified in clause 8.8.2 of TS 23.436 [28]) or performance data from OAM to find T-EAS(s) that satisfies the Expected AC service KPIs or the Minimum required AC Service KPIs. The T-EES may determine the use of statistics or prediction for evaluating KPIs based on the situation of the T-EAS discovery. The S-EES may cache the T-EAS information.

When the T-EES accepts the request and determines to trigger the EAS instantiation, then the response may indicate that the EAS instantiation is in progress so that the detailed EAS profile information will be available later. When S-EES receives the EAS instantiation in progress indication, the S-EES may send EAS discovery subscription request message if not subscribed yet or send EAS discovery request message later to the T-EES for obtaining updated EAS information.

When the bundle EAS information (i.e. list of EASID) is provided and the bundle type indicating the direct bundle, and the S-EES received associated T-EES(s) along with part of EAS ID list in the step2 from the ECS, then the S-EES discover the target direct bundle EAS(s) which belongs to same EDN for all the associated S-EES(s). The request message contains direct bundle EAS(s) information (i.e. list of EASID and direct bundle type), then the T-EES determines the direct bundle T-EAS(s) based on the bundle EAS information (e.g. list of EASIDs). Then the S-EES receives the direct bundle T-EAS(s) information from each associated T-EES(s).

NOTE 2:	T-EES(s) may belongs to same EDN.

NOTE 3:	The edge load analytics from ADAES can be either statistics or predictions on the T-EAS.

NOTE 4:	The statistical KPI value can be used for both normal ACR and service continuity planning.

If target tunnel information is received, the T-EES additionally takes the target tunnel information into consideration in identifying T-EAS(s). For instance, the IP address(es) of identified T-EAS(s) needs to be topologically close to the IP address of the target tunnel server.

When the ECS-ER is available and common EAS information corresponding to the Application Group ID (received in step 1a or ACR management event subscription) is not available, then the S-EES identifies one T-EAS for the application group and interacts with the ECS-ER to store the common EAS information as described in clause 8.20.2.3. If common EAS information is already available corresponding to the Application Group ID in the repository, then the ECS-ER returns the common EAS information to the S-EES as described in clause 8.20.2.3.

5.	If the request was received from the S-EAS, the S-EES responds to the S-EAS with the discovered T-EAS Information.

For responding to the S-EAS which is requesting EAS(s) serving the application group for EAS content synchronization purpose, only EAS endpoint and EAS ID are included in EAS profile of Discovered EAS list.

8.8.3.3	Retrieve T-EES procedure

Figure 8.8.3.3-1 illustrates the procedure for the S-EES to retrieve the T-EES information from the ECS. This procedure is also applicable for the CES to retrieve the T-EES information.

Pre-condition:

1.	The S-EES has been pre-configured with the address of the ECS; and

2.	The AC at the UE already has on-going application traffic with the S-EAS.

Figure 8.8.3.3-1: Retrieve T-EES procedure

1.	The S-EES sends the Retrieve EES request (UE location information or UE identity, EASID of the S-EAS, bundle ID, bundle type (i.e. proxy bundle case), target DNAI and UE connectivity information) to the ECS in order to identify the T-EES which has an EAS available to serve the given AC in the UE. For obtaining announcement EES list, the Retrieve EES request includes application group id. The request message may also contain the AC, EEC service continuity support information. If target tunnel information is available at the S-EES, then the S-EES provides also the tunnel information to the ECS.

2.	If the request contains the UE identity (e.g. GPSI) but the UE location is not known to the ECS, then the ECS interacts with 3GPP core network to retrieve the UE location. The ECS determines T-EES(s) as per the parameters (e.g. EASID, target DNAI) in the request and the UE location information. If the request message contains the AC, EEC service continuity support information, then the ECS may identify the T-EES taking the AC, EEC, T-EES service continuity support into consideration. The ECS may take the EAS(s) load information corresponding to the EASIDs registered with EES in the EDN(s) or use ADAES Edge load analytics as specified in clause 8.8.2 of TS 23.436 [27]) to take EDN load into consideration in identifying T-EES(s). When the bundle ID and is provided and bundle type indicating the proxy bundle case then the ECS can identify the T-EES based on the bundle ID in the EES profile and in the request message to ensure T-EAS is able to invoke the required proxy bundle EAS(s) as the S-EAS does.

If the Prediction expiration time is provided then the ECS may determine whether to identify T-EES with the instantiable but not instantiated EAS based on Prediction expiration time and predicted EAS deployment time information obtained from ADAES as specified in clause 8.11 of TS 23.436 [28] or from local configured maximum EAS deployment time.

If no ECS-ER is available and when the Retrieve EES request includes application group id to the ECS then the request EES list retrieval is for the announcement of common EAS, ECS determines the list of EESs serving the EASs (with same EASID) for the Group ID included in the Retrieve EES request.

If the ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE’s location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.17.2.3 or both.

If required by the ECSP policies, the ECS may use service provisioning information retrieval procedure as specified in clause 8.17.2.4 to obtain service provisioning information from the partner ECS.

When the bundle EAS information is provided, then if bundle EAS information includes a list of EASIDs, then the ECS identifies the one or more T-EES associated with the same EDN which support all of the EASs within the same EDN based on the EES EDN information obtained in the EES profile and bundle EAS information (e.g. list of EASIDs).

NOTE 1:	T-EES(s) may belongs to same EDN.

If tunnel information is received, the ECS additionally takes the tunnel information into consideration in identifying EES(s). For instance, the IP address(es) of identified EES(s) needs to be topologically close to the IP address of the tunnel server.

3.	The ECS sends the Retrieve EES response. If the ECS has determined the EDN configuration information, the retrieve EES response includes the list of EDN configuration information to the S-EES. The list of EDN configuration information includes the EDN details with the endpoint information of T-EES(s) as described in table 8.3.3.3.3-2. If the ECS has determined suitable partner ECS(s) instead, the retrieve EES response includes a list of ECS configuration information.

The ECS may provide associated T-EES(s) information (one or more T-EES information) to the S-EES in the Retrieve T-EES response along with the bundle EAS information (i.e. list of EASIDs). When S-EES receives multiple associated T-EES(s), then each associated T-EES information is provided along with the part of the EAS ID list.

If the retrieve EES response contains a list of ECS configuration information, the S-EES may initiate retrieve T-EES procedure with one or more ECS(s) listed in the retrieve EES response.

NOTE 2:	The Retrieve EES request initiated by the S-EES can be restricted only to its registered ECS.

8.8.4.6	Retrieve EES request

Table 8.8.4.6-1 describes the information elements to retrieve T-EES information from the ECS.

Table 8.8.4.6-1: Retrieve EES request

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | Identifier of the requesting EES or CES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | M | The identifier of the EAS. |
| Application Group ID | O | Application group identifier as defined in 7.2.11. Indicates to the ECS that the request is to obtain EES list for the announcement of common EAS |
| Bundle ID (NOTE) | O | A bundle ID as described in clause 7.2.10. |
| List of EASIDs (NOTE 2) | O | List of EASIDs associated with the EAS bundle. |
| > Bundle type | O | Type of the EAS bundle as described in clause 7.2.10 |
| Target DNAI | O | The target DNAI information which can be associated with potential T-EES(s) and/or T-EAS(s). |
| UE Identifier | O | The identifier of the UE (i.e,. GPSI) |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| EEC service continuity support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| Prediction expiration time | O | The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest. |
| AC service continuity support | O | Indicates if the AC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the AC. |
| ENS indication | O | Indicates whether edge node sharing is used. |
| Serving MNO information | O | The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber. |
| Tunnel information | O | It includes service provider ID, the endpoint address (e.g. IP address) of the tunnel server. |
| NOTE: The bundle ID is provided only when bundle type indicates the proxy bundle. |  |  |

* * * END of Change * * * *