3GPP TSG-SA WG6 Meeting #63	S6-244291

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new KI on NRM XR API

Spec:	3GPP TR 23.700-35 v0.0.0

Agenda item:	8.9

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to update the Usage of CAPIF in different phases of an Application

2. Reason for Change

XR is an emerging mobile media service expected to contribute more and more traffics to 5G network. 3GPP specifies several new capabilities to support this feature, including the enhancement of N33 interface towards the AF/AS. However, there are some issues for the 3rd party to use such API due to:

- Currently the N33 APIs are common to all the verticals which cause the input parameters for the N33 APIs are increasing more and more and become excessive complicated.

From the XR service, some parameters introduced by URLLC is not required. Here is the example of AFSessionwithQoS API:

5.2.6.9.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create

Description: The consumer requests the network to provide a specific QoS for an AF session for a UE or a list of UEs.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address and only applicable for a single UE AF session), a list of UE addresses (as described in clause 4.15.6.13, and only applicable for a Multi-member AF session). Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

NOTE 1:	In this Release, when a list of UE addresses is provided, the Flow description information is common for all UE addresses in the list. Further details are described in clause 4.15.6.13.2.

Inputs, Optional: Time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS Monitoring parameter(s) as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available.

Only applicable for a single UE AF session: flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity as described in clause 5.27.2 or clause 5.37.8.2 of TS 23.501 [2], Time domain, Survival Time, BAT Window or Capability for BAT adaptation, Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], Periodicity Range, RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description (as described in clause 5.37.5 or 5.37.8.3 of TS 23.501 [2]), Indication of ECN marking for L4S as described in clause 6.1.3.22 of TS 23.503 [20], QoS duration, QoS inactivity interval, Multi-Modal Service ID together with Multi-modal Service Requirements information for each data flow as described in clause 6.1.3.27.3 of TS 23.503 [20].

Only applicable for a Multi-member AF session: Consolidated Data Rate Threshold, a list of UE addresses subject to Consolidated Data Rate monitoring.

- The XR specific input parameters in the N33 API e.g., AFSessionWithQoS are present for various purposes (or features as defined in 3GPP TS 29.522) , but lack of vertical service applicability instruction.

| AsSessionWithQoS | - The following 5G-only features defined in clause 5.14.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "EthAsSessionQoS_5G", "QoSMonitoring_5G", "PacketDelayFailureReport", "MacAddressRange_5G", "AlternativeQoS_5G", "TSC_5G", "DisableUENotification_5G", "ExposureToEAS", "AltQosWithIndParams_5G", "EnEthAsSessionQoS_5G", "enNB_5G", "AltQoSProfilesSupportReport", "ExtQoS_5G", "EnTSCAC", "L4S", "MultiMedia", "PowerSaving", "EnQoSMon", "PDUSetHandling", "RTLatency", "ToSTC_5G", "QoSTiming_5G", "ListUE_5G" and "GMEC". and "QoSMonCapRepo". - The "LOSS_OF_BEARER", "RECOVERY_OF_BEARER" and "RELEASE_OF_BEARER" events do not apply for 5G. |
| --- | --- |

The all the yellow highlighted features can be used by eXtended Reality (XR) and interactive media services. However, only the "MultiMedia" "PDUSetHandling" and "RTLatency" are explicitly mentioned to be used by eXtended Reality (XR) and interactive media services.

(3) For the unicast resource management of NRM, several unicast resource management procedures and operations are defined. However, there are some issues to well support the XR service:

-	The existing input parameters of the SS_NetworkResourceAdaptation API are not suffcient to support the latest network XR capabilities to be used by the XR service, e.g., lack of the support of PDU set handling.

-	Only one single NRM service API, i.e., SS_NetworkResourceAdaptation API cover all the verticals XR, TSN, TSC and so on, which make it is not friend to the XR service to know how to use it.

So, from the NRM point of view, the NRM shall provide a new XR specific service API.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-35 v0.0.0.

* * * First Change * * * *

X.a	Key issue#a: NRM XR specific service

XR is an emerging mobile media service expected to contribute more and more traffics to 5G network. 3GPP specifies several new capabilities to support this feature, including the enhancement of PCF/N33 APIs towards the AF/AS. However, there are some issues for the 3rd party to use such capability due to the non-friendly API design, such as:

(1)	Various extended parameters confuse the XR service provider which parameter should be filled and what is gained from this parameter.

Currently the PCF/N33 APIs are common to all the verticals and its input parameters for the PCF/N33 APIs are increasing more and more and become excessive complicated. From the XR service point of view, some parameters introduced by URLLC is not required. Such existing APIs fails to facilitate the XR service provider to program the API during their XR application development.

(2)	The XR specific input parameters in the PCF/N33 API e.g., AFSessionWithQoS are present for various purposes (or features as defined in 3GPP TS 29.522) , but lack of vertical service applicability instruction.

The following table show the 5G added features to AsSessionWithQoS.  All the yellow highlighted features can be used by eXtended Reality (XR) and interactive media services. However, only the "MultiMedia" "PDUSetHandling" and "RTLatency" are explicitly mentioned to be used by eXtended Reality (XR) and interactive media services.

| AsSessionWithQoS | - The following 5G-only features defined in clause 5.14.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "EthAsSessionQoS_5G", "QoSMonitoring_5G", "PacketDelayFailureReport", "MacAddressRange_5G", "AlternativeQoS_5G", "TSC_5G", "DisableUENotification_5G", "ExposureToEAS", "AltQosWithIndParams_5G", "EnEthAsSessionQoS_5G", "enNB_5G", "AltQoSProfilesSupportReport", "ExtQoS_5G", "EnTSCAC", "L4S", "MultiMedia", "PowerSaving", "EnQoSMon", "PDUSetHandling", "RTLatency", "ToSTC_5G", "QoSTiming_5G", "ListUE_5G" and "GMEC". and "QoSMonCapRepo". - The "LOSS_OF_BEARER", "RECOVERY_OF_BEARER" and "RELEASE_OF_BEARER" events do not apply for 5G. |
| --- | --- |

(3) For the unicast resource management of NRM, several unicast resource management procedures and operations are defined. However, there are some issues to well support the XR service:

-	The existing input parameters of the SS_NetworkResourceAdaptation API are not suffcient to support the latest network XR capabilities to be used by the XR service, e.g., lack of the support of PDU set handling.

-	Only one single NRM service API, i.e., SS_NetworkResourceAdaptation API cover all the verticals XR, TSN, TSC and so on, which make it is not friendly for the XR service to identify which parameters to be used.

The above issues are caused due to the lack of application - oriented and friendly API design principle including:

-	The granularity of service shall be mapped onto the expected optimization objectives of per vertical application.

-	The API or operation of a service shall be well mapped onto the gain can be achieved via this API/operation towards 3rd party.

So how to design XR specific NRM APIs complied with above principle needs to be studied.