3GPP TSG-SA WG6 Meeting #63	S6-244298

Hyderabad, India, 14th – 18th October 2024	(revision of S6-243202)

Source:	Ericsson

Title:	Support of AIML Services for Assisting Edge Computing

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added descriptions and procedure for support of AIML services for assisting edge computing to TS 23.482.

2. Reason for Change

Edge computing is a distributed computing model that brings computation and data storage closer to the sources of data. Such design can reduce the latency compared to when an application runs on a centralized data centre. Some examples of edge use cases include self-driving cars, autonomous robots, smart equipment data and automated retail, etc. Solution#29 is recommended for normative work on support AIML services for assisting edge computing. See more details in 3GPP TR 23.700-82. This paper proposes to add descriptions and procedure for supporting of AIML services for assisting edge computing to TS 23.482.

3. Conclusions

This paper proposes to add descriptions and procedure for support of AIML services for assisting edge computing to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[X1]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

* * * Next Change * * * *

## 8.x	Support of AIML Services for Assisting Edge Computing

This clause describes procedure for supporting AIML services for assisting edge computing.

### 8.x.1	General

This clause describes the proedure for assisting edge computing by the AIMLE server. A CAS, CES, ECS, EES or EAS (which are defined in 3GPP TS 23.558 [X1]) may have different roles in different types of edge computing, e.g. hiearchitical computing (with one root node (e.g., CAS, EAS), the root has one or more children which also know as sub-root node(s) (e.g., EAS), and multiple leaf nodes (e.g., EES or EAS) which with no children), distributed computing (with one coordination node (e.g., CAS, CES, or ECS) and multiple execution nodes (e.g., EAS)). Here, hierarchical computing represents a computation architecture with multiple computation entities involved and multiple levels of computations for a computation task. An AI/ML task can be seen as a specical computing task, and edge comptuting can be used for completing the AI/ML tasks.

### 8.x.2	Procedures for assisting edge computing

Figure 8.x.2-1 illustrates the procedure for AIMLE server to assist an edge computing process.

Pre-conditions:

-	An AI/ML task be treated as a specical computing task being completed at consumer (e.g., CAS, EAS).

-	The consumer decides its role in the edge computing architecture (e.g., hierarchical computing, distributed computing) for an AI/ML task, and also the type of edge computing operations, based on its local configuration.

-	The consumer decides that assistance from AIMLE server for management and configuration to support the edge computing process is needed, due to lack of capability on decision making for task distribution, time for intermediate output delivery, execution node selection, edge computing preparation.

Figure 8.x.2-1: Procedure for assisting an edge computing process

Figure 8.x.2-1 illustrates the procedure for subscribe/request assistance of edge computing. The corresponding procedure in detail is as follows:

- The consumer (e.g. CAS, EAS) sends assist edge computing request or subscription request to the AIMLE Server for assistance of an edge computing process. The subscription request message includes information as described in Table 8.x.3.1-1, and the request message includes information as described in Table 8.x.3.4-1.

- The AIMLE Server derives information for the request, determines downstream entities and services needed. For example:

- Time window(s) recommendation for computing task distribution if the consumer is a root node in a hierarchical computing process or a coordination node in a distribution computing process.

- Time window(s) recommendation for intermediate output delivery if the consumer is a leaf node in a hierarchical computing process.

- Candidate execution node list provisioning or edge computing preparation status provisioning if the consumer is a coordination node of a distributed computing process.

- The AIMLE Server performs operations according to the determination in step 2 to generate assistance information. For example, the AIMLE Server may subscribe/request analytics from ADAE Server (e.g. edge load analytics, edge computing preparation analyitcs), request assistance information or configurations from 5GC NFs (e.g. Member UE selection), trigger AIMLE client selection if the computing task is an AI/ML task.

- The AIMLE Server sends assist edge computing response or notification to the consumer with the assistance information for the edge computing process. The notification contains the information as described in Table 8.x.3.2-1, and the response message contains the informaiton as described in Table 8.x.3.5-1.

The consumer (e.g. CAS, EAS) may use the assistance information for decision making on its edge computing operations.

Editor’s Note:	The procedures for task distribution, intermediate output delivery, execution node selection, and edge computing preparation status provisioning are FFS.

### 8.x.3	Information flows

#### 8.x.3.1	Assist edge computing subscription request

Table 8.x.3.1-1 shows the subscription request sent by consumer (e.g. CAS, EAS) to AIMLE server for assistance of an edge computing process.

Table 8.x.3.1-1: Assist edge computing subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Role | M | Represents the role of the requestor in an edge computing architecture (e.g. root node, sub-root node or leaf node of a hierarchical computing process, coordination node or execution node of a distributed computing process). |
| Type of operation | M | Represents the type of edge computing operations (e.g. task distribution, intermediate output delivery, execution node selection, edge computing preparation) which is used to indicate the operations that the assistance information be used for. |
| Assistance information type | M | Represents the assistance information type (e.g. time window(s) for computing task distribution and/or intermediate output delivery, candidate execution node list, edge computing preparation status), which is uded to indicate the assistance information needed. |
| Notification settings | O | Settings for how often to send notifications providing the assistance information. For example, periodic, event-based (e.g. based on status changes of the execution node), upon completion of each computing task. |

#### 8.x.3.2	Assist edge computing subscription response

Table 8.x.3.2-1 shows the response sent by AIMLE server to the consumer (e.g. CAS, EAS) for assisting the edge computing process.

Table 8.x.3.2-1: Assist edge computing subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| Subscription ID | M | Subscription identifier corresponding to the subscription only if the status is success. |

#### 8.x.3.3	Assist edge computing subscription response

Table 8.x.3.3-1 shows the notification sent by AIMLE server to the consumer (e.g. CAS, EAS) for assisting the edge computing process.

Table 8.x.3.3-1: Assist edge computing subscription notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor. |
| Assistance informaiton | M | The assistance informaiton for assisting edge computing process (e.g. time window(s) for computing task distribution and/or intermediate output delivery, candidate execution node list, edge computing preparation status). |

#### 8.x.3.4	Assist edge computing subscription request

Table 8.x.3.4-1 shows the request sent by consumer (e.g. CAS, EAS) to AIMLE server for assistance of an edge computing process.

Table 8.x.3.4-1: Assist edge computing request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Role | M | Represents the role of the requestor in an edge computing architecture (e.g. root node, sub-root node or leaf node of a hierarchical computing process, coordination node or execution node of a distributed computing process). |
| Type of operation | M | Represents the type of edge computing operations (e.g. task distribution, intermediate output delivery, execution node selection, edge computing preparation) which is used to indicate the operations that the assistance information be used for. |
| Assistance information type | M | Represents the assistance information type (e.g. time window(s) for computing task distribution and/or intermediate output delivery, candidate execution node list, edge computing preparation status), which is uded to indicate the assistance information needed. |

#### 8.x.3.5	Assist edge computing response

Table 8.x.3.5-1 shows the response sent by AIMLE server to the consumer (e.g. CAS, EAS) for assisting the edge computing process.

Table 8.x.3.5-1: Assist edge computing response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the assist edge computing request (positive or negative acknowledgement). |
| Assistance informaiton | M | The assistance informaiton for assisting edge computing process (e.g. time window(s) for computing task distribution and/or intermediate output delivery, candidate execution node list, edge computing preparation status). |

* * * Next Change * * * *

### 9.Y.A	AIMLE Assistance of Edge Computing API

Editor’s Note:	This clause provides the corresponding AIMLE assistance of edge computing API.

# *** End of Changes ***