3GPP TSG-SA WG6 Meeting #63	S6-244314

Hyderabad, India, 14th – 18th October 2024

Source:	Ericsson

Title:	Enhancements to FL Member Registration

Spec:	3GPP TS 23.482 v0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This CR brings provides enhancement to the FL member registration request procedure.

2. Reason for Change

FL members can be UE. The FL is a distributed ML and mobility of the clients can impact the FL. Therefore it is important to understand the mobility of the FL member like whether the FL member is moving at some velocity or stationary. This will help in the FL member selection. Similarly, the FL member’s location and QoS can impact the FL selection.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.2.0.

* * * First Change * * * *

#### 8.4.4.2	FL member registration request

Table 8.4.4.2-1 describes information elements for the FL member registration request from the candidate FL member (VAL server, AIMLE server) to the ML repository/registry.

Table 8.4.4.2-1: FL member registration request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration request. |
| FL member ID | M | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| FL member supported AI/ML role | M | The supported AI/ML role of FL member which can be used as FL server or FL client. |
| FL member capabilities | O (NOTE 1) | FL member capability information (e.g. ML application type, allowed resource usage level, ). |
| FL member application QoS | O | Indicates the QoS requirement of FL member. e.g. bandwidth and latency |
| FL member velocity | O See NOTE 2 | Indicates the FL member velocity. E.g. high, low, stationary |
| FL member Location information | O | Indicates the location information of the FL member |
| FL member Availability schedule | O (NOTE 1) | Availability schedule of the FL member for a certain FL member. |
| Supported ML model ID list | O | The list of ML model IDs for which the FL member can be used. |
| Area of Interest | O | The area of interest for which the registration applies. |
| Time validity | O | The time validity for the registration of the FL member. |
| NOTE 1: At least one of these shall be present. |  |  |
| NOTE 2: This IE is only present when FL member is a UE. |  |  |

* * * End of Changes * * * *