3GPP TSG-SA WG6 Meeting #63	S6-244318

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0059 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | No-transmit zones configuration and execution for UAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ECC Decision (CEPT ECC Decision 22(07)) asserts that a mechanism is necessary to ensure that aerial UEs respect no-transmit zones in order to protect incumbent radio systems from potential interference from aerial UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces new procedures on UAE layer support of NTZ (no-trasmition zone): - Management of NTZ support configuration, - NTZ configuration, - NTZ activation by UAE client, - UAE layer/SEAL/LMS assisted NTZ enforcement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of possibility to support NTZ according to CEPT ECC decision. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x.1 (new), 7.x.2 (new), 7.x.2.1 (new), 7.x.2.2 (new), 7.x.2.3 (new), 7.x.2.4 (new), 7.x.3 (new), 7.x.3.1 (new), 7.x.3.2 (new), 7.x.3.3 (new), 7.x.3.4 (new), 7.x.3.5 (new), 7.x.3.6 (new), 7.x.3.7 (new), 7.x.3.8 (new), 7.x.3.9 (new), 7.x.3.10 (new), 8.2.1, 8.2.x.1 (new), 8.2.x.2 (new), 8.2.x.3 (new), 8.2.y.1 (new), 8.2.y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 7.x	UAE layer support for NTZ

#### 7.x.1	General

This feature enables the UAS application enablement services for assisting the UAS application with NTZ handling. The UAE layer provides support for the following operations:

-	Support the distribution of the NTZ application policy from UAS application specific server to the UAE server and the UAE client as described in clause 7.x.2.1

#### 7.x.2	Procedures

#### 7.x.2.1	Management of NTZ support configuration

Figure 7.y.2.1-1 illustrates the NTZ support management procedure where the UAE server receives an application request for managing the NTZ application policy from the UAS application specific server.

Figure 7.x.2.1-1: NTZ support management procedure

1.	The UAS application specific server sends to the UAE server a NTZ management request. The request includes the UASS identifier and the NTZ policy management container. If NTZ policy container is provided, the request shall include area for deactivating transmission, information about frequency bands and reporting configuration.

2.	The UAE Server sends to the UAS application specific server a NTZ management response with a positive or negative acknowledgement of the request, based on capability of UAE server to undertake this task.

3.	UAE server executes NTZ configuration according to clause 7.x.2.2.

4.	After execution of NTZ configuration, the UAE server notifies the UAS application specific server with a NTZ management complete.

#### 7.x.2.2	NTZ configuration

This procedure enables the configuration of the UAE Client, based on an application request from UAS application specific server (which can be the USS/UTM) to manage the NTZ for a UAS.

Figure 7.x.2.2-1 illustrates the NTZ configuration procedure.

Pre-conditions:

1.	The UAS UEs are connected to 5GS and authenticated and authorized by UAS application specific server as specified in clause 5.2 of 3GPP TS 23.256 [4].

2.	UAE Server has established a UAE session with the respective UAE clients as the UAE clients are successfully registered to the UAE server.

3.	UAE Server has performed the NTZ management as in clause 7.x.2.1.

Figure 7.x.2.2-1: NTZ configuration

1.	The UAE Server sends a NTZ configuration request including the UAS identifier. If the NTZ configuration is provided, the request shall include the area for deactivating transmission, frequency bands and reporting configuration. The UAE server configures all UAS UEs registered in the UAE server.

2.	The UAE Client stores or removes the NTZ configuration parameters as per the information received in step 1.

3.	The UAE Client sends a NTZ configuration response to the UAE Server.

#### 7.4.2.3	NTZ activation by UAE Client

This procedure provides a mechanism for the UAE client to deactivate and activate its radio transmission in the respective frequency bands when entering and leaving the NTZ area based on the NTZ configuration as described in clause 7.x.2.2.

Figure 7.x.2.3-1 illustrates the NTZ activation.

Pre-conditions:

1.	The UAE Clients are configured with a NTZ configuration as described in clause 7.x.2.2.

2.	UAE Server has performed the NTZ management, as described in clause 7.x.2.1.

Figure 7.x.2.3-1: NTZ activation

1.	UAE Clients (UAV and UAV-C) detect approaching area for deactivating transmission based on their NTZ configuration. The UAE client can switch to another frequency band or entirely stop its transmission depending on its available functionality.

2.	The UAE Client sends a NTZ notification to the UAE Server indicating the time for switching off the needed frequency band and its location.

3.	The UE clients stop its transmission.

4.	The UAE Server forwards the NTZ notification to the UAS application specific server.

5.	The UAE client detects it is out of the area for deactivating transmission and starts transmission.

6.	The UAV and UAV-C sends a NTZ notification to the UAE server indicating the time for switching on the respective frequency band and its location.

7. 	The UAE Server forwards the NTZ notification to the UAS application specific server. It contains the UAS identity, the specific time when the transmission has been switched on or off and location information of the UAS.

8.	In case that the UAE client has changed its USS according to clause 7.6 then:

9. 	UAE client requests new USS NTZ policy from the UAE server.

10.	The UAE server replies with the new USS NTZ policy.

#### 7.x.2.4	UAE-layer/SEAL/LMS assisted NTZ enforcement

This procedure provides a mechanism for supporting NTZ activation and deactivation based on usage of existing SEAL/LMS functionality for monitoring location deviation specified in clause 9.3.11 of 3GPP TS 23.434 [5].

Figure 7.x.2.4-1 illustrates the procedure where the UAE server and SEAL/LMS support the assisted NTZ enforcement.

Pre-conditions:

1.	UAE Server has performed the NTZ management, as described in clause 7.x.2.1.

2.	UAE Server has subscribed for using SEAL/LMS services.

3.	UAE Server has established a UAE session with the respective UAE clients as the UAE clients are successfully registered to the UAE server.

Figure 7.x.2.4-1: UAE-layer/LMS assisted NTZ enforcement

1.	The UAE server subscribes for monitoring of location deviation according to clause 9.3.2.11 of 3GPP TS 23.434 [5] using the identities of the UAE client successfully registered to the UAE server. The area of interest is the geographical area in which the no-transmit zone is active.

2.	If one or more of the monitored UAE clients enter the area of interest, the LMS sends location area monitoring notification according to clause 9.3.2.16 of 3GPP TS 23.434 [5] containing the UAE client identifiers.

3.	The UAE Server sends enforce NTZ request to the identified UAE clients entering the area of interest, indicating to stop transmission and containing information about the NTZ (area for deactivating the transmission, frequency bands, altitude).

4.	The UAE clients send enforce NTZ response to the UAE server indicating positive or negative result of the received request.

5.	The UAE server sends NTZ notification towards the UAS application specific server. It contains the UAS identity, the specific time when the transmission has been switched off and location information of the UAS.

6.	If one or more of the monitored UAE clients leaves the area of interest, the LMS sends location area monitoring notification according to clause 9.3.2.16 of 3GPP TS 23.434 [5] containing the UAE client identifiers.

7.	The UAE Server sends enforce NTZ request to the identified UAE clients leaving the area of interest, indicating to start transmission.

8.	The UAE clients send enforce NTZ response to the UAE server indicating positive or negative result of the received request.

9.	The UAE server sends NTZ notification towards the UAS application specific server. It contains the UAS identity, the specific time when the transmission has been switched on and location information of the UAS.

### 7.x.3	Information flows

#### 7.x.3.1	NTZ management request

Table 7.x.3.1-1 describes the information flow NTZ management request from the UAS application specific server to the UAE server.

Table 7.x.3.1-1: NTZ management request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the NTZ management. This ID can be the USS identifier, when the UAS application specific server is the USS. |
| NTZ policy management container (see NOTE) | O | The NTZ policy management container consists of the requirements and policy for NTZ management. |
| > Area for deactivating transmission | M | The area where the NTZ management request applies. This can be geographical area, or topological area in which the no-transmit zone is active including specific altitude. |
| > Time | M | Specific time window for deactivating the transmission. |
| > Frequency bands | M | The information for the frequency bands not allowed to be used in no-transmit zone. |
| >Reporting configuration | M | How often the reporting of UE in NTZ is required – only when entering and exiting the NTZ or in certain time interval. |
| NOTE: If NTZ policy management container is not included, it indicates removal of the NTZ policy management related information. |  |  |

#### 7.x.3.2	NTZ management response

Table 7.x.3.2-1 describes the information flow NTZ management response from the UAE server to the UAS application specific server.

Table 7.x.3.2-1: NTZ management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the NTZ management request. |

#### 7.x.3.3	NTZ management complete

Table 7.x.3.3-1 describes the information flow NTZ management complete from the UAE server to the UAS application specific server.

Table 7.x.3.2-1: NTZ management complete

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the NTZ configuration. |

#### 7.x.3.4	NTZ configuration request

Table 7.x.3.4-1 describes the information flow NTZ configuration request from the UAE server to the UAE client.

Table 7.x.3.4-1: NTZ configuration request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS for which the NTZ configuration request applies. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| NTZ configuration (see NOTE) | O | NTZ configuration information to be configured at the UAS |
| > Area for deactivating transmission | M | The area where the NTZ management request applies. This can be geographical area, or topological area in which the no-transmit zone is active including specific altitude. |
| > Time | M | Specific time window for deactivating the transmission. |
| > Frequency bands | M | The information for the frequency bands not allowed to be used in no-transmit zone. |
| >Reporting configuration | M | How often the reporting of UE in NTZ is required – only when entering and exiting the NTZ or in certain time interval. |
| NOTE: If NTZ configuration IE is not included, it indicates removal of the NTZ configuration at the UAS ID. |  |  |

#### 7.x.3.5	NTZ configuration response

Table 7.x.3.5-1 describes the information flow NTZ configuration response from the UAE client to the UAE server.

Table 7.x.3.5-1: NTZ configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of reception and storing or removal of the NTZ configuration parameters |

#### 7.x.3.6	NTZ notification

Table 7.x.3.6-1 describes the information flow NTZ notification.

Table 7.x.3.6-1: NTZ notification

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| Transmission | M | ON/OFF indication for the transmission is switched on or off. |
| Time | M | The specific time when the transmission has stopped or has started. |
| Location information | M | Location and altitude of the UAE client when the notification is sent. |

#### 7.x.3.7	New NTZ policy request

Table 7.x.3.7-1 describes the information flow new NTZ policy request from the UAE client to the UAE server.

Table 7.x.3.7-1: New NTZ policy request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| Location information | M | Location and altitude of the UAE client when the request is sent. |

#### 7.x.3.8	New NTZ policy response

Table 7.x.3.8-1 describes the information flow new NTZ policy response from the UAE server to the UAE client.

Table 7.x.3.8-1: New NTZ policy response

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS for which the NTZ configuration request applies. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| NTZ configuration | O | NTZ configuration information to be configured at the UAS |
| > Area for deactivating transmission | M | The area where the NTZ management request applies. This can be geographical area, or topological area in which the no-transmit zone is active including specific altitude. |
| > Time | M | Specific time window for deactivating the transmission. |
| > Frequency bands | M | The information for the frequency bands not allowed to be used in no-transmit zone. |
| >Reporting configuration | M | How often the reporting of UE in NTZ is required – only when entering and exiting the NTZ or in certain time interval. |

#### 7.x.3.9	Enforce NTZ request

Table 7.x.3.9-1 describes the information flow enforce NTZ request from the UAE server to the UAE client.

Table 7.x.3.9-1: Enforce NTZ request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS for which the NTZ configuration request applies. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| NTZ enforcement information (see NOTE) | O | NTZ enforcement information to be executed at the UAS |
| > Area for deactivating transmission | M | The area where the NTZ management request applies. This can be geographical area, or topological area in which the no-transmit zone is active. |
| >Altitude | M | Altitude range for the NTZ. |
| > Time | M | Specific time window for deactivating the transmission. |
| > Frequency bands | M | The information for the frequency bands not allowed to be used in no-transmit zone. |
| NOTE: If NTZ enforcement information IE is not included, it indicates removal of the NTZ enforcement at the UAS ID. |  |  |

#### 7.x.3.10	Enforce NTZ response

Table 7.x.3.10-1 describes the information flow enforce NTZ response from the UAE client to the UAE server.

Table 7.x.3.10-1: Enforce NTZ response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the enforce NTZ request |

* * * Second Change * * *

### 8.2.1	General

Table 8.2.1-1 illustrates the UAE server APIs.

Table 8.2.1-1: List of UAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| UAE_C2OperationModeManagement API | Manage_C2OperationMode | UAS application specific server | Request/ Response |
|  | Notify_SelectedC2Mode (NOTE 1) | UAS application specific server | Subscribe/notify |
|  | Notify_C2ModeSwitching (NOTE 1) | UAS application specific server | Subscribe/notify |
|  | Notify_C2OperationModeManagementComplete (NOTE 1) | UAS application specific server | Subscribe/notify |
| UAE_RealtimeUAVStatus API | Subscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Notify_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
| UAE_ChangeUSSManagement API | Manage_USSManagement | UAS application specific server | Request/ Response |
|  | Notify_USSManagementComplete | UAS application specific server | Subscribe/notify |
|  | Manage_USSChange | UAS application specific server | Request/ Response |
|  | Notify_USSChange | UAS application specific server | Subscribe/notify |
|  | Notify_USSChangeTrigger | UAS application specific server | Subscribe/notify |
| UAE_DAASupport API | Manage_DAAManagement | UAS application specific server | Request/ Response |
|  | Notify_DAAManagementComplete | UAS application specific server | Subscribe/notify |
|  | Notify_DAAClientSupportEvent | UAS application specific server | Subscribe/notify |
|  | Manage_DAAServerSupportEvent | UAS application specific server | Request/ Response |
| UAE_UAVDynamicInfo API | Subscribe_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
|  | Notify_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
| UAE_FlightPathMonitoring API | Manage_FlightPathReportingConfiguration | UAS application specific server | Request/ Response |
|  | Notify_FlightPathReportingConfigurationComplete | UAS application specific server | Subscribe/notify |
|  | Notify_FlightPathEventReport | UAS application specific server | Subscribe/notify |
| UAE_FlightRouteSupport API | Manage_QoSFlightRoutePlan | UAS application specific server, UAE client | Request/ Response |
| UAE_NTZManagement API | Manage_NTZ | UAS application specific server | Request/response |
|  | Notify_NTZManagementComplete (NOTE 2) | UAS application specific server | Subscribe/notify |
| UAE_NTZConfigurationAPI | Configure_NTZ | UAE server | Request/response |
|  | Notify_NTZ (NOTE3) | UAE server | Subscribe/notify |
| SS_NTZenforce API | Enforce_NTZ | UAE server | Request/response |
| NOTE 1: The subscribe operation for Notify_C2OperationModeManagementComplete, Notify_SelectedC2Mode and Notify_C2ModeSwitching is part of Manage_C2OperationMode NOTE 2: The subscribe operation for Notify_NTZManagementComplete is part of Manage_NTZ NOTE 3: The subscribe operation for Notify_NTZ is part of Configure_NTZ |  |  |  |

* * * Third Change * * *

### 8.2.x	UAE_NTZ management API

#### 8.2.x.1	General

API description: This API enables the UAS application specific server to communicate with the UAE server to send configurations for NTZ management.

#### 8.2.x.2	Manage_NTZ

API operation name: Manage_NTZ

Description: Manage the configuration of the NTZ for the UAS UEs.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.x.3.1.

Outputs: Refer clause 7.x.3.2.

See clause 7.x.2.1 for the details of usage of this API operation.

#### 8.2.x.3	Notify_ NTZManagementComplete

API operation name: Notify_ NTZManagementComplete

Description: Notification about the NTZ management completion by UAE server.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.x.3.3.

Outputs: None.

See clause 7.x.2.1 for the details of usage of this API operation.

### 8.2.x	UAE_NTZ configuration API

#### 8.2.x.1	General

API description: This API enables the UAE server to communicate with the UAE client to send configuration about NTZ.

#### 8.2.x.2	Configure_NTZ

API operation name: Configure_NTZ

Description: Configure NTZ in UAE client.

Known Consumers: UAE server.

Inputs: Refer clause 7.x.3.4.

Outputs: Refer clause 7.x.3.5.

See clause 7.x.2.2 for the details of usage of this API operation.

#### 8.2.x.3	Notify_NTZ

API operation name: Notify_NTZ

Description: Notify NTZ.

Known Consumers: UAE server.

Inputs: Refer clause 7.x.3.6.

Outputs: None.

See clause 7.x.2.3 for the details of usage of this API operation.

### 8.2.y	UAE_NTZ enforce API

#### 8.2.y.1	General

API description: This API enables the UAE server or the UAE client to communicate with the UAE client to send enforced configuration on NTZ.

#### 8.2.y.2	Enforce_NTZ

API operation name: Enforce_NTZ

Description: Enforce NTZ in UAE client.

Known Consumers: UAE server.

Inputs: Refer clause 7.x.3.9.

Outputs: Refer clause 7.x.3.10.

See clause 7.x.2.4 for the details of usage of this API operation.