3GPP TSG-CT WG1 Meeting #152	C1-246355

# Orlando, US, 18-22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.379 | CR | 1004 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify list of participants by changing the criteria during an ongoing MCPTT ad hoc group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following stage 2 requirement was introduced under FRMCS_Ph5. #### 10.19.2.20 Modify ad hoc group call criteria request (MCPTT client – MCPTT server) Table 10.19.2.20-1 describes the information flow Modify ad hoc group call criteria request from the MCPTT client to the MCPTT server. Table 10.19.2.20-1: Modify ad hoc group call criteria request Information element; Status; Description. MCPTT ID; M; The MCPTT ID of the requesting MCPTT user who is authorized to modify the ad hoc group criteria. Functional alias; O; The functional alias of the requesting MCPTT user who is authorized to modify the ad hoc group call criteria. MCPTT ad hoc group ID; M; The MCPTT group ID of ad hoc group call whose participants list needs to be modified. Criteria to determine the list of participants; O; Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area. #### 10.19.2.21 Modify ad hoc group call criteria response (MCPTT server – MCPTT client) Table 10.19.2.21-1 describes the information flow Modify ad hoc group call criteria response from the MCPTT server to the MCPTT client. Table 10.19.2.21-1: Modify Ad hoc group call criteria response information elements Information Element; Status; Description. MCPTT ID; M; The MCPTT ID of the requesting MCPTT user who is authorized to modify the ad hoc group call criteria. Functional alias; O; The functional alias of the requesting MCPTT user who is authorized to modify the ad hoc group call criteria. MCPTT ad hoc group ID; M; The MCPTT group ID of ad hoc group call whose participants list needs to be modified. Result; M; Result of the modify ad hoc group call participants request (success or failure). ##### 10.19.3.1.6 Modification of ad hoc group call criteria by an authorized user Figure 10.19.3.1.6-1 below illustrates the modification of ad hoc group call criteria procedure by an authorized user. Pre-conditions: 1. The MCPTT user at the MCPTT client 1 is authorized to modify the criteria. 2. Both the MCPTT server and the MCPTT client 1 are aware of the criteria related to the ongoing ad hoc group call. Figure 10.19.3.1.6-1: Modification of ad hoc group call criteria by the authorized user 1. The MCPTT user at the MCPTT client 1 is authorized and requests to modify the criteria for determining the list of participants. The MCPTT client 1 sends the modify ad hoc group call criteria request to the MCPTT server which contains an updated criteria to determine the list of participants. 2. The MCPTT server verifies whether the MCPTT client 1 is authorized to modify the criteria which determines the list of participants during on-going ad hoc group voice communication. The MCPTT server determines the list of ad hoc group call participants based on the criteria provided and indentifies that MCPTT client 3 is to be removed from and MCPTT client 2 is to be added to the group. 3. The MCPTT server sends modify ad hoc group call criteria response to MCPTT client 1. 4. The MCPTT server sends the ad hoc group call leave request to the MCPTT client 3 to remove it from the on-going ad hoc group call. 5. The MCPTT client 3 notifies the user of the ad hoc group call leave request. 6. The MCPTT client 3 sends the ad hoc group call leave response to the MCPTT server. 7. The MCPTT server sends the ad hoc group call request towards MCPTT client 2. NOTE 1: Steps 7 to 9 can occur at any time following step 3. 8. The receiving MCPTT client 2 notifies the user about the incoming ad hoc group call. 9. The MCPTT client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server. The MCPTT server considers the MCPTT user as implicitly affiliated to the ad hoc group. 10. The MCPTT server may notify the initiating MCPTT user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call. 11. The MCPTT server may notify the participants about the change in the participants list of on-going ad hoc group call. The MCPTT server continuously checks whether other MCPTT clients meet or if participating MCPTT clients no longer meet the criteria for the ad hoc group emergency call. NOTE 2: If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly. |
| Summary of change: |  | Stage 3 implementation of the stage 2 requirement copied above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Discrepancy between stage 2 and stage 3 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.1, 17.2.1, 17.2.7 (new), 17.4.1, 17.4.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 17.1	General

This clause describes the adhoc group call procedures for on-network across single or multiple MCPTT systems, and associated functions such as emergency calls, Imminent peril calls, broadcast calls and others.

The adhoc group calls can use the participant list provided by either an initiator of the call or the MCPTT server. The MCPTT server can use the criteria provided by the initiator of the call to determine the participant list along with local criteria or local policies. The resulting adhoc group uses the configuration of a separate preconfigured MCPTT group. The MCPTT client of an authorized MCPTT user can request to modify the criteria which was used to determine the participant list.

NOTE 1:	A preconfigured group that is intended only to provide configuration for the adhoc group is identified by a parameter in the group configuration described in 3GPP TS 23.280 [82].

Editor’s Note: It is FFS an additional parameter is needed in group configuration.

The preconfigured MCPTT group that provides the configuration is not used for the MCPTT group communication, it only provides configuration for one or more adhoc group calls. The MCPTT group ID of the adhoc group call is provided by the MCPTT server when the adhoc group call originated. To establish a security context for the end-to-end secured adhoc group call, the security related information is used from this preconfigured group.

* * * Next Change * * *

### 17.2.1	General

This clause describes the adhoc group call procedures using on-demand and pre-established sessions to setup adhoc group calls, release the ongoing adhoc group calls, modify the ongoing adhoc group calls participants, and modify the participants criteria of an ongoing ad hoc group call.

* * * Next Change * * *

### 17.2.7	Modification of ad hoc group call criteria

Upon receiving a request from an authorized MCPTT user to modify an ongoing MCPTT session to modify the criteria of the participant list, the MCPTT client shall generate a SIP re-INVITE request as specified in 3GPP TS 24.229 [4], with the clarifications given below.

1)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body populated as currently established session;

2)	shall insert a <call-participants-criterias> with one or more criteria as a comma separated list into <anyExt> element of <mcptt-Params> element of <mcpttinfo> element of the application/vnd.3gpp.mcptt-info+xml MIME body;

3)	shall include an SDP offer with the media parameters as currently established according to 3GPP TS 24.229 [4]; and

4)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request, the MCPTT client may notify the user about successful criteria modification in the participant list.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the re-SIP INVITE request, the MCPTT client may notify the user about unsuccessful criteria modification in the participant list with an appropriate response along with the description.

* * * Next Change * * *

### 17.4.1	General

This clause describes the adhoc group calls controlling MCPTT function procedures using on-demand and pre-established sessions to setup adhoc group calls, release the ongoing adhoc group calls, modify the ongoing adhoc group calls participants, and modify the participants criteria of an ongoing ad hoc group call.

* * * Next Change * * *

### 17.4.7	Modification of ad hoc group call criteria

Upon receipt of a SIP re-INVITE request for an MCPTT session identifying an on-demand MCPTT adhoc group session, the controlling MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP re-INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

2)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

3)	if the originating user identified by the MCPTT ID is not authorised to modify the adhoc group session participants if the <allow-adhoc-group-call-modify> element of the <actions> element of a <rule> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the originating MCPTT user (see the MCPTT user profile document in 3GPP TS 24.484 [50]) is set to a value of "false", shall send a SIP 403 (Forbidden) response with the warning text set to: "190 user is not authorised to initiate modify adhoc group call participants" in a Warning header field as specified in clause 4.4 and skip the rest of the steps below;

4)	shall determine the MCPTT users meeting the specified criteria according to the <call-participants-criterias> in the <anyExt> element of the <mcptt-Params> element of the <mcpttinfo> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

5)	shall perform the operations described in clause 17.4.6.1.

* * * End of Change(s) * * *