3GPP TSG-CT WG1 Meeting #152	C1-246620

# Orlando, US, 18-22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.572 | CR | 0092 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the release command message in Rel-18 24.572 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As discussed in the discussion paper C1-246619, if the UE has an ongoing event report session with source LMF, after LMF reselection, the UE is not able to use the user plane connection with target LMF to continue the event report session. To assist the UE to link the ongoing event report session with the target LMF, proposal #1 is proposed: Proposal #1: The Source LMF includes the LCS correlation identifier of the target LMF in the user plane connection release command message and sends it to the UE via AMF. This CR implements above proposal #1, and adds the target LMF ID in the user plane connection release command message. Backward compatibility analysis: This proposal is backward compatible as it only adds a new IE in the existing user plane connection release command message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add Target LMF ID IE in the user plane connection release command message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE is not able to use the user plane connection with the target LMF to continue the ongoing LCS session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.6.1, 10.3.6.x(new), 11.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

10.3.6.1	Message definition

The USER PLANE CONNECTION RELEASE COMMAND message is sent by the LMF to the UE to release the LCS secured user plane connection between the UE and the LMF. See table 10.3.6.1.1.

Message type:	USER PLANE CONNECTION RELEASE COMMAND

Significance:	dual

Direction:	network to UE

Table 10.3.6.1.1: USER PLANE CONNECTION RELEASE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION RELEASE COMMAND message identity | Message type 11.1.3 | M | V | 1 |
| 10 | Back-off timer value | GPRS timer 3 11.3.2 | O | TLV | 3 |
| TBD | Target LMF ID | LCS correlation identifier 11.3.x | O | TLV | 3-257 |

* * * Next Change * * * *

10.3.6.x	Target LMF ID

The LMF may include this information element to the UE to indicate the UE performs the ongoing LCS session via the LCS secured user plane connection with the LMF identified by the Target LMF ID.

* * * Next Change * * * *

11.3.x	LCS correlation identifier

See subclause 9.11.2.1 in 3GPP TS 24.501 [4].

* * * End of Changes * * * *