3GPP TSG-CT WG1 Meeting #152	C1-246784

# Orlando, US, 18-22 November 2024

Source:	LG Electronics, Apple, Samsung, Nokia

Title:	Notification of disaster condition to the UE via E-UTRAN broadcast

Agenda item:	20.1

Document for:	Agreement

1. Introduction

This contribution proposes the solution of KI#1 for MINT_Ph2

2. Reason for Change

In TR 24.812 v0.1.0, the following Key Issue was proposed :

Key Issue #1: Notification of Disaster Condition to the UE.

-	How to deliver the information on the disaster condition of a PLMN to the UE;

-	Which network functions or entities are involved for the delivery of the information.

This p-CR proposes how to deliver the information on the disaster condition of a PLMN to the UE via E-UTRAN using the existing mechanism in 5G.

3. Conclusion

The following solution is proposed to be added to TR 24.812.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.812 v0.1.0.

* * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| x | x |  |  |  |  |  |  |  |

* * * Next Change * * * *

## 6.X	Solution #<X>: Notification of Disaster Condition to the UE via E-UTRAN broadcast

### 6.X.1	Introduction

This paper proposes a solution for Key Issue#1:

Key Issue #1: Notification of Disaster Condition to the UE

In this solution, PLMN D is the PLMN of the 5GS with disaster condition PLMN A provides a disaster roaming service for the 5G-only national roaming UE. In addition,

PLMN A is in the area covered by disaster condition by regulatory requirements or operator’s policy. This solution addresses how to deliver the information on the disaster condition of a PLMN to the UE.

### 6.X.2	Detailed description

This solution assumes that the information on the disaster condition of PLMN D can be made available to PLMN A. PLMN A broadcasts the information on the disaster condition of PLMN D when the disaster condition starts in PLMN D.

The information on the disaster condition of a PLMN may contain a 'list of one or more PLMN(s) with disaster condition for which disaster roaming service is offered by the available PLMN' in the impacted area.

PLMN A can broadcast in E-UTRAN the information on the disaster condition of a PLMN to UEs.

Editor's note:	Details of broadcast signalling depend on the RAN WGs.

The UE can determine the disaster condition by receiving the information on the disaster condition of PLMN D from PLMN A. After determining the disaster condition, the UE can manage the mobility and roaming procedure of the UE in the disaster condition through the MME.

Editor's note:	If the UE does not receive information on disaster condition of a PLMN, the handling is FFS.

### 6.X.3	Impacts on existing nodes and functionality

UE:

-	support for receiving the information on the disaster condition of PLMN D in E-UTRAN.

E-UTRAN of PLMN A:

-	support for providing the information on the disaster condition of PLMN D via broadcast.

* * * End of Changes * * * *