3GPP TSG CT WG3 Meeting #138	C3-246149

# Orlando, US, 18 – 22 November, 2024												(revision of C3-246abc)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.552 | CR | 0135 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for Horizontal Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed in S2-2411193, the terminology for Federated Learning were clarified to differentiate the Horizontal Federated Learning and Vertical Federated Learning. The clarification is also needed in stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the existing teminology from FL to HFL or Federated Learning to Horizontal Federated Learning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 requirement is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.6.1, 5.10, 5.10.1, 5.10.2, 5.10.2.1, 5.10.2.2, 5.10.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.288 CR 1198 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADRF	Analytics Data Repository Function

AF	Application Function

AMF 	Access and Mobility Management Function

AnLF	Analytics Logical Function

CEF	Charging Enablement Function

DCCF	Data Collection Coordination Function

HFL	Horizontal Federated Learning

GMLC	Gateway Mobile Location Centre

LMF	Location Management Function

MDAF	Management Data Analytics Function

MDT	Minimization of Drive Tests

MFAF	Messaging Framework Adaptor Function

ML	Machine Learning

MTLF	Model Training Logical Function

NEF	Network Exposure Function

NRF	Network Repository Function

NSACF	Network Slice Admission Control Function

NSSF	Network Slice Selection Function

NWDAF	Network Data Analytics Function

OAM	Operation, Administration, and Maintenance

PCF	Policy Control Function

RE-NWDAF	Roaming Exchange NWDAF

SMF	Session Management Function

UDM	Unified Data Management

UPF	User Plane Function

# *** Next Change ***

5.6.1	General

The ML Model provisioning procedures allow the NF service consumers (i.e. NWDAF (MTLF+AnLF), NWDAF (AnLF)) to obtain the ML model information on the related Analytics from another NWDAF (i.e. an NWDAF containing MTLF). The ML Model provisioning procedures also allow an NWDAF (MTLF) to subscribe/unsubscribe at another NWDAF (MTLF), i.e., an HFL server NWDAF, to be notified when ML Model Information on the related Analytics becomes available.

# *** Next Change ***

5.10	Horizontal Federated Learning among Multiple NWDAFs

# *** Next Change ***

5.10.1	General

The NWDAF containing MTLF can leverage Horizontal Federated Learning technique to train an ML model. To apply HFL technique for ML model training, there is no need for input data transfer (e.g. centralized into one NWDAF) but only need for cooperation among multiple NWDAFs (MTLF) distributed in different areas, i.e. sharing of ML model(s) and of the learning results among multiple NWDAFs (MTLF).

The NWDAF containing MTLF determines to train an ML model either based on local configuration or when it receives the request from NWDAF containing AnLF as described in clause 5.3 of 3GPP TS 23.288 [17]. If the NWDAF containing MTLF can act as an HFL server for the ML model training, then HFL procedure is initiated by the NWDAF containing MTLF as HFL server NWDAF directly, otherwise, the NWDAF containing MTLF should discover an HFL server NWDAF as described in clause 5.3.2.2 of 3GPP TS 29.510 [26] and request the HFL server NWDAF to provide the trained ML model as described in clause 5.10.2.1.

# *** Next Change ***

5.10.2	Procedures related to Horizontal Federated Learning

# *** Next Change ***

5.10.2.1	General Procedure for Horizontal Federated Learning among Multiple NWDAF Instances

This procedure is used by the NWDAF containing MTLF (HFL Server NWDAF) to trigger HFL among multiple NWDAF instances, by the multiple NWDAF containing MTLF (the HFL Server NWDAF and HFL Client NWDAF(s)) to execute Horizontal Federated Learning in HFL execution phase.

Figure 5.10.2.1-1: General procedure for HFL among Multiple NWDAF

0a-0b.	To send a request for ML model analytics events subscription to the NWDAF containing MTLF (the HFL Server NWDAF), the NWDAF service consumer (NWDAF containing AnLF or NWDAF containing MTLF) invokes the Nnwdaf_MLModelProvision_Subscribe service operation by sending an HTTP POST request targeting the resource "NWDAF ML Model Provision Subscriptions".

The HFL Server NWDAF responses to the Nnwdaf_MLModelProvision_Subscribe service operation. Upon receipt of the HTTP POST request, if the subscription is accepted to be created, the HFL Server NWDAF responds to the NWDAF service consumer with "201 Created", and the URI of the created subscription is included in the Location header field. Details are described in clause 4.5.2.2 of 3GPP TS 29.520 [5].

NOTE 1:	The ML model accuracy threshold can be used to indicate the target ML Model Accuracy of the training process, and the HFL Server NWDAF may stop the training process when the ML model accuracy threshold is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF or NWDAF containing MTLF) provides the Time when the ML model is needed, the HFL Server NWDAF can take this information into account to decide the maximum response time for its HFL Client NWDAFs.

0e.	The HFL Server NWDAF selects NWDAF(s) containing MTLF (HFL Client NWDAF(s)) as described in clause 5.10.2.2.

1a-1b.	To request the selected NWDAF containing MTLF (the HFL Client NWDAF) to perform the local model training, the HFL Server NWDAF may invoke Nnwdaf_MLModelTraining_Subscribe service operation by sending an HTTP POST request targeting the resource "NWDAF ML Model Training Subscriptions". The HFL Client NWDAF responses to the Nnwdaf_MLModelTraining_Subscribe service operation with an HTTP "201 Created" status code to the HFL Server NWDAF, as defined in clause 4.6.2.2 of 3GPP TS 29.520 [5].

2a-2b.	To subscribe to notification (or to modify subscriptions to notifications) of data events from the data source NF, each HFL Client NWDAF may invoke the Nnf_EventExposure_Subscribe service operation by sending an HTTP POST (or PUT, for modification) request targeting the resource representing event exposure subscriptions of that NF.

The data source NF responds to the Nnf_EventExposure_Subscribe service operation. Upon receipt of the HTTP POST request, if the subscription is accepted to be created, the NF responds to the HFL Client NWDAF with "201 Created", and the URI of the created subscription is included in the Location header field.

3a-3b.	If the data source NF observes the subscribed event(s), the NF invokes the Nnf_EventExposure_Notify service operation to report the event(s) by sending an HTTP POST request.

On success, the HFL Client NWDAF sends an HTTP "204 No Content" response to the NF.

4.	During HFL training process, each HFL Client NWDAF further trains the retrieved ML model from the HFL Server NWDAF based on its own/collected data, and reports interim local ML model information to the HFL Server NWDAF before the maximum response time elapses. Each HFL Client NWDAF also computes local model metric and reports it to the HFL Server NWDAF. The ML model information are exchanged between the HFL Client NWDAF(s) and the HFL Server NWDAF during the HFL training process.

4a-4b.	To report the ML model training information, the HFL Client NWDAF may invoke Nnwdaf_MLModelTraining_Notify service operation as defined in clause 4.6.2.4 of 3GPP TS 29.520 [5]. The HFL Server NWDAF stores the notification and responds to the Nnwdaf_MLModelTraining_Notify service operation with an HTTP "204 No Content" status code to the HFL Client NWDAF.

4c.	[Optional] If HFL Server NWDAF receives notification that the HFL Client NWDAF is not able to complete the training within the maximum response time, the HFL Server NWDAF may send to the HFL Client NWDAF an new maximum response time by invoking Nnwdaf_MLModelTraining_Subscribe, before which the HFL Client NWDAF shall report the interim local ML model information to the HFL Server NWDAF. Otherwise, the HFL Server NWDAF may indicate HFL Client NWDAF to skip reporting for this iteration.

5.	The HFL Server NWDAF aggregates all the local ML model information retrieved at step 4, to update the global ML model. The HFL Server NWDAF may also compute the global model metric.

-	If the HFL Server NWDAF provides the maximum response time for the HFL Client NWDAFs to provide the interim local ML model information in step 1 or the new maximum response time in step 4, the HFL Server NWDAF decides either to wait for the HFL Client NWDAFs which have not yet provided their interim local ML model within the new maximum response time or to aggregate only the retrieved local ML model information instances to update global ML model. The HFL Server NWDAF makes this decision, considering the notification from the HFL Client NWDAF or, if the notification is not received, based on local configuration.

6a.	[Optional] Based on the NWDAF service consumer request, the HFL Server NWDAF updates the global ML model metric to the NWDAF service consumer periodically or dynamically when some pre-determined status (e.g. ML model accuracy threshold) is achieved.

6aa-6ab.	To report the ML model training information, the HFL Server NWDAF invokes Nnwdaf_MLModelProvision_Notify service operation as defined in clause 4.5.2.4 of 3GPP TS 29.520 [5]. The NWDAF service consumer stores the notification and responds to the Nnwdaf_MLModelProvision_Notify service operation with an HTTP "204 No Content" status code to the HFL Server NWDAF.

6b.	[Optional] The NWDAF service consumer decides whether the current model can fulfil the requirement. The NWDAF service consumer modifies subscription (stops or continues the training process) according to the NWDAF service consumer decision.

6ba.	To modify the existing subscription, the NWDAF service consumer invokes Nnwdaf_MLModelProvision_Subscribe service operation by sending an HTTP PUT request with Resource URI of the resource "Individual NWDAF ML Model Provision Subscription". The HFL Server NWDAF responds to the NWDAF service consumer with an HTTP "200 OK" or "204 No Content" status code, as defined in clause 4.5.2.2.3 of 3GPP TS 29.520 [5].

6bb.	To unsubscribe the ML model, the NWDAF service consumer invokes Nnwdaf_MLModelProvision_Unsubscribe service operation by sending an HTTP DELETE request which targets the resource "Individual NWDAF ML Model Provision Subscription", to the HFL Server NWDAF, as defined in clause 4.5.2.3 of 3GPP TS 29.520 [5]. If the request is accepted, the HFL Server NWDAF deletes the subscription and responds to the NWDAF service consumer with an HTTP "204 No Content" message.

6c.	[Optional]	According to the request from the NWDAF service consumer, the HFL Server NWDAF updates or terminates the current HFL training process. If it terminates the current HFL training process, steps 7 and 8 are skipped.

7.	If the HFL procedure continues, the HFL Server NWDAF sends the aggregated ML model information to each HFL Client NWDAF for next round model training.

To modify the existing subscription, the HFL Server NWDAF invokes Nnwdaf_MLModelTraining_Subscribe service operation by sending an HTTP PUT request or an HTTP PATCH request with Resource URI of the resource "Individual NWDAF ML Model Training Subscription". The HFL Client NWDAF responds to the HFL Server NWDAF an HTTP "200 OK" or "204 No Content" status code, as defined in clauses 4.6.2.2.3 and 4.6.2.2.4 of 3GPP TS 29.520 [5].

8.	Each HFL Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the HFL Server NWDAF at step 7.

NOTE 2:	The steps 4-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

9a-9b.	To unsubscribe to the notifications of data events from the data source NF, the HFL Client NWDAF invokes the Nnf_EventExposure_Unsubscribe service operation by sending an HTTP DELETE request targeting the resource that represents the previously created individual event exposure subscription.

The data source NF responds to the Nnf_EventExposure_Unsubscribe service operation. If the subscription deletion is accepted, the NF responds with "204 No Content".

10.	If the HFL Server NWDAF determines that the subscribed ML model information is available, the HFL Server NWDAF may invoke the Nnwdaf_MLModelProvision_Notify service operation or the Nnwdaf_MLModelTraining_Notify service operation to report the ML model information by sending an HTTP POST request to the NWDAF service consumer identified by the notification URI received during the creation/modification of the subscriptions. The NWDAF service consumer responds to the HFL Server NWDAF with an HTTP "204 No Content" message.

11a-11b.	To unsubscribe from the notification(s) of the ML model information, the NWDAF service consumer invokes the Nnwdaf_MLModelProvision_Unsubscribe service operation by sending an HTTP DELETE request, which targets the resource "Individual NWDAF ML Model Provision Subscription", to the HFL Server NWDAF, as defined in clause 4.5.2.3 of 3GPP TS 29.520 [5].

If the request is accepted, the HFL Server NWDAF deletes the subscription and responds to the NWDAF service consumer with an HTTP "204 No Content" message.

# *** Next Change ***

5.10.2.2	Preparation Procedure for Horizontal Federated Learning

This procedure is used by the NWDAF containing MTLF (as HFL Server NWDAF or HFL Client NWDAF(s)) to register into NRF, discover the HFL Server NWDAF and select HFL Client NWDAF(s) for Horizontal Federated LearningHFL.

Figure 5.10.2.2-1: Preparation procedure for Horizontal Federated Learning

The NWDAF containing MTLF as HFL Server NWDAF or HFL Client NWDAF(s) registered to NRF its NF profiles. Details are described in clause 5.2.2.2 of 3GPP TS 29.510 [26].

1.	The HFL Server NWDAF and HFL Client NWDAF(s) are discovered via NRF. Details are described in clause 5.3.2.2 of 3GPP TS 29.510 [26].

2.	The Nnwdaf_MLModelTraining service may be used for the preparation information exchange between the HFL Server NWDAF and the HFL Client NWDAF(s).

2a.	The HFL Server NWDAF invokes Nnwdaf_MLModelTraining_Subscribe service operation by sending an HTTP POST request targeting the resource "NWDAF ML Model Training Subscriptions", The request shall include the "mLPreFlag" attribute and set to "true". Details are described in clause 4.6.2.2 of 3GPP TS 29.520 [5].

2b.	The HFL Client NWDAF(s) checks if it meets the ML model training requirement as described in clause 5.5.6.2.2 of 3GPP TS 29.520 [5] and/or can successfully download the model if the model information is provided in the request, and decides whether to join the HFL process based on operator policy (e.g. pre-configured list of ML models) and/or implementation.

2c.	The HFL Client NWDAF responses or notifies to the Nnwdaf_MLModelTraining_Subscribe request to indicate its decision. Upon receipt of the HTTP POST request, if the preparation request is accepted, the HFL Client NWDAF responds to the HFL Server NWDAF with "201 Created", otherwise, responds with "403 Forbidden" as described in 3GPP TS 29.520 [5].

NOTE:	Step 2 can be skipped if the HFL Server NWDAF can decide that the HFL Client NWDAF(s) supports the HFL procedure to be performed, e.g. based on information acquired from previous HFL procedures or from the NRF, or based on local configuration.

3.	The HFL Server NWDAF determines the final list of HFL Client NWDAF(s) to be involved in the HFL procedures based on the information received in step 1 and other information received in step 2 (if available).

# *** Next Change ***

5.10.2.3	Procedure for Maintenance of Horizontal Federated Learning Process

This procedure is used by the NWDAF containing MTLF (the HFL Server NWDAF) to maintain a HFL process in HFL execution phase, including: the HFL Server NWDAF triggers reselection, addition, or removal of HFL Client NWDAF(s), discovery of new HFL Client NWDAF(s) via NRF, and HFL Client NWDAF(s) joins or leaves HFL process dynamically.

In HFL execution phase, the HFL Server NWDAF monitors the status changes of HFL Client NWDAF(s), and may reselect the HFL Client NWDAF(s) based on the received information of status changes.

NOTE:	The HFL Server NWDAF checks if there is a need to carry on the HFL execution phase and then reselects HFL members for the next iteration if needed.

Figure 5.10.2.3-1: Procedure for Maintenance of Horizontal Federated Learning Process in HFL Execution Phase

The HFL Server NWDAF registered to NRF by invoking the Nnrf_NFManagement_NFRegister_request service operation about the HFL process, which includes Analytics ID.

1.	The HFL Server NWDAF may get the updated status of the current HFL Client NWDAF(s) via NRF by using the Nnrf_NFManagement service in the HFL execution phase. Details are described in clauses 5.2.2.5 and 5.2.2.6 of 3GPP TS 29.510 [26].

2.	The current HFL Client NWDAF(s) may inform the HFL Server NWDAF to leave the HFL process. The HFL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify service operation by sending an HTTP POST request to the HFL Server NWDAF identified by the notification URI received during the creation/modification of the subscriptions. The request shall include "termTrainReq" attribute and with the "termTrainCause" attribute in data type "TermMLModelTrainInfo" being provided. Details are described in clause 4.6.2.4 of 3GPP TS 29.520 [5]. The HFL Server NWDAF responds to the Nnwdaf_MLModelTraining_Notify service operation with an HTTP "204 No Content" status code to the HFL Client NWDAF(s).

3.	The HFL Server NWDAF may get the information of the new HFL Client NWDAF(s) dynamically via NRF. Details are described in clauses 5.2.2.5 and 5.2.2.6 of 3GPP TS 29.510 [26].

4.	The HFL Server NWDAF may subscribe to other NWDAF (Assist NWDAF) or the HFL Client NWDAF(s) for analytics of the HFL Client NWDAF(s), as defined in clauses 4.2.2.2 and 4.2.2.4 of 3GPP TS 29.520 [5].

5.	The HFL Client NWDAF(s) may send Status report of HFL training and Global ML Model Accuracy Information. The HFL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify service operation to the HFL Server NWDAF. The HFL Server NWDAF stores the notification and responds to the Nnwdaf_MLModelTraining_Notify service operation with an HTTP "204 No Content" status code to the HFL Client NWDAF(s). Details are described in clause 4.6.2.4 of 3GPP TS 29.520 [5].

6.	The HFL Server NWDAF checks the HFL Client NWDAF(s) status based on the received information, determines whether reselection of the HFL Client NWDAF(s) for the next round(s) of HFL is needed. The checking and determination are made according to the updated status of the HFL Client NWDAF(s) received in steps 1-5.

7.	If re-selection is needed as determined in step 6 and if step 3 is not performed, the HFL Server NWDAF may discover new candidate HFL Client NWDAF(s) via NRF by using the Nnrf_NFDiscovery services as described in clause 5.3.2.2 of 3GPP TS 29.510 [26]. The HFL Server NWDAF reselects the HFL Client NWDAF(s) from the current HFL Client NWDAF(s) and the new candidate HFL Client NWDAF(s).

8.	The HFL Server NWDAF sends termination request to the HFL Client NWDAF(s) which will be removed from the HFL process, and optionally indicating the reason. The HFL Client NWDAF(s) terminates HFL operations when receives a termination request from the HFL Server NWDAF and may perform further action to be qualified in participation of HFL training in the next cycles.

8a-8b.	To send the termination request, the HFL Server NWDAF may invoke the Nnwdaf_MLModelTraining_Unsubscribe service operation by sending an HTTP DELETE request, which targets the resource "Individual NWDAF ML Model Training Subscription", to the HFL Client NWDAF(s). If the request is accepted, the HFL Client NWDAF deletes the subscription and responds to the HFL Server NWDAF service consumer with an HTTP "204 No Content" message. Details are described in clause 4.6.2.3 of 3GPP TS 29.520 [5].

# *** End of Changes ***