3GPP TSG-CT WG3 Meeting #138	C3-246319

# Orlando, US, 18 - 22 November 2024

Source:	Nokia

Title:	Encoding of the XRM media related information between UPF and AS

Document for:	Discussion & Agreement

# Agenda Item:	19.2

# Work Item / Release:	Rel-19

Abstract of the contribution:

The encoding of the XRM media related information between UPF and AS for all three e2e encryption solutions has to be specified in TS 29.561.

# Background:

TS 23.700-70 clause 8.2:

the three mechanisms are agreed by SA2 for end-to-end encryption in UDP traffic:

-	The XRM metadata shall be encrypted and integrity protected between the UPF and the AS.

Media over QUIC (MoQ) [9] and Proxy-UDP-in-HTTP/3 [38]+QUIC-Aware Proxying[40], and UDP-option [21] should be supported in normative work.

-	For MoQ [9], the PDU set information is accommodated via Metadata in MoQ. UPF supports the MoQ relay functionality and identifies the PDU Set information from the MoQ metadata.

-	For Proxy-UDP-in-HTTP/3 [38] + QUIC-Aware proxying [40], PDU set information is included in HTTP datagram via a new datagram mode or via packet transforms in QUIC-Aware proxying [40]. UPF supports the HTTP/3 Client functionality.

-	For UDP-Option carrying XRM metadata [21], PDU set information is included in UDP-Option in UDP datagram in UDP tunnel. UPF supports the HTTP/3 Client functionality only for negotiation of security keys used for encrypting UDP-Option.

S2-2411185 (TS 23.501 CR 5728) clause 5.37.X.2:

#### 5.37.X.2	Use of connect-UDP in-band signaling protocol

When the traffic is UDP and encrypted end-to-end, the UPF may establish an in-band connection to an HTTP/3 AS proxy using connect-UDP according to RFC 9298 [170], acting as an HTTP/3 client configured to use a UDP proxy. The AS then provides Media Related Information within HTTP datagrams. The Media Related Information in the HTTP datagrams is protected by the security of the QUIC connection established between the UPF and the AS proxy. The mechanism for exchanging HTTP Datagrams and the associated format is defined in RFC 9298 [170] and RFC 9297.

The AF provides the 5GS with the address of a proxy to establish the in-band connection, which is then forwarded by the SMF to the UPF in N4 rules.

The in-band connection is established by the UPF, at the latest at reception of an UL UDP packet matching an N4 rule with Establish In-band Connection indication for which the In-band Signaling Protocol is connect-UDP, if not already established. UPF may use the same or different in-band connections for different UEs depending on implementation. In order to reuse the in-band connection, UPF sends CONNECT requests for different UEs over a single QUIC connection to the AS proxy, opening different QUIC streams to handle the traffic for the UEs.

NOTE:	The format of HTTP Datagram carrying Media Related Information is up to stage 3.

# Discussion:

Media Related information vs Metadata:

SA2 study item uses the XRM metadata between UPF and AS

The agreed CRs in SA2 normative statements uses the media related information

So, there are two options to specify in CT3

Option 1: The media related information can be used inline with normative statement. This will provide the easy reference once the normative specified between SA2 and CT3.

Option 2: The XRM metadata shall be used to align with the study.

Encoding of XRM related media related information between UPF and AS:

Option 1 (CT3 based solution):

- All HTTP enhancements are handled by CT groups so far.

- TS 29.561 handles Interworking between 5G network and external data network in CT3. This covers the control interface between UPF and AS as well.

- CT3 has vast experience in defining HTTP connections across NFs and Afs.

- CT3 is well positioned as stage 3 inline with SA2 recommendation.

Option 2 (SA4 based solutions):

- The exchanged XRM information is related to signaling and not the actual media.

- SA4 can define the actual media related enhancements.

Format of HTTP datagram handing:

SA2 recommends to create the format of HTTP datagram in stage 3.

Option 1 (CT3 based solution):

- TS 29.561 handles Interworking between 5G network and external data network in CT3

- As SA2 recommends to Stage3, CT3 is the correct stage 3 WG to update these HTTP datagrams format enhancement related specifications.

Option 2 (SA4 based solutions):

- The study of XRM_Ph2 WID is ongoing

- There is no plan to enhance HTTP datagram so far.

# 4.Conclusion:

Nokia prefers Option 1 mentioned for the above discussion which is as follows:

- Media related information terminology

- CT3 based solution for encoding of XRM related media related information between UPF and AS

- Format of HTTP datagram handing.

Nokia proposes to the updated of XRM Ph2 WID CT3 objective impact relatd to point 2 with additional requirement as

- Potential impacts for specifying the encoding of the XRM media related information between UPF and AS for all three e2e encryption solutions.

-Potential impacts to define QUIC aware proxy HTTP datagram carrying the e2e encrypted payload and XRM media related information between UPF and AS.

TS impact in as

| 29.561 | Potential Impacts to define the format of HTTP datagram carrying media related information between UPF and AS when the traffic is UDP and encrypted end-to-end. Potential impacts for specifying the encoding of the XRM media related information between UPF and AS for all three e2e encryption solutions. | TSG CT#109 (September 2025) | CT3 responsibility |
| --- | --- | --- | --- |