3GPP TSG-CT WG4 Meeting #126	C4-245078

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0819 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Nsmf_PDUSession API to support UE level measurement trace |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to LS from SA5(C4-240552), the Trace activation messages in 5GC need to be extended to support the signalling-based activation of 5GC UE level measurements collection. According to the LS reply from SA5 LS(C4-244033), the trace activation on SMF shall allign with clause 4.1.2.18 of TS 32.422, which implies, - refer to the figure 4.1.2.15.1.2 in clause 4.1.2.15.1, step 11. AMF sends the Nsmf_PDUSession_CreateSMContext request to the selected SMF with the trace control and configuration parameters. - Refer to the Figure 4.1.2.15.1.4 in clause 4.1.2.15.1, step 6. 6. AMF sends Nsmf_PDUSession_UpdateSMContext request with the trace control and configuration parameters information to the SMF This CR propose to add the configuration parameters of 5GC UE level measurements into the messages when the AMF send the Nsmf_PDUSession_CreateSMContext or Nsmf_PDUSession_UpdateSMContex request to the selected SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the 5GC UE level measurements related data to Nsmf_PDUSession API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The signalling-based activation of 5GC UE level measurements collection in Stage 2 is not supported in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.2, 6.1.6.2.4, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to TS29502_Nsmf_PDUSession.yaml. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service-based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Data within Create SM Context Request |
| SmContextCreatedData | 6.1.6.2.3 | Data within Create SM Context Response |
| SmContextUpdateData | 6.1.6.2.4 | Data within Update SM Context Request |
| SmContextUpdatedData | 6.1.6.2.5 | Data within Update SM Context Response |
| SmContextReleaseData | 6.1.6.2.6 | Data within Release SM Context Request |
| SmContextRetrieveData | 6.1.6.2.7 | Data within Retrieve SM Context Request |
| SmContextStatusNotification | 6.1.6.2.8 | Data within Notify SM Context Status Request |
| PduSessionCreateData | 6.1.6.2.9 | Data within Create Request |
| PduSessionCreatedData | 6.1.6.2.10 | Data within Create Response |
| HsmfUpdateData | 6.1.6.2.11 | Data within Update Request towards H-SMF, or from I-SMF to SMF |
| HsmfUpdatedData | 6.1.6.2.12 | Data within Update Response from H-SMF to V-SMF, or from SMF to I-SMF |
| ReleaseData | 6.1.6.2.13 | Data within Release Request |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF to V-SMF, or from SMF to I-SMF |
| VsmfUpdateData | 6.1.6.2.15 | Data within Update Request towards V-SMF, or from SMF to I-SMF |
| VsmfUpdatedData | 6.1.6.2.16 | Data within Update Response from V-SMF, or from I-SMF to SMF |
| StatusNotification | 6.1.6.2.17 | Data within Notify Status Request |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |
| SmContextRetrievedData | 6.1.6.2.27 | Data within Retrieve SM Context Response |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF, or from I-SMF to SMF |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF, or from SMF to I-SMF |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF, or from SMF to I-SMF |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |
| SmContext | 6.1.6.2.39 | Complete SM Context |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |
| PsaInformation | 6.1.6.2.41 | PSA Information |
| DnaiInformation | 6.1.6.2.42 | DNAI Information |
| N4Information | 6.1.6.2.43 | N4 Information |
| IndirectDataForwardingTunnelInfo | 6.1.6.2.44 | Indirect Data Forwarding Tunnel Information |
| SmContextReleasedData | 6.1.6.2.45 | Data within Release SM Context Response |
| ReleasedData | 6.1.6.2.46 | Data within Release Response |
| SendMoDataReqData | 6.1.6.2.47 | Data within Send MO Data Request |
| CnAssistedRanPara | 6.1.6.2.48 | SMF derived CN assisted RAN parameters tuning |
| UlclBpInformation | 6.1.6.2.49 | UL CL or BP Information |
| TransferMoDataReqData | 6.1.6.2.50 | Data within Transfer MO Data Request |
| TransferMtDataReqData | 6.1.6.2.51 | Data within Transfer MT Data Request |
| TransferMtDataError | 6.1.6.2.52 | Transfer MT Data Error Response |
| TransferMtDataAddInfo | 6.1.6.2.53 | Transfer MT Data Error Response Additional Information |
| VplmnQos | 6.1.6.2.54 | VPLMN QoS |
| DdnFailureSubs | 6.1.6.2.55 | DDN Failure Subscription |
| RetrieveData | 6.1.6.2.56 | Data within Retrieve Request |
| RetrievedData | 6.1.6.2.57 | Data within Retrieve Response |
| SecurityResult | 6.1.6.2.58 | Security Result |
| UpSecurityInfo | 6.1.6.2.59 | User Plane Security Information |
| DdnFailureSubInfo | 6.1.6.2.60 | DDN Failure Subscription Information |
| AlternativeQosProfile | 6.1.6.2.61 | Alternative QoS Profile |
| ProblemDetailsAddInfo | 6.1.6.2.62 | Problem Details Additional Information |
| ExtProblemDetails | 6.1.6.2.63 | Extended Problem Details |
| QosMonitoringInfo | 6.1.6.2.64 | QoS Monitoring Information |
| IpAddress | 6.1.6.2.65 | IP Address |
| RedundantPduSessionInformation | 6.1.6.2.66 | Redundant PDU Session Information |
| QosFlowTunnel | 6.1.6.2.67 | Tunnel Information per QoS Flow |
| TargetDnaiInfo | 6.1.6.2.68 | Target DNAI Information |
| AfCoordinationInfo | 6.1.6.2.69 | AF Coordination Information |
| NotificationInfo | 6.1.6.2.70 | Notification Correlation ID and Notification URI provided by the NF service consumer |
| AnchorSmfFeatures | 6.1.6.2.71 | Anchor SMF supported features |
| HrsboInfoFromVplmn | 6.1.6.2.72 | HR-SBO Information signaled from VPLMN |
| HrsboInfoFromHplmn | 6.1.6.2.73 | HR-SBO Information signaled from HPLMN |
| EasInfoToRefresh | 6.1.6.2.74 | EAS information to be refreshed for EAS re-discovery |
| EcnMarkingCongestionInfoReq | 6.1.6.2.75 | ECN Marking or Congestion Information Request |
| EcnMarkingCongestionInfoStatus | 6.1.6.2.76 | ECN Marking or Congestion Information Status |
| TscAssistanceInformation | 6.1.6.2.77 | TSC Assistance Information |
| N6JitterInformation | 6.1.6.2.78 | N6 Jitter Information |
| TrafficInfluenceInfo | 6.1.6.2.79 | Traffic influence information applicable at the VPLMN for an HR-SBO PDU session. |
| TrafficInfluenceData | 6.1.6.2.80 | Traffic influence data comprising the Service Data Flow description and the Application Function influence on traffic routing Enforcement Control parameters of a PCC rule. |
| Teid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |
| EpsBearerContextStatus | 6.1.6.3.2 | EPS Bearer context status |
| DrbId | 6.1.6.3.2 | Data Radio Bearer Identifier |
| DnsServerSecurityInformation | 6.1.6.3.2 | DNS server security information |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |
| HoState | 6.1.6.3.4 | Handover State |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |
| Cause | 6.1.6.3.8 | Cause information |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication |
| SmContextType | 6.1.6.3.15 | Type of SM Context information |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed |
| N4MessageType | 6.1.6.3.17 | N4 Message Type |
| QosFlowAccessType | 6.1.6.3.18 | Access type associated with the QoS Flow |
| UnavailableAccessIndication | 6.1.6.3.19 | Indicates the access type of a MA PDU session that is unavailable |
| ProtectionResult | 6.1.6.3.20 | Protection Result of the security policy indicated as "preferred" |
| QosMonitoringReq | 6.1.6.3.21 | Indicates to measure UL, or DL, or both UL/DL delays, or to stop on-going measurements. |
| Rsn | 6.1.6.3.22 | Redundancy Sequence Number |
| SmfSelectionType | 6.1.6.3.23 | SMF Selection Type |
| PduSessionContextType | 6.1.6.3.24 | PDU Session Context Type |
| PendingUpdateInfo | 6.1.6.3.25 | Pending Update Information |
| EstablishmentRejectionCause | 6.1.6.3.26 | PDU Session Establishment Rejection Cause |
| EcnMarkingReq | 6.1.6.3.27 | ECN Marking Request |
| CongestionInfoReq | 6.1.6.3.28 | Congestion Information Request |
| ActivationStatus | 6.1.6.3.29 | Activation Status |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |
| PduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |
| PduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |
| UserLocation | 3GPP TS 29.571 [13] | User Location |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |
| RatType | 3GPP TS 29.571 [13] | RAT Type |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status |
| ApnRateStatus | 3GPP TS 29.571 [13] | APN Rate Control Status |
| StationaryIndication | 3GPP TS 29.571 [13] | Stationary Indication |
| ScheduledCommunicationTime | 3GPP TS 29.571 [13] | Scheduled Communication Time |
| ScheduledCommunicationType | 3GPP TS 29.571 [13] | Scheduled Communication Type |
| TrafficProfile | 3GPP TS 29.571 [13] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [13] | Battery Indication |
| NfSetId | 3GPP TS 29.571 [13] | NF Set Identifier |
| MoExpDataCounter | 3GPP TS 29.571 [13] | MO Exception Data Counter |
| DddTrafficDescriptor | 3GPP TS 29.571 [13] | Traffic Descriptor |
| NfServiceSetId | 3GPP TS 29.571 [13] | NF Service Set ID |
| RedirectResponse | 3GPP TS 29.571 [13] | Response body of the redirect response message |
| ServerAddressingInfo | 3GPP TS 29.571 [13] | Information of a Provisioning Server |
| PcfUeCallbackInfo | 3GPP TS 29.571 [13] | The callback information of the PCF for the UE to allow the PCF for the PDU session to send SM Policy Association Establishment and Termination events notification |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [13] | Satellite backhaul category |
| Uint16 | 3GPP TS 29.571 [13] | Unsigned 16-bit integer |
| GeoSatelliteId | 3GPP TS 29.571 [13] | GEO satellite ID (string) |
| Uinteger | 3GPP TS 29.571 [13] | Unsigned Integer common data type |
| VplmnOffloadingInfo | 3GPP TS 29.571 [13] | VPLMN Specific Offloading Information |
| GroupId | 3GPP TS 29.571 [13] | Internal Group Id |
| OffloadIdentifier | 3GPP TS 29.571 [13] | Offload identifier uniquely identifying a VPLMN offloading policy information instance from the HPLMN. |
| Ipv4AddressRange | 3GPP TS 29.571 [13] | IPv4 address range |
| Ipv6AddressRange | 3GPP TS 29.571 [13] | IPv6 address range |
| Fqdn | 3GPP TS 29.571 [13] | FQDN |
| PduSetQosPara | 3GPP TS 29.571 [13] | PDU Set QoS Parameters |
| SmfChargingId | 3GPP TS 29.571 [13] | SMF Charging Identifier |
| Int32 | 3GPP TS 29.571 [13] | Signed 32-bit integer |
| VplmnDlAmbr | 3GPP TS 29.571 [13] | Authorized DL Session AMBR for Offloading for the V-PLMN |
| EcsAddrConfigInfo | 3GPP TS 29.503 [46] | ECS Address Configuration Information |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |
| WAgfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the W-AGF |
| TngfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TNGF |
| TwifInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TWIF |
| ChargingInformation | 3GPP TS 29.512 [30] | CHF addresses |
| TrafficControlData | 3GPP TS 29.512 [30] | Contains parameters influencing the routing of traffic for service data flows. |
| FlowInformation | 3GPP TS 29.512 [30] | Ethernet or IP flow packet filter information |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |
| SbiBindingLevel | 3GPP TS 29.518 [20] | SBI Binding Level |
| IpIndex | 3GPP TS 29.519 [38] | Information that identifies which IP pool or external server is used to allocate the IP address. |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |
| UeLevelMeasurementsConfiguration | 3GPP TS 29.571 [13] | 5GC UE level measurements configuration |

* * * Next Changes * * * *

6.1.6.2.2	Type: SmContextCreateData

Table 6.1.6.2.2-1: Definition of type SmContextCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present, except if the UE is emergency registered and UICCless. When present, it shall contain the subscriber permanent identify. |  |
| unauthenticatedSupi | boolean | C | 0..1 | This IE shall be present if the SUPI is present in the message but is not authenticated and is for an emergency registered UE. When present, it shall be set as follows: - true: unauthenticated SUPI; - false (default): authenticated SUPI. |  |
| roamingUeInd | boolean | O | 0..1 | This IE may be present, when the requestType IE indicates that the request refers to an emergency PDU session and the supi IE is present. When present, this IE shall indicate whether the UE is a roaming UE or not: - true: the UE is a roaming UE. - false: the UE is a non-roaming UE. |  |
| pei | Pei | C | 0..1 | This IE shall be present if the UE is emergency registered and it is either UIClless or the SUPI is not authenticated. For all other cases, this IE shall be present if it is available. When present, it shall contain the permanent equipment identifier. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the PDU Session ID. |  |
| dnn | Dnn | C | 0..1 | This IE shall be present, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the requested DNN; the DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present during the PDU session establishment procedure. In this case, it shall contain the requested S-NSSAI for the serving PLMN. This corresponds to an S-NSSAI from the allowed NSSAI. This IE shall also be present during an EPS to 5GS idle mode mobility or handover with I-SMF/V-SMF involved using the N26 interface. In this case, it shall contain the S-NSSAI configured in the AMF for EPS interworking. This IE shall also be present during a V-SMF insertion or an inter PLMN V-SMF change procedures. When present, it shall contain the S-NSSAI for the serving PLMN which shall be used by the (target) V-SMF instead of the one that it receives in the SmContext from the old V-SMF or the H-SMF. |  |
| altSnssai | Snssai | C | 0..1 | This IE shall be present during the PDU session establishment procedure if the NF service consumer supports the network slice replacement, and the network slice indicated in the sNssai IE is requested to be replaced. In this case, the NF service consumer shall send the S-NSSAI and the alternative S-NSSAI. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a roaming PDU session, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain the requested S-NSSAI for the HPLMN. This corresponds to an S-NSSAI from the Mapping Of Allowed NSSAI corresponding to the SNSSAI value included in the sNssai IE. |  |
| altHplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a roaming PDU session during the PDU session establishment procedure if the NF service consumer supports the network slice replacement, and the network slice indicated in the hplmnSnssai IE is requested to be replaced. In this case, the NF service consumer shall send the HPLMN S-NSSAI and the alternative HPLMN S-NSSAI. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| servingNfId | NfInstanceId | M | 1 | This IE shall contain the identifier of the serving NF (e.g. serving AMF). |  |
| guami | Guami | C | 0..1 | This IE shall contain the serving AMF's GUAMI. It shall be included if the NF service consumer is an AMF. |  |
| serviceName | ServiceName | O | 0..1 | When present, this IE shall contain the name of the AMF service to which SM context status notifications are to be sent (see clause 6.5.2.2 of 3GPP TS 29.500 [4]). This IE may be included if the NF service consumer is an AMF. |  |
| servingNetwork | PlmnIdNid | M | 1 | This IE shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. |  |
| requestType | RequestType | C | 0..1 | This IE shall be present if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing emergency PDU session. The requestType IE shall not be included for a MA-PDU session establishment request. It may be present otherwise. When present, it shall indicate whether the request refers to a new PDU session or emergency PDU session, or to an existing PDU session or emergency PDU session. For request sent from UE, this IE shall be set based on the Request type IE received (see clause 9.11.3.47 of 3GPP TS 24.501 [7]). |  |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present and reference the N1 SM Message binary data (see clause 6.1.6.4.2), except during an EPS to 5GS Idle mode mobility or handover using N26. |  |
| anType | AccessType | M | 1 | This IE shall indicate the Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present if a MA-PDU session is requested and the UE is registered over both 3GPP access and Non-3GPP access. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available. |  |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present if the DNN corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. |  |
| perLadnDnnSnssaiInd | boolean | C | 0..1 | This IE shall be present, if the AMF enforces the LADN Service Area per LADN DNN and S-NSSAI (see clause 4.3.2.2.1 of 3GPP TS 23.502 [3]). When present, it shall indicate that the PDU Session is subject to LADN per LADN DNN and S-NSSAI and be set as follows: - True: the PDU Session is subject to LADN per LADN DNN and S-NSSAI. - False (default): the PDU Session is not subject to LADN per LADN DNN and S-NSSAI. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall contain the UE location information (see clause 5.2.3.4), if it is available. (NOTE 1). |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall contain the UE Time Zone, if it is available. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| smContextStatusUri | Uri | M | 1 | This IE shall include the callback URI to receive notification of SM context status. |  |
| hSmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios, including Indirect Communication with Delegated Discovery, if the AMF and V-SMF do not support the ACSCR feature. This IE shall be present in HR roaming scenarios during a PDU session establishment procedure and EPS to 5GS mobility procedures, if the AMF and V-SMF support the ACSCR feature. When present, it shall contain the API URI of the Nsmf_PDUSession service of the selected H-SMF. The API URI shall be formatted as specified in clause 6.1.1. (NOTE 8) |  |
| hSmfId | NfInstanceId | O | 0..1 | This IE may be present when hSmfUri is present. If present, this IE shall carry the NF instance ID of the selected H-SMF. (NOTE 2) |  |
| smfUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, including Indirect Communication with Delegated Discovery, if the AMF and I-SMF do not support the ACSCR feature. This IE shall be present for a PDU session with an I-SMF during a PDU session establishment procedure and EPS to 5GS mobility procedures, if the AMF and I-SMF support the ACSCR feature. When present, it shall contain the API URI of the Nsmf_PDUSession service of the selected SMF. The API URI shall be formatted as specified in clause 6.1.1. (NOTE 8) | DTSSA |
| smfId | NfInstanceId | O | 0..1 | This IE may be present when smfUri is present. If present, this IE shall carry the NF instance ID of the selected SMF. (NOTE 2) | DTSSA |
| oldPduSessionId | PduSessionId | C | 0..1 | This IE shall be present if this information is received from the UE. When present, it shall contain the old PDU Session ID received from the UE. See clauses 4.3.2.2.1 and 4.3.5.2 of 3GPP TS 23.502 [3]. |  |
| pduSessionsActivateList | array(PduSessionId) | C | 1..N | This IE shall be present, during an EPS to 5GS Idle mode mobility using the N26 interface, if the UE indicated PDU session(s) to be activated in the Registration Request. When present, it shall indicate all the PDU session(s) requested to be re-activated by the UE. |  |
| ueEpsPdnConnection | EpsPdnCnxContainer | C | 0..1 | This IE shall be present, during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain an MME/SGSN UE EPS PDN connection including the EPS bearer context(s). |  |
| hoState | HoState | C | 0..1 | This IE shall be present during an EPS to 5GS handover using N26 interface or during a N2 handover with I-SMF insertion/change/removal procedure, to request the preparation of a handover of the PDU session. When present, it shall be set as specified in clauses 5.2.2.2.3 or 5.2.2.2.5. |  |
| additionalHsmfUri | array(Uri) | O | 1..N | This IE may be present in HR roaming scenarios. When present, it shall contain an array of API URI of the Nsmf_PDUSession service of the additional H-SMFs discovered by the AMF for the given DNN, hplmnSnssai and for this PDU session. If provided, the V-SMF shall use these additional H-SMF(s) if the V-SMF is not able to receive any response from the H-SMF identified by hSmfUri. The API URI shall be formatted as specified in clause 6.1.1. |  |
| additionalHsmfId | array(NfInstanceId) | O | 1..N | This IE may be present when additionalHsmfUri is present. If present, this IE shall carry the NF instance ID(s) of H-SMF(s) as stated in additionalHsmfUri IE, in exactly the same order. (NOTE 2) |  |
| additionalSmfUri | array(Uri) | O | 1..N | This IE may be present for a PDU session with an I-SMF. When present, it shall contain an array of API URI of the Nsmf_PDUSession service of the additional SMFs discovered by the AMF for the given DNN, Snssai and for this PDU session. If provided, the I-SMF shall use these additional SMF(s) if the I-SMF is not able to receive any response from the SMF identified by smfUri. The API URI shall be formatted as specified in clause 6.1.1. | DTSSA |
| additionalSmfId | array(NfInstanceId) | O | 1..N | This IE may be present when additionalSmfUri is present. If present, this IE shall carry the NF instance ID(s) of SMF(s) as stated in additionalSmfUri IE, in exactly the same order. (NOTE 2) | DTSSA |
| pcfId | NfInstanceId | O | 0..1 | When present, this IE shall contain the identifier of: - the H-PCF selected by the AMF (for UE Policy), for a HR PDU session; or - the V-PCF selected by the AMF (for Access and Mobility Policy), for a PDU session in LBO roaming scenarios; or - the PCF selected by the AMF (for Access and Mobility Policy and/or UE Policy), for a PDU session in non-roaming scenarios. |  |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | This IE may be present if pcfId IE is present. When present, this IE shall contain the NF Set ID of the PCF indicated by the pcfId IE. |  |
| nrfUri | Uri | O | 0..1 | This IE may be present to indicate the NRF to use for PCF selection within the same network slice instance. When present, the SMF shall use the NRF URI to select the PCF. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.8 is supported. |  |
| anchorSmfFeatures | SupportedFeatures | C | 0..1 | This IE shall be present, during the establishment of a HR PDU session or a PDU session with an I-SMF, if the information is known by the AMF. When present, this IE shall include the features of the Nsmf_PDUSession service (see clause 6.1.8) that are supported by anchor SMF. The absence of this IE shall not be interpreted as indicating that the anchor SMF does not support any feature. |  |
| additional AnchorSmfFeatures | array(SupportedFeatures) | C | 1..N | This IE shall be present, during the establishment of a HR PDU session or a PDU session with an I-SMF, if the additionalHsmfUri IE or the additionalSmfUri IE is present and the information is known for all the additional anchor SMFs. When present, this IE shall include the features of the Nsmf_PDUSession service (see clause 6.1.8) that are supported by the additional anchor SMFs, in exactly the same order. The absence of this IE shall not be interpreted as indicating that the additional anchor SMFs do not support any feature. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by the network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| backupAmfInfo | array(BackupAmfInfo) | C | 1..N | This IE shall be included if the NF service consumer is an AMF and the AMF supports the AMF management without UDSF for the following cases: - First interaction with SMF. - Modification of the BackupAmfInfo. |  |
| traceData | TraceData | C | 0..1 | This IE shall be included if trace is required to be activated (see 3GPP TS 32.422 [22]). |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 3) |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | The AMF may provide the indication when a PGW-C+SMF is selected to serve the PDU Session. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS or EPC/ePDG and whether N26 interface to be used during EPS interworking procedures. The AMF may derive the value of the indication from different sources, like UE 5GMM capabilities (e.g. "S1 mode supported"), UE subscription data (e.g. "Core Network Type Restriction to EPC" and "Interworking with EPS Indication" for the DNN) and configurations. |  |
| indirectForwardingFlag | boolean | C | 0..1 | The AMF shall include this indication during N26 based Handover procedure from EPS to 5GS (see 3GPP TS 23.502 [3], clause 4.11.1.2.2), to inform the SMF of the applicability or non-applicability of indirect data forwarding. When present, it shall be set as follows: - True: indirect data forwarding is applicable - False: indirect data forwarding is not applicable |  |
| directForwardingFlag | boolean | C | 0..1 | The AMF shall include this indication during N26 based Handover procedure from EPS to 5GS (see 3GPP TS 23.502 [3], clause 4.11.1.2.2), to inform the SMF of the applicability or non-applicability of direct data forwarding. When present, it shall be set as follows: - True: direct data forwarding is applicable - False: direct data forwarding is not applicable |  |
| targetId | NgRanTargetId | C | 0..1 | This IE shall be present in the following cases: - during an EPS to 5GS handover preparation using the N26 interface, when the hoState IE is set to the value "PREPARING"; - during N2 based handover procedure with I-SMF or V-SMF insertion/change/removal, when hostate IE is set to the value "PREPARING". When present, it shall contain the Target ID identifying the target RAN Node ID and TAI. In case of EPS to 5GS handover, the TAI is received in the Forward Relocation Request from the Source MME. |  |
| epsBearerCtxStatus | EpsBearerContextStatus | C | 0..1 | This IE shall be present during an EPS to 5GS idle mode mobility using the N26 interface, if received in the Registration Request from the UE. When present, it shall be set to the value received from the UE. |  |
| cpCiotEnabled | boolean | C | 0..1 | This IE shall be present with the value "True", if - the NF service consumer (e.g. the AMF) has verified that the CIOT feature is supported by the SMF (and for a home-routed session, that it is also supported by the H-SMF); and - Control Plane CIoT 5GS Optimisation is enabled for the PDU session (see 3GPP TS 23.502 [3], clauses 4.3.2.2.1 and 4.3.2.2.2). When present, it shall be set as follows: - True: Control Plane CIoT 5GS Optimisation is enabled. - False (default): Control Plane CIoT 5GS Optimisation is not enabled. | CIOT |
| cpOnlyInd | boolean | C | 0..1 | This IE shall be present with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation (see clause 5.31.4.1 of 3GPP TS 23.501 [2]). When present, it shall be set as follows: - True: the PDU session shall only use Control Plane CIoT 5GS Optimisation - False (default): the PDU session is not constrained to only use Control Plane CIoT 5GS Optimisation. | CIOT |
| invokeNef | boolean | C | 0..1 | This IE shall be present with the value "True", if Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for the PDU session (see 3GPP TS 23.502 [3], clause 4.3.2.2.2). When present, it shall be set as follows: - True: Data delivery via NEF is selected. - False (default): Data delivery via NEF is not selected. | CIOT |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established. When present, it shall be set as follows: - True: a MA-PDU session is requested - False (default): a MA-PDU session is not requested | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall only be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - True: the PDU session is allowed to be upgraded to MA PDU session - False (default): the PDU session is not allowed to be upgraded to MA PDU session When maRequestInd is present and set to "true", this IE shall not be present. | MAPDU |
| n3gPathSwitchSupportInd | boolean | C | 0..1 | This IE shall be present if AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the selected SMF supports non-3GPP path switching and if the UE supports non-3GPP access path switching for a MA-PDU session. When present, it shall be set as follows: - true: non-3GPP access path switching is supported - false (default): non-3GPP access path switching is not supported | N3GPS |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information needs to be sent to the I-SMF. | DTSSA |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. | DTSSA |
| n2SmInfoExt1 | RefToBinaryData | C | 0..1 | This IE shall be present if more than one N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). | DTSSA |
| n2SmInfoTypeExt1 | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfoExt1" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfoExt1" attribute. | DTSSA |
| smContextRef | Uri | C | 0..1 | This IE shall be present during an I-SMF or V-SMF insertion if available and during an I-SMF or V-SMF change or removal. When present, this IE shall contain the URI of the SM Context resource in the SMF or of the SM context resource in the source I-SMF or V-SMF during an I-SMF or V-SMF insertion or during an I-SMF or V-SMF change/removal respectively. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.3.2). (NOTE 6) | DTSSA |
| smContextSmfPlmnId | PlmnIdNid | C | 0..1 | This IE shall be present during an inter-PLMN mobility procedure if the smContextRef IE is present. It may be present otherwise, if the smContextRef IE is present. When present, this IE shall carry the PLMN ID of the SMF which hosts the SM Context resource identified by smContextRef IE. For an SNPN, the NID together with the PLMN ID shall identify the SNPN. (NOTE 7) | DTSSA |
| smContextSmfId | NfInstanceId | O | 0..1 | This IE may be present if smContextRef is present. When present, this IE shall carry the NF instance ID of the SMF which hosts the SM Context resource identified by smContextRef IE. (NOTE 2) | DTSSA |
| smContextSmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the old V-SMF or the old I-SMF or the SMF as identified by the smContextSmfId. | DTSSA |
| smContextSmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the PDUSession service instance (for this SmContext) in the old V-SMF or the old I-SMF or the SMF. | DTSSA |
| smContextSmfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the SBI binding level of the SM context resource. | DTSSA |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to request the activation of the user plane connection of the PDU session, in the following cases: - during Service Request procedure with an I-SMF insertion / change / removal, or with a V-SMF change (see clause 5.2.2.2.6); - during Registration procedure with an I-SMF insertion / change / removal, or with a V-SMF insertion / change / removal (see clause 5.2.2.2.7), if this PDU session is requested to be activated by the UE. | DTSSA |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the small data rate control status is available in AMF, see clause 5.31.14.3 of 3GPP TS 23.501 [2] and clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present if the APN rate control status is available in AMF, see clause 4.7.7.3 in 3GPP TS 23.401 [33] and clause 5.2.8.2.5 in 3GPP TS 23.502 [3]. | CIOT |
| extendedNasSmTimerInd | boolean | C | 0..1 | This IE shall be present with the value "True" if the UE supports CE mode B and use of CE mode B is not restricted according to the Enhanced Coverage Restriction information in the UE context in the AMF. When present, it shall indicate whether extended NAS SM timers shall be used for the UE as specified in 3GPP TS 24.501 [7], as follows: - True: extended NAS SM timers shall be used - False (default): normal NAS SM timers shall be used. | CIOT |
| dlDataWaitingInd | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.3A of 3GPP TS 23.502 [3]), if the same indication is received from the MME in the Context Response message. When present, it shall be set as follows: - true: DL data needs to be sent to the UE; - false (default): no DL data needs to be sent to the UE. | CIOT |
| ddnFailureSubs | DdnFailureSubs | C | 0..1 | This IE shall be present to subscribe the notification of the DDN Failure if the Availability after DDN failure event is subscribed by the UDM, see clause 4.15.3.2.7 of 3GPP TS 23.502 [3]. | CIOT |
| smfTransferInd | boolean | C | 0..1 | This IE shall be present during an SMF Context Transfer procedure, LBO or no Roaming, no I-SMF. When present, it shall be set as follows: - True: SMF Context Transfer - False (default): Not an SMF Context Transfer | CTXTR |
| oldSmfId | NfInstanceId | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, it shall indicate old SMF instance identifier. | CTXTR |
| oldSmContextRef | Uri | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, this IE shall contain the identifier of the SM Context resource in the old SMF. This IE shall also be present, without smfTransferInd set, if this information was received earlier in Notify SM Context Status, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. | CTXTR EnEDGE |
| wAgfInfo | WAgfInfo | C | 0..1 | This IE shall be present, if received from the W-AGF. When present, it shall contain information about the N3 terminations of the W-AGF. The SMF may use this information when selecting the UPF. |  |
| tngfInfo | tngfInfo | C | 0..1 | This IE shall be present, if received from the TNGF. When present, it shall contain information about the N3 terminations of the TNGF. The SMF may use this information when selecting the UPF. |  |
| twifInfo | twifInfo | C | 0..1 | This IE shall be present, if received from the TWIF. When present, it shall contain information about the N3 terminations of the TWIF. The SMF may use this information when selecting the UPF. |  |
| ranUnchangedInd | boolean | C | 0..1 | This IE shall be present if the NG-RAN is not changed in case of I-SMF/V-SMF change or insertion during CM-CONNECTED registration procedure after EPS to 5GS handover (see clause 5.2.2.2.7) or I-SMF/V-SMF selection per DNAI (see clause 5.2.2.2.12). When present, it shall be set as follows: - true: NG-RAN is not changed; - false: NG-RAN is changed. | DTSSA |
| samePcfSelectionInd | boolean | C | 0..1 | This IE shall be present, if the AMF received the PCF Selection Assistance info from the UDM indicating that the same PCF is required. (NOTE 4) When present, it shall be set as follows: - True: the SMF is indicated to select the same PCF instance for the PDU session. - False (default): the SMF is not indicated to select the same PCF instance for the PDU session. |  |
| targetDnai | Dnai | C | 0..1 | This IE shall be present, if this information was received earlier in Notify SM Context Status. I-SMF or SMF shall use this information for I-UPF / L-PSA / PSA selection, see clauses 4.3.5.1, 4.3.5.2 or 4.23.5.4 of 3GPP TS 23.502 [3]. V-SMF shall use this information for V-UPF selection, see clause 6.7.3.2 of 3GPP TS 23.548 [74]. | EnEDGE HR-SBO |
| nrfManagementUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFManagement Service (see clause 6.1.1 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfDiscoveryUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFDiscovery Service (see clause 6.2.1 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfAccessTokenUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the Access Token Service (see clause 6.3.2 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfOauth2Required | map(boolean) | C | 1..N | This IE should be present if the nrfUri IE, nrfManagementUri IE or nrfDiscoveryUri IE is present and if the information is available. When present, this IE shall indicate whether the NRF requires Oauth2-based authorization for accessing its services. The key of the map shall be the name of an NRF service, e.g. "nnrf-nfm" or "nnrf-disc". The value of each entry of the map shall be encoded as follows: - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of NRF services. |  |
| smfBindingInfo | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the Binding indications of the SM context resource and shall be set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [4], without the header name. | DTSSA |
| pvsInfo | array(ServerAddressingInfo) | C | 1..N | This IE shall be present, if the AMF received this information from AUSF during User Plane Remote Provisioning of UEs procedure (see clause 5.30.2.10.4 of 3GPP TS 23.501 [40]). When present, this IE shall contain the remote Provisioning Server(s) information. | ENPN |
| onboardingInd | boolean | C | 0..1 | This IE shall be present, if the UE is registered for onboarding in an SNPN (see clause 5.30.2.10.4 in 3GPP TS 23.501 [40] and clause 4.2.2.2.4 in 3GPP TS 23.502 [3]). - false (default): The UE is not registered in an SNPN for the purpose of onboarding; - true: The UE has registered in the SNPN for the purpose of onboarding. | ENPN |
| oldPduSessionRef | Uri | C | 0..1 | This IE shall be present if this information was received earlier in Notify SM Context Status, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. When present, this IE shall contain the URI of the PDU session resource in the old SMF. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.6.2). | EnEDGE |
| smPolicyNotifyInd | boolean | O | 0..1 | When present, this IE shall indicate whether the SM Policy Association Establishment and Termination events shall be reported for the PDU session by the PCF for the SM Policy to the PCF for the UE: - true: SM Policy Association Establishment and Termination events shall be reported - false (default): SM Policy Association Establishment and Termination events is not required (NOTE 5) | SPAE |
| pcfUeCallbackInfo | PcfUeCallbackInfo | C | 0..1 | This IE shall be present when the smPolicyNotifyInd IE is present with value true. When present, this IE shall contain the callback information of the PCF for the UE to receive SM Policy Association Establishment and Termination events notification from the PCF for the SM Policy. (NOTE 5) | SPAE |
| satelliteBackhaulCat | SatelliteBackhaulCategory | O | 0..1 | This IE may be present if the AMF supports the 5GSAT feature and the AMF is aware that there is a satellite backhaul towards the 5G AN serving the UE. When present, this IE shall indicate the category of the satellite backhaul used towards the 5G AN serving the UE. | 5GSAT |
| upipSupported | boolean | C | 0..1 | This IE shall be present during the PDU session establishment procedure, if the UE supports User Plane Integrity Protection with EPS and if the AMF supports the related functionality. It may be present otherwise. When present, this IE shall be set as follows: - true: User Plane Integrity Protection with EPS is supported; - false (default): User Plane Integrity Protection with EPS is not supported. | UPIPE |
| disasterRoamingInd | boolean | O | 0..1 | This IE may be set when the UE is registered for Disaster Roaming service. When present, this IE shall be set as follows: - true: the UE is registered for Disaster Roaming service - false (default): the UE is not registered for Disaster Roaming service |  |
| anchorSmfOauth2Required | boolean | O | 0..1 | When present, this IE shall indicate whether the H-SMF or SMF for a PDU session with an I-SMF requires Oauth2-based authorization for accessing its Nsmf_PDUSession service. - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of the anchor SMF's Nsmf_PDUSession service. (NOTE 2) |  |
| smContextSmfOauth2Required | boolean | O | 0..1 | This IE may be present during an I-SMF insertion, change or removal. This IE may be present when V-SMF changes within the same PLMN. When present, this IE shall indicate whether the SMF hosting the SM Context requires Oauth2-based authorization for accessing its Nsmf_PDUSession service. - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of the Nsmf_PDUSession service on the SMF hosting the SM Context. (NOTE 2) |  |
| geoSatelliteId | GeoSatelliteId | O | 0..1 | The AMF may include this IE during a PDU Session Establishment procedure (see clause 4.3.2.2.1 of 3GPP TS 23.502 [3]). When present, this IE shall contain a GEO satellite ID. | 5GSATB |
| hrsboAllowedInd | boolean | O | 0..1 | This IE may be present in HR roaming scenarios. When present, this IE shall Indicate whether Session Breakout for HR Session in VPLMN is allowed: - true: Allowed. - false (default): Not allowed. | HR-SBO |
| estabRejectionInd | boolean | O | 0..1 | This IE may be present with the value true if the AMF has determined that the PDU Session Establishment shall be rejected, e.g. according to the ODB configuration of the UE retrieved from the UDM. Presence of this IE with the value false shall be prohibited. | PSER |
| estabRejectionCause | EstablishmentRejectionCause | C | 0..1 | This IE shall be included if the estabRejectionInd is present with the value true. When present, this IE shall indicate the cause of the PDU session establishment rejection. | PSER |
| sliceAreaRestrictInd | boolean | C | 0..1 | This IE shall be present and set to true if the AMF determines that the PDU Session is subject to area restriction for the S-NSSAI, i.e. if the S-NSSAI is part of the partially allowed NSSAI or if the support of the S-NSSAI is restricted to a NS-AoS. See clauses 5.15.17 and 5.15.18 of 3GPP TS 23.501 [2]. Presence of this IE with the false value shall be prohibited. | SAR |
| ueLevelMeasurementsConfiguration | UeLevelMeasurementsConfiguration | C | 0..1 | This IE shall be present if the signalling based UE Level Measurement task is activated. When present, this IE shall contain UE Level Measurement configuration data for UE (see clause 4.1.2.18 of 3GPP TS 32.422 [22]). |  |
| NOTE 1: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. In shared networks, when the AMF and SMF pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI or NCGI to the SMF. The Core Network Operator PLMN ID shall be communicated in the TAI and the Serving Network. NOTE 2: If the SMF is aware that Oauth is enabled for the indicated next hop SMF, e.g. when the anchorSmfOauth2Required IE and/or the smContextSmfOauth2Required IE are received with the value true or when received a "401 Unauthorized" response code from next hop SMF, the SMF shall use the NF instance Identifier to acquire the access token for the Nsmf_PduSession service on the indicated SMF. NOTE 3: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 4: If present, this attribute shall be used together with the PCF ID received from the AMF for selecting the same PCF instance for the PDU session. NOTE 5: If the AMF has received the callback information of the PCF for the UE together with the information of the PDU sessions (i.e. Slice and DNN combination) that are applicable for notification of SM Policy Association events, the AMF shall identify whether the non-roaming or local breakout PDU session to be created is applicable for SM Policy Association events, i.e, whether the slice and DNN combination of the PDU session is listed in the received PDU session information from the PCF for the UE. If the PDU session is applicable for notification of SM Policy Association events, the AMF shall include the smPolicyNotifyInd IE with the value "true" and the callback information of the PCF for the UE in the request. The SMF shall forward the callback information of the PCF for the UE to the PCF for SM Policy during SM Policy Association Establishment. See clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. NOTE 6: See NOTE 2 of Table 6.1.6.2.3-1. NOTE 7: If the PLMN ID of the SMF holding the SM context received in smContextSmfPlmnId attribute is different from the PLMN ID of the target V-SMF/I-SMF/anchor SMF, the target V-SMF/I-SMF/anchor SMF shall retrieve the SM Context from the SMF via the SEPP. In this case, the smContextSmfPlmnId attribute may also be used for the discovery and selection of the local SEPP. NOTE 8: The smfUri and hSmfUri attributes need not be included in Create SM Context request in procedures other than PDU session establishment procedure and EPS to 5GS mobility procedures if the NF Service Consumer (e.g. AMF) and I-SMF/V-SMF support the "ACSCR" feature. See clause 6.1.8. |  |  |  |  |  |

* * * Next Changes * * * *

6.1.6.2.4	Type: SmContextUpdateData

Table 6.1.6.2.4-1: Definition of type SmContextUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pei | Pei | C | 0..1 | This IE shall be present if it is available and has not been provided earlier to the SMF. When present, this IE shall contain the permanent equipment identifier. |  |
| servingNfId | NfInstanceId | C | 0..1 | This IE shall be present upon inter-AMF change or mobility, or upon a N2 handover execution with AMF change. When present, it shall contain the identifier of the serving NF (e.g. AMF). |  |
| smContextStatusUri | Uri | C | 0..1 | This IE shall be present if the servingNfId IE is present. It may be present otherwise. When present, this IE shall include the callback URI to receive notification of SM context status. |  |
| guami | Guami | C | 0..1 | This IE shall be present if the servingNfId of AMF is present. When present, it shall contain the serving AMF's GUAMI. |  |
| servingNetwork | PlmnIdNid | C | 0..1 | This IE shall be present if the servingNfId IE is present. When present, it shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. |  |
| backupAmfInfo | array(BackupAmfInfo) | C | 1..N | This IE shall be included for the modification of the BackupAmfInfo if the NF service consumer is an AMF and the AMF supports the AMF management without UDSF. For deleting the backupAmfInfo, it shall contain the Null value. |  |
| anType | AccessType | C | 0..1 | This IE shall be present upon a change of the Access Network Type associated to the PDU session, e.g. during a handover of the PDU session between 3GPP access and untrusted non-3GPP access (see clause 5.2.2.3.5.2). When present, this IE shall indicate the Access Network Type (3GPP or non-3GPP access) to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated, if the UE is registered over both 3GPP and non-3GPP accesses. This IE shall be present when the UE requests to establish resources for MA PDU session over the other access. | MAPDU |
| anTypeToReactivate | AccessType | C | 0..1 | This IE shall indicate the Access Network Type for which the UP connection is requested to be re-activated, for a MA PDU session. | MAPDU |
| anTypeOfN1N2Info | AccessType | C | 0..1 | This IE should be present if the n1SmMsg IE and/or the n2SmInfo IE is present, for a MA PDU session associated with both a 3GPP and a non-3GPP access type. When present, this IE shall indicate the Access Network Type from which the N1 and/or N2 information was received. |  |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available, upon a change of RAT Type. |  |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present during a Service Request procedure (see clause 5.2.2.3.2.2) ), an Xn handover (see clause 5.2.2.3.3) or a N2 handover execution (see clause 5.2.2.3.4.3), if the DNN of the PDU session corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall be present if it is available and if it needs to be reported to the SMF (e.g. the user location has changed or the user plane of the PDU session is deactivated). When present, this IE shall contain: - the UE location information (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the UeLocation information was acquired. (NOTE 1) |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall be present if it is available, the UE Time Zone has changed and needs to be reported to the SMF. When present, this IE shall contain the UE Time Zone. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to request the activation or the deactivation of the user plane connection of the PDU session. When present, it shall be set as specified in clauses 5.2.2.3.2, 5.2.2.3.15 and 5.2.2.3.16. |  |
| hoState | HoState | C | 0..1 | This IE shall be present to request the preparation, execution or cancellation of a handover of the PDU session. When present, it shall be set as specified in clause 5.2.2.3.4. |  |
| toBeSwitched | boolean | C | 0..1 | This IE shall be present during an Xn Handover (see clause 5.2.2.3.3) to request to switch the PDU session to a new downlink N3 tunnel endpoint. When present, it shall be set as follows: - true: request to switch to the PDU session. - false (default): no request to switch the PDU session. |  |
| failedToBeSwitched | boolean | C | 0..1 | This IE shall be present during an Xn Handover (see clause 5.2.2.3.3) if the PDU session failed to be setup in the target RAN. When present, it shall be to true to indicate that the PDU session failed to be setup in the target RAN. |  |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present if N1 SM Information has been received from the UE. When present, this IE shall reference the N1 SM Message binary data (see clause 6.1.6.4.2). |  |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). |  |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |  |
| targetId | NgRanTargetId | C | 0..1 | This IE shall be present during a N2 handover preparation, when the hoState IE is set to the value "PREPARING". When present, it shall contain the Target ID identifying the target RAN Node ID and TAI received in the Handover Required from the Source RAN. |  |
| targetServingNfId | NfInstanceId | C | 0..1 | This IE shall be present during a N2 handover preparation with AMF change, when the hoState IE is set to the value "PREPARING". When present, it shall contain the identifier of the target serving NF (e.g. AMF). |  |
| dataForwarding | boolean | C | 0..1 | This IE shall be present and set as specified in clause 5.2.2.3.9 during a 5GS to EPS handover, or as specified in 5.2.2.3.13 during a N2 based handover with I-SMF insertion/change/removal, or as specified in clause 5.2.2.3.21 during N9 Forwarding Tunnel establishment between Branching Points or UL CLs controlled by different I-SMFs. When present, it shall be set as follows: - true: setup the direct or indirect data forwarding tunnels; - false (default): data forwarding tunnels are not required to be setup. |  |
| n9ForwardingTunnel | TunnelInfo | C | 0..1 | This IE shall be present in the following case: - UE triggered Service Request with I-SMF change/removal, if requesting to forward buffered downlink data packets at I-UPF (See clause 4.23.4 of 3GPP TS 23.502 [3]). When present, it shall carry the N9 forwarding tunnel info of I-UPF. | DTSSA |
| n9DlForwardingTnlList | array (IndirectDataForwardingTunnelInfo) | C | 1..N | This IE shall be present in the following case: - N2 based handover with I-SMF insertion/change/removal, if downlink indirect data forwarding tunnels are requested to be established between target I-UPF and source I-UPF / source UPF (see clause 4.23.7 and 4.23.11 of 3GPP TS 23.502 [3]). When present, it shall carry the list of N9 downlink indirect data forwarding tunnel(s) info of I-UPF. | DTSSA |
| n9UlForwardingTnlList | array (IndirectDataForwardingTunnelInfo) | C | 1..N | This IE shall be present in the following case: - N2 based handover with I-SMF insertion/change/removal, if uplink indirect data forwarding tunnels are requested to be established between target I-UPF and source I-UPF / source UPF (see clause 4.23.7 and 4.23.11 of 3GPP TS 23.502 [3]). When present, it shall carry the list of N9 uplink indirect data forwarding tunnel(s) info of I-UPF. | DTSSA |
| n9DlForwardingTunnel | TunnelInfo | C | 0..1 | This IE shall be present in the following case: - simultaneous change of Branching Points or UL CLs controlled by different I-SMFs, if downlink data forwarding tunnel is requested to be established from source BP / UL CL to target BP / UL CL (see clause 4.23.9.5 of 3GPP TS 23.502 [3]). When present, it shall carry the N9 downlink data forwarding tunnel info of the target BP / UL CL. | N9FSC |
| n9InactivityTimer | DurationSec | O | 0..1 | When present, this IE shall indicate an inactivity detection timer, in seconds, that the I-SMF may use to set the N9 forwarding tunnel inactive traffic detection timer in Branching Point or UL CL as specified in clause 4.23.9.5 of 3GPP TS 23.502 [3]. | N9FSC |
| epsBearerSetup | array(EpsBearerContainer) | C | 0..N | This IE shall be present during a 5GS to EPS handover using the N26 interface. When present, it shall include the EPS bearer context(s) successfully setup in EPS. The array shall be empty if no resource was successfully allocated in EPS for any PDU session. |  |
| revokeEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present to request the SMF to revoke some EBIs (see clause 4.11.1.4.1 of 3GPP TS 23.502 [3]). When present, it shall contain the EBIs to revoke. |  |
| release | boolean | C | 0..1 | This IE shall be used to indicate a network initiated PDU session release is requested. This IE shall be present and set as specified in clause 5.2.2.3.10 during P-CSCF restoration procedure, in clause 5.2.2.3.11 during AMF requested PDU Session Release due to duplicated PDU Session Id, in clause 5.2.2.3.12 during AMF requested PDU Session Release due to slice not available, in clause 5.2.2.3.17 during AMF requested PDU Session Release due to Network Slice-Specific Authentication and Authorization failure or revocation, in clause 5.2.2.3.19 during AMF requested PDU Session Release due to Control Plane Only indication associated with PDU Session is not applicable any longer, in clause 5.2.2.3.20 during AMF requested PDU Session Release due to ODB changes, and in clause 5.2.2.3.27 during AMF requested PDU Session Release due to MBSR not authorized. When present, it shall be set as follows: - true: PDU session release is required; - false (default): PDU session release is not required. |  |
| cause | Cause | O | 0..1 | When present, this IE shall indicate the cause for the requested modification, e.g. the NF Service Consumer cause for requesting to deactivate the user plane connection of the PDU session. |  |
| ngApCause | NgApCause | C | 0..1 | This IE shall be present, if the information is available. When present, this IE shall indicate the cause for the requested modification, e.g. the NGAP cause for requesting to deactivate the user plane connection of the PDU session. |  |
| 5gMmCauseValue | 5GMmCause | C | 0..1 | This IE shall be included if the AMF received a 5GMM cause code from the UE during any network initiated PDU session modification or release procedure. (e.g 5GMM Status message in response to a Downlink NAS Transport message carrying 5GSM payload). |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present and sent to the V-SMF, during an EPS to 5GS mobility registration using the N26 interface, if the S-NSSAI for the serving PLMN derived from the S-NSSAI of the home PLMN differs from the S-NSSAI provided in the Create SM Context Request. When present, it shall contain the S-NSSAI for the serving PLMN. For the Network slice replacement if nsReplTerminInd attribute is set to true, the sNssai attribute shall indicate the replaced SNSSAI. |  |
| traceData | TraceData | C | 0..1 | This IE shall be included if trace is required to be activated, modified or deactivated (see 3GPP TS 32.422 [22]). For trace modification, it shall contain a complete replacement of trace data. For trace deactivation, it shall contain the Null value. |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | This IE may be present if its value has changed after session creation or last update. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS or EPC/ePDG and whether N26 interface to be used during EPS interworking procedures. |  |
| anTypeCanBeChanged | boolean | C | 0..1 | This IE shall be present and set to true to indicate that the Access Network Type associated to the PDU session can be changed (see clause 5.2.2.3.2.4), during a Service Request procedure (see clause 4.2.3.2 of 3GPP TS 23.502 [3])), in response to paging or NAS notification indicating non-3GPP access, when the PDU Session for which the UE was paged or notified is in the List Of Allowed PDU Sessions provided by the UE, and the AMF received N2 SM Information only or N1 SM Container and N2 SM Information from the SMF in step 3a of clause 4.2.3.3 of 3GPP TS 23.502 [3]. When present, it shall be set as follows: - true: the access type of the PDU session can be changed. - false (default): the access type of the PDU session cannot be changed. |  |
| n2SmInfoExt1 | RefToBinaryData | C | 0..1 | This IE shall be present if more than one N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). |  |
| n2SmInfoTypeExt1 | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfoExt1" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfoExt1" attribute. |  |
| maReleaseInd | MaReleaseIndication | C | 0..1 | This IE shall be present if one access of a MA PDU session is requested to be released, in the following cases: - when UE/AMF initiates MA PDU session release over one access; or - when UE deregisters from one access. When present, it indicates the access to be released. | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 6.4.2.2 of 3GPP TS 24.501 [7]). When present, it shall be set as follows: - true: the PDU session is allowed to be upgraded to MA PDU session - false (default): the PDU session is not allowed to be upgraded to MA PDU session | MAPDU |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established (see clause 4.22.6.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: a MA-PDU session is requested - false (default): a MA-PDU session is not requested | MAPDU |
| n3gPathSwitchExecutionInd | boolean | O | 0..1 | This IE may be present and set to true if the AMF receives the indication "Non-3GPP access path switching while using old AN resources" in the registration request message from the UE and if the SMF supports non-3GPP path switching, so to request the SMF to add a new non-3GPP access path (while also keeping the existing one) during a UE requested non-3GPP access switching for a MA-PDU session. | N3GPS |
| exemptionInd | ExemptionInd | C | 0..1 | This IE shall be present if the AMF has exempted the NAS message from a NAS SM congestion control activated in the AMF. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if the servingNfId or the targetServingNfId is present (i.e. during a change of AMF) and at least one feature defined in clause 6.1.8 is supported by the new AMF. If this IE is absent when the servingNfId or the targetServingNfId is present, the new serving AMF or the target AMF respectively shall be considered as not supporting any feature. |  |
| moExpDataCounter | MoExpDataCounter | C | 0..1 | This IE shall be included if the UE has accessed the network by using "MO exception data" RRC establishment cause and when the AMF decides to send a non-zero value to the SMF. (NOTE 2) When present, this IE shall contain the MO Exception Data Counter. | CIOT |
| extendedNasSmTimerInd | boolean | C | 0..1 | This IE shall be present if the UE supports CE mode B and use of CE mode B changes from restricted to unrestricted or vice versa in the Enhanced Coverage Restriction information in the UE context in the AMF. When present, it shall indicate whether extended NAS SM timers shall be used for the UE as specified in 3GPP TS 24.501 [7], as follows: - True: extended NAS SM timers shall be used - False: normal NAS SM timers shall be used. | CIOT |
| forwardingFTeid | Bytes | C | 0..1 | This IE shall be present during a 5GS to EPS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.2A of 3GPP TS 23.502 [3]), if the Forwarding F-TEID IE is present in the Context Acknowledge message received from the MME. When present, it shall contain Base64-encoded characters, encoding the Forwarding F-TEID in the Context Acknowledge message, as specified in Figure 8.22-1 of 3GPP TS 29.274 [16] (starting from octet 1). | CIOT |
| forwardingBearerContexts | array(ForwardingBearerContainer) | C | 1..N | This IE shall be present during a 5GS to EPS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.2A of 3GPP TS 23.502 [3]), if the Bearer Contexts IE is present in the Context Acknowledge message received from the MME. When present, it shall contain the Bearer Contexts in the Context Acknowledge message. | CIOT |
| ddnFailureSubs | DdnFailureSubs | C | 0..1 | This IE shall be present to subscribe or unsubscribe to the notification of the DDN Failure if the Availability after DDN failure event is subscribed/unsubscribed by the UDM, see clause 4.15.3.2.7 of 3GPP TS 23.502 [3]. This IE shall also be present if it is required to add, modify or remove DDN failure subscriptions. If it is present and the included dnnFailureSubsInd indicates notification of DDN failure is subscribed, the content of the received ddnFailureSubs shall overwrite any ddnFailureSubs received earlier. | CIOT |
| skipN2PduSessionResRelInd | boolean | O | 0..1 | This IE may be present when the release IE is present with value "true". When present, this IE shall indicate whether N2 message shall be skipped for the PDU session RAN resources release, if the UP connection is active: - true: N2 message shall be skipped. - false (default): N2 message shall not be skipped. |  |
| secondaryRatUsageDataReportContainer | array(SecondaryRatUsageDataReportContainer) | C | 1..N | The IE shall be present during an EPS to 5GS handover procedure, if one or more instance of Secondary RAT Usage Data Report IE(s) are present and applicable to the PDU session. When present, it shall contain Base64-encoded characters, encoding the Secondary RAT Usage Data Report in the Forward Relocation Complete Acknowledge message, as specified in Figure 8.132-1 of 3GPP TS 29.274 [16] (starting from octet 1). |  |
| smPolicyNotifyInd | boolean | O | 0..1 | When present, this IE shall indicate that the SM Policy Association Establishment and Termination events shall be reported for the PDU session by the PCF for the SM Policy to the PCF for the UE: - true: SM Policy Association Establishment and Termination events shall be reported (NOTE 3) | SPAE |
| pcfUeCallbackInfo | PcfUeCallbackInfo | C | 0..1 | This IE shall be present when the smPolicyNotifyInd IE is present with value true. When present, this IE shall contain the callback information of the PCF for the UE to receive SM Policy Association Establishment and Termination events notification from the PCF for the SM Policy. (NOTE 3) | SPAE |
| satelliteBackhaulCat | SatelliteBackhaulCategory | O | 0..1 | This IE may be present if the AMF and the SMF supports the 5GSAT feature and the AMF is aware that: - there is a change of the satellite backhaul category; or - the UE is newly served by a 5G-AN without any satellite backhaul and a satellite backhaul category had been signalled to the SMF; or - the UE is newly served by a 5G-AN with a satellite backhaul and no satellite backhaul category had been signalled to the SMF; or - there is a satellite backhaul towards the 5G AN serving the UE, but it does not know whether a satellite backhaul category had been signalled to the SMF (e.g. upon Inter-AMF mobility). When present, this IE shall indicate the category of the satellite backhaul used towards the new 5G AN serving the UE, or that there is no longer any satellite backhaul towards the new 5G AN serving the UE. | 5GSAT |
| cnBasedMt | boolean | O | 0..1 | When present, this IE shall indicate to the SMF that UE is in RRC_INACTIVE mode with eDRX and CN Based MT handling is applied for the PDU session, i.e. the user plane connection is suspended and the UPF is requested to buffer subsequent DL Data and send a report upon subsequent DL data arrival, the SMF will then invoke the AMF EnableUEReachability service, see also clauses 4.8.1.1a and 4.8.2.2b of 3GPP TS 23.502 [3]. - true: CN Based MT handling is to be applied. |  |
| geoSatelliteId | GeoSatelliteId | C | 0..1 | The AMF shall include this attribute when the Geo satellite ID changes. When present, this IE shall contain a GEO satellite ID. | 5GSATB |
| altSnssai | Snssai | C | 0..1 | This IE shall be present when the PDU Session is to be transferred to an alternative S-NSSAI. When present, this IE shall indicate the alternative network slice to be used by the PDU session. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| altHplmnSnssai | Snssai | C | 0..1 | This IE shall be present for HR PDU session when the PDU Session is to be transferred to an alternative S-NSSAI in HPLMN. When present, this IE shall indicate the alternative network slice to be used by the PDU session in the HPLMN. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| nsReplTerminInd | boolean | C | 0..1 | This IE shall be present for a notification of termination of Network Slice Replacement, if the PDU Session is associated with the Alternative S-NSSAI and the replaced S-NSSAI is available again. The SMF shall transfer the PDU Session to the replaced S-NSSAI. When present, it shall be set as follows: - true: the network slice replacement is terminated The presence of this IE with false value shall be prohibited. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| sliceAreaRestrictInd | boolean | C | 0..1 | This IE shall be present if the AMF determines that the PDU Session is newly subject to area restriction for the S-NSSAI (i.e. if the S-NSSAI is newly part of the partially allowed NSSAI or if the support of the S-NSSAI is newly restricted to a NS-AoS) or if the PDU session becomes no longer subject to area restriction for the S-NSSAI. See clauses 5.15.17 and 5.15.18 of 3GPP TS 23.501 [2]. When present, it shall be set as follows: true: the PDU Session is newly subject to area restriction for the S-NSSAI false: the PDU Session is no longer subject to area restriction | SAR |
| ueLevelMeasurementsConfiguration | UeLevelMeasurementsConfiguration | C | 0..1 | This IE shall be present if the signalling based UE Level Measurement task is activated. When present, this IE shall contain UE Level Measurement configuration data for UE (see clause 4.1.2.18 of 3GPP TS 32.422 [22]). |  |
| NOTE 1: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. In shared networks, when the AMF and SMF pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI or NCGI to the SMF. The Core Network Operator PLMN ID shall be communicated in the TAI and the Serving Network. NOTE 2: The AMF increments the MO Exception Data Counter when the UE establishes/resumes RRC with "MO Exception Data" RRC cause. The AMF may defer sending the moExpDataCounter attribute to the SMF based on local configuration. The AMF resets the MO Exception Data Counter when receiving successful response from the SMF. The SMF however keeps incrementing the counter locally. NOTE 3: If the AMF has received the callback information of the PCF for the UE together with the information of the PDU sessions (i.e. Slice and DNN combination) that are applicable for notification of SM Policy Association events, the AMF shall identify whether any ongoing non-roaming or local breakout PDU session is applicable for SM Policy Association events, i.e, whether the slice and DNN combination of the PDU session is listed in the received PDU session information from the PCF for the UE. If the PDU session is applicable for notification of SM Policy Association events , the AMF shall invoke Update SM context service operation for the PDU session and include the smPolicyNotifyInd IE with the value "true" and the callback information of the PCF for the UE in the request. The SMF shall forward the callback information of the PCF for the UE to the PCF for SM Policy if exists via SM Policy Association Modification. See clause 4.3.3.2 of 3GPP TS 23.502 [3]. The AMF needs not update the SMF if the subscription to the SM Policy Association events for the PDU session is cancelled by the PCF for UE. |  |  |  |  |  |

* * * Next Changes * * * *

A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.3.0'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.502 V18.7.0; 5G System; Session Management Services; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.502/

servers:

- url: '{apiRoot}/nsmf-pdusession/v1'

variables:

apiRoot:

default: https://example.com

description: >

apiRoot as defined in clause 4.4 of 3GPP TS 29.501. The sm-contexts and pdu-sessions

resources can be distributed on different processing instances or hosts. Thus the

authority and/or deployment-specific string of the apiRoot of the created individual

sm context and pdu-session resources' URIs may differ from the authority and/or

deployment-specific string of the apiRoot of the sm-contexts and pdu-sessions

collections' URIs.

[…]

# STRUCTURED DATA TYPES

#

SmContextCreateData:

description: Data within Create SM Context Request

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

unauthenticatedSupi:

type: boolean

default:  false

roamingUeInd:

type: boolean

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

altSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

altHplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

requestType:

$ref: '#/components/schemas/RequestType'

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

perLadnDnnSnssaiInd:

type: boolean

default: false

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

smContextStatusUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

additionalHsmfUri:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

additionalHsmfId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

additionalSmfUri:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

additionalSmfId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

oldPduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

pduSessionsActivateList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

minItems: 1

ueEpsPdnConnection:

$ref: '#/components/schemas/EpsPdnCnxContainer'

hoState:

$ref: '#/components/schemas/HoState'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

nrfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

anchorSmfFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

additionalAnchorSmfFeatures:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

minItems: 1

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

backupAmfInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BackupAmfInfo'

minItems: 1

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

indirectForwardingFlag:

type: boolean

directForwardingFlag:

type: boolean

targetId:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/NgRanTargetId'

epsBearerCtxStatus:

$ref: '#/components/schemas/EpsBearerContextStatus'

cpCiotEnabled:

type: boolean

default: false

cpOnlyInd:

type: boolean

default: false

invokeNef:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

maNwUpgradeInd:

type: boolean

default: false

n3gPathSwitchSupportInd:

type: boolean

default: false

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

n2SmInfoExt1:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoTypeExt1:

$ref: '#/components/schemas/N2SmInfoType'

smContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smContextSmfPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

smContextSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smContextSmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

smContextSmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

smContextSmfBinding:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/SbiBindingLevel'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

extendedNasSmTimerInd:

type: boolean

default: false

dlDataWaitingInd:

type: boolean

default: false

ddnFailureSubs:

$ref: '#/components/schemas/DdnFailureSubs'

smfTransferInd:

type: boolean

default: false

oldSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

oldSmContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

wAgfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/WAgfInfo'

tngfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TngfInfo'

twifInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TwifInfo'

ranUnchangedInd:

type: boolean

samePcfSelectionInd:

type: boolean

default: false

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

nrfManagementUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfDiscoveryUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfAccessTokenUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfOauth2Required:

type: object

description: 'Map indicating whether the NRF requires Oauth2-based authorization for

accessing its services. The key of the map shall be the name of an NRF service, e.g.

"nnrf-nfm" or "nnrf-disc"'

additionalProperties:

type: boolean

minProperties: 1

smfBindingInfo:

type: string

pvsInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServerAddressingInfo'

minItems: 1

onboardingInd:

type: boolean

default: false

oldPduSessionRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smPolicyNotifyInd:

type: boolean

default: false

pcfUeCallbackInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

upipSupported:

type: boolean

default: false

disasterRoamingInd:

type: boolean

default: false

anchorSmfOauth2Required:

type: boolean

smContextSmfOauth2Required:

type: boolean

geoSatelliteId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GeoSatelliteId'

hrsboAllowedInd:

type: boolean

default: false

estabRejectionInd:

type: boolean

enum:

- true

estabRejectionCause:

$ref: '#/components/schemas/EstablishmentRejectionCause'

sliceAreaRestrictInd:

type: boolean

enum:

- true

ueLevelMeasurementsConfiguration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UeLevelMeasurementsConfiguration'

required:

- servingNfId

- servingNetwork

- anType

- smContextStatusUri

SmContextUpdateData:

description: Data within Update SM Context Request

type: object

properties:

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

backupAmfInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BackupAmfInfo'

minItems: 1

nullable: true

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

anTypeToReactivate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

anTypeOfN1N2Info:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

hoState:

$ref: '#/components/schemas/HoState'

toBeSwitched:

type: boolean

default: false

failedToBeSwitched:

type: boolean

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

targetId:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/NgRanTargetId'

targetServingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smContextStatusUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

dataForwarding:

type: boolean

default: false

n9ForwardingTunnel:

$ref: '#/components/schemas/TunnelInfo'

n9DlForwardingTnlList:

type: array

items:

$ref: '#/components/schemas/IndirectDataForwardingTunnelInfo'

minItems: 1

n9UlForwardingTnlList:

type: array

items:

$ref: '#/components/schemas/IndirectDataForwardingTunnelInfo'

minItems: 1

n9DlForwardingTunnel:

$ref: '#/components/schemas/TunnelInfo'

n9InactivityTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

epsBearerSetup:

type: array

items:

$ref: '#/components/schemas/EpsBearerContainer'

minItems: 0

revokeEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

release:

type: boolean

default: false

cause:

$ref: '#/components/schemas/Cause'

ngApCause:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

5gMmCauseValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GMmCause'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

anTypeCanBeChanged:

type: boolean

default: false

n2SmInfoExt1:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoTypeExt1:

$ref: '#/components/schemas/N2SmInfoType'

maReleaseInd:

$ref: '#/components/schemas/MaReleaseIndication'

maNwUpgradeInd:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

n3gPathSwitchExecutionInd:

type: boolean

enum:

- true

exemptionInd:

$ref: '#/components/schemas/ExemptionInd'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

moExpDataCounter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MoExpDataCounter'

extendedNasSmTimerInd:

type: boolean

forwardingFTeid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

forwardingBearerContexts:

type: array

items:

$ref: '#/components/schemas/ForwardingBearerContainer'

minItems: 1

ddnFailureSubs:

$ref: '#/components/schemas/DdnFailureSubs'

skipN2PduSessionResRelInd:

type: boolean

default: false

secondaryRatUsageDataReportContainer:

type: array

items:

$ref: '#/components/schemas/SecondaryRatUsageDataReportContainer'

minItems: 1

smPolicyNotifyInd:

type: boolean

enum:

- true

pcfUeCallbackInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

cnBasedMt:

type: boolean

enum:

- true

geoSatelliteId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GeoSatelliteId'

altSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

altHplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nsReplTerminInd:

type: boolean

enum:

- true

sliceAreaRestrictInd:

type: boolean

ueLevelMeasurementsConfiguration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UeLevelMeasurementsConfiguration'

[…]

* * * Next Changes * * * *

* * * Next Changes * * * *

* * * Next Changes * * * *

* * * End of Changes * * * *