3GPP TSG-CT WG4 Meeting #126	C4-245091

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1091 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N6 delay measurement protocols |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR 23.502 #4985 requests the possibility for a UPF to register the list of protocols it supports to measure the N6 delay between an external measurement endpoint and the UPF N6 interface. CR 23.501 #5443 further specifies: The SMF configures the candidate UPF(s) to perform the N6 delay measurement between the UPF and measurement endpoint and to report the N6 delay to SMF. The UPF leverages the measurement protocols (e.g., ICMP, TWAMP, OWAMP, STAMP, or other protocols) to measure the N6 delay. Then, the candidate UPF(s) report the N6 delay measurement to the SMF via N4 interface as described in clause 4.4.3 of TS 23.502 [3]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UPF profile is extended to support registering the list of N6 delay measurement protocols the UPF supports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.13, 6.1.6.3.x (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4985 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR introduces new backward compatible features to the NFManagement and NFDiscovery APIs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.13	Type: UpfInfo

Table 6.1.6.2.13-1: Definition of type UpfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sNssaiUpfInfoList | array(SnssaiUpfInfoItem) | M | 1..N | List of parameters supported by the UPF per S-NSSAI (NOTE 1) |
| smfServingArea | array(string) | O | 1..N | The SMF service area(s) the UPF can serve. If not provided, the UPF can serve any SMF service area. |
| interfaceUpfInfoList | array(InterfaceUpfInfoItem) | O | 1..N | List of User Plane interfaces configured on the UPF. When this IE is provided in the NF Discovery response, the NF Service Consumer (e.g. SMF) may use this information for UPF selection. (NOTE 7) |
| n6TunnelInfoList | map(InterfaceUpfInfoItem) | O | 1..N | This IE may be present for a UPF which may serve as a V-UPF for HR-SBO PDU sessions in the VPLMN. When present, it shall contain a map of N6 tunnelling information at the V-UPF side for establishing N6 tunnels between the V-UPF and the V-EASDF to differentiate HR-SBO PDU sessions from different DNN/S-NSSAI/HPLMNs having the same private UE IP Address, and the UPInterfaceType shall be set to "N6". The key of the map shall be a (unique) valid JSON string per clause 7 of IETF RFC 8259 [22], with a maximum of 32 characters. (NOTE 8) |
| iwkEpsInd | boolean | O | 0..1 | Indicates whether interworking with EPS is supported by the UPF. true: Supported false (default): Not Supported |
| sxaInd | boolean | O | 0..1 | Indicates whether the UPF is configured to support Sxa interface. true: Supported false: Not Supported |
| pduSessionTypes | array(PduSessionType) | O | 1..N | List of PDU session type(s) supported by the UPF. The absence of this attribute indicates that the UPF can be selected for any PDU session type. |
| atsssCapability | AtsssCapability | C | 0..1 | If present, this IE shall indicate the ATSSS capability of the UPF. If not present, the UPF shall be regarded with no ATSSS capability. |
| ueIpAddrInd | boolean | O | 0..1 | Indicates whether the UPF supports allocating UE IP addresses/prefixes. true: supported false (default): not supported |
| taiList | array(Tai) | O | 1..N | The list of TAIs the UPF can serve. It may contain one or more non-3GPP access TAIs. The absence of this attribute and the taiRangeList attribute indicates that the UPF can serve the whole SMF service area defined by the smfServingArea attribute. |
| taiRangeList | array(TaiRange) | O | 1..N | The range of TAIs the UPF can serve. It may contain non-3GPP access TAIs. The absence of this attribute and the taiList attribute indicates that the UPF can serve the whole SMF service area defined by the smfServingArea attribute. (NOTE 6) |
| wAgfInfo | WAgfInfo | C | 0..1 | If present, this IE shall indicate that the UPF is collocated with W-AGF. If not present, the UPF is not collocated with W-AGF. |
| tngfInfo | TngfInfo | C | 0..1 | If present, this IE shall indicate that the UPF is collocated with TNGF. If not present, the UPF is not collocated with TNGF. |
| twifInfo | TwifInfo | C | 0..1 | If present, this IE shall indicate that the UPF is collocated with TWIF. If not present, the UPF is not collocated with TWIF. |
| preferredEpdgInfoList | array(EpdgInfo) | O | 1..N | If present, this IE shall indicate that ePDG(s) that are preferred (e.g. for traffic effiency, distance wise or topology wise) to be served by the UPF/PGW-U. |
| preferredWagfInfoList | array(WAgfInfo) | O | 1..N | If present, this IE shall indicate that W-AGF(s) that are preferred (e.g. for traffic effiency, distance wise or topology wise) to be served by the UPF. |
| preferredTngfInfoList | array(TngfInfo) | O | 1..N | If present, this IE shall indicate that TNGF(s) that are preferred (e.g. for traffic effiency, distance wise or topology wise) to be served by the UPF. |
| preferredTwifInfoList | array(TwifInfo) | O | 1..N | If present, this IE shall indicate that TWIF(s) that are preferred (e.g. for traffic effiency, distance wise or topology wise) to be served by the UPF. |
| priority | integer | O | 0..1 | Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection for a service request matching the attributes of the UpfInfo; lower values indicate a higher priority. See the precedence rules in the description of the priority attribute in NFProfile, if Priority is also present in NFProfile. The NRF may overwrite the received priority value when exposing an NFProfile with the Nnrf_NFDiscovery service. (NOTE 2) |
| redundantGtpu | boolean | O | 0..1 | Indicates whether the UPF supports redundant GTP-U path. true: supported false (default): not supported |
| ipups | boolean | O | 0..1 | Indicates whether the UPF is configured for IPUPS. (NOTE 3) true: the UPF is configured for IPUPS. false (default): the UPF is not configured for IPUPS. |
| dataForwarding | boolean | O | 0..1 | Indicates whether the UPF is configured for data forwarding. (NOTE 4) When present, this IE shall be set as following: - true: the UPF is configured for data forwarding - false (default): the UPF is not configured for data forwarding If the UPF is configured for data forwarding, it shall support UP network interface with type "DATA_FORWARDING". |
| supportedPfcpFeatures | string | O | 0..1 | Supported PFCP Features. A string used to indicate the PFCP features supported by the UPF, which encodes the "UP Function Features" IE as specified in Table 8.2.25-1 of 3GPP TS 29.244 [21] (starting from Octet 5), in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and each two characters shall represent one octet of "UP Function Features" IE (starting from Octet 5, to higher octets). For each two characters representing one octet, the first character representing the 4 most significant bits of the octet and the second character the 4 least significant bits of the octet. (NOTE 5) |
| upfEvents | array(EventType) | O | 1..N | UPF event(s) exposed by the UPF |
| n6DelayMeastProtocs | array(N6DelayMeasurementProtocol) | O | 1..N | List of N6 delay measurement protocols supported by the UPF |
| NOTE 1: If this S-NSSAIs is present in the UpfInfo and in the NFprofile, the S-NSSAIs from the UpfInfo shall prevail. NOTE 2: An UPF profile may e.g. contain multiple UpfInfo entries, with each entry containing a different list of TAIs and a different priority, to differentiate the priority to select the UPF based on the user location. The priority in UpfInfo has the least precedence, i.e. it applies between UPFs with the same priority. NOTE 3: Any UPF can support the IPUPS functionality. In network deployments where specific UPFs are used to provide IPUPS, UPFs configured for providing IPUPS services shall be selected to provide IPUPS. NOTE 4: Based on operator policies, if dedicated UPFs are preferred to be used for indirect data forwarding during handover scenarios, when setting up the indirect data forwarding tunnel, the SMF should preferably select a UPF configured for data forwarding and use the network instance indicated in the Network Instance ID associated to the DATA_FORWARDING interface type in the interfaceUpfInfoList attribute. NOTE 5: The supportedPfcpFeatures shall be provisioned in addition and be consistent with the existing UPF features (atsssCapability, ueIpAddrInd, redundantGtpu and ipups) in the upfInfo, e.g. if the ueIpAddrInd is set to "true", then the UEIP flag shall also be set to "1" in the supportedPfcpFeatures. NOTE 6: This attribute should only be used by the UPF if, based on specific operator's deployment, the NRF and the SMFs intended to interwork with this UPF, have been upgraded to support this feature (i.e. to understand the definition of TAIs in the UPF profile based on ranges of TAIs). NOTE 7: The information elements included in the InterfaceUpfInfoItems, e.g. the Network Instance, can be used for any S-NSSAI and/or DNN which have no InterfaceUpfInfoList provisioned in the corresponding SnssaiUpfInfoItem and/or DnnUpfInfoItem. NOTE 8: It is deployment specific how to ensure that all the V-SMFs supporting the same V-UPF and V-EASDF select the same N6 tunnel for a given Home DNN/S-NSSAI/HPLMN ID. |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.3.x	Enumeration: N6DelayMeasurementProtocol

Table 6.1.6.3.x-1: Enumeration N6DelayMeasurementProtocol

| Enumeration value | Description |
| --- | --- |
| "TWAMP" | A Two-Way Active Measurement Protocol |
| "OWAMP" | A One-way Active Measurement Protocol |
| "STAMP" | Simple Two-Way Active Measurement Protocol |

Editor's note: the complete list of protocols to be defined in this data type is FFS.

* * * Next Change * * * *

# A.2	Nnrf_NFManagement API

openapi: 3.0.0

[…]

UpfInfo:

description: Information of an UPF NF Instance

type: object

required:

- sNssaiUpfInfoList

properties:

sNssaiUpfInfoList:

type: array

items:

$ref: '#/components/schemas/SnssaiUpfInfoItem'

minItems: 1

smfServingArea:

type: array

items:

type: string

minItems: 1

interfaceUpfInfoList:

type: array

items:

$ref: '#/components/schemas/InterfaceUpfInfoItem'

minItems: 1

n6TunnelInfoList:

description: >

A map of InterfaceUpfInfoItems containing the N6 tunnelling information for establishing

a N6 tunnel between the V-UPF and the V-EASDF, where a valid JSON string serves as key.

type: object

additionalProperties:

$ref: '#/components/schemas/InterfaceUpfInfoItem'

minProperties: 1

iwkEpsInd:

type: boolean

default: false

sxaInd:

type: boolean

pduSessionTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

atsssCapability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AtsssCapability'

ueIpAddrInd:

type: boolean

default: false

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

taiRangeList:

type: array

items:

$ref: '#/components/schemas/TaiRange'

minItems: 1

wAgfInfo:

$ref: '#/components/schemas/WAgfInfo'

tngfInfo:

$ref: '#/components/schemas/TngfInfo'

twifInfo:

$ref: '#/components/schemas/TwifInfo'

preferredEpdgInfoList:

type: array

items:

$ref: '#/components/schemas/EpdgInfo'

minItems: 1

preferredWAgfInfoList:

type: array

items:

$ref: '#/components/schemas/WAgfInfo'

minItems: 1

preferredTngfInfoList:

type: array

items:

$ref: '#/components/schemas/TngfInfo'

minItems: 1

preferredTwifInfoList:

type: array

items:

$ref: '#/components/schemas/TwifInfo'

minItems: 1

priority:

type: integer

minimum: 0

maximum: 65535

redundantGtpu:

type: boolean

default: false

ipups:

type: boolean

default: false

dataForwarding:

type: boolean

default: false

supportedPfcpFeatures:

type: string

upfEvents:

type: array

items:

$ref: 'TS29564_Nupf_EventExposure.yaml#/components/schemas/EventType'

minItems: 1

n6DelayMeastProtocs:

type: array

items:

$ref: '#/components/schemas/N6DelayMeasurementProtocol'

minItems: 1

[…]

DnsSecurityProtocol:

description: DNS security protocol

anyOf:

- type: string

enum:

- TLS

- DTLS

- type: string

N6DelayMeasurementProtocol:

description: N6 delay measurement protocol

anyOf:

- type: string

enum:

- TWAMP

- OWAMP

- STAMP

- type: string

[…]

* * * End of Changes * * * *