3GPP TSG-CT WG4 Meeting #126	C4-245117

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1100 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discovery of SMF based on Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3GPP TS 23.501 CR#5604 (S2-2410930) introduces I-SMF discovery and selection based on Local Offloading Management indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Local Offloading Management capability indication to SMF information registered in NRF. Add a query parameter to enable discovery of SMF based on this capability. Update supported features and OpenAPIs accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage-2 and stage-3. Discovery of I-SMF based on Local Offloading Management is not supported by NRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.12, 6.2.3.2.3.1, 6.2.9, A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5604 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new feature to: Nnrf_NFManagement, Nnrf_NFDiscovery. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.12	Type: SmfInfo

Table 6.1.6.2.12-1: Definition of type SmfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sNssaiSmfInfoList | array(SnssaiSmfInfoItem) | M | 1..N | List of parameters supported by the SMF per S-NSSAI (NOTE 1). |
| taiList | array(Tai) | O | 1..N | The list of TAIs the SMF can serve. It may contain one or more non-3GPP access TAIs. The absence of this attribute and the taiRangeList attribute indicate that the SMF can be selected for any TAI in the serving network. |
| taiRangeList | array(TaiRange) | O | 1..N | The range of TAIs the SMF can serve. It may contain non-3GPP access TAIs. The absence of this attribute and the taiList attribute indicate that the SMF can be selected for any TAI in the serving network. |
| pgwFqdn | Fqdn | O | 0..1 | The FQDN of the PGW if the SMF is a combined SMF/PGW-C. |
| pgwIpAddrList | array(IpAddr) | O | 1..N | When present, this IE shall contain the PGW IP addresses of the combined SMF/PGW-C. Each PGW IP address shall be encoded as an IPv4 or an IPv6 address (i.e. not as an IPv6 prefix). This IE allows the NF Service consumer to find the target combined SMF/PGW-C by PGW IP Address, e.g. when only PGW IP Address is available. |
| accessType | array(AccessType) | C | 1..2 | If included, this IE shall contain the access type (3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF. If not included, it shall be assumed the both access types are supported. |
| priority | integer | O | 0..1 | Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection for a service request matching the attributes of the SmfInfo; lower values indicate a higher priority. The NRF may overwrite the received priority value when exposing an NFProfile with the Nnrf_NFDiscovery service. Absence of this attribute equals to having the same smfInfo priority as the priority defined at NFProfile/NFService level. (NOTE 2) |
| vsmfSupportInd | boolean | O | 0..1 | This IE may be used by an SMF to explicitly indicate the support of V-SMF capability and its preference to be selected as V-SMF. When present, this IE shall indicate whether the V-SMF capability are supported by the SMF: - true: V-SMF capability supported by the SMF - false: V-SMF capability not supported by the SMF. Absence of this IE indicates the V-SMF capability support of the SMF is not specified. (NOTE 3) |
| ismfSupportInd | boolean | O | 0..1 | This IE may be used by an SMF to explicitly indicate the support of I-SMF capability and its preference to be selected as I-SMF. When present, this IE shall indicate whether the I-SMF capability are supported by the SMF: - true: I-SMF capability supported by the SMF - false: I-SMF capability not supported by the SMF. Absence of this IE indicates the I-SMF capability support of the SMF is not specified. (NOTE 3) |
| pgwFqdnList | array(Fqdn) | O | 1..N | When present, this attribute provides additional FQDNs to the FQDN indicated in the pgwFqdn attribute. The pgwFqdnList attribute may be present if the pgwFqdn attribute is present. |
| smfOnboardingCapability | boolean | O | 0..1 | When present, this IE indicates the SMF supports SNPN Onboarding capability and User Plane Remote Provisioning. This is used for the case of Onboarding of UEs for SNPNs (see 3GPP TS 23.501 [2], clauses 5.30.2.10 and 6.2.6.2). - false (default): SMF does not support SNPN Onboarding; - true: SMF supports SNPN Onboarding. (NOTE 4) |
| smfUPRPCapability | boolean | O | 0..1 | When present, this IE indicates the SMF supports User Plane Remote Provisioning (UPRP) capability. This is used for the case of Onboarding of UEs for SNPNs (see 3GPP TS 23.501 [2], clauses 5.30.2.10 and 6.2.6.2). - false (default): SMF does not support UPRP; - true: SMF supports UPRP. |
| loOffloadManInd | boolean | O | 0..1 | When present, this IE indicates whether the SMF supports local offloading management. - false (default): SMF does not support local offloading management; - true: SMF supports local offloading management. |
| NOTE 1: If this S-NSSAIs is present in the SmfInfo and in the NFprofile, the S-NSSAIs from the SmfInfo shall prevail. NOTE 2: An SMF profile may e.g. contain multiple SmfInfo entries, with each entry containing a different list of TAIs and a different priority, to differentiate the priority to select the SMF based on the user location. The priority in SmfInfo applies between SMFs or SMF Services with the same priority. NOTE 3: The IE should only be registered when the SMF is configured to be preferably selected as V-SMF/I-SMF. NOTE 4: The IE is deprecated and replaced by smfUPRPCapability attribute. |  |  |  |  |

* * * * Next Change * * * *

##### 6.2.3.2.3	Resource Standard Methods

6.2.3.2.3.1	GET

This operation retrieves a list of NF Instances, and their offered services, currently registered in the NRF, satisfying a number of filter criteria, such as those NF Instances offering a certain service name, or those NF Instances of a given NF type (e.g., AMF).

Table 6.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| target-nf-type | NFType | M | 1 | This IE shall contain the NF type of the target NF being discovered. |  |
| requester-nf-type | NFType | M | 1 | This IE shall contain the NF type of the Requester NF that is invoking the Nnrf_NFDiscovery service. |  |
| preferred-collocated-nf-types | array(CollocatedNfType) | O | 1..N | The IE may be present to indicate desired collocated NF type(s) when the NF service consumer wants to discover candidate NFs matching the target NF Type that are preferentially collocated with other NF types. (NOTE 19) | Collocated-NF-Selection |
| requester-nf-instance-id | NfInstanceId | O | 0..1 | If included, this IE shall contain the NF instance id of the Requester NF. | Query-Params-Ext2 |
| service-names | array(ServiceName) | O | 1..N | If included, this IE shall contain an array of service names for which the NRF is queried to provide the list of NF profiles. The NRF shall return the NF profiles that have at least one NF service matching the NF service names in this list. The NF services returned by the NRF (inside the nfServices or nfServiceList attributes) in each matching NFProfile shall be those services whose service name matches one of the service names included in this list. If not included, the NRF shall not filter based on service name. This array shall contain unique items. Example: NF1 supports services: A, B, C NF2 supports services: C, D, E NF3 supports services: A, C, E NF4 supports services: B, C, D Consumer asks for service-names=A,E NRF returns: NF1 containing service A NF2 containing service E NF3 containing services A, E NF4 is not returned |  |
| ... [non-changing rows skipped for clarity] ... |  |  |  |  |  |
| ind-com-with-del-disc | boolean | O | 0..1 | This IE may be present if the requester is an NRF or a SCP. When present, this IE shall be set to true to indicate that indirect communication with delegated discovery with NF selection at target domain feature is supported. Presence of this IE with false value shall be prohibited. | NFsel_by_tPLMN |
| ind-com-wo-del-disc | boolean | O | 0..1 | This IE may be present if the requester is an NRF or a SCP. When present, this IE shall be set to true to indicate that indirect communication without delegated discovery with NF selection at target domain feature is supported. Presence of this IE with false value shall be prohibited. | NFsel_by_tPLMN |
| local-offload-manage-ind | boolean | O | 0..1 | If pesent and set to true this IE indicates an SMF (I-SMF) that supports local offloading management is reuested to be discovered. Presence of this IE with false value shall be prohibited. | Query-EDGE-Ph3 |
| NOTE 1: If this parameter is present and no AMF supporting the requested GUAMI is available due to AMF Failure or planned AMF removal, the NRF shall return in the response AMF instances acting as a backup for AMF failure or planned AMF removal respectively for this GUAMI (see clause 6.1.6.2.11). The NRF can detect if an AMF has failed, using the Heartbeat procedure. The NRF will receive a de-registration request from an AMF performing a planned removal. NOTE 2: If the combined SMF/PGW-C is requested to be discovered, the NRF shall return in the response the SMF instances registered with the SmfInfo containing pgwFqdn. NOTE 3: If a UPF supporting interworking with EPS is requested to be discovered, the NRF shall return in the response the UPF instances registered with the upfInfo containing iwkEpsInd set to true. NOTE 4: This attribute has a different semantic than what is defined in clause 6.6.2 of 3GPP TS 29.500 [4], i.e. it is not used to signal the features of the Nnrf_NFDiscovery Service API supported by the requester NF. NOTE 5: The AMF may perform the SMF discovery based on the dnn, snssais and preferred-tai during a PDU session establishment procedure, and the NRF shall return the SMF profiles matching all if possible, or the SMF profiles only matching dnn and snssais. If the SMF profiles only matching dnn and snssais are returned, the AMF shall insert an I-SMF. An SMF may also perform a UPF discovery using this parameter. NOTE 6: The SMF may select the P-CSCF close to the UPF by setting the preferred-locality to the value of the locality of the UPF. NOTE 7: During EPS to 5GS idle mobility procedure, the Requester NF (i.e. SMF) discovers the anchor NEF for NIDD using the SCEF ID received from EPS as the value of the NEF ID, as specified in clause 4.11.1.3.3 of 3GPP TS 23.502 [3]. NOTE 8: The service consumer may include a list of preferred-nf-instance-ids in the query. If so, the NRF shall first check if the NF profiles of the preferred NF instances match the other query parameters, and if so, then the NRF shall return the corresponding NF profiles; otherwise, the NRF shall return a list of candidate NF profiles matching the query parameters other than the preferred-nf-instance-ids. For example, the target AMF may set this query parameter to the SMF Instance ID and I-SMF Instance ID during an inter AMF mobility procedure to select an I-SMF. NOTE 9: This parameter may be used by the SCP (with other query parameters) to discover and select a NF service consumer with a default notification subscription supporting the notification type of a notification request (see clause 6.10.3.3 of 3GPP TS 29.500 [4]). NOTE 10: An S-NSSAI value used in discovery request query parameters shall be considered as matching the S-NSSAI value in the NF Profile or NF Service of a given NF Instance if both the SST and SD components are identical (i.e. an S-NSSAI value where SD is absent, shall not be considered as matching an S-NSSAI where SD is present, regardless if SST is equal in both). NOTE 11: The dnn query parameter shall be considered as matching a DNN attribute in the NF Profile of a given NF Instance if: - both contain the same Network Identifier and Operator Identifier; - both contain the same Network Identifier and none contains an Operator Identifier; - the dnn query parameter contains the Network Identifier only, the DNN value in the NF Profile contains both the Network Identifier and Operator Identifier, and both contain the same Network Identifier; or - the dnn query parameter contains both the Network Identifier and Operator Identifier, the DNN value in the NF Profile contains the Network Identifier only, both contain the same Network Identifier and the Operator Identifier matches one PLMN of the NF (i.e. plmnList of the NF Profile). NOTE 12: Based on operator's policies, a discovery request not including the requester's information necessary to validate the authorization parameters in NF Profiles may be rejected or accepted but with only returning in the discovery response NF Instances whose authorization parameters allow any NF Service Consumer to access their services. The authorization parameters in NF Profile are those used by NRF to determine whether a given NF Instance / NF Service Instance can be discovered by an NF Service Consumer in order to consume its offered services (e.g. "allowedNfTypes", "allowedNfDomains", etc.). NOTE 13: Different UPF instances for data forwarding may be configured in the network e.g. for different serving areas. The SMF may use this query parameter together with others (like SMF Serving Area or TAI) in discovery to select the UPF candidate for data forwarding. NOTE 14: For HR roaming, if the V-PLMN requires Deployments Topologies with specific SMF Service Areas (DTSSA) but no H-SMF can be selected supporting V-SMF change, AMF may use this query parameter to select a V-SMF serving the full VPLMN if available. NOTE 15: The AMF may perform discovery with this parameter to find V-SMF(s)/I-SMF(s), and the NRF shall return the SMF profiles that explicitly indicated support of V-SMF/I-SMF(s) capability, if the vsmf-support-ind/ismf-support-ind query parameter is present and set to true. When performing discovery, the AMF shall use other query parameters together with this IE to ensure the required configurations and/or features are supported by the V-SMF/I-SMF(s), e.g. required Slice for the PDU session, support of DTSSA feature if V-SMF change is required for PDU Session, etc. If no SMF instances that explicitly indicated support of V-SMF/I-SMF(s) capability can be matched for the discovery, the NRF shall return matched SMF instances not indicating support of V-SMF/I-SMF(s) capability explicitly, i.e. the SMF instances not registered vsmfSupportInd/ismfSupportInd IE in the NF profile but matched to the rest query parameters, if available. NOTE 16: When required-pfcp-features is used as query parameter, the NRF shall return a list of candidate UPFs supporting all the required PFCP features. The NRF may also return UPF profiles not including the "SupportedPfcpFeatures" attribute (e.g. pre-Rel-17 UPFs) but matching the other query parameters. The NF Service Consumer, e.g. a SMF, when using required-pfcp-features as query parameter, shall also include the query parameter corresponding to the UPF features (atsss-capability, upf-ue-ip-addr-ind, redundant-gtpu) which correspond to the PFCP feature flags MPTCP and ATSSS_LL, UEIP, and RTTL respectively, if the corresponding PFCP feature is required. For example an SMF, that wishes to select a UPF supporting UE IP Address Allocation by the UP function, shall set the UEIP flag to "1" in the required-pfcp-features and also include the upf-ue-ip-addr-ind parameter set to "true". NOTE 17: In this release, the usage of Home Network Public Key identifier for AUSF/UDM discovery is limited to the scenario where the AUSF/UDM NF consumers belong to the same PLMN as AUSF/UDM. NOTE 18: The NF service consumer may derive the serving scope from e.g. the TAI of the UE, using local configuration. This parameter may be used to discover any NF that registers to the NRF, e.g. a 5GC NF or a P-CSCF. NOTE 19: If the NRF supports the "Collocated-NF-Selection" feature and the NF service consumer has included the "preferred-collocated-nf-types" attribute, the NRF shall return a list of candidates NFs (for the target-nf-type) matching the discovery query parameters and preferentially supporting CollocatedNfType(s) as indicated in the preferred-collocated-nf-types. NOTE 20: If the NRF supports this IE and the NF service consumer has included this IE with the value "true" in discovery request, the NRF shall look up and return PGW-C+SMF instances matching the other query parameters. If no matching is found, the NRF shall return a list of standalone SMF instances matching the other query parameters. If the NRF supports this IE and the NF service consumer has included this IE with the value "false" in discovery request, the NRF shall look up and return standalone SMF instances matching the other query parameters. If no matching is found, the NRF shall return a list of PGW-C+SMF instances matching the other query parameters. NOTE 21: Either pgw-ind IE or preferred-pgw-ind IE may be included in the discovery request. NOTE 22: MB-SMF may use an NRF to discover the AMF(s) serving an MBS service area (see clause 7.3.1 in 3GPP TS 23.247 [43]. For this purpose, the MB-SMF may use query parameters specified in this table, e.g. 'tai' and 'service-names', or 'snssais', or any other parameters. NOTE 23: This parameter may be set by an NF service consumer or SCP to filter-out specific NF (service) instances or NF (service) sets from the NF Discovery response, e.g. when the NFc knows that an NF service producer is not responsive or overloaded. See the 3gpp-Sbi-Selection-Info header in clause 5.2.3.3.10 of 3GPP TS 29.500 [4]. NOTE 24: A feature shall not be included in both required-features IE and preferred-features IE in the same discovery request. NOTE 25: When Locality is configured in NSACF(s), an NSACF consumer, e.g. AMF or SMF, may use a locally configured NSACF Locality to discover the candidate NSACF, or otherwise may use its own Locality to discover the candidate NSACF. When Locality is configured in MRF(s) or MRFP(s), an MRF/MRFP consumer, e.g. IMS entity, may use a locally configured MRF/MRFP Locality to discover the candidate MRF/MRFP, or otherwise may use its own Locality to discover the candidate MRF/MRFP. When Locality is configured in MF, an MF consumer, e.g. IMS AS, may use a locally configured MF Locality to discover the candidate MF, or otherwise may use its own Locality to discover the candidate MF. NOTE 26: Only one of the target-nf-instance-id and target-nf-instance-id-list query parameters may be present in an NF Discovery Request. NOTE 27: When the query parameter "ue-ipv4-address" or "ue-ipv6-prefix" is used to discover a UPF as specified in clause 4.15.10 of 3GPP TS 23.502 [3], the NRF shall find a match by looking into either natedIpv4AddressRanges or natedIpv6PrefixRanges in the DnnUpfInfoItem. NOTE 28: If the NF service consumer includes a "tai-list" query parameter and the "nf-tai-list-ind" query parameter set to true, and if the NRF supports the same and the NRF is not able to find any NF supporting all TAs included in the tai-list, the NRF may return a list of NFs in the SearchResult where each NF supports at least one TA in the tai-list; in this case, the NRF should attempt to return a list of NFs that altogether support as many TAIs from the tai-list as possible, and it should indicate in the SearchResultInfo attribute the list of TAIs from the tai-list which are not supported by these NFs. NOTE 29: The NRF should return as many matching NF profiles and/or NF Instance IDs as possible in the NF Discovery Response. If all the NF profiles matching the query parameters cannot be returned in the NF Discovery response (e.g. due to the maximum payload size), the NRF may decide to store the complete search result for subsequent retrieval by the NRF consumer (see clause 6.2.3.4). Alternatively, the NRF may include all the NF instance IDs rather than NF profiles in the NF Discovery response (using the Enh-NF-Discovery feature, see clause 5.3.2.2.2). NOTE 30: When an AMF supports the "subscription-based routing to a target core network" feature as specified in clause 5.48 of 3GPP TS 23.501 [2] and it receives the targetPlmnId as part of SmfSelectionSubscriptionData, it shall include the targetPlmnId as Operator Identifier in the DNN to select a SMF in that PLMN. NOTE 31: In the case of Indirect Network Sharing deployments, the AMF may also include the serving-scope parameter and/or preferred locality parameter for discovery of the H-SMF in participating operator's network, based on the UE location as described in clause 6.3.2 of 3GPP TS 23.501 [2]. If including the serving scope and/or preferred locality values based on UE location for the H-SMF discovery, the AMF shall be configured with the serving-scopes and the preferred localities that are applicable for the participating operator's network. |  |  |  |  |  |

When certain query parameters in the discovery request are not supported by the NRF, the NRF shall ignore the unsupported query parameters and continue processing the request with the supported query parameters. The default logical relationship among the supported query parameters is logical "AND", i.e. all the provided query parameters shall be matched, with the exception of the "preferred-locality", "ext-preferred-locality", "preferred-nf-instances", "preferred-tai", "preferred-api-versions", "preferred-full-plmn", "preferred-collocated-nf-types", "preferred-pgw-ind", "preferred-analytics-delays", "preferred-features", "mbs-session-id", "ind-com-with-del-disc" and "ind-com-wo-del-disc" query parameters (see Table 6.2.3.2.3.1-1).

The NRF may support the Complex query expression as defined in 3GPP TS 29.501 [5] for the NF Discovery service. If the "complexQuery" query parameter is included, then the logical relationship among the query parameters contained in "complexQuery" query parameter is as defined in 3GPP TS 29.571 [7].

A NRF not supporting Complex query expression shall reject a NF service discovery request including a complexQuery parameter, with a ProblemDetails IE including the cause attribute set to INVALID_QUERY_PARAM and the invalidParams attribute indicating the complexQuery parameter.

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SearchResult | M | 1 | 200 OK | The response body contains the result of the search over the list of registered NF Instances. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | The response shall be used when the intermediate NRF redirects the service discovery request. The NRF shall include in this response a Location header field containing a URI pointing to the resource located on the redirect target NRF. If an SCP redirects the message to another SCP then the location header field shall contain the same URI or a different URI pointing to the endpoint of the NF service producer to which the request should be sent. |
| ProblemDetails | O | 0..1 | 400 Bad Request | The response body contains the error reason of the request message. If the query parameter used to match the authorization parameter is required but not provided in the NF discovery request, the "cause" attribute shall be set to "MANDATORY_QUERY_PARAM_MISSING", and the missing query parameter shall be indicated. The "cause" attribute shall be set to "INVALID_CLIENT" if some or all attributes of the NF Service Consumer in query parameters do not match the requester information in, e.g. its public key certificate. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This response shall be returned if the Requester NF is not allowed to discover the NF Service(s) being queried. |
| ProblemDetails | O | 0..1 | 404 Not Found | This response shall be returned if the requested resource URI as defined in clause 6.2.3.2.2 (query parameter not considered) is not found in the server. It may also be sent in hierarchical NRF deployments when the NRF needs to forward/redirect the request to another NRF but lacks information in the request to do so; similarly, the NRF shall return this response code when it is received from the upstream NRF. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | The response body contains the error reason of the request message. |

Table 6.2.3.2.3.1-4: Headers supported by the GET method on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| If-None-Match | string | C | 0..1 | Validator for conditional requests, as described in IETF RFC 9110 [40], clause 13.1.2 |

Table 6.2.3.2.3.1-5: Headers supported by the 200 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Cache-Control | string | C | 0..1 | Cache-Control containing max-age, described in IETF RFC 9111 [20], clause 5.2 |
| ETag | string | C | 0..1 | Entity Tag containing a strong validator, described in IETF RFC 9110 [40], clause 8.8.3 |

Table 6.2.3.2.3.1-6: Headers supported by the 307 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | The URI pointing to the resource located on the redirect target NRF |

Table 6.2.3.2.3.1-7: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Parameters table | Description |
| --- | --- | --- | --- | --- |
| search | Stored Search (Document) | GET | 6.2.3.2.3.1-8 | The 'searchId' parameter returned in the response can be used as the 'searchId' parameter in the GET request to '/searches/{searchId}' |
| completeSearch | Complete Stored Search (Document) | GET | 6.2.3.2.3.1-9 | The 'searchId' parameter returned in the response can be used as the 'searchId' parameter in the GET request to '/searches/{searchId}/complete' |

* * * * Next Change * * * *

### 6.2.9	Features supported by the NFDiscovery service

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [7].

The following features are defined for the Nnrf_NFDiscovery service.

Table 6.2.9-1: Features of supportedFeatures attribute used by Nnrf_NFDiscovery service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | Complex-Query | O | Support of Complex Query expression (see clause 6.2.3.2.3.1) |
| 2 | Query-Params-Ext1 | O | Support of the following query parameters: - limit - max-payload-size - required-features - pdu-session-types |
| 3 | Query-Param-Analytics | O | Support of the query parameters for Analytics identifier: - event-id-list - nwdaf-event-list |
| 4 | MAPDU | O | This feature indicates whether the NRF supports selection of UPF with ATSSS capability. |
| 5 | Query-Params-Ext2 | O | Support of the following query parameters: - requester-nf-instance-id - upf-ue-ip-addr-ind - pfd-data - target-snpn - af-ee-data - w-agf-info - tngf-info - twif-info - target-nf-set-id - target-nf-service-set-id - preferred-tai - nef-id - preferred-nf-instances - notification-type - serving-scope - internal-group-identity - preferred-api-versions - v2x-support-ind - redundant-gtpu - redundant-transport - lmf-id - an-node-type - rat-type - ipups - scp-domain-list - address-domain - ipv4-addr - ipv6-prefix - served-nf-set-id - remote-plmn-id - data-forwarding - preferred-full-plmn - requester-snpn-list - max-payload-size-ext - client-type |
| 6 | Service-Map | M | This feature indicates whether it is supported to identify the list of NF Service Instances as a map (i.e. the "nfServiceList" attribute of NFProfile is supported). |
| 7 | Query-Params-Ext3 | O | Support of the following query parameters: - ims-private-identity - ims-public-identity - msisdn - requester-plmn-specific-snssai-list - n1-msg-class - n2-info-class |
| 8 | Query-Params-Ext4 | O | Support of the following query parameters: - realm-id - storage-id |
| 9 | Query-Param-vSmf-Capability | O | Support of the query parameters for V-SMF Capability: - vsmf-support-ind |
| 10 | Enh-NF-Discovery | O | Enhanced NF Discovery This feature indicates whether it is supported to return the nfInstanceList IE in the NF Discovery response. |
| 11 | Query-SBIProtoc17 | O | Support of the following query parameters, for Service Based Interface Protocol Improvements defined in 3GPP Rel-17: - preferred-vendor-specific-features - preferred-vendor-specific-nf-features - home-pub-key-id - pgw-ip - preferences-precedence - preferred-pgw-ind - v2x-capability - shared-data-id |
| 12 | SCPDRI | O | SCP Domain Routing Information An NRF supporting this feature shall allow a service consumer (i.e. a SCP) to get the SCP Domain Routing Information and subscribe/unsubscribe to the change of SCP Domain Routing Information with following service operations: - SCPDomainRoutingInfoGet (see clause 5.3.2.3) - SCPDomainRoutingInfoSubscribe (see clause 5.3.2.4) - SCPDomainRoutingInfoUnsubscribe (see clause 5.3.2.6) A service consumer (i.e. a SCP) supporting this feature shall be able to handle SCPDomainRoutingInfoNotify as specified in clause 5.3.2.5, if subscribed to the change of SCP Domain Routing Information in the NRF. |
| 13 | Query-Upf-Pfcp | O | This feature indicates whether the NRF supports selection of UPF with required UP function features as defined in 3GPP TS 29.244 [21]. |
| 14 | Query-5G-ProSe | O | Support of the following query parameters, for Proximity based Services in 5GS defined in 3GPP Rel-17: - prose-support-ind - prose-capability |
| 15 | NSAC | O | This feature indicates the NSACF service capability. Support of the following query parameters: - nsacf-capability |
| 16 | Query-MBS | O | Support of the following query parameters, for Multicast and Broadcast Services defined in 3GPP Rel-17: - mbs-session-id-list - mbsmf-serving-area - area-session-id |
| 17 | Query-eNA-PH2 | O | Support of the following query parameters, for Enhanced Network Automation Phase 2 defined in 3GPP Rel-17: - analytics-aggregation-ind - serving-nf-set-id - serving-nf-type - ml-analytics-info-list - analytics-metadata-prov-ind |
| 18 | Query-eLCS | O | Support of the following query parameters, for 5G LCS service: - gmlc-number |
| 19 | Query-eEDGE-5GC | O | Support of the following query parameters, for enhancement of support for Edge Computing in 5GC defined in 3GPP Rel-17: - upf-n6-ip - tai-list The query parameter tai-list may be used to discover NEF. |
| 20 | Collocated-NF-Selection | O | Support of selecting a collocated NF supporting multiple NF types. |
| 21 | Query-ENPN | O | Support of the following query parameter for the enhanced support of Non-Public Networks defined in 3GPP Rel-17: - support-onboarding-capability - target-hni - remote-snpn-id |
| 22 | Query-ID_UAS | O | Support of the following query parameters, for remote Identification of Unmanned Aerial Systems defined in 3GPP Rel-17: - uas-nf-functionality-ind |
| 23 | NRFSET | O | Support of the NRF Set feature as defined in clause 5.3.2.1. All the NRF service instances of an NRF supporting this feature shall support the NRFSET feature. |
| 24 | Query-Nw-Resolution | O | Support for the following query parameters: - target-nw-resolution |
| 25 | Query-Param-iSmf-Capability | O | Support of the query parameters for I-SMF Capability: - ismf-support-ind |
| 26 | Query-SBIProtoc17-Ext1 | O | Support of the following query parameters, for Service Based Interface Protocol Improvements defined in 3GPP Rel-17: - exclude-nfinst-list - exclude-nfservinst-list - exclude-nfserviceset-list - exclude-nfset-list |
| 27 | Query-Upf-IpIndex | O | Support of the query parameters for UPF selection with IpIndex: - ipv4-index - ipv6-index |
| 28 | Query-eNA-PH2-Ext1 | O | Support of the following query parameters, for extension of Enhanced Network Automation Phase 2 defined in 3GPP Rel-17: - preferred-analytics-delays |
| 29 | Query-HLC | O | Support of the query parameters for AMF selection with High Latency Communication capability: - high-latency-com |
| 30 | Query-SBIProtoc18 | O | Support of the following query parameters, for Service Based Interface Protocol Improvements defined in 3GPP Rel-18: - ext-preferred-locality - n32-purposes - preferred-features - sxa-ind - remote-plmn-id-roaming - nf-tai-list-ind - complete-search-result - additional-snssais |
| 31 | Complete-Profile-Discovery | O | Support discovery of complete NF profiles (including authorization attributes such as the "allowedXXX" attributes of NFProfile and NFService data types) of NF instances matching the query parameters. |
| 32 | Query-UPEAS | O | Support of the following query parameter, for UPF enhancement for Exposure and SBA defined in 3GPP Rel-18: - upf-event-list |
| 33 | Enh-NF-Discovery-Ext1 | O | Support of the following query parameter defined in 3GPP Rel-18: - target-nf-instance-id-list |
| 34 | Query-NG-RTC | O | Support of the following query parameters for NG_RTC defined in 3GPP Rel-18: - ims-domain-name - media-capability-list |
| 35 | Query-eLCS-PH3 | O | Support of the following query parameters, for 5G LCS Phase 3 service defined in 3GPP Rel-18: - pru-tai - af-data - pru-support-ind - preferred-up-positioning-ind |
| 36 | Query-eNA-PH3 | O | Support of the following query parameters, for Enhanced Network Automation Phase 3 defined in 3GPP Rel-18: - ml-accuracy-checking-ind - analytics-accuracy-checking-ind - ml-model-storage-ind - data-storage-ind - data-subscription-relocation-support-ind - roaming-exchange-ind |
| 37 | Query-A2X | O | Support of the following query parameters, for A2X in 5GS defined in 3GPP Rel-18: - a2x-support-ind - a2x-capability |
| 38 | Allowed-ruleset | O | Support receiving ruleSets in NF (Service) profile |
| 39 | Query-AIMLsys | O | Support for the following query parameters, for System Support for AI/ML-based Services defined in 3GPP Rel-18: - multi-mem-af-sess-qos-ind - member-ue-sel-assist-ind |
| 40 | Canary-Release | O | Support of "CANARY_RELEASE" value for NFStatus and NFServiceStatus, used for canary testing. The NRF shall not return, in the discovery response, NF Instances, or NF Service Instances, whose NFStatus or NFService status respectively is set to value "CANARY_RELEASE", unless the consumer of the discovery service has indicated support of the "Canary-Release" feature. |
| 41 | Query-UPF-Selection-N3GPP | O | Support of the following query parameters, for selection of preferred UPF for PDU sessions via non-3GPP access: - upf-select-epdg-info An NRF supporting this feature shall also support discovery of the preferred UPF(s) for a W-AGF/TNGF/TWIF using the following query parameters: - w-agf-info - tngf-info - twif-info |
| 42 | Query-5G- RangingSlPos | O | Support of the following query parameter, for Ranging and Sidelink positioning in 5GS defined in 3GPP Rel-18: - ranging-sl-pos-support-ind |
| 43 | Query-eNS-PH2 | O | Support of the following query parameters, for Enhanced Network Slice Phase 2 defined in 3GPP Rel-17 onwards: - nsac-sai |
| 44 | RID-NfGroupId-Mapping | O | Support the capability of mapping between Routing Indicator and NF Group ID by the NRF. If the consumer of the discovery service has not indicated support of the "RID-NfGroupId-Mapping" feature, the NRF shall not return in the discovery response NF instances (of UDMs and AUSFs) containing (in "UdmInfo" and "AusfInfo" respectively) an NF Group ID and no Routing Indicators to indicate that the mapping between both will be done by the NRF (see clauses 6.1.6.2.7 and 6.1.6.2.8). |
| 45 | Query_UEPO | O | Support of the following query parameter(s) for PCF: - ursp-delivery-eps-support-ind |
| 46 | DNN-List-Optimization | O | Support of dnnSmfInfoListId within SnssaiSmfInfoItem, and dnnUpfInfoListId within SnssaiUpfInfoItem If the consumer of the discovery service has not indicated support of the DNN-List-Optimization feature, the NRF shall provide the complete list of DNNs in dnnSmfInfoList or the dnnUpfInfoList. |
| 47 | Shared-Data | O | Support of Shared Data IDs in search results. If supported, NFProfiles within search results may contain a sharedProfileDataId identifying shared profile data and NFServices may contain a sharedServiceDataId identifying shared service data. If not supported by the consumer, the NRF shall substitute values of the referenced shared data into the NF profile it returns to the consumer. NRFs supporting this feature shall also support the Shared-Data-Retrieval feature of the NFManagement service. |
| 48 | Query-EDGE-Ph2 | O | Support of the following query parameter, for the support of Edge Computing Phase 2 defined in 3GPP Rel-18: - dns-sec-protoc |
| 49 | Query-TAI-LIST | O | Support discovery of target NF with service area within a list of TAIs. The feature supports the following query parameter(s): - tai-list The query parameter is applicable to discover any target NF type. |
| 50 | NFsel_by_tPLMN | O | Support of indirect communication with and without delegated discovery with NF selection at target domain, defined in 3GPP Rel-19. See clause 5.3.2.2.3. The feature supports the following query parameter(s) by a requester that is an NRF or a SCP: - ind-com-with-del-disc - ind-com-wo-del-disc |
| XX | Query-EDGE-Ph3 | O | Support of the following query parameter, for the support of Edge Computing Phase 3 defined in 3GPP Rel-19: - local-offload-manage-ind |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. NOTE 1: An NRF that advertises support of a given feature shall support all the query parameters associated with the feature. An NRF may support none or a subset of the query parameters of features that it does not advertise as supported. NOTE 2: For a release under development, it is recommended to define new features for new query parameters by grouping them per 3GPP work item. Any definition of new query parameters in a frozen release requires a new feature definition. |  |  |  |

* * * * Next Change * * * *

# A.2	Nnrf_NFManagement API

openapi: 3.0.0

info:

version: '1.4.0-alpha.1'

title: 'NRF NFManagement Service'

description: |

NRF NFManagement Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.510 V19.0.0; 5G System; Network Function Repository Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.510/'

[non-changing parts skipped for clarity]

SmfInfo:

description: Information of an SMF NF Instance

type: object

required:

- sNssaiSmfInfoList

properties:

sNssaiSmfInfoList:

type: array

items:

$ref: '#/components/schemas/SnssaiSmfInfoItem'

minItems: 1

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

taiRangeList:

type: array

items:

$ref: '#/components/schemas/TaiRange'

minItems: 1

pgwFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

pgwIpAddrList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

minItems: 1

accessType:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

priority:

type: integer

minimum: 0

maximum: 65535

vsmfSupportInd:

type: boolean

pgwFqdnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

minItems: 1

smfOnboardingCapability:

type: boolean

default: false

deprecated: true

ismfSupportInd:

type: boolean

smfUPRPCapability:

type: boolean

default: false

loOffloadManInd:

type: boolean

default: false

[non-changing parts skipped for clarity]

* * * * Next Change * * * *

# A.3	Nnrf_NFDiscovery API

openapi: 3.0.0

info:

version: '1.4.0-alpha.1'

title: 'NRF NFDiscovery Service'

description: |

NRF NFDiscovery Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.510 V19.0.0; 5G System; Network Function Repository Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.510/'

servers:

- url: '{apiRoot}/nnrf-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnrf-disc

- oAuth2ClientCredentials:

- nnrf-disc

- nnrf-disc:nf-instances:read-complete-profile

paths:

/nf-instances:

get:

summary: Search a collection of NF Instances

operationId: SearchNFInstances

tags:

- NF Instances (Store)

parameters:

- name: Accept-Encoding

in: header

description: Accept-Encoding, described in IETF RFC 9110

schema:

type: string

- name: target-nf-type

in: query

description: Type of the target NF

required: true

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- name: requester-nf-type

in: query

description: Type of the requester NF

required: true

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- name: preferred-collocated-nf-types

in: query

description: collocated NF types that candidate NFs should preferentially support

schema:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/CollocatedNfType'

minItems: 1

style: form

explode: false

- name: requester-nf-instance-id

in: query

description: NfInstanceId of the requester NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

- name: service-names

in: query

description: Names of the services offered by the NF

schema:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

minItems: 1

uniqueItems: true

style: form

explode: false

- name: requester-nf-instance-fqdn

in: query

description: FQDN of the requester NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

- name: target-plmn-list

in: query

description: >

Id of the PLMN of either the target NF, or in SNPN scenario the Credentials Holder

in the PLMN

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

- name: requester-plmn-list

in: query

description: Id of the PLMN where the NF issuing the Discovery request is located

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

- name: target-nf-instance-id

in: query

description: Identity of the NF instance being discovered

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

- name: target-nf-instance-id-list

in: query

description: Identities of the NF instances being discovered

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 2

style: form

explode: false

- name: target-nf-fqdn

in: query

description: FQDN of the NF instance being discovered

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

- name: hnrf-uri

in: query

description: Uri of the home NRF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

- name: snssais

in: query

description: Slice info of the target NF

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

- name: additional-snssais

in: query

description: Additional Slices supported by the target NF (Service) instances

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExtSnssai'

minItems: 1

- name: requester-snssais

in: query

description: Slice info of the requester NF

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExtSnssai'

minItems: 1

- name: plmn-specific-snssai-list

in: query

description: PLMN specific Slice info of the target NF

content:

application/json:

schema:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/PlmnSnssai'

minItems: 1

- name: requester-plmn-specific-snssai-list

in: query

description: PLMN-specific slice info of the NF issuing the Discovery request

content:

application/json:

schema:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/PlmnSnssai'

minItems: 1

- name: dnn

in: query

description: Dnn supported by the BSF, SMF or UPF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: ipv4-index

in: query

description: The IPv4 Index supported by the candidate UPF.

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/IpIndex'

- name: ipv6-index

in: query

description: The IPv6 Index supported by the candidate UPF.

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/IpIndex'

- name: nsi-list

in: query

description: NSI IDs that are served by the services being discovered

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: smf-serving-area

in: query

schema:

type: string

- name: mbsmf-serving-area

in: query

schema:

type: string

- name: tai

in: query

description: Tracking Area Identity

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

- name: amf-region-id

in: query

description: AMF Region Identity

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfRegionId'

- name: amf-set-id

in: query

description: AMF Set Identity

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfSetId'

- name: guami

in: query

description: Guami used to search for an appropriate AMF

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

- name: supi

in: query

description: SUPI of the user

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: ue-ipv4-address

in: query

description: IPv4 address of the UE

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

- name: ip-domain

in: query

description: IP domain of the UE, which supported by BSF

schema:

type: string

- name: ue-ipv6-prefix

in: query

description: IPv6 prefix of the UE

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

- name: pgw-ind

in: query

description: Combined PGW-C and SMF or a standalone SMF

schema:

type: boolean

- name: preferred-pgw-ind

in: query

description: Indicates combined PGW-C+SMF or standalone SMF are preferred

schema:

type: boolean

- name: pgw

in: query

description: PGW FQDN of a combined PGW-C and SMF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

- name: pgw-ip

in: query

description: PGW IP Address of a combined PGW-C and SMF

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

- name: gpsi

in: query

description: GPSI of the user

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

- name: external-group-identity

in: query

description: external group identifier of the user

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

- name: internal-group-identity

in: query

description: internal group identifier of the user

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

- name: pfd-data

in: query

description: PFD data

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/PfdData'

- name: data-set

in: query

description: data set supported by the NF

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/DataSetId'

- name: routing-indicator

in: query

description: routing indicator in SUCI

schema:

type: string

pattern: '^[0-9]{1,4}$'

- name: group-id-list

in: query

description: Group IDs of the NFs being discovered

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

minItems: 1

style: form

explode: false

- name: dnai-list

in: query

description: Data network access identifiers of the NFs being discovered

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

style: form

explode: false

- name: pdu-session-types

in: query

description: list of PDU Session Type required to be supported by the target NF

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

style: form

explode: false

- name: event-id-list

in: query

description: >

Analytics event(s) requested to be supported by the Nnwdaf_AnalyticsInfo service

schema:

type: array

items:

$ref: 'TS29520_Nnwdaf_AnalyticsInfo.yaml#/components/schemas/EventId'

minItems: 1

style: form

explode: false

- name: nwdaf-event-list

in: query

description: >

Analytics event(s) requested to be supported by the Nnwdaf_EventsSubscription service.

schema:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

minItems: 1

style: form

explode: false

- name: upf-event-list

in: query

description: >

Event(s) requested to be supported by the Nupf_EventExposure service.

schema:

type: array

items:

$ref: 'TS29564_Nupf_EventExposure.yaml#/components/schemas/EventType'

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: upf-iwk-eps-ind

in: query

description: UPF supporting interworking with EPS or not

schema:

type: boolean

- name: chf-supported-plmn

in: query

description: PLMN ID supported by a CHF

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: preferred-locality

in: query

description: preferred target NF location

schema:

type: string

- name: ext-preferred-locality

in: query

description: >

preferred target NF location

A map (list of key-value pairs) where the key of the map represents the relative

priority, for the requester, of each locality description among the list of locality

descriptions in this query parameter, encoded as "1" (highest priority"), "2", "3", …,

"n" (lowest priority)

content:

application/json:

schema:

type: object

additionalProperties:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/LocalityDescription'

minItems: 1

minProperties: 1

- name: access-type

in: query

description: AccessType supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

- name: limit

in: query

description: Maximum number of NFProfiles to return in the response

required: false

schema:

type: integer

minimum: 1

- name: required-features

in: query

description: Features required to be supported by the target NF

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

minItems: 1

style: form

explode: false

- name: complex-query

in: query

description: the complex query condition expression

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ComplexQuery'

- name: max-payload-size

in: query

description: Maximum content size of the response expressed in kilo octets

required: false

schema:

type: integer

maximum: 2000

default: 124

- name: max-payload-size-ext

in: query

description: >

Extended query for maximum content size of the response expressed in kilo octets

required: false

schema:

type: integer

default: 124

- name: atsss-capability

in: query

description: ATSSS Capability

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AtsssCapability'

- name: upf-ue-ip-addr-ind

in: query

description: UPF supporting allocating UE IP addresses/prefixes

schema:

type: boolean

- name: client-type

in: query

description: Requested client type served by the NF

content:

application/json:

schema:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/ExternalClientType'

- name: lmf-id

in: query

description: LMF identification to be discovered

content:

application/json:

schema:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LMFIdentification'

- name: an-node-type

in: query

description: Requested AN node type served by the NF

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/AnNodeType'

- name: rat-type

in: query

description: Requested RAT type served by the NF

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

- name: preferred-tai

in: query

description: preferred Tracking Area Identity

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

- name: preferred-nf-instances

in: query

description: preferred NF Instances

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

style: form

explode: false

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in IETF RFC 9110, 13.1.2

schema:

type: string

- name: target-snpn

in: query

description: Target SNPN Identity, or the Credentials Holder in the SNPN

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

- name: requester-snpn-list

in: query

description: SNPN ID(s) of the NF instance issuing the Discovery request

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

- name: af-ee-data

in: query

description: NEF exposured by the AF

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/AfEventExposureData'

- name: w-agf-info

in: query

description: UPF collocated with W-AGF

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/WAgfInfo'

- name: tngf-info

in: query

description: UPF collocated with TNGF

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TngfInfo'

- name: twif-info

in: query

description: UPF collocated with TWIF

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TwifInfo'

- name: upf-select-epdg-info

in: query

description: The ePDG information to find a preferred UPF

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/EpdgInfo'

- name: target-nf-set-id

in: query

description: Target NF Set ID

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

- name: target-nf-service-set-id

in: query

description: Target NF Service Set ID

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

- name: nef-id

in: query

description: NEF ID

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NefId'

- name: notification-type

in: query

description: Notification Type

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NotificationType'

- name: n1-msg-class

in: query

description: N1 Message Class

schema:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N1MessageClass'

- name: n2-info-class

in: query

description: N2 Information Class

schema:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InformationClass'

- name: serving-scope

in: query

description: areas that can be served by the target NF

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: imsi

in: query

description: IMSI of the requester UE to search for an appropriate NF (e.g. HSS, DCSF)

schema:

type: string

pattern: '^[0-9]{5,15}$'

- name: ims-private-identity

in: query

description: IMPI of the requester UE to search for a target HSS or DCSF

schema:

type: string

- name: ims-public-identity

in: query

description: IMS Public Identity of the requester UE to search for a target HSS or DCSF

schema:

type: string

- name: msisdn

in: query

description: MSISDN of the requester UE to search for a target HSS or DCSF

schema:

type: string

- name: preferred-api-versions

in: query

description: Preferred API version of the services to be discovered

content:

application/json:

schema:

description: A map (list of key-value pairs) where ServiceName serves as key

type: object

additionalProperties:

type: string

minProperties: 1

- name: v2x-support-ind

in: query

description: PCF supports V2X

schema:

type: boolean

- name: redundant-gtpu

in: query

description: UPF supports redundant gtp-u to be discovered

schema:

type: boolean

- name: redundant-transport

in: query

description: UPF supports redundant transport path to be discovered

schema:

type: boolean

- name: ipups

in: query

description: UPF which is configured for IPUPS functionality to be discovered

schema:

type: boolean

- name: sxa-ind

in: query

description: UPF which is configured to support sxa interface

schema:

type: boolean

- name: scp-domain-list

in: query

description: SCP domains the target SCP or SEPP belongs to

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: address-domain

in: query

description: Address domain reachable through the SCP

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

- name: ipv4-addr

in: query

description: IPv4 address reachable through the SCP

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

- name: ipv6-prefix

in: query

description: IPv6 prefix reachable through the SCP

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

- name: served-nf-set-id

in: query

description: NF Set ID served by the SCP

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

- name: remote-plmn-id

in: query

description: Id of the PLMN reachable through the SCP or SEPP

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: remote-snpn-id

in: query

description: Id of the SNPN reachable through the SCP or SEPP

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

- name: data-forwarding

in: query

description: UPF Instance(s) configured for data forwarding are requested

schema:

type: boolean

- name: preferred-full-plmn

in: query

description: NF Instance(s) serving the full PLMN are preferred

schema:

type: boolean

- name: requester-features

in: query

description: >

Features supported by the NF Service Consumer that is invoking

the Nnrf_NFDiscovery service

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: realm-id

in: query

description: realm-id to search for an appropriate UDSF

schema:

type: string

- name: storage-id

in: query

description: storage-id to search for an appropriate UDSF

schema:

type: string

- name: vsmf-support-ind

in: query

description: V-SMF capability supported by the target NF instance(s)

schema:

type: boolean

- name: ismf-support-ind

in: query

description: I-SMF capability supported by the target NF instance(s)

schema:

type: boolean

- name: nrf-disc-uri

in: query

description: Uri of the NRF holding the NF profile of a target NF Instance

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

- name: preferred-vendor-specific-features

in: query

description: Preferred vendor specific features of the services to be discovered

content:

application/json:

schema:

description: A map (list of key-value pairs) where ServiceName serves as key

type: object

additionalProperties:

description: >

A map (list of key-value pairs) where IANA-assigned SMI Network Management

Private Enterprise Codes serves as key

type: object

additionalProperties:

type: array

items:

$ref:  'TS29510_Nnrf_NFManagement.yaml#/components/schemas/VendorSpecificFeature'

minItems: 1

minProperties: 1

minProperties: 1

- name: preferred-vendor-specific-nf-features

in: query

description: Preferred vendor specific features of the network function to be discovered

content:

application/json:

schema:

description: >

A map (list of key-value pairs) where IANA-assigned SMI Network Management Private

Enterprise Codes serves as key

type: object

additionalProperties:

type: array

items:

$ref:  'TS29510_Nnrf_NFManagement.yaml#/components/schemas/VendorSpecificFeature'

minItems: 1

minProperties: 1

- name: required-pfcp-features

in: query

description: PFCP features required to be supported by the target UPF

schema:

type: string

- name: home-pub-key-id

in: query

description: >

Indicates the Home Network Public Key ID which shall be able to be served

by the NF instance

schema:

type: integer

- name: prose-support-ind

in: query

description: PCF supports ProSe Capability

schema:

type: boolean

- name: analytics-aggregation-ind

in: query

description: analytics aggregation is supported by NWDAF or not

schema:

type: boolean

- name: serving-nf-set-id

in: query

description: NF Set Id served by target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

- name: serving-nf-type

in: query

description: NF type served by the target NF

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- name: ml-analytics-info-list

in: query

description: Lisf of ML Analytics Filter information of Nnwdaf_MLModelProvision service

content:

application/json:

schema:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/MlAnalyticsInfo'

minItems: 1

- name: analytics-metadata-prov-ind

in: query

description: analytics matadata provisioning is supported by NWDAF or not

schema:

type: boolean

- name: nsacf-capability

in: query

description: the service capability supported by the target NSACF

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NsacfCapability'

- name: mbs-session-id-list

in: query

description: List of MBS Session ID(s)

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

minItems: 1

- name: area-session-id

in: query

description: Area Session ID

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AreaSessionId'

- name: gmlc-number

in: query

description: The GMLC Number supported by the GMLC

schema:

type: string

pattern: '^[0-9]{5,15}$'

- name: upf-n6-ip

in: query

description: N6 IP address of PSA UPF supported by the EASDF

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

- name: tai-list

in: query

description: Tracking Area Identifiers of the NFs being discovered

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

- name: nf-tai-list-ind

in: query

description: the NF service consumer supports candidate nfs supporting a subset of TAIs

schema:

type: boolean

enum:

- true

- name: preferences-precedence

in: query

description: >

Indicates the precedence of the preference query parameters (from higher to lower)

schema:

type: array

items:

type: string

minItems: 2

style: form

explode: false

- name: support-onboarding-capability

in: query

description: Indicating the support for onboarding.

schema:

type: boolean

default: false

- name: uas-nf-functionality-ind

in: query

description: UAS NF functionality is supported by NEF or not

schema:

type: boolean

- name: multi-mem-af-sess-qos-ind

in: query

description: Multi-member AF session with required QoS is supported by NEF or not

schema:

type: boolean

enum:

- true

- name: member-ue-sel-assist-ind

in: query

description: member UE selection assistance functionality is supported by NEF or not

schema:

type: boolean

enum:

- true

- name: v2x-capability

in: query

description: indicates the V2X capability that the target PCF needs to support.

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/V2xCapability'

- name: prose-capability

in: query

description: indicates the ProSe capability that the target PCF needs to support.

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ProSeCapability'

- name: shared-data-id

in: query

description: Identifier of shared data stored in the NF being discovered

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SharedDataId'

- name: target-hni

in: query

description: Home Network Identifier query.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

- name: target-nw-resolution

in: query

description: Resolution of the identity of the target PLMN based on the GPSI of the UE

schema:

type: boolean

- name: exclude-nfinst-list

in: query

description: NF Instance IDs to be excluded from the NF Discovery procedure

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

style: form

explode: false

- name: exclude-nfservinst-list

in: query

description: NF service instance IDs to be excluded from the NF Discovery procedure

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/NfServiceInstance'

minItems: 1

- name: exclude-nfserviceset-list

in: query

description: NF Service Set IDs to be excluded from the NF Discovery procedure

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

minItems: 1

style: form

explode: false

- name: exclude-nfset-list

in: query

description: NF Set IDs to be excluded from the NF Discovery procedure

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

minItems: 1

style: form

explode: false

- name: preferred-analytics-delays

in: query

description: Preferred analytics delays supported by the NWDAF to be discovered

content:

application/json:

schema:

description: >

A map (list of key-value pairs) where EventId or NwdafEvent serves as key

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

minProperties: 1

- name: high-latency-com

in: query

description: Indicating the support for High Latency communication.

schema:

type: boolean

enum:

- true

- name: nsac-sai

in: query

description: NSAC Service Area Identifier

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NsacSai'

- name: complete-profile

in: query

description: request to discover the complete profile of NF instances

schema:

type: boolean

enum:

- true

- name: n32-purposes

in: query

description: N32 purposes to be supported by the SEPP

schema:

type: array

items:

$ref: 'TS29573_N32_Handshake.yaml#/components/schemas/N32Purpose'

minItems: 1

style: form

explode: false

- name: preferred-features

in: query

description: Preferred features to be supported by the target Network Function.

content:

application/json:

schema:

description: >

A map (list of key-value pairs) where Service Name serves as the key.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

minProperties: 1

- name: remote-plmn-id-roaming

in: query

description: Id of the remote PLMN served by the target NF service producer

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: pru-tai

in: query

description: LMF(s) serving the TAI with PRU(s) existence

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

- name: pru-support-ind

in: query

description: Indicating the support of PRU function

schema:

type: boolean

- name: af-data

in: query

description: events supported by the trusted AFs being discovered

content:

application/json:

schema:

$ref: '#/components/schemas/AfData'

- name: ml-accuracy-checking-ind

in: query

description: Indicating the support for ML Model Accuracy checking.

schema:

type: boolean

enum:

- true

- name: analytics-accuracy-checking-ind

in: query

description: Indicating the support for Analytics Accuracy checking.

schema:

type: boolean

enum:

- true

- name: a2x-support-ind

in: query

description: PCF supports A2X

schema:

type: boolean

enum:

- true

- name: a2x-capability

in: query

description: indicates the A2X capability that the target PCF needs to support.

content:

application/json:

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/A2xCapability'

- name: ml-model-storage-ind

in: query

description: Indicating the support for ML model storage and retrieval capability.

schema:

type: boolean

enum:

- true

- name: data-storage-ind

in: query

description: >

Indicating the support for data and analytics storage and retrieval capability.

schema:

type: boolean

enum:

- true

- name: data-subscription-relocation-support-ind

in: query

description: Indicating the support for relocation of data subscription.

schema:

type: boolean

enum:

- true

- name: ims-domain-name

in: query

description: Indicating the IMS domain name to search for a target DCSF.

schema:

type: string

- name: media-capability-list

in: query

description: Indicating the media capability list to search for a target MF, MRF or MRFP.

schema:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/MediaCapability'

minItems: 1

style: form

explode: false

- name: roaming-exchange-ind

in: query

description: Indicating the support for roaming exchange.

schema:

type: boolean

enum:

- true

- name: ranging-sl-pos-support-ind

in: query

description: PCF or LMF supports ranging and sidelink positioning Capability

schema:

type: boolean

enum:

- true

- name: preferred-up-positioning-ind

in: query

description: LMF supporting user plane positioning capability

schema:

type: boolean

enum:

- true

- name: complete-search-result

in: query

description: >
            Indicates that all the NF profiles or NF Instance IDs matching the query parameters

are requested to be returned

schema:

type: boolean

enum:

- true

- name: ursp-delivery-eps-support-ind

in: query

description: >
            Indicates whether a PCF supporting URSP delivery in EPS needs to be discovered

schema:

type: boolean

enum:

- true

- name: dns-sec-protoc

in: query

description: DNS security protocols

schema:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/DnsSecurityProtocol'

minItems: 1

style: form

explode: false

- name: ind-com-with-del-disc

in: query

description: >
            Indicates that indirect communication with delegated discovery

with NF selection at target domain feature is supported.

schema:

type: boolean

enum:

- true

- name: ind-com-wo-del-disc

in: query

description: >
            Indicates that indirect communication without delegated discovery

with NF selection at target domain feature is supported.

schema:

type: boolean

enum:

- true

- name: local-offload-manage-ind

in: query

description: >
            Indicates that an SMF (I-SMF) that supports local offloading management

is reuested to be discovered.

schema:

type: boolean

enum:

- true

[non-changing parts skipped for clarity]

* * * * End of Changes * * *