3GPP TSG-CT WG4 Meeting #126	C4-245154

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.175 | CR | 0029 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Report the PS Data Off Status to DCSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support the PS data off of the services over IMS data channel, S2-2410914 agreed that: In the context of IMS Data Channel, the AS may further notify the DCSF about the UE 3GPP PS Data Off status. Which should be supported in the Nimsas_SessionEventControl service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Nimsas_SessionEventControl service to support reporting PS Data Off Staus to the DCSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Does not support the feature of SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.4, 6.1.6.3.x(new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features to the OpenAPI files of Nimsas_SessionEventControl. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nimsas_SessionEventControl service based interface protocol.

Table 6.1.6.1-1: Nimsas_SessionEventControl specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SessionEventNotification | 6.1.6.2.2 | The session event notification item. |
| NotificationEvent | 6.1.6.2.3 | The notification event information. |
| SessionInfo | 6.1.6.2.4 | The session information related to the notification event. |
| MediaInfo | 6.1.6.2.5 | The media information related to the notification event. |
| DcMediaSpec | 6.1.6.2.6 | The data channel media specification information which includes media description attributes. |
| EventType | 6.1.6.3.3 | The type of the event for which the notification is generated. |
| MediaType | 6.1.6.3.4 | The type of the media for which the notification is generated. |
| SessionCase | 6.1.6.3.5 | The type of the session case. |
| EventInitiator | 6.1.6.3.6 | The initiator of the event for which the notification is generated. |
| PsDataOffStatus | 6.1.6.3.x | The status of the PS Data Off. |

Table 6.1.6.1-2 specifies data types re-used by the Nimsas_SessionEventControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nimsas_SessionEventControl service based interface.

Table 6.1.6.1-2: Nimsas_SessionEventControl re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ImsPublicId | 3GPP TS 29.562 [15] | IMS Public Identity. |
| DcStream | 3GPP TS 29.571 [16] | Data Channel mapping and configuration information. |
| MaxMessageSize | 3GPP TS 29.571 [16] | Maximum message size of to be expected. |
| DcEndpoint | 3GPP TS 29.571 [16] | local or remote endpoint for the Data Channel. |
| SessionId | 3GPP TS 29.571 [16] | IMS Session Identity. |
| MediaId | 3GPP TS 29.571 [16] | IMS media flow identity. |
| BdcUsedBy | 3GPP TS 29.571 [16] | The party using the bootstrap data channel. |
| AppBindingInfo | 3GPP TS 29.571 [16] | The application binding information of the application data channel. |

* * * Next Change * * * *

6.1.6.2.4	Type: SessionInfo

Table 6.1.6.2.4-1: Definition of type SessionInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| callingIdentity | ImsPublicId | O | 0..1 | The public identity of the calling IMS subscriber. |
| calledIdentity | ImsPublicId | O | 0..1 | The public identity of the called IMS Subscriber. |
| sessionCase | SessionCase | O | 0..1 | Indicator to indicate that whether this is an originating or terminating IMS session. |
| psDataOffStatus | PsDataOffStatus | O | 0..1 | Indicates whether the UE served by the IMS AS actives the PS Data Off. |

* * * Next Change * * * *

6.1.6.3.x	Enumeration: PsDataOffStatus

The enumeration PsDataOffStatus indicates whether the UE that the IMS AS serves actives the PS Data Off or not. It shall comply with the provisions defined in table 6.1.6.3.x-1.

Table 6.1.6.3.x-1: Enumeration PsDataOffStatus

| Enumeration value | Description |
| --- | --- |
| "ACTIVE" | The UE actives the PS Data Off. |
| "INACTIVE" | The UE inactives the PS Data Off. |

* * * Next Change * * * *

A.2	Nimsas_SessionEventControl API

openapi: 3.0.0

info:

{Not shown for clarity}

SessionInfo:

description: The session information related to the notification event.

type: object

properties:

callingIdentity:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

calledIdentity:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

sessionCase:

$ref: '#/components/schemas/SessionCase'

psDataOffStatus:

$ref: '#/components/schemas/PsDataOffStatus'

{Not shown for clarity}

SessionCase:

description: The type of the session case.

anyOf:

- type: string

enum:

- ORIGINATING_IMS_SESSION

- TERMINATING_IMS_SESSION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

EventInitiator:

description: The initiator of the event for which the notification is generated.

anyOf:

- type: string

enum:

- SERVED_IMS_SUBSCRIBER

- REMOTE_IMS_SUBSCRIBER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

PsDataOffStatus:

description: The status of the PS Data Off.

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

* * * End of Changes * * * *