3GPP TSG-CT WG4 Meeting #126	C4-245190

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.128 | CR | 0084 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support monitoring event for store and forward operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2 agreed CR (S2-2410985, TS 23.682 CR#0495) to define new monitoring event and reporting for Store and Forward operation for satellite access. Accordingly, it is proposed to adapt the specification to meet the Stage 2 requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Add new supported monitoring event. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.2.Y (New), 6.4.1, 6.4.3, 6.4.XX (New), 6.4.YY (New), 6.4.ZZ (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.682 CR 0495 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.2.1	General

This procedure is used between the MME/SGSN and the SCEF, between the MME/SGSN and the IWK-SCEF and between the IWK-SCEF and the SCEF.

When the procedure is invoked by the MME or the SGSN, it is used for reporting:

-	UE Loss of Connectivity;

-	UE Reachability;

-	Location of the UE and change in location of the UE;

-	Communication Failure;

-	Availability after DNN failure;

-	Idle Status Indication;

-	PDN Connectivity Status;

-	Store and Forward Satellite operation information.

When the procedure is invoked by the IWK-SCEF, it is used for conveying the monitoring event reported by the MME or the SGSN to the SCEF, after applying the roaming policies configured at the IWK-SCEF.

This procedure is mapped to the commands Reporting-Information-Request/Answer in the Diameter application specified in clause 6. The tables 5.2.1-1 and 5.2.1-2 detail the involved information elements.

Table 5.2.1-1: Reporting Information Request

| Information Element Name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identity (See 6.4.16) | User-Identifier | C | If present, this information element shall contain the identity of the UE. This is a grouped AVP containing the MSISDN or the External-Identifier. |
| Monitoring Event Report (see 3GPP TS 29.336 [5] clause 8.4.3) | Monitoring-Event-Report | C | If the Reporting-Information-Request is sent for reporting the monitoring events, the MME/SGSN and the IWK-SCEF shall include the monitoring event(s) reported towards the SCEF. |
| Supported Features (See 3GPP TS 29.229 [4]) | Supported-Features | O | If present, this information element shall contain the list of features supported by the origin host. |

Table 5.2.1-2: Reporting Information Answer

| Information Element Name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Result (See 6.3) | Result-Code / Experimental-Result | M | Result of the request. Result-Code AVP shall be used for errors defined in the Diameter Base Protocol. Experimental-Result AVP shall be used for T6a/b errors. This is a grouped AVP, which contains the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP. |
| Supported Features (See 3GPP TS 29.229 [4]) | Supported-Features | O | If present, this information element shall contain the list of features supported by the origin host. |
| Monitoring Event Report Status | Monitoring-Event-Report-Status | O | If present, this information element shall contain the status of each individual monitoring event report received by the SCEF from the MME/SGSN. |

* * * Next Change * * * *

#### 5.2.2.Y	Store and Forward Satellite operation information

The following AVPs shall be present within the Monitoring-Event-Report AVP when the Store and Forward Satellite operation information event is reported:

-	Monitoring-Type set to SAT_SF_OPERATION_INFO (X)

-	SAT-SF-Operation-Info

* * * Next Change * * * *

### 6.4.1	General

The following table specifies the Diameter AVPs defined for the T6a/T6b interface protocol, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415).

For all AVPs which contain bit masks and are of the type Unsigned32, bit 0 shall be the least significant bit. For example, to get the value of bit 0, a bit mask of 0x00000001 should be used.

Table 6.4.1-1: T6a/T6b specific Diameter AVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encr. |
| Communication-Failure-Information | 4300 | 6.4.4 | Grouped | M,V |  |  |  | No |
| Cause-Type | 4301 | 6.4.5 | Unsigned32 | M,V |  |  |  | No |
| S1AP-Cause | 4302 | 6.4.6 | Unsigned32 | M,V |  |  |  | No |
| RANAP-Cause | 4303 | 6.4.7 | Unsigned32 | M,V |  |  |  | No |
| BSSGP-Cause | 4309 | 6.4.8 | Unsigned32 | M,V |  |  |  | No |
| GMM-Cause | 4304 | 6.4.9 | Unsigned32 | M,V |  |  |  | No |
| SM-Cause | 4305 | 6.4.10 | Unsigned32 | M,V |  |  |  | No |
| Number-Of-UE-Per-Location-Configuration | 4306 | 6.4.11 | Grouped | M,V |  |  |  | No |
| Number-Of-UE-Per-Location-Report | 4307 | 6.4.12 | Grouped | M,V |  |  |  | No |
| UE-Count | 4308 | 6.4.13 | Unsigned32 | M,V |  |  |  | No |
| Connection-Action | 4314 | 6.4.18 | Unsigned32 | M,V |  |  |  | No |
| Non-IP-Data | 4315 | 6.4.19 | OctetSstring | M,V |  |  |  | No |
| Serving-PLMN-Rate-Control | 4310 | 6.4.21 | Grouped | M,V |  |  |  | No |
| Uplink-Rate-Limit | 4311 | 6.4.23 | Unsigned32 | M,V |  |  |  | No |
| Downlink-Rate-Limit | 4312 | 6.4.22 | Unsigned32 | M,V |  |  |  | No |
| Extended-PCO | 4313 | 6.4.26 | OctetString | M,V |  |  |  | No |
| SCEF-Wait-Time | 4316 | 6.4.24 | Time | M,V |  |  |  | No |
| CMR-Flags | 4317 | 6.4.25 | Unsigned32 | M,V |  |  |  | No |
| RRC-Cause-Counter | 4318 | 6.4.27 | Grouped | M,V |  |  |  | No |
| Counter-Value | 4319 | 6.4.28 | Unsigned32 | M,V |  |  |  | No |
| RRC-Counter-Timestamp | 4320 | 6.4.29 | Time | M,V |  |  |  | No |
| TDA-Flags | 4321 | 6.4.31 | Unsigned32 | V |  |  | M | No |
| Idle-Status-Indication | 4322 | 6.4.32 | Grouped | V |  |  | M | No |
| Idle-Status-Timestamp | 4323 | 6.4.33 | Time | V |  |  | M | No |
| Active-Time | 4324 | 6.4.34 | Unsigned32 | V |  |  | M | No |
| Reachability-Cause | 4325 | 6.4.35 | Unsigned32 | V |  |  | M | No |
| APN-Rate-Control-Status | 4326 | 6.4.36 | Grouped | V |  |  | M | No |
| Uplink-Number-Of-Packets-Allowed | 4327 | 6.4.37 | Unsigned32 | V |  |  | M | No |
| Number-Of-Additional-Exception-Reports | 4328 | 6.4.38 | Unsigned32 | V |  |  | M | No |
| Downlink-Number-Of-Packets-Allowed | 4329 | 6.4.39 | Unsigned32 | V |  |  | M | No |
| APN-Rate-Control-Status-Validity-Time | 4330 | 6.4.40 | Unsigned64 | V |  |  | M | No |
| SF-Operation-Status | XXXX | 6.4.XX | Unsigned32 | V |  |  | M | No |
| DL-Estimated-Delivery-Time | YYYY | 6.4.YY | Unsigned32 | V |  |  | M | No |
| SAT-SF-Operation-Info | ZZZZ | 6.4.ZZ | Grouped | V |  |  | M | No |
| NOTE 1: The AVP header bit denoted as "M" indicates whether support of the AVP is required. The AVP header bit denoted as "V" indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [32]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |  |  |  |  |  |

The following table specifies the Diameter AVPs re-used by the T6a/T6b interface protocol from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their use within T6a/T6b.

Any other AVPs from existing Diameter Applications, except for the AVPs from Diameter Base Protocol, do not need to be supported. The AVPs from Diameter Base Protocol are not included in table 6.4.1-2, but they may be re-used for the T6a/T6b protocol.

Table 6.4.1-2: T6a/T6b re-used Diameter AVPs

| Attribute Name | Reference | Comments |
| --- | --- | --- |
| Monitoring-Event-Configuration | 3GPP TS 29.336 [5] | This AVP shall contain the monitoring event to be configured at the MME/SGSN or the IWK-SCEF. See 6.4.2. |
| Monitoring-Event-Report | 3GPP TS 29.336 [5] | This AVP shall contain the monitoring event reported by the MME/SGSN or the IWK-SCEF. See 6.4.3. |
| SCEF-Reference-ID | 3GPP TS 29.336 [5] |  |
| SCEF-ID | 3GPP TS 29.336 [5] |  |
| SCEF-Reference-ID-for-Deletion | 3GPP TS 29.336 [5] |  |
| Supported-Features | 3GPP TS 29.229 [4] |  |
| Feature-List-ID | 3GPP TS 29.229 [4] |  |
| Feature-List | 3GPP TS 29.229 [4] | See 6.4.14 |
| OC-Supported-Features | IETF RFC 7683 [9] |  |
| OC-OLR | IETF RFC 7683 [9] |  |
| Monitoring-Event-Config-Status | 3GPP TS 29.336 [5] | This AVP shall contain the status of configuration of each monitoring event identified by an SCEF-ID and SCEF-Reference-ID. |
| DRMP | IETF RFC 7944 [15] | see 6.4.15 |
| User-Identifier | 3GPP TS 29.336 [5] | See 6.4.16 |
| Bearer-Identity | 3GPP TS 29.212 [10] | See 6.4.17 |
| Monitoring-Type | 3GPP TS 29.336 [5] |  |
| Loss-Of-Connectivity-Reason | 3GPP TS 29.336 [5] |  |
| Maximum-Number-of-Reports | 3GPP TS 29.336 [5] |  |
| Monitoring-Duration | 3GPP TS 29.336 [5] |  |
| Charged-Party | 3GPP TS 32.299 [20] |  |
| UE-Reachability-Configuration | 3GPP TS 29.336 [5] |  |
| Location-Information-Configuration | 3GPP TS 29.336 [5] |  |
| Reachability-Information | 3GPP TS 29.336 [5] |  |
| EPS-Location-Information | 3GPP TS 29.272 [16] |  |
| Service-Selection | IETF RFC 5778 [21] | See 6.4.20 |
| PDN-Connection-Charging-Id | 3GPP TS 32.299 [22] |  |
| Maximum-Retransmission-Time | 3GPP TS 29.338 [27] |  |
| Requested-Retransmission-Time | 3GPP TS 29.338 [27] |  |
| Maximum-UE-Availability-Time | 3GPP TS 29.338 [27] |  |
| 3GPP-Charging-Characteristics | 3GPP TS 29.061 [29] |  |
| RAT-Type | 3GPP TS 29.212 [10] |  |
| Terminal-Information | 3GPP TS 29.272 [16] | See 6.4.30 |
| Visited-PLMN-Id | 3GPP TS 29.272 [16] |  |
| Load | IETF RFC 8583 [31] |  |
| Subscribed-Periodic-RAU-TAU-Timer | 3GPP TS 29.272 [16] |  |
| Monitoring-Event-Report-Status | 3GPP TS 29.336 [5] |  |
| IMSI-Group-Id | 3GPP TS 29.272 [16] |  |
| Reporting-Time-Stamp | 3GPP TS 29.336 [5] |  |
| eDRX-Cycle-Length | 3GPP TS 29.272 [16] |  |
| DL-Buffering-Suggested-Packet-Count | 3GPP TS 29.272 [16] |  |
| PDN-Connectivity-Status-Report | 3GPP TS 29.336 [5] |  |
| SCEF-Reference-ID-Ext | 3GPP TS 29.336 [5] |  |
| SCEF-Reference-ID-for-Deletion-Ext | 3GPP TS 29.336 [5] |  |

* * * Next Change * * * *

### 6.4.3	Monitoring-Event-Report

The Monitoring-Event-Report AVP is of type Grouped. It shall contain the Monitoring event report data. It is originally defined in 3GPP TS 29.336 [5].

For the T6a/T6b interface, the Monitoring-Event-Report AVP format is specified as following:

AVP format:

Monitoring-Event-Report ::= <AVP header: 3123 10415>

{ SCEF-Reference-ID }

[ SCEF-Reference-ID-Ext ]

[ SCEF-ID ]

[ Monitoring-Type ]

[ Reachability-Information ]

[ EPS-Location-Information ]

[ Communication-Failure-Information ]

*[ Number-Of-UE-Per-Location-Report ]

[ Loss-Of-Connectivity-Reason ]

[ Visited-PLMN-Id ]

[ Idle-Status-Indication ]

[ Reporting-Time-Stamp ]

[ Maximum-UE-Availability-Time ]

*[ PDN-Connectivity-Status-Report ]

[ SAT-SF-Operation-Info ]

[ Reachability-Cause ]

*[AVP]

The AVPs applicable for each Monitoring-Type reported by the MME/SGSN are specified under clause 5.2.2.

When the "Extended Reference IDs" feature is supported by the SCEF and MME/SGSN, the SCEF-Reference-ID-Ext AVP shall be used insted of SCEF-Reference-ID; in such case, the required AVP "SCEF-Reference-ID" shall be included in the grouped AVP by the sender, but its content shall be discarded by the receiver.

* * * Next Change * * * *

### 6.4.XX	SF-Operation-Status

The SF-Operation-Status AVP is of type Unsigned32. It shall contain the Store and Forward operation status of the UE. The following values are defined:

IN_SF_MODE (0)

OUT_OF_SF_MODE (1)

* * * Next Change * * * *

### 6.4.YY	DL-Estimated-Delivery-Time

DL-Estimated-Delivery-Time AVP is of type Unsigned32 and shall contain the downlink estimated delivery time.

* * * Next Change * * * *

### 6.4.ZZ	SAT-SF-Operation-Info

The SAT-SF-Operation-Info AVP is of type Grouped, and it shall contain the Store and Forward operation information for satellite access.

AVP format:

SAT-SF-Operation-Info::=	<AVP header: ZZZZ 10415>

{ SF-Operation-Status }

[ DL-Estimated-Delivery-Time ]

*[AVP]

* * * End of Changes * * * *