3GPP TSG-CT WG4 Meeting #126	C4-245204

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0303 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on the Time Windows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3GPP TS 23.273#0615 (S2-2410896) specifies the reference for the time window to provide a good guidance for stage 3 work. NOTE x: The time window(s) is the Time Window Information SRS List and the Time Window Information Measurement List specified in TS 38.455[15] when network assisted positioning is used and is the NR-DL-PRS-MeasurementTimeWindowsConfig specified in TS 37.355[20] when UE assisted positioning is used. “The time window(s) is the Time Window Information SRS List and the Time Window Information Measurement List specified in TS 38.455[15] when network assisted positioning is used” is not mentioned in Stage 3. It proposes to update the description of Time Windows |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of Time Windows |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with Stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0615 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not introduce any changes to OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[7]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[12]	IETF RFC 9113: "HTTP/2".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[15]	IETF RFC 9457: "Problem Details for HTTP APIs".

[16]	3GPP TR 21.900: "Technical Specification Group working methods".

[17]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[18]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[19]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[20]	3GPP TS 24.080: "Mobile radio interface layer 3 Supplementary services specification; Formats and coding".

[21]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 29.518: "Access and Mobility Management Services".

[24]	3GPP TS 29.171: "Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[25]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[26]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[27]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[28]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[29]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[30]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[xx]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

* * * * Next Change * * * *

6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Altitude | number | Double-precision float value of the altitude, expressed in meters. Minimum: -32767. Maximum: 32767. Format: double. |
| Angle | integer | Integer value of the angle, expressed in degrees. Minimum: 0. Maximum: 360. |
| Uncertainty | number | Float value of uncertainty, expressed in meters. Minimum: 0 Format: float. |
| Orientation | integer | Integer value of the orientation angle, expressed in degrees. Minimum: 0. Maximum: 180. |
| HorizAxesOrientation | integer | Integer value of the "y" axis orientation angle clockwise from northing, expressed in 0.1 degrees. Minimum: 0. Maximum: 3600. |
| Confidence | integer | Integer value of the confidence, expressed in percentage value. Minimum: 0. Maximum: 100. |
| Accuracy | number | Float value of accuracy, expressed in meters. Minimum: 0 Format: float. |
| InnerRadius | integer | Integer value of the inner radius, expressed in meters. Minimum: 0. Maximum: 327675. Format: int32. |
| CorrelationID | string | LCS Correlation ID. The correlation ID shall be of a minimum length of 1 character and maximum length of 255 characters. |
| AgeOfLocationEstimate | integer | Integer value of the age of the location estimate, expressed in minutes. Minimum: 0. Maximum: 32767. |
| HorizontalSpeed | number | Float value of horizontal speed, expressed in kilometres per hour. Minimum: 0. Maximum: 2047. Format: float. |
| VerticalSpeed | number | Float value of horizontal speed, expressed in kilometres per hour. Minimum: 0. Maximum: 255. Format: float. |
| SpeedUncertainty | number | Float value of speed uncertainty, expressed in kilometres per hour. Minimum: 0. Maximum: 255. Format: float. |
| BarometricPressure | integer | This IE specifies the measured uncompensated atmospheric pressure in units of Pascal (Pa). Minimum: 30000. Maximum: 115000. |
| LcsServiceType | integer | The LCS service type as defined in 3GPP TS 22.071 [17] and clause 17.7.8 of 3GPP TS 29.002 [18]. Minimum: 0. Maximum: 127. |
| LdrReference | string | LDR Reference encoded as a string of hexadecimal characters. The LdrReference shall be of a minimum length of 2 characters and maximum length of 510 characters. |
| LirReference | string | LIR Reference encoded as a string of hexadecimal characters. The LdrReference shall be of a minimum length of 2 characters and maximum length of 510 characters. |
| ReportingAmount | integer | Number of required periodic event reports. Minimum: 1. Maximum: 8639999. |
| ReportingInterval | integer | Event reporting periodic interval in seconds. Minimum: 1. Maximum: 8639999. ReportingInterval x ReportingAmount shall not exceed 8639999. |
| ReportingIntervalMs | integer | Event reporting periodic interval in milliseconds. Minimum: 1. Maximum: 999. |
| MinimumInterval | integer | Minimum interval between event reports in seconds. Minimum: 1. Maximum: 32767. |
| MaximumInterval | integer | Maximum interval between event reports in seconds. Minimum: 1. Maximum: 86400. |
| SamplingInterval | integer | Maximum time interval between consecutive evaluations by a UE of a trigger event, in seconds. Minimum: 1. Maximum: 3600 |
| ReportingDuration | integer | Maximum duration of event reporting, in seconds. Minimum: 1. Maximum: 8640000. |
| LinearDistance | integer | The minimum straight line distance moved by a UE to trigger a motion event report, in meters. Minimum: 1. Maximum: 10000. |
| LMFIdentification | string | The serving LMF identification as defined in 3GPP TS 23.273 [19], encoded as a string of hexadecimal characters. |
| EventReportCounter | integer | Number of event reports received from the target UE. Minimum: 1. Maximum: 8640000. Note: the current event report is included in the count. |
| EventReportDuration | integer | Duration of event reporting, in seconds. Minimum: 0. Maximum: 8640000. Note: the duration starts when event reporting is activated in the UE and extends to the current time. |
| UePositioningCapabilities | Bytes | Positioning capabilities supported by the UE. String with format "byte" as defined in OpenAPI Initiative [14], i.e. base64-encoded characters, encoding the "ProvideCapabilities-r9-IEs" IE as specified in clause 6.3 of 3GPP TS 37.355 [21] (start from octet 1). |
| TimeWindow | Bytes | Time window(s) of the target UE. String with format "byte" as defined in OpenAPI Initiative [14], i.e. base64-encoded characters, encoding the "NR-DL-PRS-MeasurementTimeWindowsConfig-r18" IE as specified in clause 6.4.3 of 3GPP TS 37.355 [21] when UE assisted positioning is used, or Time Window Information SRS List and the Time Window Information Measurement List specified in clause 9.2.90 and clause 9.2.91 of 3GPP TS 38.455 [xx] when network assisted positioning is used (start from octet 1). |
| RangingSlCapability | Bytes | Ranging/Sidelink Positioning Capability supported by the UE. String with format "byte" as defined in OpenAPI Initiative [14], i.e. base64-encoded characters, encoding the "ProvideCapabilities" IE as specified in clause 6.2.2 of 3GPP TS 38.355 [30] (start from octet 1). |

* * * * End of Changes * * * *