3GPP TSG-CT WG4 Meeting #126	C4-245233

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 1369 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of GET method to the PP Data resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2024-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The resource "PP Data" on Numd_PP does not have a GET method defined (it only has a PATCH method). It is quite unusual to define a resource without a GET method, specically when the resource follows the "Document" archetype, since it makes it impossible for the consumer to retrieve the representation of such resource. This is very awkward in a REST architecture since, normally, a consumer should typically be expected to retrieve the representation of a given resource in order to do further operations with it (e.g. PATCH). In addition, not being able to retrieve the resource representation, creates a problem when the resource needs to be updated (PATCH) using the Merge-Patch format: When the consumer needs to modify an atribute of PpData defined as an array (e.g. to add a new array item, or to delete an existing array item), the consumer needs to replace the entire array. But, since a GET operation is not available for the PpData resource, it is not possible to, first, read the resource to obtain the entire array, then modify its content locally at the consumer side, and lastly replace (via Merge-Patch) on the producer side the existing array with the new list of values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a GET method to the PP Data resource. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Updates of the PP Data resource becomes unnecessarily complex (or impossible) without a GET method to retrieve the resource representation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.1, 5.6.2.5.1, 5.6.2.5.x (new), 6.5.3.1, 6.5.3.2.3.x (new), A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features with impacts on the following APIs: - TS29503_Ndum_PP.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.6.2.1	Introduction

For the Nudm_ParameterProvision service the following service operations are defined:

-	Update

-	Create

-	Delete

-	Get

The Nudm_ParameterProvision service is used by consumer NFs (e.g. NEF) to update a UE's or a group of UEs' subscription data by means of the Update service operation. For details see 3GPP TS 23.502 [3] clause 4.15.6.2.

The Nudm_ParameterProvision service can also be used by a NF Service Consumer (e.g. SOR-AF) to send updated Steering of Roaming Information for a UE to the UDM at any time, as specified in Annex C.3 of 3GPP TS 23.122 [20].

The Nudm_ParameterProvision service can also be used by a NF Service Consumer (e.g. AF) to configure network slice usage control information (e.g. slice de-registration inactivity timer, PDU session inactivity timer for a network slice), as specified in 3GPP TS 23.501 [2] clause 5.15.15.2 and clause 5.15.15.3.

* * * Next Change * * * *

##### 5.6.2.5.1	General

The following procedures using the Get service operation are supported:

-	5G-VN-Group get

-	Parameter Provisioning Data Entry per AF get

-	5G-MBS-Group get

-	Subscription Data (PP Data) get

* * * Next Change * * * *

##### 5.6.2.5.x	Subscription Data (PP Data) get

Figure 5.6.2.5.x-1 shows a scenario where the NF service consumer (e.g. NEF) sends a request to the UDM to get Parameter Provisioning Data. The NF consumer shall send a GET request towards the resource URI of the Parameter Provisioning Data (.../{ueId}/pp-data). The URI variable {ueId} shall take the value of the identity of the UE, or the identity of the group of Ues, as described in Table 6.5.3.2.2-1.

Figure 5.6.2.5.x-1: NF service consumer gets Parameter Provisioning Data

1.	The NF service consumer sends a GET request to the resource .../{ueId}/pp-data, to get the Parameter Provisioning Data.

2a.	On success, the UDM responds with "200 OK" with the Parameter Provisioning Data.

2b.	If the Parameter Provisioning Data does not exist in the UDM, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" data structure).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

* * * Next Change * * * *

#### 6.5.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.5.3.1-1 depicts the resource URIs structure for the Nudm_PP API.

Figure 6.5.3.1-1: Resource URI structure of the Nudm_PP API

Table 6.5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PpData | /{ueId}/pp-data | PATCH | Modify the UE's modifiable subscription data Send updated SoR Information for a UE to the UDM |
|  |  | GET | Retrieve the UE's subscription data (PP Data) |
| 5GVnGroupConfiguration | /5g-vn-groups/{extGroupId} | PUT | Create a 5G VN Group |
|  |  | DELETE | Delete a 5G VN Group |
|  |  | PATCH | Modify a 5G VN Group |
|  |  | GET | Retrieve a 5G VN Group |
| ParameterProvisioningDataEntry | /{ueId}/pp-data-store/{afInstanceId} | PUT | Create or update a Parameter Provisioning Data entry per AF |
|  |  | DELETE | Delete a Parameter Provisioning Data entry per AF |
|  |  | GET | Retrieve a Parameter Provisioning Data entry per AF |
| MulticastMbsGroupMemb | /mbs-group-membership/{extGroupId} | PUT | Create a 5G MBS Group |
|  |  | DELETE | Delete a 5G MBS Group |
|  |  | PATCH | Delete a 5G MBS Group |
|  |  | GET | Retrieve a 5G MBS Group |

* * * Next Change * * * *

6.5.3.2.3.x	GET

This method shall support the URI query parameters specified in table 6.5.3.2.3.x-1.

Table 6.5.3.2.3.x-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the response data structures and response codes specified in table 6.5.3.2.3.x-3.

Table 6.5.3.2.3.x-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.5.3.2.3.x-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PpData | M | 1 | 200 OK | Upon success, the request body contain the target PP Data. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND |

* * * Next Change * * * *

# A.6	Nudm_PP API

(... text not shown for clarity ...)

paths:

/{ueId}/pp-data:

get:

summary: Get Parameter Provisioning Data

operationId: Get PP Data

tags:

- Parameter Provisioning Data (Document)

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

anyOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PpData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: provision parameters

operationId: Update

tags:

- Parameter Provisioning Data (Document)

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

anyOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PpData'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Expected response to a valid request

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/5g-vn-groups/{extGroupId}:

put:

(... text not shown for clarity ...)

* * * End of Changes * * * *