3GPP TSG-CT WG4 Meeting #126	C4-245235

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.527 | CR | 0090 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time-based restoration with subscriber data migration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SUBDMIG |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When a UDM consumer is informed about an individual UE to be restored, there is no mechanism to ensure that the restoration is finished for all affected users in a deterministic manner. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a time-based restoration in notifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operators cannot ensure that the restoration and users' migration is finished in a certain period of time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not introduce any changes in OpenAPI descriptions. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.7.3	UDR Restoration during Subscriber Data Migration

During operation of a PLMN, it is a common task to have to re-allocate subscriber's data records across subscriber servers/databases, due to multiple reasons such as, e.g., subscriber re-distribution, network re-dimensioning / scaling-out, replacement / swap of legacy databases (e.g. 4G UDC systems), etc.

The migration of the temporary data (context data) to the target UDR system may be realized by triggering the restoration of profiles related to UDR.

Additionally, network traffic shall be properly routed to the target UDR handling the new subscribers, affecting in many cases the traffic from 5GC to UDR consumers, i.e. UDM/PCF. For example, when subscribers are re-allocated to a new UDR group Id and also to a new UDM/PCF group Id, UDM/PCF consumers shall switch traffic related to the migrated subscribers to UDM/PCF instances in the new UDM/PCF group Id. In those cases, UDR rediscovery and/or UDM/PCF rediscovery is required to guarantee proper traffic routing.

The procedures followed to support subscriber data migration includes both traffic switch and recovery of temporary data in the target UDR (data re-synchronization). These procedures are initiated via OAM once the network has been properly configured, e.g. once static data has been provisioned in the target UDR for the migrated subscribers and other network functions (e.g. NRF and UDR supporting the NF Group ID mapping service), have been configured according to the new subscriber distribution.

Depending on the network deployment, data re-synchronization, traffic switch with rediscovery or both may be required for the data migration service. For example, in case UDM hides UDR network topology, traffic switch is not necessary for the UDM consumers, or in case temporary data is recovered from origin UDR but subscribers are distributed to a new UDM/UDR/PCF group, traffic switch with rediscovery without data re-synchronization shall be requested to UDM consumers. In case both are required, UDM/PCF/UDR consumers shall execute the traffic switch before the re-synchronization of the impacted resources is initiated.

For data migration, the restoration procedures described in figure 6.7.2-1 are used with the following adaptations. The notification to UDR consumers and UDM/PCF consumers may specifically include:

-	SUPI or GPSI ranges and PLMNId of the UDR: to identify the migrated subscribers

-	NF Re-discovery indication: to indicate traffic switch with NF rediscovery is required due to subscriber reallocation in a different NF group Id

-	No resynchronization required: to indicate temporary data does not need to be restored in UDR after subscriber reallocation in a different NF group Id

-	Time-based indication: to indicate that all identified migrated users need to be restored before a specified absolute time is reached.

When UDM/PCF/UDR consumers receive a NF rediscovery indication, it means the relationship between the migrated User and UDM/PCF/UDR GroupId has changed, so UDM/PCF/UDR consumers shall use the user identity to discover and select UDM/PCF/UDR instances serving the affected users in the new UDM/PCF/UDR Group Id, discarding the UDM/PCF/UDR Group Id previously available in the UE context, if any. AMF may additionally update the UDM/PCF group Id information in other NFs in case that information is changed in the UE context as a result of the new UDM/PCF selection.

* * * End of Changes * * * *