3GPP TSG-CT WG4 Meeting #126	C4-245267

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0903 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of additional MPQUIC-IP functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CR5493(S2-2410017) to 3GPP TS 23.501 has approved the definition of a new steering functionality known as the “MPQUIC-IP” functionality: “MPQUIC-IP” steering functionality that can be applied to steer, switch and split IP traffic identified in the ATSSS/N4 rules. However, it has not yet been included in TS 29.244 to facilitate support for the “MPQUIC-IP” steering functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for the MPQUIC-IP steering functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support for the MPQUIC-IP steering functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20.2A, 5.20.2A.1, 5.20.2A.2, 5.20.2A.X(new), 5.20.2A.Y(new), 7.5.2.2, 7.5.3.7, 8.2.25, 8.2.124, 8.2.158, 8.2.181, 8.2.225 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5493 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# **** First Change ****

5.20.2A	MPQUIC and MPQUIC-IP functionality

5.20.2A.1	General

The SMF shall instruct the UPF(PSA) to activate MPQUIC steering functionality for a given MA-PDU session if MPQUIC needs to be used for UDP traffic flows.

The SMF shall instruct the UPF(PSA) to activate MPQUIC-IP steering functionality for a given MA-PDU session if MPQUIC needs to be used for IP traffic flows.

The UPF(PSA) shall allocate resources for MPQUIC or MPQUIC-IP steering functionality (e.g. MPQUIC Proxy address, MPQUIC link-specific multipath IP addresses, etc.), and perform traffic steering, switching and splitting according to the instructions from the SMF.

# **** Next Change ****

5.20.2A.2	Activate MPQUIC or MPQUIC-IP functionality and Exchange MPQUIC Parameters

If MPQUIC or MPQUIC-IP steering functionality is required for a Multi-Access PDU session and if the UPF indicates supports of the corresponding MPQUIC feature (see clause 8.2.25), the SMF shall send MPQUIC Control information to the UPF in PFCP Session Establishment Request, to instruct the UPF to activate the corresponding MPQUIC functionality for this PFCP session. The SMF may also request to activate the PMF functionality for this PFCP session by sending PMF Control Information to the UPF. As a result, the UPF(PSA) shall allocate necessary resources for the corresponding MPQUIC and PMF and return the corresponding MPQUIC Parameters (e.g. MPQUIC IP address and port, MPQUIC link-specific multipath IP address) and PMF Parameters (e.g. PMF IP address and port, etc.) to the SMF in PFCP Session Establishment Response. The SMF may also activate the MPTCP functionality for this PFCP session as specified in clause 5.20.2.2.

The SMF may send MPQUIC Control Information and/or PMF Control Information (and/or MPTCP Control Information if the MPTCP functionality is also activated) to the UPF in PFCP Session Modification Request, with updated value, e.g. if the SMF receives updated ATSSS rules form the PCF.

NOTE 1: Such MPQUIC Parameters received by the SMF are sent to the UE together with the ATSSS rules, as specified in 3GPP TS 24.193 [59].

When provisioning an UL PDR for user plane traffic for which MPQUIC is applicable, the SMF shall include a Multipath Applicable Indication IE with the MQAI flag set to "1" in the Create PDR IE.

NOTE 2: This indication can be used by the UPF to prepare itself for the reception of MPQUIC traffic for the PDU session.

When provisioning an UL PDR for IP traffic for which MPQUIC-IP is applicable, the SMF shall include a Multipath Applicable Indication IE with the MIQAI flag set to "1" in the Create PDR IE.

NOTE X: This indication can be used by the UPF to prepare itself for the reception of MPQUIC-IP traffic for the PDU session.

# **** Next Change ****

5.20.2A.X	Control of Multipath QUIC IP Connection Establishment by MPQUIC-IP Proxy

When the MPQUIC-IP steering functionality is utilized, the UDP connection establishment and data exchange between the UE and a remote host shall be proxied by the MPQUIC-IP Proxy in the UPF(PSA), as specified in clause 5.32.6.2.2 of 3GPP TS 23.501 [28] and clause 6.1.4.1.0 of 3GPP TS 24.193 [59].

Once a multipath QUIC IP connection is set up, the MPQUIC-IP Proxy shall store the MPQUIC session entry which includes the following information:

-	MPQUIC link-specific multipath IP addresses and the associated UDP ports;

-	the QFI associated with the multipath QUIC connection.

The stored MPQUIC-IP session entry shall be used by the MPQUIC-IP Proxy for subsequent IP translation when receiving uplink or downlink MPQUIC-IP traffic.

Editor’s Note:	More implementation details needs to be further discussed.

# **** Next Change ****

5.20.2A.Y	IP Traffic Steering and IP Translation by MPQUIC-IP Proxy

Once IP traffic for which MPQUIC-IP is applicable is detected by the UPF, the UPF shall internally forward this traffic to the MPQUIC-IP Proxy for IP translation. The MPQUIC-IP Proxy shall use the stored information in MPQUIC-IP session entry to perform IP translation to the detected MPQUIC IP packets, e.g. adding or removing the outer IP/UDP header according to the MPQUIC link-specific multipath IP addresses and the associated UDP ports stored in the MPQUIC session entry.

The MPQUIC-IP Proxy shall send the DL traffic of a QoS flow using the multipath QUIC connection associated with the corresponding QoS flow.

The UPF may detect the uplink IP packets for which MPQUIC-IP is applicable by checking whether the UL PDR matching the user plane IP traffic is set with a Multipath Applicable Indication, or (UPF implementation choice) by checking whether the source or destination IP address of the user plane packets (after removing the GTP-U header) correspond to MPQUIC link-specific multipath IP address(es) and the corresponding MPQUIC proxy IP address.

The UPF may detect the downlink IP packets for which MPQUIC-IP is applicable, by checking the information in the associated MAR (e.g. checking if the steering functionality is set to MPQUIC-IP) or by checking the destination IP address.

Editor’s Note:	More implementation details needs to be further discussed.

# **** Next Change ****

7.5.2.2	Create PDR IE within PFCP Session Establishment Request

The Create PDR grouped IE shall be encoded as shown in Figure 7.5.2.2-1.

Table 7.5.2.2-1: Create PDR IE within PFCP Session Establishment Request

| Octet 1 and 2 |  | Create PDR IE Type = 1(decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| PDR ID | M | This IE shall uniquely identify the PDR among all the PDRs configured for that PFCP session. | X | X | X | X | X | PDR ID |
| Precedence | M | This IE shall indicate the PDR's precedence to be applied by the UP function among all PDRs of the PFCP session, when looking for a PDR matching an incoming packet. | - | X | X | X | X | Precedence |
| PDI | M | This IE shall contain the PDI against which incoming packets will be matched. See Table 7.5.2.2-2. | X | X | X | X | X | PDI |
| Outer Header Removal | C | This IE shall be present if the UP function is required to remove one or more outer header(s) from the packets matching this PDR. | X | X | - | X | X | Outer Header Removal |
| FAR ID | C | This IE shall be present if the Activate Predefined Rules IE is not included or if it is included but it does not result in activating a predefined FAR, and if the MAR ID is not included. This IE may be present if the CP function activated a predefined rule name with a predefined FAR but the CP function wishes to overwrite the predefined FAR by another FAR. (NOTE 2) When present this IE shall contain the FAR ID to be associated to the PDR. | X | X | X | X | X | FAR ID |
| URR ID | C | This IE shall be present if a measurement action shall be applied to packets matching this PDR. When present, this IE shall contain the URR IDs to be associated to the PDR. Several IEs within the same IE type may be present to represent a list of URRs to be associated to the PDR. | X | X | X | X | - | URR ID |
| QER ID | C | This IE shall be present if a QoS enforcement or QoS marking action shall be applied to packets matching this PDR. When present, this IE shall contain the QER IDs to be associated to the PDR. Several IEs within the same IE type may be present to represent a list of QERs to be associated to the PDR. | - | X | X | X | X | QER ID |
| Activate Predefined Rules | C | This IE shall be present if Predefined Rule(s) shall be activated for this PDR. When present this IE shall contain one Predefined Rules name. Several IEs with the same IE type may be present to represent multiple "Activate Predefined Rules" names. | - | X | X | X | - | Activate Predefined Rules |
| Activation Time | O | This IE may be present if the PDR activation shall be deferred. (NOTE 1) | - | X | X | X | - | Activation Time |
| Deactivation Time | O | This IE may be present if the PDR deactivation shall be deferred. (NOTE 1) | - | X | X | X | - | Deactivation Time |
| MAR ID | C | This IE shall be present if the PDR is provisioned to match the downlink traffic of non-GBR QoS flows towards the UE for a PFCP session established for a MA PDU session. | - | - | - | X | - | MAR ID |
| Packet Replication and Detection Carry-On Information | C | This IE shall be present if the PDR is provisioned to match a broadcast packet. When present, it contains the information to instruct the UPF to replicate the packet and to carry-on the look-up of other PDRs of other PFCP sessions matching the packet (see clause 5.2.1). | - | - | - | X | - | Packet Replication and Detection Carry-On Information |
| IP Multicast Addressing Info | O | This IE may be present in an UL PDR controlling UL IGMP/MLD traffic (see 5.25). When present, it shall contain a (range of) IP multicast address(es), and optionally source specific address(es), identifying a set of IP multicast flows. See Table 7.5.2.2-4. Several IEs with the same IE type may be present to represent multiple IP multicast flows. | - | - | - | X | - | IP Multicast Addressing Info |
| UE IP address Pool Identity | O | This IE may be present if UE IP Addresses Pools are configured in the UPF. When present, this IE shall contain the identity of a UE IP address Pool configured in the UPF. Two IEs with the same IE type shall be present to represent UE IPv4 Address Pool Identity and UE IPv6 Address Pool Identity if different pool identities are used for UE IPv4 address and UE IPv6 address and both an UE IPv4 and an UE IPv6 address are requested to be assigned for the PFCP session. In this case, the UE IPv4 Address Pool Identity shall be encoded before the UE IPv6 Address Pool Identity. | - | X | - | X | - | UE IP address Pool Identity |
| Multipath Applicable Indication | C | This IE shall be present if the PDR is used to detect UL user plane traffic for which MPTCP, MPQUIC-IP or MPQUIC is applicable. | - | - | - | X | - | Multipath Applicable Indication |
| Transport Delay Reporting | C | This IE shall be present to request the UPF to add the delay of the GTP-U path with the preceding uplink GTP-U entity to the "N3/N9 Delay Result received in the GTP-U PDU Session Container extension header (see 3GPP TS 38.415 [34]) of the uplink packet, when monitoring the QoS of a PDU session based on GTP-U path monitoring (see clause 5.24.5.3). See Table 7.5.2.2-6. | - | - | - | X | - | Transport Delay Reporting |
| RAT Type | O | This IE may be present to provide the UP Function the current RAT Type for the UL PDR for statistics purpose if the PFCP session is established for a MA PDU session. | - | - | - | X | - | RAT Type |
| NOTE 1: When the Activation Time and Deactivation Time are not present, the PDR shall be activated immediately at receiving the message. NOTE 2: If a predefined FAR is or has been activated using a predefined rule name, it is UP function implementation specific whether this predefined FAR can be overwritten by a FAR ID pointing to another predefined FAR (i.e. with the most significant bit set to 1). If not, the UP function shall reject such a request if received from the CP function. |  |  |  |  |  |  |  |  |

Table 7.5.2.2-2: PDI IE within PFCP Session Establishment Request

| Octet 1 and 2 |  |  |  | PDI IE Type = 2 (decimal) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  |  | Length = n |  |  |  |  |  |  |  |  |
| Information elements |  | P |  | Condition / Comment |  | Appl. |  |  |  |  |  | IE Type |
|  |  |  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |  |
| Source Interface |  | M |  | This IE shall identify the source interface of the incoming packet. |  | X | X | X | X | X |  | Source Interface |
| Local F-TEID |  | O |  | This IE shall not be present if Traffic Endpoint ID is present. If present, this IE shall identify the local F-TEID to match for an incoming packet. The CP function shall set the CHOOSE (CH) bit to 1 if the CP function requests the UP function to assign a local F-TEID to the PDR. |  | X | X | - | X | - |  | F-TEID |
| Local Ingress Tunnel |  | C |  | This IE shall not be present if Traffic Endpoint ID is present. If present, this IE shall identify the IP address and the UDP port for a UDP/IP tunnel. The CP function shall set the CHOOSE (CH) bit to 1 if the CP function requests the UP function to assign a local ingress tunnel to the PDR. |  | - | - | - | - | X |  | Local Ingress Tunnel |
| Network Instance |  | O |  | This IE shall not be present if Traffic Endpoint ID is present. It shall be present if the CP function requests the UP function to allocate a UE IP address/prefix and the Traffic Endpoint ID is not present. If present, this IE shall identify the Network instance to match for the incoming packet. See NOTE 1, NOTE2. |  | X | X | X | X | X |  | Network Instance |
| Redundant Transmission Detection Parameters |  | O |  | If present, this IE shall contain the information used for the reception of redundant uplink packets on N3/N9 interfaces. |  | - | - | - | X | - |  | Redundant Transmission Detection Parameters |
| UE IP address |  | O |  | This IE shall not be present if Traffic Endpoint ID is present. If present, this IE shall identify the UE IP address as the source or destination IP address to match for the incoming packet. (NOTE 5). The CP function shall set the CHOOSE IPV4 (CHV4) and/or the CHOOSE IPV6 (CHV6) bits to 1 if the UP function supports the allocation of UE IP address/ prefix and the CP function requests the UP function to assign a UE IP address/prefix to the PDR. This IE may also present to identify the IP address of the CP function as the destination IP address to match for the incoming packet, e.g. Radius, Diameter or DHCP signalling packet. (NOTE 8). This IE may be present for a PFCP session established in the V-UPF (local PSA) for a HR-SBO PDU session if the UPF supports the UN6TU feature (see clause 8.2.25). In the 5GC, several IEs with the same IE type may be present to represent multiple UE IP addresses, if the UPF indicated support of the IP6PL feature (see clause 5.21). |  | - | X | X | X | - |  | UE IP address |
| Mapped N6 IP Address |  | O |  | When present, the V-SMF shall set the CHOOSE IPV4 (CHV4) bit to 1 if the UP function supports the UMN6IP feature and the V-SMF requests the UP function to assign a mapped N6 IP address for one or more DL PDRs as described in clause 5.33.7. |  | - | - | - | X | - |  | Mapped N6 IP Address |
| N6 Routing Information |  | O |  | This IE may be present in a DL PDR when using distinct N6 tunnels from the V-EASDF to the V-UPF for differentiating HR-SBO PDU sessions with overlapping private UE IP addresses. See clause 5.33.7. |  | - | - | - | X | - |  | N6 Routing Information |
| Traffic Endpoint ID |  | C |  | This IE may be present if the UP function has indicated the support of PDI optimization. If present, this IE shall uniquely identify the Traffic Endpoint for that PFCP session. Several IEs with the same IE type may be present to provision several Traffic Endpoints with different Traffic Endpoint IDs, from which the UPF may receive packets pertaining to the same service data flow, which is subject for the same FAR, QER and URR, if the UPF has indicated it supports MTE feature as specified in clause 8.2.25. See NOTE 6. |  | X | X | X | X | X |  | Traffic Endpoint ID |
| SDF Filter |  | O |  | If present, this IE shall identify the SDF filter to match for the incoming packet. Several IEs with the same IE type may be present to provision a list of SDF Filters. The full set of applicable SDF filters, if any, shall be provided during the creation or the modification of the PDI. See NOTE 3. |  | - | X | X | X | X |  | SDF Filter |
| Application ID |  | O |  | If present, this IE shall identify the Application ID to match for the incoming packet. |  | - | X | X | X | X |  | Application ID |
| Ethernet PDU Session Information |  | O |  | This IE may be present to identify all the (DL) Ethernet packets matching an Ethernet PDU session (see clause 5.13.1). |  | - | - | - | X | - |  | Ethernet PDU Session Information |
| Ethernet Packet Filter |  | O |  | If present, this IE shall identify the Ethernet PDU to match for the incoming packet. Several IEs with the same IE type may be present to represent a list of Ethernet Packet Filters. The full set of applicable Ethernet Packet filters, if any, shall be provided during the creation or the modification of the PDI. |  | - | - | - | X | - |  | Ethernet Packet Filter |
| QFI |  | O |  | This IE shall not be present if Traffic Endpoint ID is present and the QFI(s) are included in the Traffic Endpoint. If present, this IE shall identify the QoS Flow Identifier to match for the incoming packet. Several IEs with the same IE type may be present to provision a list of QFIs. When present, the full set of applicable QFIs shall be provided during the creation or the modification of the PDI. |  | - | - | - | X | - |  | QFI |
| Framed-Route |  | O |  | This IE may be present for a PDR if the UPF indicated support of Framed Routing (see clauses 8.2.25 and 5.16). If present, this IE shall describe a framed route. Several IEs with the same IE type may be present to provision a list of framed routes. (NOTE 5) | - | X | - | X |  | - | Framed-Route |  |
| Framed-Routing |  | O |  | This IE may be present for a DL PDR if the UPF indicated support of Framed Routing (see clauses 8.2.25 and 5.16). If present, this IE shall describe the routing method for the UP function for the IP route related to Framed-Routes or Framed-IPv6-Routes. (NOTE 7) | - | X | - | X |  | - | Framed-Routing |  |
| Framed-IPv6-Route |  | O |  | This IE may be present for a PDR if the UPF indicated support of Framed Routing (see clauses 8.2.25 and 5.16). If present, this IE shall describe a framed IPv6 route. Several IEs with the same IE type may be present to provision a list of framed IPv6 routes. (NOTE 5) | - | X | - | X |  | - | Framed-IPv6-Route |  |
| Source Interface Type |  | C |  | When present, this IE shall indicate the 3GPP interface type of the source interface. This IE shall be present if this is a PDR for a DL data forwarding tunnel. Otherwise this IE may be present, if required by functionalities in the UP Function, e.g. for performance measurements. | X | X | - | X |  | X | 3GPP Interface Type |  |
| IP Multicast Addressing Info |  | O |  | This IE may be present in a DL PDR controlling DL IP multicast traffic (see clause 5.25). When present, it shall contain a (range of) IP multicast address(es), and optionally source specific address(es), identifying a set of IP multicast flows. See Table 7.5.2.2-4. This IE shall be present over N4mb, if multicast transport is used over N6mb and if Traffic Endpoint ID is not present. Several IEs with the same IE type may be present to represent multiple IP multicast flows. | - | - | - | X |  | X | IP Multicast Addressing Info |  |
| DNS Query Filter |  | O |  | This IE may be present for an UL PDR if the UPF indicated support of DNS traffic steering based on FQDN in the DNS Query message. If present, this IE shall identify the DNS Query filter to match for the incoming DNS Query packets. Several IEs with the same IE type may be present to provision a list of DNS Query Filters. The full set of applicable DNS Query filters, if any, shall be provided during the creation or the modification of the PDI. | - | - | - | X |  | - | DNS Query/Response Filter |  |
| MBS Session Identifier |  | C |  | This IE shall be present when the PDR is created to receive MBS session data. (NOTE 9) | - | - | - | X |  | - | MBS Session Identifier |  |
| Area Session ID |  | C |  | This IE shall be present for a location dependent MBS service. When present, it shall contain the Area Session ID, which together with the MBS Session Identifier, uniquely identify the service area part of the content data of the MBS service. (NOTE 9) | - | - | - | X |  | - | Area Session ID |  |
| Protocol Description |  | O |  | This IE may be present to indicate the transport protocol (e.g. RTP/SRTP), transport protocol header extensions (e.g. RTP Header Extension for PDU Set Marking as defined in 3GPP TS 26.522 [79]), payload type and format (e.g. H.264, H.265) used by the packets matching this PDR when PDU Set marking and/or End of Data Burst marking applies to the QER (i.e. QoS flow) with which the PDR is associated. See Table 7.5.2.2-7. (NOTE 10) | - | - | - | X |  | - | Protocol Description |  |
| DNS Response Filter |  | O |  | This IE may be present for an DL PDR to support EAS discovery procedure with V-EASDF using IP replacement mechanism in HR-SBO. If present, this IE shall identify the DNS Response filter to match for the incoming DNS Response packets. Several IEs with the same IE type may be present to provision a list of DNS Response Filters. The full set of applicable DNS Response filters, if any, shall be provided during the creation or the modification of the PDI. | - | - | - | X |  | - | DNS Query/Response Filter |  |
| NOTE 1: The Network Instance parameter is needed e.g. in the following cases: - PGW/TDF UP function supports multiple PDNs with overlapping IP addresses; - SGW UP function is connected to PGWs in different IP domains (S5/S8); - PGW UP function is connected to SGWs in different IP domains (S5/S8); - SGW UP function is connected to eNodeBs in different IP domains; - UPF is connected to 5G-ANs in different IP domains; - Separation of multiple 5G VN groups communication in the UPF; - Indirect data forwarding. NOTE 2: When a Local F-TEID is provisioned in the PDI, the Network Instance shall relate to the IP address of the F-TEID. Otherwise, the Network Instance shall relate to the UE IP address if provisioned or the destination IP address in the SDF filter if provisioned NOTE 3: SDF Filter IE(s) shall not be present if Ethernet Packet Filter IE(s) is present. NOTE 4: When several SDF filter IEs are provisioned, the UP function shall consider that the packets are matched if matching any SDF filter. The same principle shall apply for Ethernet Packet Filters and QFIs. NOTE 5: If both the UE IP Address and the Framed-Route (or Framed-IPv6-Route) are present, the packets which are considered being matching the PDR shall match at least one of them. NOTE 6: Maximum two Traffic Endpoint ID containing different Local TEIDs per PDI may be provisioned over the N4 interface for a PFCP session which is established for a PDU session subject for 5G to EPS mobility with N26 supported. Several Traffic Endpoint ID containing different UE IP Addresses may be provisioned over the N4 interface for a PFCP session if the UPF also indicated support of the IP6PL feature (see clause 5.21.1). NOTE 7: In this release of specification, the UP function shall announce the IP route(s) for Framed-Route(s) or Framed-IPv6-Route(s) to the PDN regardless of the value of the Framed-Routing. NOTE 8: The IP address of the CP function is needed e.g. perform Router Advertisements and match the destination IP address of the incoming packet if the Radius, Diameter or DHCP signaling packets between the SMF and the external DN is forwarded by the UPF. NOTE 9: The Inclusion of the MBS Session Identifier, or MBS Session Identifier and Area Session ID for a location dependent MBS session, enables the UPF to allocate the same N19mb tunnel for the same MBS Session, or for the same MBS session and Area Session ID for a location dependent MBS session, when receiving multiple PFCP Session Establishment or Modification Request messages for different PFCP sessions being associated with the same MBS session, or with the same MBS session and Area Session ID for a location dependent MBS session, and also helps the UPF to identify the PFCP sessions associated with a MBS Session, or with the same MBS session and Area Session ID for a location dependent MBS session. NOTE 10: The Protocol Description IE is not used for packets matching against the PDR but to assist PDU Set identification and/or End of Data Burst marking by the UPF. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.5.2.2-3: Ethernet Packet Filter IE within PFCP Session Establishment Request

| Octet 1 and 2 |  |  | Ethernet Packet Filter IE Type = 132 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Ethernet Filter ID | C | This shall be present if Bidirectional Ethernet filter is required. This IE shall uniquely identify an Ethernet Filter among all the Ethernet Filters provisioned for a given PFCP session. |  | - | - | - | X | - | Ethernet Filter ID |
| Ethernet Filter Properties | C | This IE shall be present when provisioning a bidirectional Ethernet Filter the first time (see clause 5.13.4). |  | - | - | - | X | - | Ethernet Filter Properties |
| MAC address | O | If present, this IE shall identify the MAC address. This IE may be present up to 16 times. |  | - | - | - | X | - | MAC address |
| Ethertype | O | If present, this IE shall identify the Ethertype. |  | - | - | - | X | - | Ethertype |
| C-TAG | O | If present, this IE shall identify the Customer-VLAN tag. |  | - | - | - | X | - | C-TAG |
| S-TAG | O | If present, this IE shall identify the Service-VLAN tag. |  | - | - | - | X | - | S-TAG |
| SDF Filter | O | If packet filtering is required, for Ethernet frames with Ethertype indicating IPv4 or IPv6 payload, this IE shall describe the IP Packet Filter Set. Several IEs with the same IE type may be present to represent a list of SDF filters. |  | - | - | - | X | - | SDF Filter |

Table 7.5.2.2-4: IP Multicast Addressing Info IE within PFCP Session Establishment Request

| Octet 1 and 2 |  | IP Multicast Addressing Info IE Type = 188 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| IP Multicast Address | M | This IE shall contain the IP multicast address(es) of the DL multicast flow(s) or indicate "any" IP multicast address. For N4mb, the IP Multicast Address shall be set to the value of IP Multicast Address in the SSM for the MBS Session. | - | - | - | X | X | IP Multicast Address |
| Source IP Address | O | When present, this IE shall contain the source specific IP address of the DL multicast flow. Several IEs with the same IE type may be present to represent multiple source specific addresses. If this IE is not present, this indicates "any" source IP address. | - | - | - | X |  | Source IP Address |
|  | M | This IE shall be set to the value of the IP Source Address in the SSM for the MBS Session. | - | - | - | - | X |  |

Table 7.5.2.2-5: Redundant Transmission Detection Parameters IE in PDI

| Octet 1 and 2 |  |  | Redundant Transmission Detection Parameters IE Type = 255 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Local F-TEID for Redundant Transmission | M | This IE shall identify the local F-TEID to match for an incoming packet for redundant transmission. The CP function shall set the CHOOSE (CH) bit to 1 if it requests the UP function to assign a local F-TEID to the PDR. |  | - | - | - | X | - | F-TEID |
| Network Instance for Redundant Transmission | C | This IE shall be included if the Local F-TEID for Redundant Transmission uses a different network Instance than the Network Instance used for the Local F-TEID for the primary GTP-U tunnel. |  | - | - | - | X | - | Network Instance |

Table 7.5.2.2-6: Transport Delay Reporting IE in Create PDR IE

| Octet 1 and 2 |  |  | Transport Delay Reporting IE Type = 271 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Preceding UL GTP-U Peer | M | This IE shall identify the preceding UL GTP-U peer. |  | - | - | - | X | - | Remote GTP-U Peer |
| DSCP | O | If present, this IE shall contain the DSCP to use to measure the GTP-U path delay with the preceding UL GTP-U peer. |  | - | - | - | X | - | Transport Level Marking |

Table 7.5.2.2-7: Protocol Description IE

| Octet 1 and 2 |  |  | Protocol Description IE Type = 334 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Transport Protocol | O | When present, it shall indicate the transport protocol used by the media flow. |  | - | - | - | X | - | Media Transport Protocol |
| RTP Header Extension Information | C | This IE shall be present if RTP or SRTP is used and the RTP payload packets contains a RTP Header Extension that can be used for PDU Set identification and/or End of Data Burst marking. When present, this IE shall contain information on the RTP header extension that can be used for PDU Set identification and/or End of Data Burst marking. See Table 7.5.2.2-8. (NOTE 1) |  | - | - | - | X | - | RTP Header Extension Information |
| RTP Payload Information | O | When present, it shall contain RTP Payload information for the RTP stream, which can be used to derive the PDU Set information and/or End of Data Burst marking. See Table 7.5.2.2-9. Several IEs with the same IE type may be present to describe multiple RTP Payload Information. (NOTE 1) (NOTE 2) |  | - | - | - | X | - | RTP Payload Information |
| NOTE 1: If the RTP Payload Information is present and contains one or more Payload Type values, the UPF may only parse the RTP packets with an RTP header containing any of these Payload Type value(s). Otherwise, if the RTP Payload Information is absent or does not contain any Payload Type value, the UPF should parse all the RTP packets of the media flow and use either the RTP Header Extension if included, or the Payload format to derive the PDU set information (see Guidelines for PDU Set identification in clauses A.1 and A.2 of 3GPP TS 26.522 [79]). NOTE 2: In this release of the specification, only one RTP Payload Information IE may be provisioned. NOTE 3: Vendor/operator specific attributes may be supported as defined in clauses 5.9 and 8.1.1. |  |  |  |  |  |  |  |  |  |

Table 7.5.2.2-8: RTP Header Extension Information IE

| Octet 1 and 2 |  |  | RTP Header Extension Information IE Type = 340 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| RTP Header Extension Type | C | This IE shall be present if RTP or SRTP is used and the RTP payload packets contains a RTP Header Extension that can be used for PDU Set identification and/or End of Data Burst marking. When present, it shall indicate the RTP header extension type. |  | - | - | - | X | - | RTP Header Extension Type |
| RTP Header Extension ID | C | This IE shall be present if the RTP Header Extension Type IE is present. When present, the RTP Header Extension ID shall be set to the Id of the RTP header extension identified by the RTP Header Extension Type IE, as defined in IETF RFC 8285 [86]. |  | - | - | - | X | - | RTP Header Extension ID |
| RTP Header Extension Additional Information | O | This IE may be present to provide additional information on the RTP header extension. |  | - | - | - | X | - | RTP Header Extension Additional Information |
| NOTE: Vendor/operator specific attributes may be supported as defined in clauses 5.9 and 8.1.1. |  |  |  |  |  |  |  |  |  |

Table 7.5.2.2-9: RTP Payload Information IE

| Octet 1 and 2 |  |  | RTP Payload Information IE Type = 341 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| RTP Payload Type | C | This IE shall be present when the RTP Payload Format is present, otherwise it may be present. When present, this IE shall contain the list of Payload Type (PT) values in the RTP header of RTP packets the UPF may parse to derive the PDU Set Information. Several IEs with the same IE type may be present to describe multiple Payload Types. (NOTE) |  | - | - | - | X | - | RTP Payload Type |
| RTP Payload Format | O | When present, it shall indicate the RTP Payload format as defined in 3GPP TS 26.522 [79]. (NOTE) |  | - | - | - | X | - | RTP Payload Format |
| NOTE: The RTP Payload Type(s) shall correspond to the RTP Payload Format if the RTP Payload Format is present. |  |  |  |  |  |  |  |  |  |

# **** Next Change ****

7.5.3.7	ATSSS Control Parameters IE within PFCP Session Establishment Response

The ATSSS Control Parameters grouped IE shall be encoded as shown in Figure 7.5.3.7-1.

Table 7.5.3.7-1: ATSSS Control Parameters IE within PFCP Session Establishment Response

| Octet 1 and 2 |  |  | ATSSS Control Parameters IE Type = 221 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| MPTCP Parameters | C | This IE shall be present if the TCI flag in the MPTCP Control Information IE is set to "1" in the Request message and the UPF allocated resources for MPTCP. |  | - | - | - | X | - | MPTCP Parameters |
| ATSSS-LL Parameters | C | This IE shall be present if the LLI flag in ATSSS-LL Control Information IE is set to "1" in the Request message and the UPF allocated resources for ATSSS-LL. |  | - | - | - | X | - | ATSSS-LL Parameters |
| PMF Parameters | C | This IE shall be present if the PMFI flag in the PFM Control Information IE is set to "1" in the Request message and the UPF allocated resources for PMF. Several IEs with the same IE type may be present to provide multiple PMF Parameters. |  | - | - | - | X | - | PMF Parameters |
| MPQUIC Parameters | C | This IE shall be present if the CUDP flag in the MPQUIC Control Information IE is set to "1" in the Request message and the UPF allocated resources for MPQUIC. |  | - | - | - | X | - | MPQUIC Parameters |
| MPQUIC-IP Parameters | C | This IE shall be present if the CIP flag in the MPQUIC Control Information IE is set to "1" in the Request message and the UPF allocated resources for MPQUIC. |  | - | - | - | X | - | MPQUIC Parameters |

The MPTCP Parameters grouped IE shall be encoded as shown in Figure 7.5.3.7-2.

Table 7.5.3.7-2: MPTCP Parameters IE within PFCP Session Establishment Response

| Octet 1 and 2 |  |  | MPTCP Parameters IE Type = 225 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| MPTCP Address Information | M | This IE shall carry the information of allocated MPTCP address. |  | - | - | - | X | - | MPTCP Address Information |
| MPTCP Link-Specific IP Address | M | This IE shall carry the information of allocated MPTCP link-specific multipath IP address. |  | - | - | - | X | - | Link-Specific Multipath IP Address |

The ATSSS-LL Parameters grouped IE shall be encoded as shown in Figure 7.5.3.7-3.

Table 7.5.3.7-3: ATSSS-LL Parameters IE within PFCP Session Establishment Response

| Octet 1 and 2 |  |  | ATSSS-LL Parameters IE Type = 226 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| ATSSS-LL Information | M | This IE shall indicate that resources have been allocated to the ATSSS functionality. |  | - | - | - | X | - | ATSSS-LL Information |

The PMF Parameters grouped IE shall be encoded as shown in Figure 7.5.3.7-4.

Table 7.5.3.7-4: PMF Parameters IE within PFCP Session Establishment Response

| Octet 1 and 2 |  |  | PMF Parameters IE Type = 227 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| PMF Address Information | M | This IE shall contain the PMF Address Information. |  | - | - | - | X | - | PMF Address Information |
| QoS flow identifier | O | If present, this IE shall identify the QoS Flow Identifier of the QoS flow related to the PMF address information. |  | - | - | - | X | - | QFI |

The MPQUIC Parameters grouped IE shall be encoded as shown in Figure 7.5.3.7-5.

Table 7.5.3.7-5: MPQUIC Parameters IE within PFCP Session Establishment Response

| Octet 1 and 2 |  |  | MPQUIC Parameters IE Type = 331 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| MPQUIC Address Information | M | This IE shall carry the information of allocated MPQUIC address |  | - | - | - | X | - | MPQUIC Address Information |
| MPQUIC Link-Specific IP Address | M | This IE shall carry the information of allocated MPQUIC link-specific multipath IP address. |  | - | - | - | X | - | Link-Specific Multipath IP Address |

# **** Next Change ****

8.2.25	UP Function Features

The UP Function Features IE indicates the features supported by the UP function. It is coded as depicted in Figure 8.2.25-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 43 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to 6 | Supported-Features |  |  |  |  |  |  |  |  |
|  | 7 to 8 | Additional Supported-Features 1 |  |  |  |  |  |  |  |  |
|  | 9 to 10 | Additional Supported-Features 2 |  |  |  |  |  |  |  |  |
|  | 11 to 12 | Additional Supported-Features 3 |  |  |  |  |  |  |  |  |
|  | 13 to 14 | Additional Supported-Features 4 |  |  |  |  |  |  |  |  |
|  | 15 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.25-1: UP Function Features

The UP Function Features IE takes the form of a bitmask where each bit set indicates that the corresponding feature is supported. Spare bits shall be ignored by the receiver. The same bitmask is defined for all PFCP interfaces.

The following table specifies the features defined on PFCP interfaces and the interfaces on which they apply.

Table 8.2.25-1: UP Function Features

| Feature Octet / Bit | Feature | Interface | M/O | Description |
| --- | --- | --- | --- | --- |
| 5/1 | BUCP | Sxa, N4 | O (NOTE 2) | Downlink Data Buffering in CP function is supported by the UP function. |
| 5/2 | DDND | Sxa, N4 | O | The buffering parameter 'Downlink Data Notification Delay' is supported by the UP function. |
| 5/3 | DLBD | Sxa, N4 | O | The buffering parameter 'DL Buffering Duration' in PFCP Session Report Response is supported by the UP function. |
| 5/4 | TRST | Sxb, Sxc, N4 | O | Traffic Steering is supported by the UP function. |
| 5/5 | FTUP | Sxa, Sxb, N4 | M | F-TEID allocation / release in the UP function is supported by the UP function. |
| 5/6 | PFDM | Sxb, Sxc, N4 | O | The PFD Management procedure is supported by the UP function. |
| 5/7 | HEEU | Sxb, Sxc, N4 | O | Header Enrichment of Uplink traffic is supported by the UP function. |
| 5/8 | TREU | Sxb, Sxc, N4 | O (NOTE 3) | Traffic Redirection Enforcement in the UP function is supported by the UP function. |
| 6/1 | EMPU | Sxa, Sxb, N4 | M | Sending of End Marker packets supported by the UP function. |
| 6/2 | PDIU | Sxa, Sxb, Sxc, N4, N4mb | O (NOTE 4) | Support of PDI optimised signalling in UP function (see clause 5.2.1A.2). |
| 6/3 | UDBC | Sxb, Sxc, N4 | O | Support of UL/DL Buffering Control |
| 6/4 | QUOAC | Sxb, Sxc, N4 | O | The UP function supports being provisioned with the Quota Action to apply when reaching quotas. |
| 6/5 | TRACE | Sxa, Sxb, Sxc, N4 | O | The UP function supports Trace (see clause 5.15). |
| 6/6 | FRRT | Sxb, N4 | O | The UP function supports Framed Routing (see IETF RFC 2865 [37] and IETF RFC 3162 [38]). |
| 6/7 | PFDE | Sxb, N4 | O | The UP function supports a PFD Contents including a property with multiple values. |
| 6/8 | EPFAR | Sxa, Sxb, Sxc, N4 | O | The UP function supports the Enhanced PFCP Association Release feature (see clause 5.18). |
| 7/1 | DPDRA | Sxb, Sxc, N4 | O | The UP function supports Deferred PDR Activation or Deactivation. |
| 7/2 | ADPDP | Sxa, Sxb, Sxc, N4 | O | The UP function supports the Activation and Deactivation of Pre-defined PDRs (see clause 5.19). |
| 7/3 | UEIP | Sxb, N4 | O (NOTE 5) | The UP function supports allocating UE IP addresses or prefixes (see clause 5.21). |
| 7/4 | SSET | N4, N4mb | O | (MB-)UPF support of PFCP sessions successively controlled by different (MB-)SMFs of a same (MB-)SMF Set (see clause 5.22). |
| 7/5 | MNOP | Sxa, Sxb, Sxc, N4 | O | The UP function supports measurement of number of packets which is instructed with the flag 'Measurement of Number of Packets' in a URR. See also clause 5.2.2.2.1. |
| 7/6 | MTE | N4 | O | UPF supports multiple instances of Traffic Endpoint IDs in a PDI. |
| 7/7 | BUNDL | Sxa, Sxb, Sxc, N4, N4mb | O | PFCP messages bundling (see clause 6.5) is supported by the UP function. |
| 7/8 | GCOM | N4 | O | UPF support of 5G VN Group Communication. (See clause 5.23) |
| 8/1 | MPAS | N4, N4mb | O | (MB-)UPF support for multiple PFCP associations to the (MB-)SMFs in an (MB-)SMF set (see clause 5.22.3). |
| 8/2 | RTTL | N4 | O | UPF supports redundant transmission at transport layer. |
| 8/3 | VTIME | Sxb,N4 | O | UP function support of quota validity time feature. |
| 8/4 | NORP | Sxa, Sxb, Sxc, N4 | O | UP function support of Number of Reports as specified in clause 5.2.2.2. |
| 8/5 | IPTV | N4 | O | UPF support of IPTV service (see clause 5.25) |
| 8/6 | IP6PL | N4 | O | UPF supports: - UE IPv6 address(es) allocation with IPv6 prefix length other than default /64 (including allocating /128 individual IPv6 addresses), as specified in clause 4.6.2.2 of of 3GPP TS 23.316 [57]; and - multiple UE IPv6 addresses allocation using multiple instances of the UE IP Address IE in a same PDI or Traffic Endpoint, or using multiple PDIs or Traffic Endpoints with a different UE IP Address as specified in clause 5.21.1. |
| 8/7 | TSN | N4 | O | Integration of 5GS into a TSN data network is supported by the UPF (see clause 5.26.1). |
| 8/8 | MPTCP | N4 | O | UPF support of MPTCP Proxy functionality (see clause 5.20) |
| 9/1 | ATSSS-LL | N4 | O | UPF support of ATSSS-LL steering functionality (see clause 5.20) |
| 9/2 | QFQM | N4 | O | UPF support of per QoS flow per UE QoS monitoring for packet delay (see clause 5.24.4). |
| 9/3 | GPQM | N4 | O | UPF support of per GTP-U Path QoS monitoring (see clause 5.24.5). |
| 9/4 | MT-EDT | Sxa | O | SGW-U support of reporting the size of DL Data Packets. (see clause 5.2.4.1). |
| 9/5 | CIOT | Sxb, N4 | O | UP function support of CIoT feature, e.g. small data packet rate enforcement. (see 5.4.15) |
| 9/6 | ETHAR | N4 | O | UPF support of Ethernet PDU Session Anchor Relocation (see clause 5.13.6). |
| 9/7 | DDDS | N4 | O | UPF support of reporting the first buffered / first discarded downlink data after buffering / directly dropped downlink data for downlink data delivery status notification. |
| 9/8 | RDS | Sxb, N4 | O | UP function support of Reliable Data Service (see clause 5.29). |
| 10/1 | RTTWP | N4 | O | UPF support of RTT measurements towards the UE Without PMF. |
| 10/2 | QUASF | Sxb, Sxc, N4 | O | The UP function supports being provisioned in a URR with an Exempted Application ID for Quota Action or an Exempted SDF Filter for Quota Action which is to be used when the quota is exhausted. See also clauses 5.2.2.2.1 and 5.2.2.3.1. |
| 10/3 | NSPOC | Sxa, Sxb, N4 | O | UP function supports notifying start of Pause of Charging via user plane. |
| 10/4 | L2TP | Sxb, N4 | O | UP function supports the L2TP feature as described in clause 5.31. |
| 10/5 | UPBER | N4 | O | UP function supports the uplink packets buffering during EAS relocation. |
| 10/6 | RESPS | Sxb, N4 | O | UP function supports Restoration of PFCP Sessions associated with one or more PGW-C/SMF FQ-CSID(s), Group Id(s) or CP IP address(es) (see clause 5.22.4) |
| 10/7 | IPREP | N4 | O | UP function supports IP Address and Port number replacement (see clauses 5.33.3, 5.33.4 and 5.33.6). |
| 10/8 | DNSTS | N4 | O | UP function support DNS Traffic Steering based on FQDN in the DNS Query message (see clause 5.33.4) |
| 11/1 | DRQOS | N4 | O | UP function supports Direct Reporting of QoS monitoring events to Local NEF or AF (see clause 5.33.5). |
| 11/2 | MBSN4 | N4 | O | UPF supports sending MBS multicast session data to associated PDU sessions using 5GC individual delivery. |
| 11/3 | PSUPRM | N4, N4mb | O | UP function supports Per Slice UP Resource Management (see clause 5.35). |
| 11/4 | EPPPI | N4 | O | UP function supports Enhanced Provisioning of Paging Policy Indicator feature as specified in clause 5.36.2. |
| 11/5 | RATP | Sxb, N4 | O | UP function supports Redirection Address Types set to "Port", "IPv4 address and Port", "IPv6 address and Port", or "IPv4 and IPv6 addresses and Port". |
| 11/6 | UPIDP | N4 | O | UP function supports User Plane Inactivity Detection and reporting per PDR feature as specified in clause 5.11.3. |
| 11/7 | AFSFC | N4 | O | UP function supports inserting metadata when Application Function influence on Service Function Chaining is used as specified in clause 5.4.8. |
| 11/8 | MPQUIC | N4 | O | UPF support of MPQUIC Proxy functionality (see clause 5.20). |
| 12/1 | REDSM | N4 | O | UP function supports the Redundant Steering Mode (see clause 5.32.8 of 3GPP TS 23.501 [28] and clause 5.2.7.1) |
| 12/2 | DBDM | Sxa, N4 | O | The buffering parameters 'DL Buffering Duration' and 'DL Buffering Suggested Packet Count' in PFCP Session Modification Request are supported by the UP function. |
| 12/3 | TSCTS | N4 | O | AF requested support of Time Synchronization and/or Time Sensitive Communication is supported by the UP function (see clause 5.26.1). |
| 12/4 | DRTSC | N4 | O | UP function supports Direct Reporting of TSC management information events to TSN AF or TSCTSF (see clause 5.26.3.2). |
| 12/5 | N6JEDB | N4 | O | UPF support of N6 Jitter, DL Periodicity and UL Periodicity Measurement and Reporting, and End of Data Burst marking as specified in clause 5.38.4. |
| 12/6 | QMCON | N4 | O | UPF support of QoS monitoring of Congestion information (see clause 5.39.3.3). |
| 12/7 | DETNET | N4 | O | Deterministic Networking is supported by the UP function (see clause 5.26.1). |
| 12/8 | EML4S | N4 | O | UP function supports ECN Marking for L4S (see clause 5.38.1). |
| 13/1 | PDUSM | N4 | O | UP function supports PDU Set Marking (see clause 5.38.3). |
| 13/2 | CN-TL | N4 | O | UPF support of the TSN Talker and Listener (TL) functionality, i.e. CN-TL. See clause 5.26a. |
| 13/3 | QMDRM | N4 | O | UPF support of QoS monitoring of Data Rate Measurement (see clause 5.39.3.4). |
| 13/4 | EDBNC | N4 | O | UP function supports Extended DL Buffering Notification Control (see clause 5.2.4.1). |
| 13/5 | MT-SDT | N4 | O | UPF support of reporting the size of DL Data Packets per QoS flow (see clause 5.2.4.1). |
| 13/6 | UPSBIES | N4 | O | UPF support of event subscription via Service Based Interface. (See clause 5.40). |
| 13/7 | UMN6IP | N4 | O | UPF supports to allocate a mapped N6 IP address and perform IP replacement as described in clause 5.33.7. |
| 13/8 | UN6TU | N4 | O | UPF supports N6 tunnelling using N6 routing information as described in clause 5.33.7. |
| 14/1 | MBSCH | N4mb | O | MB-UPF support of MBS charging (see clause 5.34.2.5) |
| 14/X | MPQUIC-IP | N4 | O | UPF support of MPQUIC-IP Proxy functionality (see clause 5.20). |
| NOTE 1: Features are defined as follows: - Feature Octet / Bit: The octet and bit number within the Supported-Features IE, e.g. "5 / 1". - Feature: A short name that can be used to refer to the octet / bit and to the feature. - Interface: A list of applicable interfaces to the feature. - M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O") for a UP function complying with this release of the specification. A feature defined as mandatory in this release of the specification may not necessarily be defined or mandatory to support in earlier releases (see the respective specifications), therefore a CP function complying with this release of the specification shall be prepared to receive a UP Function Features IE with a mandatory feature bit set to "0". - Description: A clear textual description of the feature. NOTE 2: Downlink data should be buffered preferably in the UP function. Downlink data may be buffered in the CP function when desired, e.g. for UEs using power saving methods. NOTE 3: If the traffic that the UP function can support may be subject to traffic redirection, traffic redirection enforcement in the UP function shall be supported by the UP function. NOTE 4: CP and UP functions complying with this release of the specification should support this feature. NOTE 5: A UPF that supports the SSET or MPAS feature shall support this feature. |  |  |  |  |

# **** Next Change ****

8.2.124	Steering Functionality

The Steering Functionality IE type shall be encoded as shown in Figure 8.2.124-1. It indicates the steering functionality (ATSSS feature) to be used in a MAR.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 171 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  | Steering Functionality Value |  |  |  |  |
|  | 6 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.124-1: Steering Functionality

The Steering Functionality value shall be encoded as a 4 bits binary integer as specified in Table 8.2.124-1.

Table 8.2.124-1: Steering Functionality value

| Steering Functionality Value | Values (Decimal) |
| --- | --- |
| ATSSS-LL | 0 |
| MPTCP | 1 |
| MPQUIC | 2 |
| MPQUIC-IP | X |
| Spare | Y to 15 |

# **** Next Change ****

8.2.158	Link-Specific Multipath IP Address

The Link-Specific Multipath IP Address IE shall carry link-specific IP address used for MPTCP, MPQUIC-IP or MPQUIC steering function. It shall be encoded as shown in Figure 8.2.158-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 229 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  | NV6 | NV4 | V6 | V4 |  |
|  | p to (p+3) | Link-Specific Multipath IPv4 Address for 3GPP Access |  |  |  |  |  |  |  |  |
|  | q to (q+15) | Link-Specific Multipath IPv6 Address for 3GPP Access |  |  |  |  |  |  |  |  |
|  | r to (r+3) | Link-Specific Multipath IPv4 Address for Non-3GPP Access |  |  |  |  |  |  |  |  |
|  | s to (s+3) | Link-Specific Multipath IPv6 Adress for Non-3GPP Access |  |  |  |  |  |  |  |  |
|  | m to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.158-1: Link-Specific Multipath IP Address

The following flags are coded within Octet 5:

-	Bit 1 – V4: If this bit is set to "1", then the Link-Specific Multipath IPv4 Address for 3GPP Access shall be present in the Link-Specific Multipath IP Address IE.

-	Bit 2 – V6: If this bit is set to "1", then the Link-Specific Multipath IPv6 Address for 3GPP Access shall be present in the Link-Specific Multipath IP Address IE.

-	Bit 3 – NV4: If this bit is set to "1", then the Link-Specific Multipath IPv4 Address for Non-3GPP Access shall be present in the Link-Specific Multipath IP Address IE.

-	Bit 4 – NV6: If this bit is set to "1", then the Link-Specific Multipath IPv6 Address for Non-3GPP Access shall be present in the Link-Specific Multipath IP Address IE.

-	Bit 5 to 8 Spare, for future use and set to "0".

Octets "p to (p+3)" or "q to (q+15)" (IPv4 address / IPv6 address fields), if present, shall contain the value for Link-Specific Multipath IP Address for 3GPP access.

Octets "r to (r+3)" or "s to (s+15)" (IPv4 address / IPv6 address fields), if present, shall contain the value for Link-Specific Multipath IP Address for Non-3GPP access.

# **** Next Change ****

8.2.181	Multipath Applicable Indication

Multipath Applicable Indication shall be coded as depicted in Figure 8.2.181-1.

|  |  | Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 265 (decimal) |  |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  |  | MIQAI |  | MQAI | MTAI |  |
|  | 6 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |  |

Figure 8.2.181-1: Multipath Applicable Indication

The following flags are coded within Octet 5:

-	Bit 1 – MTAI (MPTCP Applicable Indication): When this bit is set to "1", it indicates that the PDR is used to detect user plane traffic for which MPTCP is applicable (see clause 5.20.2.4).

-	Bit 2 – MQAI (MPQUIC Applicable Indication): When this bit is set to "1", it indicates that the PDR is used to detect user plane traffic for which MPQUIC is applicable (see clause 5.20.2A.4).

-	Bit X – MIQAI (MPQUIC-IP Applicable Indication): When this bit is set to "1", it indicates that the PDR is used to detect IP traffic for which MPQUIC-IP is applicable (see clause 5.20.2A.X).

-	Bit Y to 8 are spare and reserved for future use.

# **** Next Change ****

8.2.225	MPQUIC Control Information

The MPTCP Control Information IE shall provide details of the required MPQUIC-based functionality. It shall be encoded as shown in Figure 8.2.a-1.

|  |  | Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |
|  | 1 to 2 | Type = 330 (decimal) |  |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  |  |  | CIP |  | CUDP |  |
|  | m to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |  |

Figure 8.2.225-1: MPQUIC Control Information

The following flags are coded within Octet 5:

-	Bit 1– CUDP (CONNECT-UDP): If this bit is set to "1", it indicates that the proxy type of required MPQUIC is "connect-udp" (see clause 5.32.6.2.2 of 3GPP TS 23.501 [28]) and the UPF shall allocate relevant resource as specified in clause 5.20.2A.1

-	Bit X– CIP (CONNECT-IP): If this bit is set to "1", it indicates that the proxy type of required MPQUIC is "connect-ip" (see clause 5.32.6.2.2 of 3GPP TS 23.501 [28]) and the UPF shall allocate relevant resource as specified in clause 5.20.2A.1

-	Bit Y to 8 Spare, for future use and set to "0".

# **** End of Changes ****