3GPP TSG-CT WG4 Meeting #125	C4-244388

Hefei, P.R. China; 14th – 18th October 2024	was C4-244264

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 1133 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PWS N2 Subscription per CBCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1/ According to TS 29.518, the CBCF/PWS-IWF will subscribe to AMF for N2 PWS information notification by invoking NonUeN2InfoSubscribe service operation. When multiple CBCF/PWS-IWF instances are deployed in the network, the subscription should be associated to the NF consumer who created the subscription, so the AMF can check whether the corresponding N2 information subscription for PWS information from the NF service consumer is available in a subsequent N2 Message Transfer service operation. The service logic is not described in NonUeN2InfoSubscribe service procedure. 2/ After restart, a CBCF/PWS-IWF will create the subscription again to the AMF if the CBCF/PWS-IWF doesn't support redundant architecture, which resulting duplicate subscriptions in the AMF from the same CBCF/PWS-IWF. When uplink N2 PWS information received, the AMF will send multiple notifications (one per subscription) to the CBCF. When the AMF manages a large number of RAN nodes, the N2 PWS information may contain a big size of RAN information. Duplicate traffic for such large size messages should de avoided. It is recommended that the AMF remove the duplicated subscription when new subscription is created by the same CBCF/PWS-IWF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clause service procedure for NonUeN2InfoSubscribe for N2 PWS information, describing that the AMF should store the NF instance ID of the NF consumer should be included conditionally and the AMF should store it for later use, also with description that the AMF should remove duplicated subscriptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear AMF behavior on handling N2 PWS subscription when multiple CBCF/PWS-IWF deployed in the network. Redundant N2 PWS notification with potentially large size send between AMF and CBCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.2.x(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not require version update on OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: 1/ Change "Should" to "May" 2/ Remove the callback URI in the descritpion 3/ Describe the AMF may use the CBCF ID when handling N2 PWS Transfer. 4/ Correct the cover page |  |  |  |  |  |  |  |  |

* * * First Change * * * *

###### 5.2.2.4.2.x	NonUeN2InfoSubscribe for N2 PWS Information

The NonUeN2InfoSubscribe service operation shall be invoked by the NF Service Consumer (i.e. CBCF, PWS-IWF) towards the NF Service Producer (i.e. target AMF) to subscribe for notifications of N2 PWS information classes ("PWS", "PWS-BCAL" or "PWS-RF").

The requirements specified in clause 5.2.2.4.2.1 shall apply with the following additions:

1.	Same as step 1 of Figure 5.2.2.4.2.1-1. The POST request body may contain the NF instance ID of the NF consumer when more than one CBCF/PWS-IWF instances are deployed in the network. The AMF may store the NF instance ID in the subscription and use it to identify whether the same CBCF/PWS-IWF instance has subscribed for corresponding N2 PWS information when handling N2 PWS information transfer request from a CBCF/PWS-IWF.

2.	Same as step 1 of Figure 5.2.2.4.2.1-1. If the request is accepted, the AMF may remove any duplicated subscription, i.e. an existing subscription from the same NF consumer (identified by the NF instance ID) for notification of the same N2 PWS information class.

* * * End of Changes * * * *

* * * Information * * * *

###### 5.2.2.4.1.3	Warning Request Transfer Procedure

The NonUeN2MessageTransfer service operation shall be invoked by the NF Service Consumer, e.g. CBCF/PWS-IWF, to send non-UE specific messages of N2 information class PWS to the NG-RAN.

The requirements specified in clause 5.2.2.4.1.1 shall apply with the following modifications:

1.	Same as step 1 of Figure 5.2.2.4.1.1-1, the request body shall include the N2 Message Container and:

- the globalRanNodeList IE, or;

- the taiList IE and the ratSelector IE, or;

- the ratSelector IE.

The AMF shall forward the N2 Message Container to ng-eNBs or to gNBs indicated in the globalRanNodeList IE if present. If the globalRanNodeList IE if not present, the AMF shall forward the N2 Message Container to ng-eNBs or to gNBs, subject to the value of the ratSelector IE, that serve Tracking Areas as listed in the taiList IE if present. If the taiList IE and the globalRanNodeList IE are not present, the AMF shall forward the N2 Message Container to all attached ng-eNBs or all attached gNBs, subject to the value of the ratSelector IE.

NOTE:	The globalRanNodeList IE can be present when transferring WRITE-REPLACE WARNING REQUEST. When present, the globalRanNodeList IE only contains RAN nodes of the same type, i.e. only ng-eNBs or only gNBs.

The request body may additionally include the omcId IE and/or the sendRanResponse IE.

2a.	Same as step 2a of Figure 5.2.2.4.1.1-1, and the POST response body shall contain the mandatory elements from the Write-Replace-Warning Confirm response (see clause 9.2.17 in TS 23.041 [20]) or the mandatory elements and optionally the unknown TAI List IE from the Stop-Warning Confirm response (see clause 9.2.19 in TS 23.041 [20]).

If the sendRanResponse IE with the value "true" was received in the request, but the corresponding N2 information subscription for PWS information from the NF service consumer is not available in the AMF, the AMF should include the n2PwsSubMissInd IE with the value "true" in the response.

2b.	Same as step 2b of Figure 5.2.2.4.1.1-1, and the POST response body shall contain following additional information:

-	PWS specific information, if any, e.g. PWS Cause information.

* * * Information * * * *