3GPP TSG-CT WG4 Meeting #125	C4-244409

# Hefei, P.R. China; 14th – 18th October 2024						Revision of C4-244251, C4-244101

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.274 | CR | 2117 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Trace Reporting Consumer URI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2024-10-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Trace Collection Entity IP Address is supported for the file-based trace reporting and Trace Reporting Consumer URI is supported from Rel-16 for the streaming trace reporting. For backward compatibility issue, both collection Entity IP Address and Trace Reporting Consumer URI are included, the Trace Reporting Consumer URI shall have priority if both file-based trace reporting and streaming trace reporting are supported. Trace Collection Entity URI is already supported in clause 9.2.1.4 of 3GPP TS 36.413. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support the Trace Reporting Consumer URI for streaming trace reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage2. Streaming trace reporting can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.2.1, 7.3.1, 7.3.6, 7.3.9, 7.12.1, 8.1, 8.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[4]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[5]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[6]	IETF RFC 791 (STD 0005): "Internet Protocol", J. Postel.

[7]	IETF RFC 768 (STD 0006): "User Datagram Protocol", J. Postel.

[8]	3GPP TS 32.251: "Telecommunication Management; Charging Management; Packet Switched (PS) domain charging.

[9]	3GPP TS 32.298: "Telecommunication Management; Charging Management; Charging Data Record (CDR) parameter classification.

[10]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[11]	3GPP TS 33.102: "3G security; Security architecture".

[12]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[13]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[14]	3GPP TS 29.276: "3GPP Evolved Packet System (EPS); Optimized handover procedures and protocols between E-UTRAN Access and cdma2000 HRPD Access; Stage 3".

[15]	3GPP TS 29.280: "Evolved Packet System (EPS); 3GPP Sv interface (MME to MSC, and SGSN to MSC) for SRVCC".

[16]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[17]	3GPP TS 23.007: "Restoration procedures".

[18]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management ".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[20]	3GPP TS 36.414: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 data transport".

[21]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[22]	3GPP TS 29.118: "Mobility Management Entity (MME) - Visitor Location Register (VLR) SGs interface specification".

[23]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[24]	void

[25]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[26]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3".

[27]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol".

[28]	3GPP TS 48.008: "Mobile Switching Centre - Base Station System (MSC-BSS) interface; Layer 3 specification".

[29]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[30]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[31]	IETF RFC 1035: "Domain Names - Implementation and Specification".

[32]	3GPP TS 29.303: "Domain Name System Procedures; Stage 3".

[33]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[34]	3GPP TS 48.018: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS protocol (BSSGP)".

[35]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[36]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[37]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[38]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN) ".

[39]	IETF RFC 3588: "Diameter Base Protocol ".

[40]	IETF RFC 4607: "Source-Specific Multicast for IP".

[41]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[42]	3GPP TS 29.010: "Information element mapping between Mobile Station - Base Station System (MS - BSS) and Base Station System - Mobile-services Switching Centre (BSS - MSC); Signalling procedures and the Mobile Application Part (MAP)".

[43]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[44]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[45]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses.

[46]	3GPP TR 25.999: "HSPA Evolution (FDD)".

[47]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services".

[48]	3GPP TS 23.203: "Policy and charging control architecture; Stage 2".

[49]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[50]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[51]	3GPP TS 23.139: "3GPP system - fixed broadband access network interworking; Stage 2".

[52]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[53]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[54]	3GPP TS 32.299: "Telecommunication Management; Charging Management; Diameter charging applications.

[55]	3GPP TS 23.251: " Network Sharing; Architecture and Functional Description".

[56]	3GPP TS 23.271: "Location Services".

[57]	3GPP TS 29.173: "Diameter-based SLh interface for Control Plane LCS".

[58]	IETF RFC 5453: "Reserved IPv6 Interface Identifiers".

[59]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[60]	IETF RFC 3046: "DHCP Relay Agent Information Option".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 22.153: "Multimedia Priority Service".

[63]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC)
via non-3GPP access networks; stage 3".

[64]	ITU-T Recommendation E.212: "The international identification plan for mobile terminals and mobile users".

[65]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[66]	3GPP TS 24.244: "Wireless LAN control plane protocol for trusted WLAN access to EPC".

[67]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[68]	3GPP TS 29.273: "3GPP EPS AAA Interfaces".

[69]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[70]	3GPP TS 29.272: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[71]	3GPP TS 23.161: "Network-Based IP Flow Mobility (NBIFOM); Stage 2".

[72]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[73]	3GPP TS 24.161: "Network-Based IP Flow Mobility (NBIFOM); Stage 3".

[74]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[75]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[76]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[77]	IETF RFC 4995: "The RObust Header Compression (ROHC) Framework".

[78]	3GPP TS 43.020: "Security related network functions".

[79]	3GPP TS 36.444: "EUTRAN M3 Application Protocol (M3AP)".

[80]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane of EPC Nodes; stage 3".

[81]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[82]	3GPP TS 23.501:"System Architecture for the 5G System; Stage 2".

[83]	3GPP TS 23.502:"Procedures for the 5G System; Stage 2".

[84]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[85]	3GPP TS 33.250: "Security assurance specification for the PGW network product class".

[86]	3GPP TS 33.501:" Security Architecture and Procedures for 5G System".

[87]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[88]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[89]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[90]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[91]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[92]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[93]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

* * * Next Change * * * *

7.2.1	Create Session Request

The direction of this message shall be from MME/S4-SGSN to SGW and from SGW to PGW, and from ePDG/TWAN to the PGW (see Table 6.1-1).

The Create Session Request message shall be sent on the S11 interface by the MME to the SGW, and on the S5/S8 interface by the SGW to the PGW as part of the procedures:

-	E-UTRAN Initial Attach when a PDN connection needs to be established through the SGW and PGW

-	Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN with GTP on S5/S8 interface (see clauses 8.2, 8.6 and 16.11 of 3GPP TS 23.402 [45])

-	UE requested PDN connectivity when a PDN connection needs to be established through the SGW and PGW

-	Addition of a 3GPP access of NBIFOM procedure as specified by 3GPP TS 23.161 [71]

-	Restoration of PDN connections after an PGW-C/SMF change as specified in clause 31 of 3GPP TS 23.007 [17]

The message shall also be sent on S4 interface by the SGSN to the SGW, and on the S5/S8 interface by the SGW to the PGW as part of the procedures:

-	PDP Context Activation

-	Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN with GTP on S5/S8 interface (see clauses 8.2, 8.6 and 16.11 of 3GPP TS 23.402 [45])

-	Addition of a 3GPP access of the NBIFOM procedure as specified by 3GPP TS 23.161 [71]

The message shall also be sent on the S11 interface by the MME to the SGW as part of the procedures:

-	Tracking Area Update procedure with Serving GW change

-	S1/X2-based handover with SGW change

-	UTRAN Iu mode to E-UTRAN Inter RAT handover with SGW change

-	GERAN A/Gb mode to E-UTRAN Inter RAT handover with SGW change

-	3G Gn/Gp SGSN to MME combined hard handover and SRNS relocation procedure

-	Gn/Gp SGSN to MME Tracking Area Update procedure

-	Restoration of PDN connections after an SGW failure if the MME and PGW support these procedures as specified in 3GPP TS 23.007 [17]

-	MME triggered Serving GW relocation

-	Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN with PMIP on S5/S8 interface (see clauses 8.2 and 16.11 of 3GPP TS 23.402 [45])

and on the S4 interface by the SGSN to the SGW as part of the procedures:

-	Routing Area Update with MME interaction and with SGW change

-	Gn/Gp SGSN to S4 SGSN Routing Area Update

-	Inter SGSN Routeing Area Update Procedure and Combined Inter SGSN RA / LA Update using S4 with SGW change

-	Iu mode RA Update Procedure using S4 with SGW change

-	E-UTRAN to UTRAN Iu mode Inter RAT handover with SGW change

-	E-UTRAN to GERAN A/Gb mode Inter RAT handover with SGW change

-	Serving RNS relocation using S4 with SGW change

-	Combined hard handover and SRNS relocation using S4 with SGW change

-	Combined Cell / URA update and SRNS relocation using S4 with SGW change

-	Enhanced serving RNS relocation with SGW relocation

-	Restoration of PDN connections after an SGW failure if the SGSN and PGW support these procedures as specified in 3GPP TS 23.007 [17]

-	S4-SGSN triggered Serving GW relocation

-	Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN with PMIP on S5/S8 interface (see clauses 8.2 and 16.11 of 3GPP TS 23.402 [45])

and on the S2b interface by the ePDG to the PGW as part of the procedures:

-	Initial Attach with GTP on S2b

-	UE initiated Connectivity to Additional PDN with GTP on S2b

-	Handover to Untrusted Non-3GPP IP Access with GTP on S2b (See clause 8.6 of 3GPP TS 23.402 [45])

-	Initial Attach for emergency session (GTP on S2b)

-	Addition of an access using S2b of NBIFOM procedure as specified by 3GPP TS 23.161 [71]

and on the S2a interface by the TWAN to the PGW as part of the procedure:

-	Initial Attach in WLAN on GTP S2a

-	Initial Attach in WLAN for Emergency Service on GTP S2a

-	UE initiated Connectivity to Additional PDN with GTP on S2a

-	Handover to TWAN with GTP on S2a (See clause 16.10 of 3GPP TS 23.402 [45])

-	Addition of an access using S2b of NBIFOM procedure as specified by 3GPP TS 23.161 [71]

-	PDN connection establishment over untrusted non-3GPP access as user-plane resource associated with a MA PDU session (see clause 4.22.2.4.2 of 3GPP TS 23.502 [83])

If the new Create Session Request received by the SGW collides with an existing active PDN connection context (the existing PDN connection context is identified with the tuple [IMSI, EPS Bearer ID], where IMSI shall be replaced by TAC and SNR part of ME Identity for emergency or RLOS attached UE without UICC or authenticated IMSI), this Create Session Request shall be treated as a request for a new session. Before creating the new session, the SGW should delete:

-	the existing PDN connection context locally, if the Create Session Request is received with the TEID set to zero in the header, or if it is received with a TEID not set to zero in the header and it collides with the default bearer of an existing PDN connection context;

-	the existing dedicated bearer context locally, if the Create Session Request collides with an existing dedicated bearer context and the message is received with a TEID not set to zero in the header.

In the former case, if the PGW S5/S8 IP address for control plane received in the new Create Session Request is different from the PGW S5/S8 IP address for control plane of the existing PDN connection, the SGW should also delete the existing PDN connection in the corresponding PGW by sending a Delete Session Request message.

NOTE 1:	The SGW can send the Create Session Request and Delete Session Request over S5/S8 asynchronously, e.g. the SGW can send the Delete Session Request and then the Create Session Request without having to wait for the Delete Session Response. It does not matter if the PGW happens to receive the Delete Session Request after the Create Session Request since the PGW assigns a different S5/S8 F-TEID for control plane to the new PDN connection.

In some network deployment, e.g. when 5G Network Slice is deployed and the combined PGW-C/SMFs are connected to the UDM, the PGW may select another PGW supporting the network slice for which the UE has subscription and then forward the Create Session Request to that PGW. When forwarding the Create Session Request message, the PGW shall forward the Create Session Request message as received from the SGW but with the following modifications:

-	the destination IP address of the message shall be set to the selected PGW IP address;

-	the CSRMFI flag shall be set to "1";

-	the source IP address and UDP port of the message shall be set to the IP address and port of the forwarding PGW.

NOTE 2:	The Create Session Response message is sent back to the forwarding PGW that forwards it to the SGW. It is assumed that GTPv2/UDP/IP connectivity between the source PGW/SMF and the target PGW/SMF which are in different slices is allowed in such network deployment; otherwise, if there is no GTPv2/UDP/IP connectivity between the source PGW/SMF and the target PGW/SMF, or if the source PGW/SMF does not support forwarding the request to the target PGW/SMF, the source PGW/SMF can proceed as specified in clause 7.2.2.

If the new Create Session Request received by the PGW collides with an existing PDN connection context (the existing PDN connection context is identified with the triplet [IMSI, EPS Bearer ID, Interface type], where applicable Interface type here is S2a TWAN GTP-C interface or S2b ePDG GTP-C interface or S5/S8 SGW GTP-C interface, and where IMSI shall be replaced by TAC and SNR part of ME Identity for emergency or RLOS attached UE without UICC or authenticated IMSI), this Create Session Request shall be treated as a request for a new session. Before creating the new session, the PGW should delete:

-	the existing PDN connection context, if the Create Session Request collides with the default bearer of an existing PDN connection context;

-	the existing dedicated bearer context, if the Create Session Request collides with a dedicated bearer of an existing PDN connection context.

The PGW shall allocate a new PGW S5/S8 F-TEID for control plane to the new PDN connection, i.e. not the same F-TEID value as the one which was assigned to the existing PDN connection.

NOTE 3:	With GTP based S2a and S2b, the EPS Bearer IDs assigned for specific UE over S2a between the TWAN and PGW and over S2b between an ePDG and PGW are independent of the EPS Bearer IDs assigned for the same UE over S5/S8 and may overlap in value (see 3GPP TS 23.402 [45] clause 4.6.2).

NOTE 4:	Only the TAC and SNR part of the ME Identity is used to identify an emergency or RLOS attached UE without UICC or authenticated IMSI.

During a PDN connection restoration after a PGW-C/SMF change procedure specified in clause 31 of 3GPP TS 23.007 [17], the MME/ePDG:

-	shall include the IEs defined as Mandatory IEs;

-	shall include the IMSI or the ME Identity (MEI), as defined for these IEs in Table 7.2.1-1;

-	shall include the MSISDN, the Selection Mode, the PDN Type, the PDN Address Allocation (PAA) and the Charging Characteristics, as defined for these IEs in Table 7.2.1-1, if a new SGW is selected, otherwise these IEs should not be included;

-	shall include the User Location Information (ULI), the Serving Network, the UE Time Zone, the User CSG Information (UCI), the Presence Reporting Area Information, the Serving PLMN Rate Control, as defined for these IEs in Table 7.2.1-1, if a new SGW is selected, otherwise these IEs may be included;

NOTE 5:	The IEs documented in above two bullets are required for a new SGW, e.g. for SGW-CDR (see 3GPP TS 32.251 [8]).

-	may include the UE Local IP address, the UE UDP Port, the UE TCP Port, the WLAN Location Information and the WLAN Location Timestamp as defined for these IEs in Table 7.2.1-1;

NOTE 6:	The User Location Information (ULI), the Serving Network, the UE Time Zone, the User CSG Information (UCI), the Presence Reporting Area Information, the Serving PLMN Rate Control, the UE Local IP address, the UE UDP Port, the UE TCP Port, the WLAN Location Information and the WLAN Location Timestamp can be included if there is a change in comparison with the value reported to the PGW-C/SMF before the restoration procedure.

-	shall set the "Restoration of PDN connections after an PGW-C/SMF Change Support Indication" and "PGW Change Indication" flags in the Indication Flags IE to "1", and when receiving these two flags set to "1", the (alternative) PGW-C/SMF shall consider that other flags in the Indication Flags IE set for the PDN connection (before the restoration) remain unchanged.

NOTE 7:		The Create Session Request and Response messages used during a PDN connection restoration after a PGW-C/SMF change procedure are to restore the PDN connection in an alternative PGW-C/SMF of the same PGW-C/SMF set, therefore the information elements included in the Create Session Request are not intended to enable the alternative PGW-C/SMF to re-establish the PDN connection context; instead, the alternative PGW-C/SMF will retrieve the PDN connection context which is stored in a shared context in the PGW-C/SMF set.

NOTE 8:	During a PDN connection restoration after a PGW-C/SMF change procedure, if the MME receives a NAS ESM signaling message, e.g.  Bearer resource allocation request or Bearer resource modification request, where these NAS message can contain a PCO/ePCO, the MME will send a Create Session Request message without PCO/ePCO to restore the PDN connection in the (alternative) PGW-C/SMF and then send Bearer Resource Modification Command to address the NAS ESM message.

NOTE 9:	The (alternative) PGW-C/SMF considers that IEs which are not received in the Create Session Request message, e.g. the Maximum APN Restriction, UE Local IP Address, User Location Information, are not changed in comparison with the ones stored.

NOTE 10:	The PGW-C/SMF does not change APN-AMBR, Bearer Level QoS for the PDN connection in the Create Session Response message since these parameters can trigger signalling towards the UE/eNB and this is not supported by the restoration procedure; the PGW-C/SMF can change those information elements using Bearer Context Modification procedures as specified in clauses 5.4.2 and 5.4.3 of 3GPP TS 23.401 [3] after the PDN connection is restored.

Table 7.2.1-1: Information Elements in a Create Session Request

| Information elements | P | Condition / Comment | IE Type | Ins. |
| --- | --- | --- | --- | --- |
| IMSI | C | The IMSI shall be included in the message on the S4/S11 interface, and on S5/S8 interface if provided by the MME/SGSN, except for the case: - If the UE is emergency or RLOS attached and the UE is UICCless. The IMSI shall be included in the message on the S4/S11 interface, and on S5/S8 interface if provided by the MME/SGSN, but not used as an identifier - if UE is emergency or RLOS attached but IMSI is not authenticated. The IMSI shall be included in the message on the S2a/S2b interface, except for the case: - if the UE is emergency attached and the UE is UICCless. The IMSI shall be included in the message on the S2a/S2b interface, but not used as an identifier: - if UE is emergency attached but IMSI is not authenticated. | IMSI | 0 |
| MSISDN | C | For an E-UTRAN Initial Attach and a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN the IE shall be included when used on the S11 interface, if provided in the subscription data from the HSS. For a PDP Context Activation procedure and a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN the IE shall be included when used on the S4 interface, if provided in the subscription data from the HSS. The IE shall be included for the case of a UE Requested PDN Connectivity, if the MME has it stored for that UE. It shall be included when used on the S5/S8 interfaces if provided by the MME/SGSN. The ePDG shall include this IE on the S2b interface during an Attach with GTP on S2b, UE initiated Connectivity to Additional PDN with GTP on S2b and a Handover to Untrusted Non-3GPP IP Access with GTP on S2b, Initial Attach for emergency session (GTP on S2b) if provided by the HSS/AAA. The TWAN shall include this IE on the S2a interface during an Initial Attach in WLAN on GTP S2a, Initial Attach in WLAN for Emergency Service on GTP S2a, UE initiated Connectivity to Additional PDN with GTP on S2a and a Handover to TWAN with GTP on S2a, if provided by the HSS/AAA. | MSISDN | 0 |
|  | CO | During a PDN connection restoration procedure after an PGW-C/SMF change, the MME shall include this IE if a new SGW is selected and if the MSISDN is available at the MME. |  |  |
|  | CO | This IE shall be present on the S11 interface during an S1/X2-based handover, 5GS to EPS handover or a TAU procedure with SGW relocation or insertion (for 5GS to EPS mobility), if the MSISDN is available at the MME. |  |  |
| ME Identity (MEI) | C | The MME/SGSN shall include the ME Identity (MEI) IE on the S11/S4 interface: - If the UE is emergency attached and the UE is UICCless; or - If the UE is emergency attached and the IMSI is not authenticated For all other cases the MME/SGSN shall include the ME Identity (MEI) IE on the S11/S4 interface if it is available. | MEI | 0 |
|  | CO | The MME shall include the ME Identity (MEI) IE on the S11 interface: - If the UE is RLOS attached and the UE is UICCless; or - If the UE is RLOS attached and the IMSI is not authenticated. |  |  |
|  | CO | If the SGW receives this IE, it shall forward it to the PGW on the S5/S8 interface. |  |  |
|  | CO | The TWAN/ePDG shall include the ME Identity (MEI) IE on the S2a/S2b interface: - If the UE is emergency attached and the UE is UICCless; or - If the UE is emergency attached and the IMSI is not authenticated. For all other cases, the TWAN/ePDG shall include the ME Identity (MEI) IE on the S2a/S2b interface, if it is available. |  |  |
| User Location Information (ULI) | C | This IE shall be included on the S11 interface for E-UTRAN Initial Attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN and UE-requested PDN Connectivity procedures. It shall include ECGI and TAI. The MME/SGSN shall also include it on the S11/S4 interface for TAU/RAU/X2-Handover/Enhanced SRNS Relocation procedure if the PGW/PCRF has requested location information change reporting and MME/SGSN support location information change reporting. | ULI (NOTE 10) | 0 |
|  | CO | This IE shall also be included on the S4 interface for PDP Context Activation procedure. It shall include CGI/SAI, together with RAI. |  |  |
|  | CO | This IE shall also be included on the S4/S11 interface for a TAU/RAU procedure if - the level of support (User Location Change Reporting and/or CSG Information Change Reporting) changes; or - the target MME/S4-SGSN cannot derive the level of support (User Location Change Reporting and/or CSG Information Change Reporting) for the source Gn/Gp SGSN. See NOTE 9. The MME shall include the ECGI and /or TAI in the ULI, the S4-SGSN shall include either the CGI or SAI or RAI, or CGI/SAI together with RAI in the ULI. |  |  |
|  | CO | This IE shall also be included on the S4/S11 interface for: - a TAU procedure with an S4-SGSN interaction, if the MME supports location information change reporting; - a RAU procedure with an MME interaction, if the S4-SGSN supports location information change reporting. The MME shall include the ECGI and TAI in the ULI, the S4-SGSN shall include the CGI/SAI together with RAI in the ULI. |  |  |
|  | CO | During a PDN connection restoration after a PGW-C/SMF change procedure, the MME shall include this IE on the S11 interface if a new SGW is selected, otherwise the MME may include this IE. When present, it shall be set to the last known location of the UE (which may differ from the last ULI information signaled to the PGW-C/SMF before the restoration of the PDN connection). |  |  |
|  | CO | The SGW shall include this IE on S5/S8 if it receives the ULI from MME/SGSN. |  |  |
| Serving Network | C | This IE shall be included on the S4/S11, S5/S8 and S2b interfaces for an E-UTRAN initial attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, a PDP Context Activation, a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN, a UE requested PDN connectivity, an Attach with GTP on S2b, a UE initiated Connectivity to Additional PDN with GTP on S2b, a Handover to Untrusted Non-3GPP IP Access with GTP on S2b and an Initial Attach for emergency session (GTP on S2b). See NOTE 10. | Serving Network | 0 |
|  | CO | This IE shall be included on S4/S11 for RAU/TAU/Handover with SGW relocation procedures. See NOTE 10. |  |  |
|  | CO | During a PDN connection restoration after a PGW-C/SMF change procedure procedure, the MME shall include this IE on the S11 interface if a new SGW is selected, otherwise the MME/ePDG may include this IE. When present, it shall contain the last known Serving Network for the UE. |  |  |
|  | CO | This IE shall be included on the S2a interface for an Initial Attach in WLAN on GTP S2a, an Initial Attach in WLAN for Emergency Service on GTP S2a, a UE initiated Connectivity to Additional PDN with GTP on S2a and a Handover to TWAN with GTP on S2a. The TWAN shall set this IE to the PLMN identity of the selected PLMN used for 3GPP-based access authentication. The selected PLMN is the PLMN of the 3GPP AAA Proxy in roaming case and the PLMN of the 3GPP AAA Server in non-roaming case. |  |  |
| RAT Type | M | This IE shall be set to the 3GPP access type or to the value matching the characteristics of the non-3GPP access the UE is using to attach to the EPS. The MME shall set the RAT Type to LTE-M if it has received the LTE-M indication from the eNodeB, otherwise it shall set it to the RAT type the UE is using. The ePDG may use the access technology type of the untrusted non-3GPP access network if it is able to acquire it; otherwise it shall indicate Virtual as the RAT Type. The TWAN shall set the RAT Type value to "WLAN" on the S2a interface. If the LTE-M RAT type is received from the MME, the SGW shall signal the following RAT type to the PGW: - LTE-M RAT type, if the 'LTE-M RAT type reporting to PGW' flag is received from the MME; or - WB-E-UTRAN RAT type, otherwise. If the Satellite related RAT type is received from the MME, the SGW shall signal the following RAT type to the PGW: - The received Satellite related RAT type, if the 'Satellite RAT Type reporting to PGW Indication' flag is received from the MME; or - The RAT type without Satellite, i.e. LTE-M / EUTRAN-NB-IoT / WB-E-UTRAN, otherwise. See NOTE 3, NOTE 4, NOTE 22, NOTE 28. | RAT Type | 0 |
| Indication Flags | C | This IE shall be included if any one of the applicable flags is set to 1. Applicable flags are: - S5/S8 Protocol Type: This flag shall be set to 1 on the S11/S4 interfaces if the chosen protocol type for the S5/S8 interface is PMIP. - Dual Address Bearer Flag: This flag shall be set to 1 on the S2b, S11/S4 and S5/S8 interfaces when the PDN Type, determined based on UE request and subscription record, is set to IPv4v6 and all SGSNs which the UE may be handed over to support dual addressing. This shall be determined based on node pre-configuration by the operator. (see also NOTE 5). The TWAN shall set this flag to 1 on the S2a interface if it supports IPv4 and IPv6 and the PDN Type determined from the UE request if single-connection mode or multi-connection mode is used (see 3GPP TS 23.402 [45]) and the user subscription data is set to IPv4v6. - Handover Indication: This flag shall be set to 1 on the S11/S4 and S5/S8 interface during a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, or a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN procedures, or an Addition of a 3GPP access of NBIFOM procedure, or during a 5GS to EPS handover without the N26 interface. This flag shall be set to 1 on the S2b interface during a Handover from 3GPP access to Untrusted Non-3GPP IP Access with GTP on S2b and IP address preservation is requested by the UE, or an Addition of an access using S2b of NBIFOM procedure. This flag shall be set to 1 on the S2a interface during a Handover from 3GPP access to TWAN with GTP on S2a and IP address preservation is requested by the UE, or an Addition of an access using S2a of NBIFOM procedure. - Operation Indication: This flag shall be set to 1 on the S4/S11 interface for a TAU/RAU procedure with SGW relocation, Enhanced SRNS Relocation with SGW relocation, X2-based handovers with SGW relocation and MME triggered Serving GW relocation. - Direct Tunnel Flag: This flag shall be set to 1 on the S4 interface if Direct Tunnel is used. - Piggybacking Supported: This flag shall be set to 1 on the S11 interface only if the MME supports the piggybacking feature as described in Annex F of 3GPP TS 23.401 [3]. This flag shall be set to 1 on S5/S8 only if both the MME and the SGW support piggybacking. - Change Reporting support Indication: This flag shall be set to 1 on S4/S11 and S5/S8 interfaces if the SGSN/MME supports location Info Change Reporting and if the SGSN/MME's operator policy permits reporting of location change to the operator of the PGW with which the session is being established. See NOTE2. - CSG Change Reporting Support Indication: This flag shall be set to 1 on S4/S11 and S5/S8 interfaces if the SGSN/MME supports CSG Information Change Reporting and if the SGSN/MME's operator policy permits reporting of CSG Information change to the operator of the PGW with which the session is being established. See NOTE 2. - Unauthenticated IMSI: This flag shall be set to 1 on the S4/S11, S5/S8 and S2a/S2b interfaces if the IMSI present in the message is not authenticated and is for an emergency attached UE. It shall also be set to 1 on the S11 and S5/S8 interfaces if the IMSI is present in the message is not authenticated and is for an RLOS attached UE. - PDN Pause Support Indication: this flag shall be set to 1 on the S5/S8 interface if the SGW supports the PGW Pause of Charging procedure. - NBIFOM Support Indication: This flag shall be set to 1 on S11/S4 if the MME/SGSN supports NBIFOM. This flag shall be set to 1 on S5/S8 if both the SGW and the MME/SGSN support NBIFOM. This flag shall be set to 1 on S2a/S2b if the TWAN/ePDG supports NBIFOM. - WLCP PDN Connection Modification Support Indication: This flag shall be set to 1 on the S2a interface if the TWAN supports the WLCP PDN Connection Modification procedure. - UE Not Authorised Cause Code Support Indication: This flag shall be set to 1 on S4/S11 and S5/S8 interface if the SGSN/MME supports the "UE not authorised by OCS or external AAA Server" Cause Code. - UE Available for Signalling Indication: this flag shall be set to 1 on S11/S4 during a TAU/RAU with SGW relocation procedure if there is pending network initiated PDN connection signalling for this PDN connection. The SGW shall include this IE on S5/S8 if it receives the flag from the MME/SGSN. - S11-U Tunnel Flag: this flag shall be set to 1 on the S11 interface if user data is transported in NAS signalling. - Extended PCO Support Indication: this flag shall be set to 1 on S11 interface by the MME if the UE and the MME support ePCO; and this flag shall be set to 1 on S5/S8 interface by the SGW if the SGW supports ePCO and MME has set the flag to 1. - Control Plane Only PDN Connection Indication: this flag shall be set to 1 over S11 and S5/S8 if the PDN Connection is set to Control Plane Only. - eNB Change Reporting Support Indication: This flag shall be set to 1 on S11 and S5/S8 interfaces if the MME supports location Info Change Reporting and if the MME's operator policy permits reporting of location change to the operator of the PGW with which the session is being established. See NOTE 19. - LTE-M RAT Type reporting to PGW Indication: this flag shall be set to 1 on S11, based on operator policy or roaming agreements (for Home Routed PDN connections), if the SGW needs to forward the LTE-M RAT type to the PGW. - 5GS Interworking Indication(5GSIWKI): this flag shall be set to 1 on S11, S5/S8 and S2b interfaces if the UE supports N1 mode and the PDN connection is not restricted from interworking with 5GS by user subscription (see "5GC" bit within Core-Network-Restrictions AVP and Interworking-5GS-Indicator AVP specified in 3GPP TS 29.272 [70] and 3GPP TS 29.273 [68]). (NOTE 27) - 5GS Interworking without N26 Indication: this flag shall be set to 1 on S11 and S5/S8 interfaces if the 5GS Interworking Indication (5GSIWKI) is set to 1 and the N26 interface is not supported. See clause 4.11.1.1 in 3GPP TS 23.502 [83]). (NOTE 23) - 5GCNRI (5GC Not Restricted Indication): this flag shall be set to 1 on S11, S5/S8 and S2b interfaces if access to the 5GC is not restricted for the PDN connection by user subscription (see "5GC" bit within Core-Network-Restrictions AVP and Interworking-5GS-Indicator AVP specified in 3GPP TS 29.272 [70] and 3GPP TS 29.273 [68]). (NOTE 27) - 5GCNRS (5GC Not Restricted Support): this flag shall be set to 1 on S11, S5/S8 and S2b interfaces if the sending node (i.e. MME or ePDG) supports setting the 5GCNRI flag. An MME or an ePDG compliant with this version of the specification shall support setting the 5GCNRI flag. - MTEDTA (MT-EDT Applicable): this flag shall be set to 1 on the S11 interface if MT-EDT is applicable for the PDN connection, i.e. if the UE has indicated its support of MT-EDT as part of the UE network capability and if the local policy requires so. (NOTE 26) - CSRMFI (Create Session Request Message Forwarded Indication): this shall be set to 1 on S5/S8 interfaces by a PGW if the Create Session Request message has been forwarded by that PGW. - Restoration of PDN connections after an PGW-C/SMF Change Support Indication: this flag shall be set to 1 on the S11/S2b interface if the MME/ePDG supports the Restoration of PDN connections after an PGW-C/SMF Change procedure as specified in clause 31 of 3GPP TS 23.007 [17]. This flag shall be set to 1 on the S5/S8 interface if the SGW and the MME support the Restoration of PDN connections after an PGW-C/SMF Change procedure as specified in clause 31 of 3GPP TS 23.007 [17]. - PGW Change Indication: this flag shall be set to 1 by the MME/ePDG over the S11/S2b interface if the Create Session Request is to move an existing PDN connection to the new PGW-C/SMF as specified in clauses 31.3 and 31.3A of 3GPP TS 23.007 [17]. The SGW shall forward the flag on the S5/S8 interface. - PGWRNSI (PGW Redirection due to mismatch with Network Slice subscribed by UE Support Indication): this flag shall be set to 1 on the S11 and S5/S8 interfaces, if the MME supports receiving a Create Session rejection response with the cause "PGW redirection due to mismatch with network slice subscribed by the UE". - UPIPSI (User Plane Integrity Protection Support Indication): this flag shall be set to 1 by the MME on the S11 interface, if the UE, MME and E-UTRAN supports User Plane Integrity Protection with EPS. This flag shall be set to 1 by the SGW on S5/S8 interface if the MME has set the flag to 1 over the S11 interface. - Satellite RAT Type reporting to PGW Indication (SRTPI): this flag shall be set to 1 on S11, based on operator policy or roaming agreements (for Home Routed PDN connections), if the SGW needs to forward the Satellite RAT type to the PGW. - Large Additional Protocol Configuration Options Support Indication: this flag shall be set to 1 on S2b interface by the ePDG if the ePDG support receiving an APCO content larger than 253 octets. | Indication | 0 |
| Sender F-TEID for Control Plane | M |  | F-TEID | 0 |
| PGW S5/S8 Address for Control Plane or PMIP | C | This IE shall be sent on the S11 / S4 interfaces. The TEID or GRE Key is set to "0" in the E-UTRAN initial attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, the PDP Context Activation, a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN and the UE requested PDN connectivity procedures. | F-TEID | 1 |
| Access Point Name (APN) | M |  | APN | 0 |
| Selection Mode | C | This IE shall be included on the S4/S11 and S5/S8 interfaces for an E-UTRAN initial attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, a PDP Context Activation, a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN and a UE requested PDN connectivity. This IE shall be included on the S2b interface for an Initial Attach with GTP on S2b, a Handover to Untrusted Non-3GPP IP Access with GTP on S2b, a UE initiated Connectivity to Additional PDN with GTP on S2b and an Initial Attach for emergency session (GTP on S2b). It shall indicate whether a subscribed APN or a non- subscribed APN chosen by the UE/MME/SGSN/ePDG/TWAN was selected, see NOTE 17. This IE shall be included on the S2a interface for an Initial Attach in WLAN on GTP S2a, an Initial Attach in WLAN for Emergency Service on GTP S2a, a Handover to TWAN with GTP on S2a and a UE initiated Connectivity to Additional PDN with GTP on S2a. The value shall be set to "MS or network provided APN, subscription verified". | Selection Mode | 0 |
|  | CO | During a PDN connection restoration after an PGW-C/SMF change procedure, the MME shall include this IE on the S11 interface if a new SGW is selected. When present, it shall contain the same value as the value signaled during the PDN connection creation procedure. |  |  |
|  | CO | When available, this IE shall be sent by the MME/SGSN on the S11/S4 interface during TAU/RAU/HO with SGW relocation. |  |  |
| PDN Type | C | This IE shall be included on the S4/S11 and S5/S8 interfaces for an E-UTRAN initial attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, a PDP Context Activation, a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN and a UE requested PDN connectivity. This IE shall be set to IPv4, IPv6, IPv4v6, Non-IP or Ethernet. This is based on the UE request and the subscription record retrieved from the HSS (for MME see 3GPP TS 23.401 [3], clause 5.3.1.1, and for SGSN see 3GPP TS 23.060 [35], clause 9.2.1). See NOTE 1. See NOTE 14. See NOTE 24 and 25. | PDN Type | 0 |
|  | CO | During a PDN connection restoration after an PGW-C/SMF change procedure, the MME shall include this IE on the S11 interface if a new SGW is selected. When present, it shall contain the same value as the value signaled during the PDN connection creation procedure. |  |  |
| PDN Address Allocation (PAA) | C | This IE shall be included the S4/S11, S5/S8 and S2a/S2b interfaces for an E-UTRAN initial attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, a PDP Context Activation, a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN, a UE requested PDN connectivity, an Attach with GTP on S2b, a UE initiated Connectivity to Additional PDN with GTP on S2b, a Handover to Untrusted Non-3GPP IP Access with GTP on S2b, an Initial Attach for emergency session (GTP on S2b, an Initial Attach in WLAN on GTP S2a, an Initial Attach in WLAN for Emergency Service on GTP S2a, a UE initiated Connectivity to Additional PDN with GTP on S2a and a Handover to TWAN with GTP on S2a. For PMIP-based S5/S8, this IE shall also be included on the S4/S11 interfaces for TAU/RAU/Handover cases involving SGW relocation. The PDN type field in the PAA shall be set to IPv4, or IPv6 or IPv4v6, Non-IP or Ethernet by MME/SGSN, based on the UE request and the subscription record retrieved from the HSS (see clause 8.12 and also NOTE 5. See also NOTE 24). The TWAN shall set the PDN type field in the PAA to IPv4, or IPv6 or IPv4v6 based on the UE request if single-connection mode or multi-connection mode is used (see 3GPP TS 23.402 [45]), the IP versions the TWAN supports and the PDN type received in the user subscription data from the HSS/3GPP AAA Server, or based on the UE request and the TWAN Emergency Configuration Data for an Initial Attach in WLAN for Emergency Service on GTP S2a. The ePDG shall set the PDN type field in the PAA to IPv4, or IPv6 or IPv4v6 based on the UE request and the subscription record retrieved from the HSS/3GPP AAA Server, or based on the UE request and the ePDG Emergency Configuration Data for an Initial Attach for emergency session (GTP on S2b). For static IP address assignment (for MME see 3GPP TS 23.401 [3], clause 5.3.1.1, for SGSN see 3GPP TS 23.060 [35], clause 9.2.1, for ePDG see 3GPP TS 23.402 [45] clause 4.7.3, and for TWAN see 3GPP TS 23.402 [45] clause 16.1.5), the MME/SGSN/ePDG/TWAN shall set the IPv4 address and/or IPv6 prefix length and IPv6 prefix and Interface Identifier based on the subscribed values received from HSS, if available. For PDN Type IPv4v6, either one of the IP versions (i.e. IPv4 address or IPv6 prefix and Interface Identifier) or both the IP versions may be statically provisioned in the HSS. If only one of the IP versions is statically provisioned in the HSS, the MME/SGSN/ePDG/TWAN shall set the other IP version as all zeros. The value of PDN Type field shall be consistent with the value of the PDN Type IE, if present in this message. For a Handover to Untrusted Non-3GPP IP Access with GTP on S2b, the ePDG shall set the IPv4 address and/or IPv6 prefix length and IPv6 prefix and Interface Identifier based on the IP address(es) received from the UE. For IP PDN connections, if static IP address assignment is not used (e.g. static address is not received from the HSS), and for scenarios other than a Handover to Untrusted Non-3GPP IP Access with GTP on S2b, the IPv4 address shall be set to 0.0.0.0, and/or the IPv6 Prefix Length and IPv6 prefix and Interface Identifier shall all be set to zero. For Non-IP or Ethernet PDN connections, the PDN Address and Prefix field shall not be present. See NOTE 14 and 25. | PAA | 0 |
|  | CO | This IE shall be sent by the MME/SGSN on S11/S4 interface during TAU/RAU/HO with SGW relocation. |  |  |
|  | CO | During a restoration of a PDN connection after an PGW-C/SMF change procedure, the MME shall include this IE on the S11 interface if a new SGW is selected. When present, it shall be set to the IP address that was assigned during the PDN connection creation. |  |  |
| Maximum APN Restriction | C | This IE shall be included on the S4/S11 and S5/S8 interfaces in the E-UTRAN initial attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, PDP Context Activation, a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN and UE Requested PDN connectivity procedures. This IE denotes the most stringent restriction as required by any already active bearer context. If there are no already active bearer contexts, this value is set to the least restrictive type. | APN Restriction | 0 |
| Aggregate Maximum Bit Rate (APN-AMBR) | C | This IE represents the APN-AMBR. It shall be included on the S4/S11, S5/S8 and S2a/S2b interfaces for an E-UTRAN initial attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, UE requested PDN connectivity, PDP Context Activation procedure using S4, a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN, TAU/RAU/Handover from the Gn/Gp SGSN to the S4 SGSN/MME procedures, Attach with GTP on S2b, a Handover to Untrusted Non-3GPP IP Access with GTP on S2b, UE initiated Connectivity to Additional PDN with GTP on S2b, an Initial Attach for emergency session (GTP on S2b), Initial Attach in WLAN on GTP S2a, an Initial Attach in WLAN for Emergency Service on GTP S2a, a Handover to TWAN with GTP on S2a and UE initiated Connectivity to Additional PDN with GTP on S2a. | AMBR | 0 |
| Linked EPS Bearer ID | C | This IE shall be included on: - S4/S11 in RAU/TAU/HO except in the Gn/Gp SGSN to MME/S4-SGSN RAU/TAU/HO procedures with SGW change; - S11/S5/S8 or S2b during a restoration of a PDN connection after an PGW-C/SMF change procedureas specified in clause 31 of 3GPP TS 23.007 [17]. When present, the Linked EPS bearer ID shall identify the default bearer of the PDN Connection. | EBI | 0 |
| Trusted WLAN Mode Indication | CO | The TWAN shall include this IE on S2a interface (during initial attach, handover to TWAN with GTP on S2a procedure, UE-initiated additional PDN connectivity procedures), if the single-connection mode or multiple-connection mode is used. The TWAN shall not include this IE if transparent single-connection mode is used. The PGW shall assume that transparent single-connection mode is used if it receives this message without this IE from the TWAN. | TWMI | 0 |
| Protocol Configuration Options (PCO) | C | If MME/SGSN receives PCO from the UE during the Attach, PDN connectivity or Handover to 3GPP access procedures, the MME/SGSN shall forward the PCO IE to SGW. The SGW shall also forward it to PGW. (NOTE 29) | PCO | 0 |
|  | CO | If the TWAN receives a PCO from the UE during an initial attach, handover to TWAN or UE-initiated additional PDN connectivity with GTP on S2a procedures (in multi-connection mode or single connection mode), the TWAN shall forward the PCO IE to the PGW, see 3GPP TS 23.402 [45]. |  |  |
| Bearer Contexts to be created | M | Several IEs with the same type and instance value shall be included on the S4/S11 and S5/S8 interfaces as necessary to represent a list of Bearers. One single IE shall be included on the S2a/S2b interface, except during an ePDG triggered PDN connection restoration where one or more bearers shall be included (for the default bearer and dedicated bearers if any) as specified in clause 31.3A of 3GPP TS 23.007 [17]. One bearer shall be included for E-UTRAN Initial Attach, PDP Context Activation, UE requested PDN Connectivity, Attach with GTP on S2b, UE initiated Connectivity to Additional PDN with GTP on S2b, Handovers between Untrusted Non-3GPP IP Access with GTP on S2b and 3GPP Access, Initial Attach for emergency session (GTP on S2b), Initial Attach in WLAN on GTP S2a, an Initial Attach in WLAN for Emergency Service on GTP S2a, Handovers between TWAN with GTP on S2a and 3GPP Access and UE initiated Connectivity to Additional PDN with GTP on S2a. One or more bearers shall be included for a Handover/TAU/RAU with an SGW change, and for an MME triggered PDN connection restoration as specified in clause 31.3 of 3GPP TS 23.007 [17]. See NOTE 6 and NOTE 7. | Bearer Context | 0 |
| Bearer Contexts to be removed | C | This IE shall be included on the S4/S11 interfaces for the TAU/RAU/Handover cases where any of the bearers existing before the TAU/RAU/Handover procedure will be deactivated as consequence of the TAU/RAU/Handover procedure. For each of those bearers, an IE with the same type and instance value shall be included. See NOTE 6 and NOTE 7. | Bearer Context | 1 |
| Trace Information | C | This IE shall be included on the S4/S11 interface if an SGW trace is activated, and/or on the S5/S8 and S2a/2b interfaces if a PGW trace is activated. See 3GPP TS 32.422 [18]. | Trace Information | 0 |
| Recovery | C | This IE shall be included on the S4/S11, S5/S8 and S S2a/2b interfaces if contacting the peer node for the first time. | Recovery | 0 |
| MME-FQ-CSID | C | This IE shall be included by the MME on the S11 interface and shall be forwarded by an SGW on the S5/S8 interfaces according to the requirements in 3GPP TS 23.007 [17]. | FQ-CSID | 0 |
| SGW-FQ-CSID | C | This IE shall be included by the SGW on the S5/S8 interfaces according to the requirements in 3GPP TS 23.007 [17]. | FQ-CSID | 1 |
| ePDG-FQ-CSID | C | This IE shall be included by the ePDG on the S2b interface according to the requirements in 3GPP TS 23.007 [17]. | FQ-CSID | 2 |
| TWAN-FQ-CSID | C | This IE shall be included by the TWAN on the S2a interface according to the requirements in 3GPP TS 23.007 [17]. | FQ-CSID | 3 |
| UE Time Zone | CO | This IE shall be included by the MME over S11 during Initial Attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN and UE Requested PDN Connectivity procedure. This IE shall be included by the SGSN over S4 during PDP Context Activation procedure and a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN. This IE shall be included by the MME/SGSN over S11/S4 TAU/RAU/Handover with SGW relocation. | UE Time Zone | 0 |
|  | CO | During a PDN connection restoration after an PGW-C/SMF change procedure, the MME shall include this IE on the S11 interface if a new SGW is selected, otherwise the MME may include this IE. When present, it shall be set to the last known UE Time Zone (which may differ from the last UE Time Zone information signaled to the PGW-C/SMF before the restoration of the PDN connection). |  |  |
|  | C | If SGW receives this IE, SGW shall forward it to PGW across S5/S8 interface. |  |  |
|  | O | This IE shall be included by the TWAN on the S2a interface for Initial Attach in WLAN procedure, UE-initiated Connectivity to Additional PDN with GTP on S2a and handover to TWAN with GTP on S2a procedure as specified in 3GPP TS 23.402 [45]. |  |  |
| User CSG Information (UCI) | CO | This IE shall be included on the S4/S11 interface for E-UTRAN Initial Attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, UE-requested PDN Connectivity, PDP Context Activation and a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN using S4 procedures, if the UE is accessed via CSG cell or hybrid cell. The MME/SGSN shall also include it for TAU/RAU/Handover procedures with SGW relocation if the UE is accessed via a CSG cell or hybrid cell or leaves a CSG or hybrid cell and the PGW/PCRF has requested CSG info reporting and MME/SGSN support CSG info reporting. NOTE 11. The SGW shall include this IE on S5/S8 if it receives the User CSG information from MME/SGSN. See NOTE 10. | UCI | 0 |
|  | CO | During a PDN connection restoration after an PGW-C/SMF change procedure, the MME shall include this IE on the S11 interface if a new SGW is selected and if the UE is accessed via CSG cell or hybrid cell, otherwise the MME may include this IE. When present, it shall be set to the last known UCI (which may differ from the last UCI information signaled to the PGW-C/SMF before the restoration of the PDN connection). |  |  |
| Charging Characteristics | C | This IE shall be included on the S4/S11, S5/S8 and S2a/S2b interfaces according to 3GPP TS 32.251 [8] | Charging Characteristics | 0 |
| MME/S4-SGSN LDN | O | This IE may be sent by the MME to the SGW on the S11 interface and by the S4-SGSN to the SGW on the S4 interface (see 3GPP TS 32.423 [44]), when communicating the LDN to the peer node for the first time. | Local Distinguished Name (LDN) | 0 |
| SGW LDN | O | This IE may be sent by the SGW to the PGW on the S5/S8 interfaces (see 3GPP TS 32.423 [44]), when communicating the LDN to the peer node for the first time. | Local Distinguished Name (LDN) | 1 |
| ePDG LDN | O | This IE may be sent by the ePDG to the PGW on the S2b interfaces (see 3GPP TS 32.423 [44]), when contacting the peer node for the first time. | Local Distinguished Name (LDN) | 2 |
| TWAN LDN | O | This IE may be sent by the TWAN to the PGW on the S2a interfaces (see 3GPP TS 32.423 [44]), when contacting the peer node for the first time. | Local Distinguished Name (LDN) | 3 |
| Signalling Priority Indication | CO | The SGSN/MME shall include this IE on the S4/S11 interface if the UE indicates low access priority when requesting to establish the PDN connection. The SGW shall forward this IE in the Create Session Request message on the S5/S8 interfaces if received from the MME/SGSN. | Signalling Priority Indication | 0 |
| UE Local IP Address | CO | The ePDG shall include this IE on the S2b interface during an Initial Attach for emergency session (GTP on S2b). Otherwise the ePDG shall include this IE on the S2b interface based on local policy. | IP Address | 0 |
|  | O | This IE may be included by the ePDG on the S2b interface, during the restoration of a PDN connection after a PGW-C/SMF change. When present, it shall be set to the last known UE Local IP Address (which may differ from the last UE Local IP address information signaled to the PGW-C/SMF before the restoration of the PDN connection). |  |  |
| UE UDP Port | CO | The ePDG shall include this IE on the S2b interface if NAT is detected, the UDP encapsulation is used and the UE Local IP Address is present. | Port Number | 0 |
|  | O | This IE may be included by the ePDG on the S2b interface, during the restoration of a PDN connection after a PGW-C/SMF change. When present, this IE shall be set to the last known UE UDP Port (which may differ from the last UE UDP Port information signaled to the PGW-C/SMF before the restoration of the PDN connection). |  |  |
| Additional Protocol Configuration Options (APCO) (NOTE 29) | CO | If multiple authentications are supported by the ePDG, the ePDG shall include this IE on the S2b interface and perform the corresponding procedures as specified for PAP and CHAP authentication of the UE with external networks in 3GPP TS 33.402 [50]. | Additional Protocol Configuration Options (APCO) | 0 |
|  | O | If the UE requests the DNS IPv4/IPv6 address in the Configuration Payload (CFG_REQ) during the IPsec tunnel establishment procedure (as specified in 3GPP TS 33.402 [50]), and if the ePDG supports the Additional Protocol Configuration Options IE, the ePDG may include this IE over S2b interface and correspondingly set the "DNS Server IPv4/v6 Address Request" parameter as defined in 3GPP TS 24.008 [5]. |  |  |
|  | CO | If the UE includes the P-CSCF_IP6_ADDRESS attribute, or the P-CSCF_IP4_ADDRESS attribute or both in the CFG_REQUEST configuration payload during the IPsec tunnel establishment procedure as specified in 3GPP TS 24.302 [63]), and if the ePDG supports these IKEv2 attributes and the Additional Protocol Configuration Options IE, the ePDG shall include this IE over the S2b interface and correspondingly set the P-CSCF IPv6 Address Request, or P-CSCF IPv4 Address Request, or both parameters as defined in 3GPP TS 24.008 [5]. |  |  |
|  | CO | If the UE includes the P-CSCF_RESELECTION_SUPPORT Private Status Type in a Notify payload within the IKE_AUTH request message during the IPsec tunnel establishment procedure asspecified in 3GPP TS 24.302 [63], and if the ePDG supports the P-CSCF restoration extension procedure for the untrusted WLAN access (see 3GPP TS 23.380 [61]), the ePDG shall include this IE over the S2b interface and correspondingly set the P-CSCF_RESELECTION_SUPPORT, as defined in 3GPP TS 24.008 [5]. |  |  |
|  | O | The TWAN may include this IE on the S2a interface to retrieve additional IP configuration parameters from the PGW (e.g. DNS server) if the transparent single-connection mode is used as specified in 3GPP TS 23.402 [45]. |  |  |
|  | CO | The ePDG shall include this IE over the S2b interface and correspondingly set the PDU session ID, as defined in 3GPP TS 24.008 [5] if: - the UE includes the N1_MODE_CAPABILITY Notify payload within the IKE_AUTH request message during the IPsec tunnel establishment procedure as specified in 3GPP TS 24.302 [63]; - the ePDG supports this IKEv2 attribute and the Additional Protocol Configuration Options IE; and - the PDN connection is not restricted from interworking with 5GS by user subscription (see "5GC" bit within Core-Network-Restrictions AVP and Interworking-5GS-Indicator AVP specified in 3GPP TS 29.273 [68]). |  |  |
|  | CO | The ePDG shall include this IE over S2b interface and correspondingly set the "ATSSS request" parameter as defined in 3GPP TS 24.008 [5] if: - the UE includes the ATSSS_REQUEST Notify payload within the IKE_AUTH request message during UE establishing a PDN connection over untrusted non-3GPP access network as a user-plane resource of an MA PDU session as specified in clause 5.3a of 3GPP TS 24.193 [93]; and - the ePDG supports this IKEv2 attribute and the Additional Protocol Configuration Options IE. |  |  |
| H(e)NB Local IP Address | CO | The MME/SGSN shall include this IE on S11/S4 interface if the MME/SGSN receives this information from H(e)NB in UE associated S1/Iu signalling according (see 3GPP TS 23.139 [51]) during: - E-UTRAN Initial Attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, UE-requested PDN Connectivity, PDP Context Activation and a a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN using S4; - TAU/RAU/X2-based handover/Enhanced Serving RNS Relocation Procedure with SGW change, if the PGW/PCRF has requested H(e)NB information reporting for the PDN connection. The SGW shall forward this IE on S5/S8 interface if the SGW receives it from the MME/SGSN. | IP Address | 1 |
|  | O | During a PDN connection restoration after an PGW-C/SMF change procedure, the MME may include this IE on the S11 interface if the MME receives this information from H(e)NB in UE associated S1 signalling. |  |  |
| H(e)NB UDP Port | CO | The MME/SGSN shall include this IE on S11/S4 interface if the MME/SGSN receives this information from H(e)NB in UE associated S1/Iu signalling according (see 3GPP TS 23.139 [51]) during: - E-UTRAN Initial Attach, a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, UE-requested PDN Connectivity, PDP Context Activation and a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN using S4; - TAU/RAU/X2-based handover/Enhanced Serving RNS Relocation Procedure with SGW relocation, if the PGW/PCRF has requested H(e)NB information reporting for the PDN connection. The SGW shall forward this IE on S5/S8 interface if the SGW receives it from the MME/SGSN. | Port Number | 1 |
|  | O | During a PDN connection restoration after an PGW-C/SMF change procedure, the MME may include this IE on the S11 interface if the MME receives this information from H(e)NB in UE associated S1 signalling. |  |  |
| MME/S4-SGSN Identifier | CO | If the PGW triggered SGW restoration procedure is supported, the MME/S4-SGSN shall include this IE on S11/S4 interface and the SGW shall forward this IE on S5 interface in the existing signalling as specified in 3GPP TS 23.007 [17]. If the overload control feature is supported by the MME/S4-SGSN and is activated for the PLMN to which the PGW belongs (see clause 12.3.11), the MME/S4-SGSN shall include this IE on the S11/S4 interface. In that case, the SGW shall forward this IE on the S5/S8 interface. | IP Address | 2 |
| TWAN Identifier | CO | This IE shall be included on the S2a interface for Initial Attach in WLAN procedure, UE-initiated Connectivity to Additional PDN with GTP on S2a and handover to TWAN with GTP on S2a procedure as specified in 3GPP TS 23.402 [45]. | TWAN Identifier | 0 |
| ePDG IP Address | O | This IE may be included on the S2b interface based on local policy for Fixed Broadband access network interworking, see 3GPP TS 23.139 [51]. If present, it shall contain the ePDG IP address which is used as IKEv2 tunnel endpoint with the UE. | IP Address | 3 |
| CN Operator Selection Entity | CO | In shared networks, the SGSN shall include this IE on the S4 interface for a PDP Context Activation, a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN and RAU with SGW relocation procedures, if the information is available, to indicate whether the Serving Network has been selected by the UE or by the network. | CN Operator Selection Entity | 0 |
|  | CO | The SGW shall include this IE on S5/S8 if it receives it from the SGSN. |  |  |
| Presence Reporting Area Information | CO | The MME/SGSN shall include this IE in the following procedures, if the PGW/PCRF/OCS requested reporting changes of UE presence in the Presence Reporting Area(s) and the MME/SGSN supports such reporting: - TAU/RAU/X2 Handover/Enhanced SRNS Relocation procedures with SGW relocation and MME/SGSN change. The new MME/SGSN shall then indicate whether the UE is inside or outside the PRA for each of the active Presence Reporting Area(s), or indicate that the Presence Reporting Area (s) is inactive; - TAU/RAU/X2 Handover/Enhanced SRNS Relocation procedures with SGW relocation and without MME/SGSN change, if the UE enters or leaves the Presence Reporting Area(s). In this case, this IE shall only include the active PRA(s) that the UE has newly entered or left. During a PDN connection restoration after an PGW-C/SMF change procedure, the MME shall include this IE on the S11 interface if a new SGW is selected, otherwise the MME may include this IE. Several IEs with the same type and instance value may be included as necessary to represent a list of Presence Reporting Area Actions. See NOTE 20. | Presence Reporting Area Information | 0 |
| MME/S4-SGSN's Overload Control Information | O | During an overload condition, the MME/S4-SGSN may include this IE on the S11/S4 interface if the overload control feature is supported by the MME/S4-SGSN and is activated for the PLMN to which the PGW belongs (see clause 12.3.11). When present, the MME/S4-SGSN shall provide only one instance of this IE, representing its overload information. | Overload Control Information | 0 |
|  | CO | If the SGW receives this IE and if it supports the overload control feature, it shall forward it to the PGW on the S5/S8 interface. |  |  |
| SGW's Overload Control Information | O | During an overload condition, the SGW may include this IE over the S5/S8 interface if the overload control feature is supported by the SGW and is activated for the PLMN to which the PGW belongs (see clause 12.3.11). When present, the SGW shall provide only one instance of this IE, representing its overload information. | Overload Control Information | 1 |
| TWAN/ePDG's Overload Control Information | O | During an overload condition, the TWAN/ePDG may include this IE over the S2a/S2b interface if the overload control feature is supported by the TWAN/ePDG and is activated for the PLMN to which the PGW belongs (see clause 12.3.11). When present, the TWAN/ePDG shall provide only one instance of this IE, representing its overload information. | Overload Control Information | 2 |
| Origination Time Stamp | CO | The MME/SGSN and the TWAN/ePDG shall include this IE on the S11/S4 and S2a/S2b interface respectively, in the conditions specified in clause 13.2. When present, the Origination Time Stamp shall contain the UTC time when the originating entity initiated the request. | Millisecond Time Stamp | 0 |
|  | CO | The SGW shall include this IE on the S5/S8 interface if it receives the Origination Time Stamp from the MME/SGSN and if it supports the procedure specified in clause 13.2. |  |  |
| Maximum Wait Time | CO | The MME/SGSN and the TWAN/ePDG shall include this IE on the S11/S4 and S2a/S2b interface respectively, in the conditions specified in clause 13.3. When present, the Maximum Wait Time shall contain the duration (number of milliseconds since the Origination Time Stamp) during which the originator of the request waits for a response. | Integer Number | 0 |
|  | CO | The SGW shall include this IE on the S5/S8 interface if it receives the Maximum Wait Time from the MME/SGSN and if it supports the procedure specified in clause 13.3. |  |  |
| WLAN Location Information | CO | This IE shall be included on the S2b interface if the WLAN Location Information is available. | TWAN Identifier | 1 |
|  | O | This IE may be included by the ePDG on the S2b interface, during the restoration of a PDN connection after a PGW-C/SMF change. When present, this IE shall be set to the last known WLAN Location Information (which may differ from the last WLAN Location Information signaled to the PGW-C/SMF before the restoration of the PDN connection). |  |  |
| WLAN Location Timestamp | CO | This IE shall be included on the S2b interface, if the WLAN Location Timestamp is available. | TWAN Identifier Timestamp | 0 |
|  | O | This IE may be included by the ePDG on the S2b interface, during the restoration of a PDN connection after a PGW-C/SMF change. When present, this IE shall be set to the last known WLAN Location Timestamp (which may differ from the last WLAN Location Timestamp signaled to the PGW-C/SMF before the restoration of the PDN connection). |  |  |
| NBIFOM Container | CO | This IE shall be included on the S11/S4 or S2a/S2b interfaces if the MME/S4-SGSN or the TWAN/ePDG receives an NBIFOM Container from the UE as specified in TS 24.161 73]. The Container Type shall be set to 4. | F-Container | 0 |
|  | CO | If the SGW receives an NBIFOM Container from the MME/S4-SGSN, the SGW shall forward this IE to the PGW on the S5/S8 interface. |  |  |
| Remote UE Context Connected | CO | The MME shall include this IE on the S11 interface during a SGW relocation procedure if such information is available. Several IEs with the same type and instance value may be included as necessary to represent a list of remote UEs connected. | Remote UE Context | 0 |
| 3GPP AAA Server Identifier | O | The ePDG/TWAN may include this IE on the S2a/S2b interface to provide the selected 3GPP AAA server identifier to the PGW. See NOTE 13. | Node Identifier | 0 |
| Extended Protocol Configuration Options (ePCO) | CO | If the MME receives ePCO from the UE during the Initial Attach, UE requested PDN Connectivity procedures, the MME shall forward the ePCO IE to the SGW if the MME supports ePCO. (NOTE 29) The SGW shall also forward it to the PGW if the SGW supports ePCO. See NOTE 15. | ePCO | 0 |
| Serving PLMN Rate Control | CO | The MME shall include this IE on the S11 interface if Serving PLMN Rate control is configured by the MME operator and the PDN Connection is set to Control Plane Only: - during an Initial Attach, or a UE Requested PDN Connectivity procedure; - during an inter MME TAU with SGW relocation procedure; - during a PDN connection restoration after an PGW-C/SMF change procedure, if a new SGW is selected. See NOTE 18. The SGW shall include this IE on S5/S8 if it receives this IE from MME. | Serving PLMN Rate Control | 0 |
| MO Exception Data Counter | CO | The MME shall include the counter if it has received the counter for RRC cause "MO Exception data" in the Context Response message during a TAU with an MME and SGW change. | Counter | 0 |
| UE TCP Port | CO | The ePDG shall include this IE on the S2b interface if NAT is detected, the TCP encapsulation is used and the UE Local IP Address is present. | Port Number | 2 |
|  | O | This IE may be included by the ePDG on the S2b interface, during the restoration of a PDN connection after a PGW-C/SMF change. When present, this IE shall be set to the last known UE TCP Port (which may differ from the last UE TCP Port information signaled to the PGW-C/SMF before the restoration of the PDN connection). |  |  |
| Mapped UE Usage Type | CO | The MME/SGSN shall include this IE on the S11/S4 interface, if available. When present, this IE shall contain the mapped UE usage type applicable to the PDN connection. See NOTE 21. | Mapped UE Usage Type | 0 |
|  | CO | The SGW shall include this IE on S5 if it receives it from the MME/SGSN. |  |  |
| User Location Information for SGW | CO | The MME/SGSN shall include this IE on the S11/S4 interface, based on operator policy for the User Location Information to be sent to the SGW, if the user location information to be passed to the SGW is not already reported in the ULI IE in this message. When present, this IE shall include the ECGI, TAI, eNodeB ID, RAI and/or RNC-ID, based on local policy. See NOTE 21. | ULI | 1 |
| SGW-U node name | CO | The SGW-C shall include this IE on the S5 interface, if available. See NOTE 21. | FQDN | 0 |
| Secondary RAT Usage Data Report | CO | If the PLMN has configured secondary RAT usage reporting and PDN GW Secondary RAT reporting is active, the MME shall include this IE on the S11 interface if it has received Secondary RAT usage data from eNodeB in an X2-based handover with Serving GW relocation. The MME shall also set the IRSGW flag to "0", to indicate that the Secondary RAT usage data is reported for the Source SGW, and sent via the Target SGW to the PGW. Several IEs with the same type and instance value may be included, to represent multiple usage data reports. | Secondary RAT Usage Data Report | 0 |
| UP Function Selection Indication Flags | CO | Based on operator policy, the MME/S4-SGSN shall include this IE on the S4/S11 interface, if any of the applicable flags is set to 1. Applicable flags are: - DCNR: This flag shall be set to 1 if it is desired to select a specific SGW-U and PGW-U for UEs supporting Dual Connectivity with NR and not restricted from using NR by user subscription, e.g. due to requirements of higher bitrates. See NOTE 21. | UP Function Selection Indication Flags | 0 |
|  | CO | The SGW shall include this IE on S5/S8 if it receives it from the MME/S4-SGSN. |  |  |
| APN RATE Control Status | CO | If APN RATE Control Status is available in MME/S4-SGSN, APN RATE Control Status shall be transfered on the S4/S11 interface. The SGW shall include the APN RATE Control Status IE on the S5/S8 interface, if received from the MME/S4-SGSN. | APN RATE Control Status | 0 |
| PSCell ID | CO | The MME shall include this IE on the S11 interface, if it has received this information from the eNodeB. | PSCell ID | 0 |
| Trace Collection Entity URI | O | This IE may be present on the S4/S11 interface if an SGW trace is activated, and/or on the S5/S8 and S2a/2b interfaces if a PGW trace is activated and if the Trace Information IE is included. When present, this IE shall include the Trace Reporting Consumer URI for Streaming based Reporting as defined in 3GPP TS 32.422 [18]. If both the IP Address of the Trace Collection Entity and Trace Collection Entity URI are present, the Trace Collection Entity URI shall take the higher precedence. | URI | 0 |
| Private Extension | O | This IE may be sent on the S5/S8, S4/S11 and S2a/S2b interfaces. | Private Extension | VS |
| NOTE 1: The conditional PDN Type IE is redundant on the S4/S11 and S5/S8 interfaces (as the PAA IE contains exactly the same field). The receiver may ignore it. This IE is never sent on the S2a/S2b interface. NOTE 2: 3GPP TS 23.401 [3] (e.g. clause 5.3.2.1) and 3GPP TS 23.060 [35] (e.g. clause 9.2.2.1) defines the MME/SGSN shall send the MS Info Change Reporting Support Indication to the PGW. In such case MME/SGSN shall use the Change Reporting Support Indication and/or CSG Change Reporting Support Indication (whichever is applicable), even if stage 2 refers to MS Info Change Reporting Support Indication. NOTE 3: The methods that the ePDG may use to acquire the RAT type of the untrusted non-3GPP IP access network are not specified in this release. NOTE 4: The PDN-GW can be informed about the type of access network used by the UE over several reference points, see 3GPP TS 29.212 [30] for the mapping between the code values for the different access network types. NOTE 5: 3GPP TS 23.401 [3] (see clause 5.3.1.1) and 3GPP TS 23.060 [35] (see clause 9.2.1) specify the handling of the cases when UE has requested IPv4v6 PDN Type, but MME does not set the Dual Address Bearer Flag due to the MME operator using single addressing per bearer to support interworking with nodes of earlier releases. NOTE 6: The Bearer Context to be created IE and Bearer Context to be removed IE, together, shall contain all the bearers belonging to the given PDN connection with each bearer appearing in only one of these IEs. NOTE 7: During S1 based handover/ Inter RAT handover/TAU/RAU with S4-SGSN/MME and SGW change, and handover/RAU/TAU from Gn/Gp SGSN to S4-SGSN/MME, if the target MME/S4-SGSN cannot accept one or more PDN connection(s) but can accept at least one or more remaining PDN Connection(s) of the UE, the target MME/SGSN shall indicate all the non GBR bearers of the unaccepted PDN Connection in the Bearer Contexts to be created IE. The (target) MME/SGSN shall indicate all the GBR bearers of the unaccepted PDN connection in the Bearer Contexts to be removed IE. NOTE 8: The conditions of presence of the IEs in the Create Session Request for the MME and S4-SGSN triggered Serving GW relocation (see clause 5.10.4 of 3GPP TS 23.401 [3] and clause 9.2.2.4 of 3GPP TS 23.060 [35]) are identical to those specified respectively for X2 handover with SGW relocation and for Enhanced Serving RNS Relocation with SGW relocation. NOTE 9: During the TAU/RAU/Handover from Gn/Gp SGSN, the target MME/S4-SGSN cannot derive the level of support for User Location Change Reporting and/or CSG Information Change Reporting at the source Gn/Gp SGSN. NOTE 10: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator for supporting UEs, or that of the allocated Core Network Operator for non-supporting UEs. As an exception, based on inter-operator roaming/sharing agreement, if the information on whether the UE is a supporting or non-supporting UE is available, the PLMN ID that is communicated to the HPLMN for non-supporting UEs shall be the Common PLMN ID. In shared networks, when the MME/S4-SGSN and PGW pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI to the PGW, and the Common PLMN ID shall be communicated in SAI/CGI to the PGW, for both supporting and non-supporting UEs. The Core Network Operator PLMN ID (selected by the UE for supporting UEs or allocated by the network for non-supporting UEs) shall be communicated in the TAI, RAI, UCI and the Serving Network. See clause 4.4 of 3GPP TS 23.251 [55]. NOTE 11: If the UE initiates a TAU or RAU procedure back to the old MME/old S4 SGSN before completing the ongoing TAU or RAU procedure and the UE is not accessed via a CSG cell or hybrid cell, the old MME/old S4-SGSN shall treat this case as the UE leaves a CSG or hybrid cell. . NOTE 12: Void NOTE 13: If supported, the PGW shall contact the 3GPP AAA server (identified by this IE which carries the Origin-Host and Origin-Realm included in the DEA message received by the ePDG/TWAN over SWm or STa interface) for establishing the S6b session. NOTE 14: Before contacting an EPC entity, e.g. to send a Create Session Request message, the MME/SGSN shall ensure, during the selection procedure, that the receiving entities support Non-IP PDN type, as specified in clause 5.9 of 3GPP TS 29.303 [32], e.g. using the Notification of Supported features procedure to learn if the candidate SGW supports the CIOT feature. See also the clause 8.83. NOTE 15: An MME, SGW and PGW which supports NB-IoT and/or Non-IP or Ethernet PDN type and/or inter-system change with 5GS and/or UAS services shall support ePCO. A UE supporting NB-IoT access and/or Non-IP or Ethernet PDN type and/or N1 mode and/or UAS services also support ePCO. NOTE 16: All the UE's SGi PDN Connections shall either have the Control Plane Only PDN Connection Indication set or not set. NOTE 17: If the APN was authorized based on the wildcard APN, the Selection Mode Value shall be set to indicate that the subscription is not verified, see Annex A of 3GPP TS 23.060 [35]. NOTE 18: The MME can set the Control Plane Only Indication only during a PDN connection creation procedure, and the Serving PLMN Rate Control is only applicable to the PDN connection with Control Plane Only Indication set. During an inter MME with SGW relocation procedure, when the source MME has not set the Control Plane Only Indication, and the target MME supports only the Control Plane CIoT Optimizations, then the target MME shall not include the Serving PLMN Rate Control IE as the PDN connection cannot be changed to Control Plane Only. During an inter MME with SGW relocation procedure, when the source MME has set the Control Plane Only Indication and included Serving PLMN rate control IE in the Context Response message, and the target MME supports both the Control Plane CIoT Optimisation and the establishment of the User Plane, the target MME cannot stop the Serving PLMN Rate Control, i.e. the PGW will continue to enforce Serving PLMN Rate Control as the Control Plane Only Indication for this PDN connection cannot be changed during this mobility procedure. NOTE 19: An MME which supports eNB Change Reporting shall also support Change reporting and therefore shall set both the Change Report Supporting indication and the eNB Changing Reporting Support Indication. NOTE 20: Upon inter MME/SGSN mobility, the target MME/SGSN shall report Presence Reporting Area Information for all the active and inactive PRAs requested by the PGW. Upon intra MME/SGSN mobility with SGW relocation, the MME/SGSN shall only report active PRAs whose Presence Reporting Area Information is changed, e.g. from inside to outside, or vice versa. NOTE 21: This information is used for the SGW-U, PGW-U or combined SGW-U/PGW-U selection (see Annex B.2 of 3GPP TS 29.244 [80]). NOTE 22: An MME shall send the LTE-M RAT type to the SGW only if the latter is known to support it. The forwarding of the LTE-M RAT type to the PGW is controlled by the LTE-M RAT Type reporting to PGW Indication. NOTE 23: It is assumed that the N26 interface is supported homogeneously across a PLMN. NOTE 24: PDN connections of PDN Type "Ethernet" are not supported in GERAN/UTRAN. For PDN connections of PDN type "Ethernet", mobility to GERAN/UTRAN or Non 3GPP access from E-UTRAN is not supported. See clause 4.3.17.8a of TS 23.401 [3]. NOTE 25: Before contacting an EPC entity, e.g. to send a Create Session Request message, the MME shall ensure, during the selection procedure, that the receiving entities support Ethernet PDN type, as specified in clause 5.13 of 3GPP TS 29.303 [32], e.g. using the Notification of Supported features procedure to learn if the candidate SGW supports the ETH feature. See also the clause 8.83. NOTE 26: The MME should select an SGW supporting MT-EDT if MT-EDT is applicable for the PDN connection. NOTE 27: If the PGW-C/SMF receives the PDU session ID from the UE in the PCO, ePCO or APCO IE, the 5GCNRI flag is set to 1, and the PGW-C/SMF allows to transfer the PDN connection from EPC to 5GC, the PGW-C/SMF shall return the 5GS parameters as specified in 3GPP TS 23.501 [82] and 3GPP TS 23.502 [83] (e.g. mapped QoS parameters and S-NSSAI for interworking with N26 in the PCO or ePCO, S-NSSAI for interworking without N26 in the PCO or ePCO, or S-NSSAI for interworking between ePDG connected to EPC and 5GS in the APCO) in the Create Session Response message, regardless of the 5GSIWKI flag. The S-NSSAI included in either PCO, ePCO or APCO contains the PLMN ID (of the PGW-C/SMF) that the S-NSSAI relates to as specified in clause 10.5.6.3 of 3GPP TS 24.008 [5]. NOTE 28: If the LTE-M Satellite related RAT type is received from the MME, the SGW shall send the LTE-M RAT type to the PGW if the 'Satellite RAT Type reporting to PGW Indication' is not received and the 'LTE-M RAT Type reporting to PGW Indication' is received, or the SGW shall send the WB-E-UTRAN RAT type to the PGW if both of the indications are absent. NOTE 29: The MME and ePDG does not include the PCO, ePCO or APCO IE during the restoration of a PDN connection after a PGW-C/SMF change procedure. In other words, the PCO, ePCO or APCO information that was signaled before the restoration procedure is not signaled again during the restoration procedure and the PGW-C/SMF shall consider that any PCO, ePCO or APCO IE received before the restoration procedure still apply (e.g. if 5G related information had been exchanged before the restoration to support the ability to move the PDN connection later on to the 5GC). |  |  |  |  |

Table 7.2.1-2: Bearer Context to be created within Create Session Request

| Octet 1 |  | Bearer Context IE Type = 93 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| EPS Bearer ID | M |  | EBI | 0 |
| TFT | O | This IE may be included on the S4/S11 interfaces. | Bearer TFT | 0 |
| S1-U eNodeB F-TEID | C | This IE shall be included on the S11 interface for X2-based handover with SGW relocation. | F-TEID | 0 |
|  | CO | During a PDN connection restoration after an PGW-C/SMF change procedure, the MME shall include this IE on the S11 interface if the S1-U eNodeB F-TEID for the EPS Bearer Context has been allocated by the eNB and if it is available at the MME. |  |  |
| S4-U SGSN F-TEID | C | This IE shall be included on the S4 interface if the S4-U interface is used. | F-TEID | 1 |
| S5/S8-U SGW F-TEID | C | This IE shall be included on the S5/S8 interface for an "E-UTRAN Initial Attach", a Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN, a "PDP Context Activation", a Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN, a "UE Requested PDN Connectivity" and an MME triggered PDN connection restoration (see clause 31.3 of 3GPP TS 23.007 [17]). | F-TEID | 2 |
| S5/S8-U PGW F-TEID | C | This IE shall be included on the S4 and S11 interfaces for the TAU/RAU/Handover cases when the GTP-based S5/S8 is used. | F-TEID | 3 |
|  |  | This IE shall be included on the S11 interface for an MME triggered PDN connection restoration (see clause 31.3 of 3GPP TS 23.007 [17]). |  |  |
|  | CO | For PMIP-based S5/S8, this IE shall be included on the S11/S4 interface for the TAU/RAU/Handover cases if the PGW provided an alternate address for user plane, i.e. an IP address for user plane which is different from the IP address for control plane. When present, this IE shall contain the alternate IP address for user plane and the uplink GRE key. See NOTE 1. |  |  |
| S12 RNC F-TEID | CO | This IE shall be included on the S4 interface if the S12 interface is used in the Enhanced serving RNS relocation with SGW relocation procedure. | F-TEID | 4 |
| S2b-U ePDG F-TEID | C | This IE shall be included on the S2b interface for an Attach with GTP on S2b, a UE initiated Connectivity to Additional PDN with GTP on S2b, a Handover to Untrusted Non-3GPP IP Access with GTP on S2b, an Initial Attach for emergency session (GTP on S2b) and an ePDG triggered PDN connection restoration (see clause 31.3A of 3GPP TS 23.007 [17]). | F-TEID | 5 |
| S2a-U TWAN F-TEID | C | This IE shall be included on the S2a interface for an Initial Attach in WLAN on GTP S2a, an Initial Attach in WLAN for Emergency Service on GTP S2a, a UE initiated Connectivity to Additional PDN with GTP on S2a and a Handover to TWAN with GTP on S2a. | F-TEID | 6 |
| Bearer Level QoS | M | Bearer Level QoS information. During a PDN connection restoration after an PGW-C/SMF change procedure, this IE shall be set to the Bearer Level QoS that was set by the PGW-C/SMF before the restoration of the PDN connection. | Bearer QoS | 0 |
| S11-U MME F-TEID | CO | This IE shall be sent on the S11 interface, if S11-U is being used, during the E-UTRAN Initial Attach and UE requested PDN connectivity procedures. This IE may also be sent on the S11 interface, if S11-U is being used, during a Tracking Area Update procedure with Serving GW change, if the MME needs to establish the S11-U tunnel. See NOTE 2. | F-TEID | 7 |
| NOTE 1: The capability to receive from the LMA an alternate LMA address for user plane shall be supported homogeneously across all the SGWs, when supported over PMIP-based S5/S8. NOTE 2: Establishing the S11-U tunnel at once during the Create Session Request/Response procedure avoids the need for a subsequent Modify Bearer Request/Response exchange to transfer DL or UL user data. |  |  |  |  |

Table 7.2.1-3: Bearer Context to be removed within Create Session Request

| Octet 1 |  | Bearer Context IE Type = 93 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| EPS Bearer ID | M |  | EBI | 0 |
| S4-U SGSN F-TEID | C | This IE shall be sent on the S4 interface if the S4-U interface is used. See NOTE 1. | F-TEID | 0 |
| NOTE 1: The conditional S4-U SGSN F-TEID IE is redundant. |  |  |  |  |

Table 7.2.1-4: Overload Control Information within Create Session Request

| Octet 1 |  | Overload Control Information IE Type = 180 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| Overload Control Sequence Number | M | See clause 12.3.5.1.2.1 for the description and use of this parameter. | Sequence Number | 0 |
| Overload Reduction Metric | M | See clause 12.3.5.1.2.3 for the description and use of this parameter. | Metric | 0 |
| Period of Validity | M | See clause 12.3.5.1.2.2 for the description and use of this parameter. This IE should be set to "0" if the "Overload Reduction Metric" is null. This IE shall be ignored by the receiver if the "Overload Reduction Metric" is null. | EPC Timer | 0 |

Table 7.2.1-5: Remote UE Context Connected within Create Session Request

| Octets 1 |  | Remote UE Context IE Type = 191 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octets 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| Remote User ID | M | See clause 8.123 for the description and use of this parameter | Remote User ID | 0 |
| Remote UE IP Information | M | See clause 8.124 for the description and use of this parameter | Remote UE IP Information | 0 |

* * * Next Change * * * *

7.3.1	Forward Relocation Request

A Forward Relocation Request message shall be sent from:

-	the source MME to the target MME over the S10 interface as part of an S1-based handover relocation procedure;

-	the source MME to the target SGSN, or from the source SGSN to the target MME over the S3 interface as part of an Inter RAT handover and combined hard handover and SRNS relocation procedures;

-	the source SGSN to the target SGSN over the S16 interface as part of an SRNS Relocation and PS handover procedures;

-	the source MME to the target SGSN over the S3 interface as part of an SRVCC from E-UTRAN to UTRAN or GERAN with DTM HO support procedures and from the source SGSN to the target SGSN over the S16 interface as part of SRVCC from UTRAN (HSPA) to UTRAN or GERAN with DTM HO support;

-	the source MME to the target AMF, or from the source AMF to the target MME over the N26 interface as part of the EPS to 5GS handover and 5GS to EPS handover procedures.

-	the source AMF to the target MME_SRVCC over the N26 interface as part of the 5G-SRVCC from NG-RAN to UTRAN procedure.

A source MME/SGSN which supports CIoT knows whether the target MME/SGSN pool or target AMF (5GS) supports some CIoT optimisations either by using DNS procedures enhanced for DCNs or by local configuration, as specified in clause 5.9 of 3GPP TS 29.303 [32]. A source AMF knows whether the target MME pool supports SGi Non-IP or Ethernet PDN connections either by using DNS procedures enhanced for DCNs or by local configuration, as specified in clause 5.9 and 5.13 of 3GPP TS 29.303 [32]. The target MME/SGSN may forward the Forward Relocation Request to another MME/SGSN in the target MME/SGSN pool which is more suitable to serve the UE, based on the information received in the Forward Relocation Request message, e.g. required CIoT EPS optimisation(s) applicable to the given UE's attachment.

NOTE 1:	The source MME/SGSN/AMF does not need to know each individual CIoT feature the target MME/SGSN pool or target AMF (5GS) supports. The source MME/SGSN/AMF can behave as if the target MME/SGSN/AMF pool supports all CIoT features when the target MME/SGSN pool is known to support CIoT; the source MME/SGSN/AMF determines then which bearer contexts were successfully transferred as specified in sunclause 7.3.2.

NOTE 2:	Among the CIoT optimization features, only the support of SCEF Non-IP PDN connection and the support of SGi Non-IP PDN connection are applicable to a SGSN.

NOTE 3:	5GS supports Attach without PDU session. 5GS can also support Unstructured and Ethernet PDU session types, which are assimilated to "SGi Non-IP PDN connections" over N26.

Forward Relocation procedure across S10 interface (when KASME is taken into use) shall be performed according to the Rules on Concurrent Running of Security Procedures, which are specified in 3GPP TS 33.401 [12].

When the source MME/SGSN supports one or more of the CIoT optimization features as specified in clause 8.125, the source MME/SGSN shall transfer EPS bearer context(s) for SGi Non-IP PDN connections or for PDN connections to a SCEF only if the target serving node is known to support SGi Non-IP PDN connections or SCEF Non-IP PDN connections respectively, as specified in clause 5.5.1.2.1 and 5.5.2 of 3GPP TS 23.401 [3]. Likewise, a source AMF shall transfer EPS bearer context(s) for SGi Non-IP PDN connections only if the target MME is known to support SGi Non-IP PDN connections. The source MME/SGSN/AMF shall not proceed with the Forward Relocation Request procedure if the UE does not have any EPS bearer context(s) for SGi IP or Non-IP PDN connections that can be transferred to the target serving node, i.e. under the following conditions:

-	If the UE is attached to the source MME/SGSN with only the PDN connection(s) of PDN type "non-IP" through the SGW and the PGW, with or without SCEF PDN connections, and the target serving node is known to not support SGi Non-IP PDN Connection (as specified in clause 4.3.17.8.3.3 of 3GPP TS 23.401 [3] and in clause 5.3.13.8 of 3GPP TS 23 060 [35]); or

-	If the UE is attached to the source AMF (5GS) with only PDU session(s) of type "Ethernet" or "Unstructured", and the target MME is known to not support SGi Non-IP and Ethernet PDN Connection.

The source MME shall transfer the EPS bearer context(s) for the Ethernet PDN connection only if the target serving node is known to support the Ethernet PDN connection, as specified in clause 5.3.3.1 of 3GPP TS 23.401 [3]. The source MME shall not proceed with the Forward Relocation Request procedure if the UE does not have any EPS bearer context(s) that can be transferred to the target serving node, i.e. under the following conditions:

-	If the UE is attached to the source MME with only the PDN connection(s) of PDN type "Ethernet" through the SGW and the PGW, and the target serving node is known to not support the Ethernet PDN connection.

Table 7.3.1-1 specifies the presence requirements and conditions of the IEs in the message.

Table 7.3.1-1: Information Elements in a Forward Relocation Request

| Information elements | P | Condition / Comment | IE Type | Ins. |
| --- | --- | --- | --- | --- |
| IMSI | C | The IMSI shall be included in the message, except if the UE is emergency or RLOS attached and the UE is UICCless. The IMSI shall be included in the message but not used as an identifier if UE is emergency or RLOS attached but IMSI is not authenticated. | IMSI | 0 |
| Sender's F-TEID for Control Plane | M |  | F-TEID | 0 |
| MME/SGSN/AMF UE EPS PDN Connections | C | This IE shall be present, except: - over the S16 interface if there is no active PDP context and the source and target SGSNs supports SRNS relocation w/o PDN connection over GTPv2 (see NOTE 2); or - over the N26 interface, during 5G SRVCC handover procedure from 5GS to UTRAN, as specified in clause 6.5 of 3GPP TS 23.216 [43]. If the target MME/SGSN is known to not support SGi Non-IP PDN Connection (as specified in clause 4.3.17.8.3.3 of 3GPP TS 23.401 [3] and in clause 5.3.13.8 of 3GPP TS 23 060 [35]), then the source MME/SGSN/AMF shall only include PDN Connections of IP PDN type. If the target MME is known to not support Ethernet PDN connections, or the target is an SGSN, then the source MME shall not include any Ethernet PDN connection. (See NOTE 10) Several IEs with this type and instance values shall be included as necessary to represent a list of PDN Connections | PDN Connection | 0 |
| SGW S11/S4 IP Address and TEID for Control Plane | C | This IE shall be present, except over the S16 interface if there is no active PDP context and the source and target SGSNs supports SRNS relocation w/o PDN connection over GTPv2 (see NOTE 2). Over the N26 interface, the source AMF shall set the IP address and TEID to the following values: - any reserved TEID (e.g. all 0's, or all 1's); - IPv4 address set to 0.0.0.0, or IPv6 Prefix Length and IPv6 prefix and Interface Identifier all set to zero. | F-TEID | 1 |
| SGW node name | C | This IE identifies the SGW that was used by the old MME/S4-SGSN. It shall be included by the source MME/S4-SGSN, except over the S16 interface if there is no active PDP context and the source and target SGSNs supports SRNS relocation w/o PDN connection over GTPv2 (see NOTE 2). | FQDN | 0 |
| MME/SGSN/AMF UE MM Context | M |  | MM Context | 0 |
| Indication Flags | C | This IE shall be included if any of the flags are set to 1. - Direct Forwarding Indication: This flag shall be set to 1 if direct forwarding is supported in the S1 based handover procedure or during an inter-system handover between 5GS and EPS. This flag shall not be set to 1 if the message is used for other handover procedures. - Idle mode Signalling Reduction Supported Indication flag: This flag shall be set to 1 if the source MME/SGSN and associated SGW are capable to establish ISR for the UE. - Unauthenticated IMSI: This flag shall be set to 1 if the IMSI present in the message is not authenticated and is for an emergency or RLOS attached UE. - Change Reporting support indication flag: This flag shall be set to 1 if the Source S4-SGSN/MME supports Location Change Reporting mechanism. See NOTE1. See NOTE 3. - CSG Change Reporting Support Indication flag: This flag shall be set to 1 if the Source S4-SGSN/MME supports CSG Information Change Reporting mechanism. See NOTE1. See NOTE 3. - Management Based MDT allowed flag: This flag shall be set to 1 for the S1 based inter-MME handover procedure over the S10 interface, if Management Based Minimization of Drive Tests (MDT) is allowed. See 3GPP TS 36.413 [10] and 3GPP TS 32.422 [18]. - CSFB Indication: when configured to support the return to the last used PLMN after CSFB, the MME shall set this flag to 1 on the S3 interface if the PS handover procedure is due to CSFB (see clause 4.3.2 of 3GPP TS 23.272 [21]). See NOTE 5. - Pending MT Short Message Indication: This flag shall be set to 1 on the S10/S16/S3 interface if the source S4-SGSN/MME knows that there is one (or more) pending MT Short Message(s) in the SMS-GMSC for the UE as specified in clause 10.1 of 3GPP TS 23.040 [75], Figure 17c). See NOTE 8. - LTE-M UE Indication (LTEMUI): This flag shall be set to 1 if the MME/AMF knows that the UE is an LTE-M UE. - Return Preferred Indication (REPREFI): the MME or the AMF may set this flag to 1 on the N26 interface to indicate a preferred return of the UE to the last used EPS or 5GS PLMN (respectively) at a later access change to an EPS or 5GS shared network. See NOTE 5. - 5G-SRVCC HO Indication (5SRHOI): the AMF may set this flag to 1 on the N26 interface to indicate the HO is used for 5G-SRVCC as specified in 3GPP TS 23.216 [43]. - Emergency PDU Session Indication (EMCI): the AMF may set this flag to 1 on the N26 interface to indicate that the UE has emergency PDU session, as specified in 3GPP TS 23.126 [43]. The EMCI bit shall be set together with the 5SRHOI bit. - LTE-M Satellite Access Indication (LTEMSAI): This flag shall be set to 1 if the MME/AMF knows that the LTE-M UE is accessing Satellite E-UTRAN. | Indication | 0 |
| E-UTRAN Transparent Container | C | This IE shall be included to contain the "Source to Target Transparent Container", if the message is used for UTRAN/GERAN to E-UTRAN inter RAT handover procedure, E-UTRAN intra RAT handover procedure, 3G SGSN to MME combined hard handover and SRNS relocation procedure, 5GS to EPS handover and EPS to 5GS handover. The Container Type shall be set to 3. | F-Container | 0 |
| UTRAN Transparent Container | C | This IE shall be included to contain the "Source to Target Transparent Container", if the message is used for PS handover to UTRAN Iu mode procedures, SRNS relocation procedure and E-UTRAN to UTRAN inter RAT handover procedure. The Container Type shall be set to 1. | F-Container | 1 |
| BSS Container | C | This IE shall be included to contain the "Source BSS to Target BSS Transparent Container" if the message is used for PS handover to GERAN A/Gb mode and E-UTRAN to GERAN A/Gb mode inter RAT handover procedure. The Container Type shall be set to 2. | F-Container | 2 |
| Target Identification | C | This IE shall be included if the message is used for SRNS relocation procedure, handover to UTRAN/E-UTRAN, 5GS to EPS handover, EPS to 5GS handover procedures and 5G-SRVCC from NG-RAN to UTRAN procedure. | Target Identification | 0 |
|  | CO | This IE shall be included on the S3 interface if the message is used for PS handover from E-UTRAN to GERAN A/Gb mode. |  |  |
| HRPD access node S101 IP address | C | This IE shall be included only if the HRPD pre-registration was performed at the source MME. | IP-Address | 0 |
| 1xIWS S102 IP address | C | This IE shall be included only if the 1xRTT CS fallback pre- registration was performed at the source MME. | IP-Address | 1 |
| S1-AP Cause | C | This IE is the information received from the source eNodeB or derived from the information received from the source NG-RAN. The source MME or AMF shall include this IE in the message. Refer to 3GPP TS 29.010 [42] for the mapping of cause values between S1AP, RANAP, BSSGP and NGAP. | F-Cause | 0 |
| RANAP Cause | C | This IE is the information from the source RNC. The source SGSN shall include this IE in the message. Refer to the 3GPP TS 29.010 [42] for the mapping of cause values between S1AP, RANAP and BSSGP. | F-Cause | 1 |
| BSSGP Cause | C | This IE is the information received from source BSS. The source SGSN shall include this IE in the message. Refer to 3GPP TS 29.010 [42] for the mapping of cause values between S1AP, RANAP and BSSGP. | F-Cause | 2 |
| Source Identification | C | This IE shall be included on the S16 interface if the message is used for PS handover from GERAN/UTRAN to GERAN A/Gb mode. | Source Identification | 0 |
| Selected PLMN ID | C | The old MME/SGSN/AMF shall include this IE if the selected PLMN identity is available. The Selected PLMN ID IE indicates the target core network operator selected for the UE in a shared network. | PLMN ID | 0 |
| Recovery | C | This IE shall be present when contacting the peer for the first time. | Recovery | 0 |
| Trace Information | C | This IE shall be included over S10/S16/S3 when session trace is active for this IMSI/IMEI. | Trace Information | 0 |
| Subscribed RFSP Index | CO | This IE shall be included by the source MME/SGSN/AMF, if received from the HSS or UDM. | RFSP Index | 0 |
| RFSP Index in Use | CO | This IE shall be included by the source MME/SGSN/AMF, if it supports the feature. | RFSP Index | 1 |
| RFSP Index in Use Validity Time | O | This IE may be included by the source MME/AMF during 5GS to EPS mobility and subsequent inter-MME mobility procedures if the validity time has not expired yet. It indicates the validity time of RFSP Index in Use to be used in the target MME as specified in clause 4.11.1.5.8 of 3GPP TS 23.502 [83]. When present, this IE shall be set to the remaining validity time of the RFSP Index in Use. The AMF shall ignore this IE if it is received from the MME. | Timer in Seconds | 0 |
| CSG ID | CO | This IE shall be included on the S3/S10/S16 interfaces if the source MME/SGSN receives it from the source eNodeB/RNC. It indicates the target CSG ID in case of a handover to a CSG cell or hybrid cell. | CSG ID | 0 |
| CSG Membership Indication | CO | This IE shall be included on the S3/S10/S16 interfaces by the source MME/SGSN if the CSG access mode is received from the source eNodeB/RNC and indicates the target cell is a hybrid cell, or if the UE has emergency bearer(s) and the target cell is a CSG cell. | CMI | 0 |
| UE Time Zone | CO | When available, this IE shall be included by the source MME/S4-SGSN/AMF. | UE Time Zone | 0 |
| Serving Network | CO | This IE shall be included to indicate the current Serving Network. | Serving Network | 0 |
| MME/S4-SGSN LDN | O | This IE may be sent by the MME/S4-SGSN to the peer MME/S4-SGSN on the S3/S10/S16 interfaces (see 3GPP TS 32.423 [44]), when communicating the LDN to the peer node for the first time. | Local Distinguished Name (LDN) | 0 |
| Additional MM context for SRVCC | CO | This IE shall be sent by the source MME/S4-SGSN to the target MME/S4-SGSN on the S3/S10/S16 interfaces if MS Classmark2, MS Classmark3 and the Supported Codec are available in the source MME/S4-SGSN, or by the source AMF to the target MME_SRVCC (during a 5G-SRVCC procedure as specified in clause 6.5.4 of 3GPP TS 23.216 [43]) or to the target MME (during a 5G to 4G handover) on the N26 interface if MS Classmark2 and the Supported Codec are available in the source AMF. | Additional MM context for SRVCC | 0 |
| Additional flags for SRVCC | CO | This IE shall be included if any one of the applicable flags needs to be forwarded. Applicable flags: - ICS Indicator: This IE shall be sent by the source MME/S4-SGSN to the target MME/S4-SGSN on the S3/S10/S16 interfaces if ICS Indicator is available in the source MME/S4-SGSN. - vSRVCC flag: This IE shall be sent by the source MME to the target MME on the S10 interface if vSRVCC flag is available in the source MME. | Additional flags for SRVCC | 0 |
| STN-SR | CO | This IE shall be sent by the source MME/S4-SGSN to the target MME/S4-SGSN on the S3/S10/S16 interfaces if STN-SR is available in the source MME/S4-SGSN, or by the source AMF to the target MME_SRVCC during a 5G-SRVCC procedure as specified in clause 6.5.4 of 3GPP TS 23.216 [43]) or to the target MME (during a 5G to 4G handover) on the N26 interface if STN-SR is available in the source AMF. | STN-SR | 0 |
| C-MSISDN | CO | This IE shall be sent by the source MME/S4-SGSN to the target MME/S4-SGSN on the S3/S10/S16 interfaces if C-MSISDN is available in the source MME/S4-SGSN, or by the source AMF to the target MME_SRVCC during a 5G-SRVCC procedure as specified in clause 6.5.4 of 3GPP TS 23.216 [43]) or to the target MME (during a 5G to 4G handover) on the N26 interface if C-MSISDN is available in the source AMF. The C-MSISDN is defined in 3GPP TS 23.003 [2]. | MSISDN | 0 |
| MDT Configuration | CO | This IE shall be sent by the source MME to the target MME on the S10 interface for the S1-based handover relocation procedure, if the Job Type indicates Immediate MDT. See 3GPP TS 32.422 [18] clause 4.4. | MDT Configuration | 0 |
| SGSN node name | CO | This IE shall be sent by the source SGSN on the S3 interface if both source SGSN and associated SGW support ISR. See NOTE 4. | FQDN | 1 |
| MME node name | CO | This IE shall be sent by the source MME on the S3 interface if both source MME and associated SGW support ISR. See NOTE 4. | FQDN | 2 |
| User CSG Information (UCI) | CO | This IE shall be sent by the source MME/S4-SGSN on the S3/S10/S16 interfaces if the source MME/SGSN has reported to the PGW that the UE is in a CSG or hybrid cell. It shall then contain the last User CSG information that the source MME/S4-SGSN has reported to the PGW. The absence of this IE indicates that the UE has not been reported to the PGW as being in a CSG or hybrid cell. See NOTE 6. | UCI | 0 |
| Monitoring Event Information | CO | This IE shall be sent by the source MME/S4-SGSN on the S3/S10/S16 interfaces if monitoring events are to be continued in the target MME/S4-SGSN. More than one IE with this type and instance values may be included to represent multiple monitoring events. | Monitoring Event Information | 0 |
| Monitoring Event Extension Information | CO | This IE shall be sent by the source MME/S4-SGSN on the S3/S10/S16 interfaces if monitoring event extensions are to be sent to the target MME/S4-SGSN. More than one IE with this type and instance values may be included to represent multiple monitoring event extensions. | Monitoring Event Extension Information | 0 |
| UE Usage Type | CO | This IE shall be set to the subscribed UE Usage Type, if received from the HSS, and sent by the old MME/SGSN if the old MME/SGSN supports the Dedicated Core Networks specified in TS 23.401 [3]. This IE shall also be set to the subscribed UE Usage Type, if received from the UDM, and sent by the old AMF on N26 interface. If the UE Usage Type is not available in the old MME/SGSN/AMF, the length field of this IE shall be set to 0. See NOTE 7. | Integer Number | 0 |
| MME/SGSN UE SCEF PDN Connections | CO | This IE shall be present over the S10/S3/S16 interface if there is at least one SCEF PDN connection for this UE at the source MME/SGSN and if the target MME/SGSN is known to support SCEF Non-IP PDN Connections (as specified in clause 5.13.1 of 3GPP TS 23.682 [74]). Several IEs with this type and instance values shall be included as necessary to represent a list of SCEF PDN Connections. | SCEF PDN Connection | 0 |
| MSISDN | CO | This IE shall be present over the S3/S16 interface if the UE's MSISDN is available and if there is at least one instance of the MME/SGSN UE SCEF PDN Connection IE included in the message. | MSISDN | 1 |
|  | CO | This IE shall be present over the S10/N26 interface if the UE's MSISDN is available. |  |  |
| Source UDP Port Number | CO | If the target MME/SGSN/AMF, selected by the source MME/SGSN/AMF, decides to forward the Forward Relocation Request message to another more suitable MME/SGSN/AMF in the same MME/SGSN pool or AMF set, the MME/SGSN/AMF shall include this IE, set to the Source UDP Port number of the received message from the source MME/SGSN/AMF. The new target MME/SGSN/AMF shall use this UDP port as the UDP destination port in the Forward Relocation Response message. | Port Number | 0 |
| Serving PLMN Rate Control | CO | This IE shall be included by the old MME on S10 interface if the Serving PLMN Rate control was enabled when there is at least one SCEF PDN connection. See NOTE 9. | Serving PLMN Rate Control | 0 |
| Extended Trace Information | C | This IE shall be included over N26 when session trace is active for this user. | Extended Trace Information | 0 |
| Subscribed Additional RRM Policy Index | CO | This IE shall be included by the MME over the S10 interface if received from the HSS. | Additional RRM Policy Index | 0 |
| Additional RRM Policy Index in Use | CO | This IE shall be included by the MME over the S10 interface if the feature is supported by the MME. | Additional RRM Policy Index | 1 |
| Subscribed V2X Information | CO | This IE shall be present over the N26 interface if available, and sent by the old MME/AMF if the old MME/AMF supports the V2X services specified in 3GPP TS 23.287 [89]. | V2X Context | 0 |
| IWK SCEF ID for Monitoring Event | CO | This IE shall be included on the S3/S10 interface if the source MME/SGSN has selected the IWK-SCEF to relay Monitoring Events. | Node Identifier | 0 |
| Alternative IMSI | CO | This IE shall be included by the old MME over the S10 interface if it has stored an Alternative IMSI as specified in TS 23.401 [3] clause 5.7.2. | Alternative IMSI | 0 |
| Trace Collection Entity URI | O | This IE may be present over S10/S16/S3/N26 when session trace is active for this user and if the Trace Information or Extended Trace Information IE is included. When present, this IE shall include the Trace Reporting Consumer URI for Streaming based Reporting as defined in 3GPP TS 32.422 [18]. If both the IP Address of the Trace Collection Entity and Trace Collection Entity URI are present, the Trace Collection Entity URI shall take the higher precedence. | URI | 0 |
| Private Extension | O |  | Private Extension | VS |
| NOTE 1: 3GPP TS 23.401 [3] (e.g. clause 5.3.2.1) and 3GPP TS 23.060 [35] (e.g. clause 9.2.2.1) defines the MME/SGSN shall send the MS Info Change Reporting Support Indication to the PGW. In such case MME/SGSN shall use the Change Reporting Support Indication and/or CSG Change Reporting Support Indication (whichever is applicable), even if stage 2 refers to MS Info Change Reporting Support Indication. NOTE 2: GTPv2 shall be used for SRNS relocation w/o PDN connection if all the S4-SGSNs (between which SRNS relocation can take place) support this optional GTPv2 procedure. Otherwise GTPv1 shall be used for that procedure (see clause 7.10). The S4-SGSN can know by local configuration whether all peer S4-SGSNs support this procedure. NOTE 3: The receiver shall ignore the per UE Change Reporting Support Indication and CSG Change Reporting Support Indication flags, as included within the Indication Flags IE above, if these flags are included per PDN connection i.e. within the Indication Flags IE of the MME/SGSN UE EPS PDN Connections IE. NOTE 4: According to the 3GPP TS 23.401 [3], during an inter-RAT handover procedure for a UE with ISR activated, the source MME/SGSN should select the ISR associated CN node for this UE as the target CN node for the inter RAT HO when the ISR associated CN node can serve the target access. This parameter is exchanged when ISR is being activated and used in the source MME/SGSN for this decision upon subsequent inter-RAT handover. NOTE 5: If the SGSN needs to include the last used LTE PLMN ID in the Equivalent PLMN list it sends to the UE (see 3GPP TS 23.272 [21]), the SGSN shall derive the last used LTE PLMN ID from the Serving Network IE. If an MME or AMF needs to store the last used 5GS or EPS PLMN ID (respectively), the MME or AMF shall derive the last used 5GS or EPS PLMN ID (respectively) from the Serving Network IE. NOTE 6: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator for supporting UEs, or that of the allocated Core Network Operator for non-supporting UEs. As an exception, based on inter-operator roaming/sharing agreement, if the information on whether the UE is a supporting or non-supporting UE is available, the PLMN ID that is communicated to the HPLMN for non-supporting UEs shall be the Common PLMN ID. In shared networks, when the MME/S4-SGSN and PGW pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI to the PGW, and the Common PLMN ID shall be communicated in SAI/CGI to the PGW, for both supporting and non-supporting UEs. The Core Network Operator PLMN ID (selected by the UE for supporting UEs or allocated by the network for non-supporting UEs) shall be communicated in the TAI, RAI, UCI and the Serving Network. See clause 4.4 of 3GPP TS 23.251 [55]. NOTE 7: A UE Usage Type IE with the length field equal to 0 is used for the receiver to differentiate the case where the sender does not support the Dedicated Core Network feature from the case where the sender supports the Dedicated Core Network feature but no UE Usage type was received in UE's subscription. NOTE 8: There may be a pending MT Short Message at the SMS-GMSC during a handover scenario, when the UE performs a Service Request towards the source MME/SGSN and a handover procedure occurs shortly afterward, before the SMS-GMSC is alerted to retransmit the pending MT Short Message. NOTE 9: The target MME may compare the value of the Serving PLMN Rate Control received in the Forward Relocation Request message with the one configured locally, to determine if this parameter needs to be updated towards the SCEF. The Serving PLMN Rate Control does not apply to any SGi PDN Connection in this message. NOTE 10: PDN connections with PDN Type "Ethernet" are not supported in GERAN/UTRAN. For PDN connections with PDN type "Ethernet", mobility to GERAN/UTRAN is not supported. See clause 4.3.17.8a of TS 23.401 [3]. |  |  |  |  |

The PDN Connection grouped IE shall be coded as depicted in Table 7.3.1-2.

Table 7.3.1-2: MME/SGSN/AMF UE EPS PDN Connections within Forward Relocation Request

| Octet 1 |  | PDN Connection IE Type = 109 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| APN | M |  | APN | 0 |
| APN Restriction | C | This IE denotes the restriction on the combination of types of APN for the APN associated with this EPS bearer Context. The target MME or SGSN determines the Maximum APN Restriction using the APN Restriction. If available, the source MME/S4SGSN shall include this IE. | APN Restriction | 0 |  |
| Selection Mode | CO | When available, this IE shall be included by the source MME/S4-SGSN/AMF. | Selection Mode | 0 |  |
| IPv4 Address | C | This IE shall not be included if no IPv4 Address is assigned. See NOTE 1. | IP Address | 0 |
| IPv6 Address | C | This IE shall not be included if no IPv6 Address is assigned. | IP Address | 1 |
| Linked EPS Bearer ID | M | This IE identifies the default bearer of the PDN Connection. | EBI | 0 |
| PGW S5/S8 IP Address for Control Plane or PMIP | M | This IE shall include the TEID in the GTP based S5/S8 case and the uplink GRE key in the PMIP based S5/S8 case. See NOTE 4. | F-TEID | 0 |
| PGW node name | C | This IE shall be included if the source MME, SGSN or AMF has the PGW FQDN. | FQDN | 0 |
|  | CO | This IE shall be included by the source MME over the N26 interface. See NOTE 6. |  |  |
| Bearer Contexts | C | Several IEs with this type and instance values may be included as necessary to represent a list of Bearers. | Bearer Context | 0 |
| Aggregate Maximum Bit Rate (APN-AMBR) | M |  | AMBR | 0 |
| Charging characteristics | C | This IE shall be present if charging characteristics was supplied by the HSS to the MME/SGSN, or by the UDM to the SMF, as a part of subscription information. | Charging characteristics | 0 |
| Change Reporting Action | C | This IE shall be included whenever available at the source MME/SGSN. See NOTE 5. | Change Reporting Action | 0 |
| CSG Information Reporting Action | CO | This IE shall be included whenever available at the source MME/SGSN. | CSG Information Reporting Action | 0 |
| H(e)NB Information Reporting | CO | This IE shall be included whenever available at the source MME/SGSN. | H(e)NB Information Reporting | 0 |
| Indication Flags | CO | This IE shall be included if any of the flags are set to 1. - Change Reporting support indication flag: This flag shall be set to 1 if the Source S4-SGSN/MME supports Location Change Reporting mechanism and if the S4-SGSN/MME has indicated the support for the Location Change Reporting mechanism to the PGW, during the session establishment and/or modification procedures. See NOTE 2. - CSG Change Reporting Support Indication flag: This flag shall be set to 1 if the Source S4-SGSN/MME supports CSG Information Change Reporting mechanism and if the S4-SGSN/MME has indicated the support for the CSG Informatoin Change Reporting to the PGW, during the session establishment and/or modification procedures. See NOTE 2. - Delay Tolerant Connection Indication flag: This flag shall be set to 1 on the S3/S10/S16 interface by the source MME/SGSN if the PGW indicated that this PDN Connection is delay tolerant. - Extended PCO Support Indication flag: This flag shall be set to 1 on S10/N26 interface by the source MME if the UE and the source MME support Extended PCO. It shall be set to 1 on the N26 interface during a 5GS to EPS handover. - NO 5GS N26 mobility Indication flag: This flag shall be set to 1 on S10 interface if the PDN connection cannot be moved to 5GS via N26. - Pending PDN connection Restoration Indication: This flag shall be set to 1 on S10 interface if the PDN connection restoration is to be performed in the target MME as specified in clause 31.5 of 3GPP TS 23.007 [17]. | Indication | 0 |
| Signalling Priority Indication | CO | The source SGSN/MME shall include this IE if the UE indicated low access priority when establishing the PDN connection. | Signalling Priority Indication | 0 |
| Change to Report Flags | CO | This IE shall be included by the SGSN if any one of the applicable flags is set to 1. See NOTE3. Applicable flags: - Serving Network Change to Report: This flag shall be set to 1 if the source SGSN has detected a Serving Network change during a RAU procedure without SGSN change but has not yet reported this change to the PGW. - Time Zone Change to Report: This flag shall be set to 1 if the source SGSN has detected a UE Time Zone change during a RAU procedure without SGSN change but has not yet reported this change to the PGW. | Change to Report Flags | 0 |
| Local Home Network ID | CO | This IE shall be sent over the S3/S10/S16 interface if SIPTO at the Local Network is active for the PDN connection in the SIPTO at the Local Network architecture with stand-alone GW. | FQDN | 1 |
| Presence Reporting Area Action | CO | This IE shall be included if the PGW requested the source MME/SGSN to report changes of UE presence in a Presence Reporting Area. The source MME/SGSN shall include the Presence Reporting Area Identifier and, if received from the PGW, the list of the Presence Reporting Area elements. Several IEs with the same type and instance value may be included as necessary to represent a list of Presence Reporting Area Actions. One IE shall be included for each Presence Reporting Area. | Presence Reporting Area Action | 0 |
| WLAN Offloadability Indication | CO | If the MME/SGSN supports WLAN/3GPP Radio Interworking with RAN rules then this IE shall be included on S3/S10/S16 if the UE has been authorized to perform WLAN offload for at least one RAT. | WLAN Offloadability Indication | 0 |
| Remote UE Context Connected | CO | The source MME shall include this IE on the S10 interface during an inter MME mobility procedure if such information is available. Several IEs with the same type and instance value may be included as necessary to represent a list of remote UEs connected. | Remote UE Context | 0 |
| PDN Type | CO | The source MME/SGSN/AMF shall include this IE on the S10/S3/S16/N26 interface, for a Non-IP PDN Connection, during an inter MME/SGSN/AMF mobility procedure, if the target MME/SGSN/AMF supports SGi Non-IP or Ethernet PDN connections. | PDN Type | 0 |
| Header Compression Configuration | CO | This IE shall be sent over the S10 interface if the use of IP Header Compression for Control Plane CIoT EPS optimisations has been negotiated with the UE and the target MME is known to support CIoT EPS optimisations. | Header Compression Configuration | 0 |
| PGW Change Info | CO | This IE shall be sent over the S10/N26 interface if available from a source AMF/MME to a target MME. See clause 31.5 of 3GPP TS 23.007 [17]. | PGW Change Info | 0 |
| New PGW-C/SMF IP Address | CO | This IE shall be included when the Pending PDN connection Restoration Indication flag is set to "1" and if a new PGW-C/SMF IP address is available. (NOTE 7) | IP Address | 2 |
| New PGW Node Name | CO | This IE shall be included when the Pending PDN connection Restoration Indication flag is set to "1" and if a new PGW-C/SMF Node Name is available. (NOTE 7) | FQDN | 2 |
| UP Security Policy | CO | This IE shall be sent by the old MME on the S10 interface or by the old AMF on the N26 interface if this information is available. When present, it shall indicate whether User Plane integrity protection is required, preferred or not needed for the traffic of the PDN connection. | UP Security Policy | 0 |
| NOTE 1: For deferred IPv4 address allocation, if the MME/S4-SGSN receives the PDN address "0.0.0.0" from PGW during "eUTRAN Initial Attach", "PDP Context Activation", "UE requested PDN Connectivity", then the MME/S4-SGSN shall include this IPv4 address "0.0.0.0". NOTE 2: 3GPP TS 23.401 [3] (e.g. clause 5.3.2.1) and 3GPP TS 23.060 [35] (e.g. clause 9.2.2.1) defines the MME/SGSN shall send the MS Info Change Reporting Support Indication to the PGW. In such case MME/SGSN shall use the Change Reporting Support Indication and/or CSG Change Reporting Support Indication (whichever is applicable), even if stage 2 refers to MS Info Change Reporting Support Indication. NOTE 3: When UE is camping on the 3G and performs a Service Request procedure, as specified in the clause 6.12.1 of 3GPP TS 23.060 [35], if Service Type indicates Signalling, the signalling connection is established between the MS and the SGSN for sending upper-layer signalling messages, e.g. Activate PDP Context Request, but the resources for active PDP context(s) are not allocated, therefore the change of Serving Network or UE Time zone may not be reported to SGW/PGW for the existing PDP Contexts. NOTE 4: For PMIP based S5/S8, the 'PGW S5/S8 IP Address and TEID for user plane' IE and the 'PGW S5/S8 IP Address for Control Plane or PMIP' IE shall contain the same uplink GRE key; the Interface Type in these IEs shall be set to the value 9 (S5/S8 PGW PMIPv6 interface). NOTE 5: The target MME (respectively S4-SGSN) shall ignore this IE if it is received from an S4-SGSN (respectively an MME), i.e. over the S3 interface. In this case, the target serving node shall consider that no ULI change reporting is requested by the PGW for the target RAT, and the PGW shall request the target serving node to start ULI change reporting for the target RAT if so desired. NOTE 6: The PGW Node Name is used by the target AMF in the NF Service Discovery procedure to find the combined PGW-C/SMF for the PDU Session during an MME to AMF mobility procedure. NOTE 7: The New PGW-C/SMF IP Address and New PGW Node Name may be available, e.g., when the PDN connection was requested to be moved to the new PGW-C/SMF during a PGW triggered PDN connection restoration procedure, or when the source MME has selected an alternative PGW-C/SMF and sent the Create Session request message to the new PGW-C/SMF to restore the PDN connection but it has not received the Create Session Response yet. |  |  |  |  |

The Bearer Context grouped IE shall be coded as depicted in Table 7.3.1-3.

Table 7.3.1-3: Bearer Context within MME/SGSN/AMF UE EPS PDN Connections within Forward Relocation Request

| Octet 1 |  | Bearer Context IE Type = 93 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| EPS Bearer ID | M | See NOTE 3. | EBI | 0 |
| TFT | C | This IE shall be present if a TFT is defined for this bearer. | Bearer TFT | 0 |
| SGW S1/S4/S12 IP Address and TEID for user plane | M | This IE shall contain the SGW S1/S4/S12 IP Address and TEID for user plane. Over the N26 interface, the SMF (on behalf of the source AMF) shall set the IP address and TEID to the following values: - any reserved TEID (e.g. all 0's, or all 1's); - IPv4 address set to 0.0.0.0, or IPv6 Prefix Length and IPv6 prefix and Interface Identifier all set to zero. See NOTE2, NOTE 4. | F-TEID | 0 |
| PGW S5/S8 IP Address and TEID for user plane | C | This IE shall be present for GTP based S5/S8. | F-TEID | 1 |
|  | CO | For PMIP-based S5/S8, this IE shall be included if the PGW provided an alternate address for user plane, i.e. an IP address for user plane which is different from the IP address for control plane. When present, this IE shall contain the alternate IP address for user plane and the uplink GRE key. See NOTE 1. |  |  |
| Bearer Level QoS | M |  | Bearer QoS | 0 |
| BSS Container | CO | The MME/S4 SGSN shall include the Packet Flow ID, Radio Priority, SAPI, PS Handover XID parameters in the TAU/RAU/Handover procedure, if available. See Figure 8.48-2. The Container Type shall be set to 2. | F-Container | 0 |
| Transaction Identifier | C | This IE shall be sent over S3/S10/S16 if the UE supports A/Gb and/or Iu mode. This IE should be sent over N26 if the MME has a TI stored that is linked with this EPS Bearer ID, or, the SMF provides the TI to the AMF (as part of a procedure to deliver SM context to AMF). | TI | 0 |
| Bearer Flags | CO | Applicable flags: - vSRVCC indicator: This IE shall be sent by the source MME to the target MME on the S10 interface if vSRVCC indicator is available in the source MME. - ASI (Activity Status Indicator): the source S4-SGSN shall set this indicator to 1 on the S16 interface if the bearer context is preserved in the CN without an associated RAB. | Bearer Flags | 0 |
| SGW S11 IP Address and TEID for user plane | CO | This IE shall be present if available. See NOTE 2. | F-TEID | 2 |
| NOTE 1: For PMIP based S5/S8, the 'PGW S5/S8 IP Address and TEID for user plane' IE and the 'PGW S5/S8 IP Address for Control Plane or PMIP' IE shall contain the same uplink GRE key; the Interface Type in these IEs shall be set to the value 9 (S5/S8 PGW PMIPv6 interface). NOTE 2: When separate IP address spaces are used for S1-U and S11-U, the MMEs shall include both S1-U SGW F-TEID in the SGW S1/S4/S12/S11 IP Address and TEID for user plane IE and S11-U SGW F-TEID in SGW S11 IP Address and TEID for user plane, when they are available; otherwise, the source MME shall include the SGW S1/S4/S12/S11 IP Address and TEID for user plane IE, since S1-U and S11-U SGW F-TEID are the same. NOTE 3: The support of the 15 EPS Bearers shall be homogeneously supported within an MME Pool / SGW serving area. A source MME which supports the 15 EPS Bearers, shall know whether the target MME pool also supports that by local configuration. When the target MME is known to not support the 15 EPS Bearers, the source MME shall only transfer up to 8 EPS bearer contexts with the EBI value set between '5' and '15' to the target MME and shall delete EPS bearer(s) which are not transferred, and if the default bearer is to be deleted, the corresponding PDN connection(s) shall be deleted by the source MME. NOTE 4: During 5GS to EPS mobility procedures with N26 interface, the source AMF shall transparently transfer the MME/SGSN/AMF UE EPS PDN Connections IE received from the SMF as specified in clause 6.1.6.2.27 of 3GPP TS 29.502[88]. |  |  |  |  |

Table 7.3.1-4: Remote UE Context Connected within MME/SGSN UE EPS PDN Connections within Forward Relocation Request

| Octets 1 |  | Remote UE Context IE Type = 191 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octets 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| Remote User ID | M | See clause 8.123 for the description and use of this parameter | Remote User ID | 0 |
| Remote UE IP Information | M | See clause 8.124 for the description and use of this parameter | Remote UE IP Information | 0 |

Table 7.3.1-5: MME UE SCEF PDN Connections within Forward Relocation Request

| Octets 1 |  | SCEF PDN Connection IE Type = 195 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octets 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| APN | M |  | APN | 0 |
| Default EPS Bearer ID | M | This IE shall identify the default bearer of the SCEF PDN Connection. | EBI | 0 |
| SCEF ID | M | This IE shall include the SCEF Identifier and the SCEF Realm for the APN. | Node Identifier | 0 |

Table 7.3.1-6: Subscribed V2X Information within Forward Relocation Request

| Octets 1 |  | V2X Context IE Type = 208 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octets 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| LTE V2X Services Authorized | C | This IE shall be included to indicate the authorization status of the UE to use the LTE sidelink for V2X services. | Services Authorized | 0 |
| NR V2X Services Authorized | C | This IE shall be included to indicate the authorization status of the UE to use the NR sidelink for V2X services. | Services Authorized | 1 |
| LTE UE Sidelink Aggregate Maximum Bit Rate | C | This IE shall be included if the UE is authorized for LTE V2X services. | Bit Rate | 0 |
| NR UE Sidelink Aggregate Maximum Bit Rate | C | This IE shall be included if the UE is authorized for NR V2X services. | Bit Rate | 1 |
| PC5 QoS Parameters | C | This IE shall be included if the UE is authorized for NR V2X services. | PC5 QoS Parameters | 0 |

Table 7.3.1-7: PC5 QoS Parameters within Forward Relocation Request

| Octets 1 |  | PC5 QoS Parameters IE Type = 209 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octets 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| PC5 QoS Flows | M | Several IEs with this type and same instance value may be included as necessary to represent a list of PC5 QoS Flows. | PC5 QoS Flow | 0 |
| PC5 Link Aggregated Bit Rates | O | This IE may be included for the non-GBR PC5 QoS Flows. | Bit Rate | 0 |

Table 7.3.1-8: PGW Change Info with Forward Relocation Request

| Octet 1 |  | PGW Change Info IE Type = 214 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| PGW Set FQDN | CO | This IE shall be included by the source MME if available. When present, this IE shall contain the PGW Set FQDN of the PGW-C/SMF set to which the PGW-C/SMF belongs. (NOTE) | PGW FQDN | 0 |
| Alternative PGW-C/SMF IP Address | CO | This IE shall be included by the source MME if available. When present, this IE shall contain alternative PGW-C/SMF IP addresses within the PGW-C/SMF set to which the PGW-C/SMF belongs. Several IEs with the same type and instance value may be included to represent a list of Alternative PGW-C/SMF IP Addresses. (NOTE) | IP Address | 0 |
| Alternative PGW-C/SMF FQDN | CO | This IE shall be included by the source MME if available. When present, this IE shall contain alternative PGW-C/SMF FQDN within the PGW-C/SMF set to which the PGW-C/SMF belongs. Several IEs with the same type and instance value may be included to represent a list of Alternative PGW-C/SMF FQDNs. (NOTE) | PGW FQDN | 1 |
| Group Id | CO | This IE shall be included by the source MME if available. When present, this IE shall identify the group to which the PDN connection pertains (see clause 31.6 of 3GPP TS 23.007 [17]). | Group Id | 0 |
| NOTE: Either the PGW Set FQDN IE, the Alternative PGW-C/SMF IP Address IE, or the Alternative PGW-C/SMF FQDN IE shall be present. |  |  |  |  |

* * * * Next Change * * * *

7.3.6	Context Response

A Context Response message shall be sent as a response to a previous Context Request message, in the scenarios identified in clause 7.3.5.

Possible Cause values are specified in Table 8.4-1. Message specific cause values are:

-	"IMSI/IMEI not known"

-	"P-TMSI Signature mismatch"

-	"User authentication failed"

-	"Target access restricted for the subscriber"

Based on the subscription profile, when the access to the target RAT is prohibited for the subscriber, the old MME/SGSN/AMF may reject the Context Request message with the cause "Target access restricted for the subscriber".

When the source MME/SGSN/AMF supports one or more of the CIoT optimization features as indicated through the CIoT Optimizations Support Indication IE specified in clause 8.125, and if the target node is a MME and the target MME has not set the AWOPDN bit of the CIoT Optimizations Support Indication IE set to 1 in the Context Request message as specified in clause 8.125, the source MME/SGSN/AMF shall reject the Context Request with a cause value of "Request Rejected" under the following conditions (conditions are mutually exclusive):

-	If the UE is attached to the source MME/SGSN without any PDN connection through the SGW and PGW and without any SCEF PDN connection;

-	if the UE is attached to the source MME/SGSN with only the PDN connection(s) of PDN type "non-IP", through the SGW and the PGW but the UE has not activated any SCEF PDN connection and the target MME/AMF has not set the SGNIPDN bit of the CIoT Optimizations Support Indication IE set to 1 in the Context Request message as specified in clause 8.125;

-	if the UE is attached to the the source MME/SGSN with only the SCEF PDN connection(s) but the UE has not activated any PDN connection through the SGW and PGW and the target MME has not set the SCNIPDN bit of the CIoT Optimizations Support Indication IE set to 1 in the Context Request message as specified in clause 8.125;

-	if the UE is attached to the source MME/SGSN with only PDN connection(s) of PDN type "non-IP", through the SGW and the PGW and at least one SCEF PDN connection and the target MME has neither set the SGNIPDN bit nor the SCNIPDN bit of the CIoT Optimizations Support Indication IE set to 1 in the Context Request message as specified in clause 8.125;

-	if the UE is registered to the source AMF without any PDU session;

-	if the UE is registered to the source AMF with only PDU session(s) of type "Unstructured" or "Ethernet", and the target MME has not set the SGNIPDN bit of the CIoT Optimizations Support Indication IE set to 1 in the Context Request message as specified in clause 8.125.

NOTE 2:	Among the CIoT optimization features, only the support of SCEF Non-IP PDN connection and the support of SGi Non-IP PDN connection are applicable to a SGSN.

NOTE 3:	5GS supports Attach without PDU session. 5GS can also support Unstructured and Ethernet PDU session types, which are assimilated to "SGi Non-IP PDN connections" over N26 if Ethernet PDN connection type in EPC is not supported; otherwise, the Ethernet PDU session in 5GS can move to EPC seamlessly.

If the target node is a MME and the target MME has not set the ETHPDN bit in the Indication IE to 1 in the Context Request message as specified in clause 8.12, or the target node is a SGSN, the source MME shall reject the Context Request with a cause value of "Request Rejected" if the UE is attached to the source MME with only PDN connection(s) of PDN type "Ethernet".

Table 7.3.6-1 specifies the presence requirements and conditions of the IEs in the message.

Table 7.3.6-1: Information Elements in a Context Response

| Information elements | P | Condition / Comment | IE Type | Ins. |
| --- | --- | --- | --- | --- |
| Cause | M |  | Cause | 0 |
| IMSI | C | The IMSI shall be included in the message except for the case: - If the UE is emergency or RLOS attached and the UE is UICCless. The IMSI shall be included in the message but not used as an identifier - if UE is emergency or RLOS attached but IMSI is not authenticated. | IMSI | 0 |
| MME/SGSN/AMF UE MM Context | C | This IE shall be included if the Cause IE has the value " Request Accepted ". | MM Context | 0 |
| MME/SGSN/AMF UE EPS PDN Connections | C | This IE shall be included if there is at least a PDN connection for this UE through the SGW and PGW on the sending MME/SGSN, or if there is at least a PDU session for this UE on the sending AMF. If the target MME/SGSN/AMF has not set the SGNIPDN bit of the CIoT Optimizations Support Indication IE to 1 in the Context Request, then the source MME/SGSN/AMF shall only include PDN Connections of IP PDN type (if any). Several IEs with this type and instance values shall be included as necessary to represent a list of PDN Connections. | PDN Connection | 0 |
| Sender F-TEID for Control Plane | C | This IE shall be included if the Cause IE has the value "Request Accepted". | F-TEID | 0 |
| SGW S11/S4 IP Address and TEID for Control Plane | C | This IE shall be included if a SGW is being used by the old MME/SGSN, except if: - the source and target MME/S4-SGSN support the MME/S4-SGSN triggered SGW restoration procedure, and the source MME/S4-SGSN has not performed the SGW relocation procedure after the source SGW has failed as specified in 3GPP TS 23.007 [17]. - across the S16 interface if there is no active PDP context - across the S10/N26 interface if the UE does not have any PDN connection through the SGW and PGW. This IE shall not be included by an old AMF. | F-TEID | 1 |
| SGW node name | C | This IE identifies the SGW that was used by the old MME/SGSN and it shall be included by the source MME/S4-SGSN with the following exceptions: - the source and target MME/S4-SGSN support the MME/S4-SGSN triggered SGW restoration procedure, and the source MME/S4-SGSN has not performed the SGW relocation procedure after the source SGW has failed as specified in 3GPP TS 23.007 [17]. - across the S16 interface if there is no active PDP context - across the S10/N26 interface if the UE does not have any PDN connection through the SGW and PGW. This IE shall not be included by an old AMF. | FQDN | 0 |
| Indication Flags | C | This IE shall be included if any of the flags are set to 1. Idle mode Signalling Reduction Supported Indication: - This flag shall be set to 1 if the Cause IE value indicates "Request accepted" and the old system (including old MME/SGSN and the associated SGW) has the ISR capability. Unauthenticated IMSI: - This flag shall be set to 1 if the IMSI present in the message is not authenticated and is for an emergency or RLOS attached UE. Change Reporting support indication flag: - This flag shall be set to 1 if the Source S4-SGSN/MME supports Location Change Reporting mechanism. See NOTE 1. See NOTE 2. CSG Change Reporting support indication flag: - This flag shall be set to 1 if the Source S4-SGSN/MME supports CSG Information Change Reporting mechanism. See NOTE 1. See NOTE 2. ISRAU: - This flag shall be set to 1 on S10/S16 interface if the ISR is activated for the UE before the UE moving to the new SGSN/MME. Management Based MDT allowed flag: - This flag shall be set to 1 for the inter-MME TAU procedure over the S10 interface, if Management Based Minimization of Drive Tests (MDT) is allowed. See 3GPP TS 36.413 [10] and 3GPP TS 32.422 [18]. SGW Restoration Needed Indication (SRNI): - This flag shall be set to 1 if both source and target MME/S4-SGSN support the MME/S4-SGSN triggered SGW restoration procedure and the source MME/S4-SGSN has not performed the SGW relocation procedure after the source SGW has failed as specified in 3GPP TS 23.007 [17]. CSFB Indication (CSFBI): - when configured to support the return to the last used PLMN after CSFB, the MME shall set this flag to 1 on the S3 interface if the UE has been subject to CSFB recently (see clause 4.3.2 of 3GPP TS 23.272 [21]). See NOTE 4. Buffered DL Data Waiting Indication (BDWI): - This flag shall be set to 1 on the S3/S10/S16/N26 interface, during TAU/RAU with or without SGW change procedures or during Idle mode mobility between 5GS and EPS with data forwarding, when it is required to forward to the UE DL data buffered in the (old) SGW or (V-)SMF/UPF, i.e. when the DL Data Buffer Expiration Time has not expired yet in the old MME/SGSN/(V-)SMF, as specified in clause 4.3.17.7 of 3GPP TS 23.401 [3] and in clauses 4.11.1.3.2A, 4.11.1.3.3A, 4.23.12.2a and 4.23.12.3a of 3GPP TS 23.502 [83]. Pending MT Short Message Indication (PMTSMI): - This flag shall be set to 1 on the S10/S16/S3 interface if the source S4-SGSN/MME knows that there is one (or more) pending MT Short Message(s) in the SMS-GMSC for the UE as specified in clause 10.1 of 3GPP TS 23.040 [75], Figure 17c). LTE-M UE Indication (LTEMUI): - This flag shall be set to 1 if the MME/AMF knows that the UE is an LTE-M UE. Return Preferred Indication (REPREFI): - the MME or the AMF may set this flag to 1 on the N26 interface to indicate a preferred return of the UE to the last used EPS or 5GS PLMN (respectively) at a later access change to an EPS or 5GS shared network. See NOTE 4. LTE-M Satellite UE Indication (LTEMSUI): - This flag shall be set to 1 if the MME/AMF knows that the UE is an LTE-M Satellite UE. | Indication | 0 |
| Trace Information | C | This IE shall be included over S10/S16/S3 when session trace is active for this IMSI/IMEI. | Trace Information | 0 |
| HRPD access node S101 IP address | C | This IE shall be included only if the HRPD pre registration was performed at the old MME | IP-Address | 0 |
| 1xIWS S102 IP address | C | This IE shall be included only if the 1xRTT CS fallback pre registration was performed at the old MME | IP-Address | 1 |
| Subscribed RFSP Index | CO | This IE shall be included by the MME/SGSN/AMF, if received from the HSS or UDM. | RFSP Index | 0 |
| RFSP Index in Use | CO | This IE shall be included by the MME/SGSN/AMF, if it supports the feature. | RFSP Index | 1 |
| RFSP Index in Use Validity Time | O | This IE may be included by the source MME/AMF during 5GS to EPS mobility and subsequent inter-MME mobility procedures if the validity time has not expired yet. It indicates the validity time of RFSP Index in Use to be used in the target MME as specified in clause 4.11.1.5.8 of 3GPP TS 23.502 [83]. When present, this IE shall be set to the remaining validity time of the RFSP Index in Use. The AMF shall ignore this IE if it is received from the MME. | Timer in seconds | 0 |
| UE Time Zone | CO | When available, this IE shall be included by the source MME/S4-SGSN/AMF. | UE Time Zone | 0 |
| MME/S4-SGSN LDN | O | This IE is optionally sent by the MME/S4-SGSN to the peer MME/S4-SGSN on the S3/S10/S16 interfaces (see 3GPP TS 32.423 [44]), when communicating the LDN to the peer node for the first time. | Local Distinguished Name (LDN) | 0 |
| MDT Configuration | CO | This IE shall be sent by the source MME to the target MME on the S10 interface for inter-MME TAU procedure, if the Job Type indicates Immediate MDT. See 3GPP TS 32.422 [18] clause 4.4. | MDT Configuration | 0 |
| SGSN node name | CO | This IE shall be sent by the old SGSN on the S3 interface if both old SGSN and associated SGW support ISR. See NOTE 3. | FQDN | 1 |
| MME node name | CO | This IE shall be sent by the old MME on the S3 interface if both old MME and associated SGW support ISR. See NOTE 3. | FQDN | 2 |
| User CSG Information (UCI) | CO | This IE shall be sent by the source MME/S4-SGSN on the S3/S10/S16 interfaces if the source MME/SGSN has reported to the PGW that the UE is in a CSG or hybrid cell. It shall then contain the last User CSG information that the source MME/S4-SGSN has reported to the PGW. The absence of this IE indicates that the UE has not been reported to the PGW as being in a CSG or hybrid cell. See NOTE 5. | UCI | 0 |
| Monitoring Event Information | CO | This IE shall be sent by the source MME/S4-SGSN on the S3/S10/S16 interfaces if monitoring events are to be continued in the target MME/S4-SGSN. More than one IE with this type and instance values may be included to represent multiple monitoring events. | Monitoring Event Information | 0 |
| Monitoring Event Extension Information | CO | This IE shall be sent by the source MME/S4-SGSN on the S3/S10/S16 interfaces if monitoring event extensions are to be sent to the target MME/S4-SGSN. More than one IE with this type and instance values may be included to represent multiple monitoring event extensions. | Monitoring Event Extension Information | 0 |
| UE Usage Type | CO | This IE shall be set to the subscribed UE Usage Type, if received from the HSS, and sent by the old MME/SGSN on the S3/S10/S16 interfaces if the old MME/SGSN supports the Dedicated Core Networks feature specified in TS 23.401 [3]. This IE shall be set to the subscribed UE Usage Type, if received from the UDM, and sent by the old AMF on the N26 interface. If the UE Usage Type is not available in the old MME/SGSN/AMF, the length field of this IE shall be set to 0. See NOTE 6. | Integer Number | 0 |
| MME/SGSN UE SCEF PDN Connections | C | This IE shall be included if there is at least one SCEF PDN connection for this UE at the source MME/SGSN and if the target MME/SGSN has set the SCNIPDN bit of the CIoT Optimizations Support Indication IE to 1 in the Context Request as specified in clause 8.125. Several IEs with this type and instance values shall be included as necessary to represent a list of SCEF PDN Connections. | SCEF PDN Connection | 0 |
| RAT Type | CO | This IE shall be included by the source MME, SGSN or AMF on the S10, S16, S3 and N26 interface to indicate the old RAT type where the UE was camping. See NOTE 7. | RAT Type | 0 |
| Serving PLMN Rate Control | CO | This IE shall be included by the old MME on the S10 interface if such Serving PLMN Rate control was enabled when there is at least one SGi or SCEF PDN connection with the Control Plane Only Indication set. See NOTE 8. | Serving PLMN Rate Control | 0 |
| MO Exception Data Counter | CO | This IE shall be included on the S10 interfaces if the source MME has not yet reported a non-zero MO Exception Data Counter to the PGW. The timestamp in the counter shall be set with the time at which the counter value increased from 0 to 1. | Counter | 0 |
| Remaining Running Service Gap Timer | CO | This IE shall be included on the S10 interface if the source MME has started a Service Gap Timer which has not expired yet, as specified in clauses 5.3.3.1 and 5.3.3.2 in 3GPP TS 23.401 [3]. The value of the IE indicates the remaining time before the Service Gap Timer expires in number of seconds. See NOTE 9. | Integer Number | 1 |
| Extended Trace Information | C | This IE shall be included over N26 when session trace is active for this user. | Extended Trace Information | 0 |
| Subscribed Additional RRM Policy Index | CO | This IE shall be included by the MME over the S10 interface if received from the HSS. | Additional RRM Policy Index | 0 |
| Additional RRM Policy Index in Use | CO | This IE shall be included by the MME over the S10 interface if the feature is supported by the MME. | Additional RRM Policy Index | 1 |
| IWK SCEF ID for Monitoring Event | CO | This IE shall be included on the S3/S10 interface if the source MME/SGSN has selected the IWK-SCEF to relay Monitoring Events. | Node Identifier | 0 |
| Alternative IMSI | CO | This IE shall be included by the old MME over the S10 interface if it has stored an Alternative IMSI as specified in TS 23.401 [3] clause 5.7.2. | Alternative IMSI | 0 |
| MSISDN | CO | This IE shall be present over the S10/N26 interface if the UE's MSISDN is available. | MSISDN | 0 |
| Trace Collection Entity URI | O | This IE may be present over S10/S16/S3/N26 when session trace is active for this user and if the Trace Information or Extended Trace Information IE is included. When present, this IE shall include the Trace Reporting Consumer URI for Streaming based Reporting as defined in 3GPP TS 32.422 [18]. If both the IP Address of the Trace Collection Entity and Trace Collection Entity URI are present, the Trace Collection Entity URI shall take the higher precedence. | URI | 0 |
| Private Extension | O |  | Private Extension | VS |
| NOTE 1: 3GPP TS 23.401 [3] (e.g. clause 5.3.2.1) and 3GPP TS 23.060 [35] (e.g. clause 9.2.2.1) defines the MME/SGSN shall send the MS Info Change Reporting Support Indication to the PGW. In such case MME/SGSN shall use the Change Reporting Support Indication and/or CSG Change Reporting Support Indication (whichever is applicable), even if stage 2 refers to MS Info Change Reporting Support Indication. NOTE 2: The receiver shall ignore the per UE Change Reporting Support Indication and CSG Change Reporting Support Indication flags, as included within the Indication Flags IE above, if these flags are included per PDN connection i.e. within the Indication Flags IE of the MME/SGSN UE EPS PDN Connections IE. NOTE 3: According to the 3GPP TS 23.401 [3], during an inter-RAT handover procedure for a UE with ISR activated, the source MME/SGSN should select the ISR associated CN node for this UE as the target CN node for the inter RAT HO when the ISR associated CN node can serve the target access. This parameter is exchanged when ISR is being activated and used in the source MME/SGSN for this decision upon subsequent inter-RAT handover. NOTE 4: If the SGSN needs to include the last used LTE PLMN ID in the Equivalent PLMN list it sends to the UE (see 3GPP TS 23.272 [21]), the SGSN shall derive the last used LTE PLMN ID from the Old RAI IE received in the RAU request message. If an MME or AMF needs to store the last used 5GS or EPS PLMN ID (respectively), the MME shall derive the last used 5GS PLMN ID from the Old GUTI received in the TAU request message, the AMF shall derive the last used EPS PLMN ID from the 5GS mobile identity received in Registration request message. NOTE 5: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator for supporting UEs, or that of the allocated Core Network Operator for non-supporting UEs. As an exception, based on inter-operator roaming/sharing agreement, if the information on whether the UE is a supporting or non-supporting UE is available, the PLMN ID that is communicated to the HPLMN for non-supporting UEs shall be the Common PLMN ID. In shared networks, when the MME/S4-SGSN and PGW pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI to the PGW, and the Common PLMN ID shall be communicated in SAI/CGI to the PGW, for both supporting and non-supporting UEs. The Core Network Operator PLMN ID (selected by the UE for supporting UEs or allocated by the network for non-supporting UEs) shall be communicated in the TAI, RAI, UCI and the Serving Network. See clause 4.4 of 3GPP TS 23.251 [55]. NOTE 6: A UE Usage Type IE with the length field equal to 0 is used for the receiver to differentiate the case where the sender does not support the Dedicated Core Network feature from the case where the sender supports the Dedicated Core Network feature but no UE Usage type was received in UE's subscription. NOTE 7: The old RAT shall be used by the target MME/SGSN to determine if the RAT type has changed and, if so, to decide whether to maintain or deactivate the PDN connections of the UE based on the PDN-Connection-Continuity subscription parameter and operator policy, as specified in 3GPP TS 23.401 [3] and 3GPP TS 23.060 [35]. A target MME shall consider that the RAT type has changed if the target RAT is NB-IoT and a source MME (complying with an earlier version of the specification) does not include the RAT Type IE in the Context Response. NOTE 8: The target MME may compare the value of the Serving PLMN Rate Control received in the Context Response message with the one configured locally, to determine if such parameter needs to be updated towards the SCEF and/or PGW. NOTE 9: If the serving MME has changed after the TAU procedure, the target MME shall start Service Gap timer and should use a value that is slightly shorter than the value received in this IE. |  |  |  |  |

Table 7.3.6-2: MME/SGSN/AMF UE EPS PDN Connections within Context Response

| Octet 1 |  | PDN Connection IE Type = 109 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| APN | M |  | APN | 0 |
| APN Restriction | C | This IE denotes the restriction on the combination of types of APN for the APN associated with this EPS bearer Context. The target MME or SGSN determines the Maximum APN Restriction using the APN Restriction. If available, the source MME/S4 SGSN shall include this IE. | APN Restriction | 0 |  |
| Selection Mode | CO | When available, this IE shall be included by the source MME/S4-SGSN/AMF. | Selection Mode | 0 |  |
| IPv4 Address | C | This IE shall not be included if no IPv4 Address is assigned. See NOTE 1. See NOTE 5. | IP Address | 0 |
| IPv6 Address | C | This IE shall not be included if no IPv6 Address is assigned. See NOTE 5. | IP Address | 1 |
| Linked EPS Bearer ID | M | This IE identifies the default bearer of the PDN Connection. | EBI | 0 |
| PGW S5/S8 IP Address for Control Plane or PMIP | M | This IE shall include the TEID in the GTP based S5/S8 case and the uplink GRE key in the PMIP based S5/S8 case. See NOTE 3. | F-TEID | 0 |
| PGW node name | C | This IE shall be included if the source MME, SGSN or AMF has the PGW FQDN. | FQDN | 0 |
|  | CO | This IE shall be included by the source MME over the N26 interface. See NOTE 6. |  |  |
| Bearer Contexts | M | Several IEs with this type and instance values may be included as necessary to represent a list of Bearers. | Bearer Context | 0 |
| Aggregate Maximum Bit Rate (APN-AMBR) | M |  | AMBR | 0 |
| Charging characteristics | C | This IE shall be present if charging characteristics was supplied by the HSS to the MME/SGSN, or by the UDM to the SMF, as a part of subscription information. | Charging characteristics | 0 |
| Change Reporting Action | C | This IE shall be included whenever available at the source MME/SGSN. See NOTE 4. | Change Reporting Action | 0 |
| CSG Information Reporting Action | CO | This IE shall be included whenever available at the source MME/SGSN. | CSG Information Reporting Action | 0 |
| H(e)NB Information Reporting | CO | This IE shall be included whenever available at the source MME/SGSN. | H(e)NB Information Reporting | 0 |
| Indication flags | CO | This IE shall be included if any one of the applicable flags is set to 1. Applicable flags: - Subscribed QoS Change Indication: This flag shall be set to 1 if the subscribed QoS profile of the related PDN connection has changed in the old MME/SGSN when the UE is in ECM-IDLE state and ISR is activated. - Change Reporting support indication flag: This flag shall be set to 1 if the source S4-SGSN/MME supports Location Change Reporting mechanism and if the S4-SGSN/MME has indicated the support for the Location Change Reporting mechanism to the PGW, during the session establishment and/or modification procedures. See NOTE 2. - CSG Change Reporting Support Indication flag: This flag shall be set to 1 if the Source S4-SGSN/MME supports CSG Information Change Reporting mechanism and if the S4-SGSN/MME has indicated the support for the CSG Information Change Reporting to the PGW, during the session establishment and/or modification procedures. See NOTE 2. - Pending Subscription Change Indication flag: This flag shall be set to 1 if the source MME has received Subscribed QoS profile updates for QCI/ARP/APN-AMBR from the HSS but has deferred the reporting of these updates to the PGW/PCRF because the UE was not reachable. - Pending Network Initiated PDN Connection Signalling Indication flag: This flag shall be set to 1 by the source MME/SGSN if there is pending network initiated signalling for the PDN connection. - Delay Tolerant Connection Indication flag: This flag shall be set to 1 interface by the source MME/SGSN if the PGW indicated that this PDN Connection is delay tolerant. - Extended PCO Support Indication flag: This flag shall be set to 1 on S10/N26 interface by the source MME if the UE and the source MME support Extended PCO. It shall be set to 1 on the N26 interface during a 5GS to EPS Idle mode mobility. - Control Plane Only PDN Connection Indication: This flag shall be set to 1 if the PDN Connection is set to Control Plane Only. - NO 5GS N26 mobility Indication flag: This flag shall be set to 1 on S10 interface if the PDN connection cannot be moved to 5GS via N26. - Pending PDN connection Restoration Indication: This flag shall be set to 1 on S10 interface if the PDN connection restoration is to be performed in the target MME as specified in clause 31.5 of 3GPP TS 23.007 [17]. | Indication | 0 |
| Signalling Priority Indication | CO | The source SGSN/MME shall include this IE if the UE indicated low access priority when establishing the PDN connection. | Signalling Priority Indication | 0 |
| Change to Report Flags | CO | This IE shall be included by the MME/SGSN if any one of the applicable flags is set to 1. Applicable flags: - Serving Network Change to Report: This flag shall be set to 1 if the source MME/SGSN has detected a Serving Network change during a TAU/RAU procedure without MME/SGSN change but has not yet reported this change to the PGW. - Time Zone Change to Report: This flag shall be set to 1 if the source MME/SGSN has detected a UE Time Zone change during a TAU/RAU procedure without MME/SGSN change but has not yet reported this change to the PGW. | Change To Report Flags | 0 |
| Local Home Network ID | CO | This IE shall be sent over the S3/S10/S16 interface if SIPTO at the Local Network is active for the PDN connection in the SIPTO at the Local Network architecture with stand-alone GW. | FQDN | 1 |
| Presence Reporting Area Action | CO | This IE shall be included if the PGW requested the source MME/SGSN to report changes of UE presence in a Presence Reporting Area. The source MME/SGSN shall include the Presence Reporting Area Identifier and, if received from the PGW, the list of the Presence Reporting Area elements. Several IEs with the same type and instance value may be included as necessary to represent a list of Presence Reporting Area Actions. One IE shall be included for each Presence Reporting Area. | Presence Reporting Area Action | 0 |
| WLAN Offloadability Indication | CO | If the MME/SGSN supports WLAN/3GPP Radio Interworking with RAN rules then this IE shall be included on S3/S10/S16 if the UE has been authorized to perform WLAN offload for at least one RAT. | WLAN Offloadability Indication | 0 |
| Remote UE Context Connected | CO | The source MME shall include this IE on the S10 interface during an inter MME mobility procedure if such information is available. Several IEs with the same type and instance value may be included as necessary to represent a list of remote UEs connected. | Remote UE Context | 0 |
| PDN Type | CO | The source MME/SGSN/AMF shall include this IE on the S10/S3/S16/N26 interface, for a Non-IP or Ethernet PDN Connection, during an inter MME/SGSN/AMF mobility procedure if the new MME/SGSN/AMF supports non-IP or Ethernet PDN connection using SGi as indicated in the Context Request message. | PDN Type | 0 |
| Header Compression Configuration | CO | This IE shall be sent over the S10 interface if the use of IP Header Compression for Control Plane CIoT EPS optimisations has been negotiated with the UE and the target MME has set the IHCSI bit of the CIoT Optimizations Support Indication IE to 1 in the Context Request as specified in clause 8.125. | Header Compression Configuration | 0 |
| PGW Change Info | CO | This IE shall be sent over the S10/N26 interface if available from a source AMF/MME to a target MME. See clause 31.5 of 3GPP TS 23.007 [17]. | PGW Change Info | 0 |
| New PGW-C/SMF IP Address | CO | This IE shall be included when the Pending PDN connection Restoration Indication flag is set to "1" and if a new PGW-C/SMF IP address is available. (NOTE 7) | IP Address | 2 |
| New PGW Node Name | CO | This IE shall be included when the Pending PDN connection Restoration Indication flag is set to "1" and if a new PGW-C/SMF Node Name is available. (NOTE 7) | FQDN | 2 |
| UP Security Policy | CO | This IE shall be sent by the old MME on the S10 interface or by the old AMF on the N26 interface if this information is available. When present, it shall indicate whether User Plane integrity protection is required, preferred or not needed for the traffic of the PDN connection. | UP Security Policy | 0 |
| NOTE 1: For deferred IPv4 address allocation, if the MME/S4-SGSN receives the PDN address "0.0.0.0" from PGW during "eUTRAN Initial Attach", "PDP Context Activation", "UE requested PDN Connectivity", then the MME/S4-SGSN shall include this IPv4 address "0.0.0.0". NOTE 2: 3GPP TS 23.401 [3] (e.g. clause 5.3.2.1) and 3GPP TS 23.060 [35] (e.g. clause 9.2.2.1) defines the MME/SGSN shall send the MS Info Change Reporting Support Indication to the PGW. In such case MME/SGSN shall use the Change Reporting Support Indication and/or CSG Change Reporting Support Indication (whichever is applicable), even if stage 2 refers to MS Info Change Reporting Support Indication. NOTE 3: For PMIP based S5/S8, the 'PGW S5/S8 IP Address and TEID for user plane' IE and the 'PGW S5/S8 IP Address for Control Plane or PMIP' IE shall contain the same uplink GRE key; the Interface Type in these IEs shall be set to the value 9 (S5/S8 PGW PMIPv6 interface). NOTE 4: The target MME (respectively S4-SGSN) shall ignore this IE if it is received from an S4-SGSN (respectively an MME), i.e. over the S3 interface. In this case, the target serving node shall consider that no ULI change reporting is requested by the PGW for the target RAT, and the PGW shall request the target serving node to start ULI change reporting for the target RAT if so desired. NOTE 5: For Non-IP or Ethernet PDN connections, neither an IPv4 address nor an IPv6 address shall be present. NOTE 6: The PGW Node Name is used by the target AMF in the NF Service Discovery procedure to find the combined PGW-C/SMF for the PDU Session during an MME to AMF mobility procedure. NOTE 7: The New PGW-C/SMF IP Address and New PGW Node Name may be available, e.g., when the PDN connection was requested to be moved to the new PGW-C/SMF during a PGW triggered PDN connection restoration procedure, or when the source MME has selected an alternative PGW-C/SMF and sent the Create Session request message to to the new PGW-C/SMF to restore the PDN connection but it has not received the Create Session Response yet. |  |  |  |  |

The Bearer Context shall be coded as depicted in Table 7.3.6-3.

Table 7.3.6-3: Bearer Context within MME/SGSN/AMF UE EPS PDN Connections within Context Response

| Octet 1 |  | Bearer Context IE Type = 93 |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Sparae and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| EPS Bearer ID | M | See NOTE 4. | EBI | 0 |
| TFT | C | This IE shall be present if a TFT is defined for this bearer. | Bearer TFT | 0 |
| SGW S1/S4/S12/S11 IP Address and TEID for user plane | C | The IE shall be present except if: - the source and target MME/S4-SGSN support the MME/S4-SGSN triggered SGW restoration procedure, and the source MME/S4-SGSN has not performed the SGW relocation procedure after the SGW has failed as specified in 3GPP TS 23.007 [17]. Over the N26 interface, the SMF (on behalf of the source AMF) shall set the IP address and TEID to the following values: - any reserved TEID (e.g. all 0's, or all 1's); - IPv4 address set to 0.0.0.0, or IPv6 Prefix Length and IPv6 prefix and Interface Identifier all set to zero. See NOTE 2, NOTE3 and NOTE 5. | F-TEID | 0 |
| PGW S5/S8 IP Address and TEID for user plane | C | This IE shall be included for GTP based S5/S8. | F-TEID | 1 |
|  | CO | For PMIP-based S5/S8, this IE shall be included if the PGW provided an alternate address for user plane, i.e. an IP address for user plane which is different from the IP address for control plane. When present, this IE shall contain the alternate IP address for user plane and the uplink GRE key. See NOTE 1. |  |  |
| Bearer Level QoS | M |  | Bearer QoS | 0 |
| BSS Container | CO | The MME/S4 SGSN shall include the Packet Flow ID, Radio Priority, SAPI, PS Handover XID parameters in the TAU/RAU/Handover procedure, if available. The Container Type shall be set to 2. | F-Container | 0 |
| Transaction Identifier | C | This IE shall be sent over S3/S10/S16 if the UE supports A/Gb and/or Iu mode. | TI | 0 |
| SGW S11 IP Address and TEID for user plane | CO | This IE shall be sent over S3/S10/S16 if the UE supports A/Gb and/or Iu mode. This IE should be sent over N26 if the MME has a TI stored that is linked with this EPS Bearer ID, or, the SMF provides the TI to the AMF (as part of a procedure to deliver SM context to AMF). | F-TEID | 2 |
| NOTE 1: For PMIP based S5/S8, the 'PGW S5/S8 IP Address and TEID for user plane' IE and the 'PGW S5/S8 IP Address for Control Plane or PMIP' IE shall contain the same uplink GRE key; the Interface Type in these IEs shall be set to the value 9 (S5/S8 PGW PMIPv6 interface). NOTE 2: The MME shall set the interface type in this IE to 1, i.e "S1-U SGW GTP-U interface", for S1-U and S11-U bearers. This is done for backwards compatibility reasons, when the target serving node does not support CIoT optimizations. NOTE 3: When separate IP address spaces are used for S1-U and S11-U, the MME shall include both S1-U SGW F-TEID in the SGW S1/S4/S12/S11 IP Address and TEID for user plane IE and S11-U SGW F-TEID in SGW S11 IP Address and TEID for user plane, when they are available; otherwise, the source MME shall include the SGW S1/S4/S12/S11 IP Address and TEID for user plane IE, since S1-U and S11-U SGW F-TEID are the same. NOTE 4: The support of the 15 EPS Bearers shall be homogeneously supported within an MME Pool / SGW serving area. A source MME which supports the 15 EPS Bearers, shall know whether the target MME pool also supports that by local configuration. When the target MME is known to not support the 15 EPS Bearers, the source MME shall only transfer up to 8 EPS bearer contexts with the EBI value set between '5' and '15' to the target MME and shall delete EPS bearer(s) which are not transferred, and if the default bearer is to be deleted, the corresponding PDN connection(s) shall be deleted by the source MME. NOTE 5: During 5GS to EPS mobility procedures with N26 interface, the source AMF shall transparently transfer the MME/SGSN/AMF UE EPS PDN Connections IE received from the SMF as specified in clause 6.1.6.2.27 of 3GPP TS 29.502[88]. |  |  |  |  |

Table 7.3.6-4: Remote UE Context Connected within MME/SGSN UE EPS PDN Connections within Context Response

| Octets 1 |  | Remote UE Context IE Type = 191 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octets 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| Remote User ID | M | See clause 8.123 for the description and use of this parameter | Remote User ID | 0 |
| Remote UE IP Information | M | See clause 8.124 for the description and use of this parameter | Remote UE IP Information | 0 |

Table 7.3.6-5: MME/SGSN UE SCEF PDN Connections within Context Response

| Octets 1 |  | SCEF PDN Connection IE Type = x (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octets 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| APN | M |  | APN | 0 |
| Default EPS Bearer ID | M | This IE identifies the default bearer of the SCEF PDN Connection. | EBI | 0 |
| SCEF ID | M | This IE shall include the SCEF Identifier and the SCEF Realm for the APN. | Node Identifier | 0 |

Table 7.3.6-6: PGW Change Info with Context Response

| Octet 1 |  | PGW Change Info IE Type = 214 (decimal) |  |  |
| --- | --- | --- | --- | --- |
| Octets 2 and 3 |  | Length = n |  |  |
| Octet 4 |  | Spare and Instance fields |  |  |
| Information elements | P | Condition / Comment | IE Type | Ins. |
| PGW Set FQDN | CO | This IE shall be included by the source MME if available. When present, this IE shall contain the PGW Set FQDN of the PGW-C/SMF set to which the PGW-C/SMF belongs. (NOTE) | PGW FQDN | 0 |
| Alternative PGW-C/SMF IP Address | CO | This IE shall be included by the source MME if available. When present, this IE shall contain alternative PGW-C/SMF IP addresses within the PGW-C/SMF set to which the PGW-C/SMF belongs. Several IEs with the same type and instance value may be included to represent a list of Alternative PGW-C/SMF IP Addresses. (NOTE) | IP Address | 0 |
| Alternative PGW-C/SMF FQDN | CO | This IE shall be included by the source MME if available. When present, this IE shall contain alternative PGW-C/SMF FQDN within the PGW-C/SMF set to which the PGW-C/SMF belongs. Several IEs with the same type and instance value may be included to represent a list of Alternative PGW-C/SMF FQDNs. (NOTE) | PGW FQDN | 1 |
| Group Id | CO | This IE shall be included by the source MME if available. When present, this IE shall identify the group to which the PDN connection pertains (see clause 31.6 of 3GPP TS 23.007 [17]). | Group Id | 0 |
| NOTE: Either the PGW Set FQDN IE, the Alternative PGW-C/SMF IP Address IE, or the Alternative PGW-C/SMF FQDN IE shall be present. |  |  |  |  |

* * * * Next of Change * * * *

7.3.9	Identification Response

The old SGSN/MME/AMF shall send an Identification Response message to the new MME/SGSN/AMF as a response to a previous Identification Request message over S3/S10/S16/N26 interface.

Table 7.3.9-1 specifies the presence requirements and conditions of the IEs in the message.

For Intra Domain Connection of RAN Nodes to Multiple CN Nodes, if an old SGSN within an SGSN pool receives an Identification Request message that contains the optional parameter Address for Control Plane, the old SGSN shall use this address as destination IP address of the Identification Response message.

Possible Cause values are specified in Table 8.4-1. Message specific cause values are:

- "P-TMSI Signature mismatch"

- "User authentication failed"

Only the Cause information element shall be included in the response if the Cause contains another value than "Request accepted".

Table 7.3.9-1: Information Elements in an Identification Response

| Information elements | P | Condition / Comment | IE Type | Ins. |
| --- | --- | --- | --- | --- |
| Cause | M |  | Cause | 0 |
| IMSI | C | This IE shall be included if the Cause contains the value "Request accepted". | IMSI | 0 |
| MME/SGSN UE MM Context | C | This IE shall be included if the integrity check or the P-TMSI signature check for the Attach Request succeeds. | MM Context | 0 |
| Trace Information | CO | This IE shall be included over S10/S16/S3 when session trace is active for this IMSI/IMEI. | Trace Information | 0 |
| UE Usage Type | CO | This IE shall be set to the subscribed UE Usage Type, if received from the HSS, and sent by the old MME/SGSN on the S3/S10/S16 interfaces if the old MME/SGSN supports the Dedicated Core Networks specified in TS 23.401 [3]. It shall also be sent by the AMF on the N26 interface, if received from the UDM. If the UE Usage Type is not available in the old MME/SGSN/AMF, the length field of this IE shall be set to 0. See NOTE 1. | Integer Number | 0 |
| Monitoring Event Information | CO | This IE shall be sent by the source MME/S4-SGSN on the S3/S10/S16 interfaces if monitoring events are to be continued in the target MME/S4-SGSN. More than one IE with this type and instance values may be included to represent multiple monitoring events. | Monitoring Event Information | 0 |
| Monitoring Event Extension Information | CO | This IE shall be sent by the source MME/S4-SGSN on the S3/S10/S16 interfaces if monitoring event extensions are to be sent to the target MME/S4-SGSN. More than one IE with this type and instance values may be included to represent multiple monitoring event extensions. | Monitoring Event Extension Information | 0 |
| Extended Trace Information | C | This IE shall be included over N26 when session trace is active for this user. | Extended Trace Information | 0 |
| Trace Collection Entity URI | O | This IE may be present over S10/S16/S3/N26 when session trace is active for this user and if the Trace Information or Extended Trace Information IE is included. When present, this IE shall include the Trace Reporting Consumer URI for Streaming based Reporting as defined in 3GPP TS 32.422 [18]. If both the IP Address of the Trace Collection Entity and Trace Collection Entity URI are present, the Trace Collection Entity URI shall take the higher precedence. | URI | 0 |
| Private Extension | O |  | Private Extension | VS |
| NOTE 1: A UE Usage Type IE with the length field equal to 0 is used for the receiver to differentiate the case where the sender does not support the Dedicated Core Network feature from the case where the sender supports the Dedicated Core Network feature but no UE Usage type was received in UE's subscription. |  |  |  |  |

* * * * Next of Change * * * *

7.12.1	Trace Session Activation

The Trace Session Activation message shall be sent on S11/S4 by the MME/SGSN to the SGW, on S2a/S2b by the TWAN/ePDG to the PGW, and on S5/S8 by the SGW to the PGW when session trace is activated for a particular IMSI or IMEI for a UE that is attached and active or attached and idle.

Table 7.12.1-1 specifies the presence of the IEs in the message.

Table 7.12.1-1: Information Elements in a Trace Session Activation

| Information elements | P | Condition / Comment | IE Type | Ins. |
| --- | --- | --- | --- | --- |
| IMSI | C | The MME/SGSN shall include the IMSI in the message on the S11/S4 interface except for the case: - If the UE is emergency or RLOS attached and the UE is UICCless. The IMSI shall be included in the message on the S11/S4 interface but not used as an identifier - if UE is emergency or RLOS attached but IMSI is not authenticated. The SGW shall forward this IE to the PGW on S5/S8 if received on S11/S4. The TWAN/ePDG shall include this IE on the S2a/S2b interface except for the case: - If the UE is emergency attached and the UE is UICCless. The IMSI shall be included in the message on the S2a/S2b interface if available, but not used as an identifier - if UE is emergency attached but IMSI is not authenticated. | IMSI | 0 |
| Trace Information | M |  | Trace Information | 0 |
| ME Identity (MEI) | C | The MME/SGSN shall include the ME Identity (MEI) IE on the S11/S4 interface: - If the UE is emergency attached and the UE is UICCless; or - If the UE is emergency attached and the IMSI is not authenticated. In other cases, the MME shall include the ME Identity (MEI) IE on the S11 interface, if available. The SGW shall forward this IE to the PGW on S5/S8 if received on S11/S4. | MEI | 0 |
|  | CO | The MME/SGSN shall include the ME Identity (MEI) IE on the S11 interface: - If the UE is RLOS attached and the UE is UICCless; or - If the UE is RLOS attached and the IMSI is not authenticated. |  |  |
|  | CO | The TWAN/ePDG shall include the ME Identity (MEI) IE on the S2a/S2b interface: - If the UE is emergency attached and the UE is UICCless - If the UE is emergency attached and the IMSI is not authenticated In other cases, the TWAN/ePDG shall include the ME Identity (MEI) IE on the S2a/S2b interface, if available. |  |  |
| Trace Collection Entity URI | O | This IE may be present to include the Trace Reporting Consumer URI for Streaming based Reporting as defined in 3GPP TS 32.422 [18]. If both the IP Address of the Trace Collection Entity and Trace Collection Entity URI are present, the Trace Collection Entity URI shall take the higher precedence. | URI | 0 |

* * * * Next of Change * * * *

8.1	Information Element Types

A GTP control plane (signalling) message may contain several information elements. In order to have forward compatible type definitions for the GTPv2 information elements, all of them shall be TLIV (Type, Length, Instance, Value) coded. GTPv2 information element type values are specified in the Table 8.1-1. The last column of this table indicates whether the information element is:

-	Fixed Length: the IE has a fixed set of fields, and a fixed number of octets.

-	Variable Length: the IE has a fixed set of fields, and has a variable number of octets.
For example, the last octets may be numbered similar to "5 to (n+4)". In this example, if the value of the length field, n, is 0, then the last field is not present.

-	Extendable: the IE has a variable number of fields, and has a variable number of octets.
The last fields are typically specified with the statement: "These octet(s) is/are present only if explicitly specified". The legacy receiving entity shall ignore the unknown octets.

In order to improve the efficiency of troubleshooting, it is recommended that the information elements should be arranged in the signalling messages as well as in the grouped IEs, according to the order the information elements are listed in the message definition table or grouped IE definition table in clause 7. However the receiving entity shall be prepared to handle the messages with information elements in any order.

Within information elements, certain fields may be described as spare. These bits shall be transmitted with the value set to 0. To allow for future features, the receiver shall not evaluate these bits. GTPv2-C information elements that have similar semantics in GTPv1-C shall be converted into GTPv1-C format, as specified in TS 29.060 [4], before sending them to a pre-R8 GSN.

Table 8.1-1: Information Element types for GTPv2

| IE Type value (Decimal) | Information elements | Comment / Reference | Number of Fixed Octets |
| --- | --- | --- | --- |
| 0 | Reserved |  |  |
| 1 | International Mobile Subscriber Identity (IMSI) | Variable Length / 8.3 | Not Applicable |
| 2 | Cause | Variable Length / 8.4 | Not Applicable |
| 3 | Recovery (Restart Counter) | Variable Length / 8.5 | Not Applicable |
| 4 to 34 | Reserved for S101 interface | See 3GPP TS 29.276 [14] | See 3GPP TS 29.276 [14] |
| 35 to 50 | Reserved for S121 interface | See 3GPP TS 29.276 [14] | See 3GPP TS 29.276 [14] |
| 51 | STN-SR | See 3GPP TS 29.280 [15] | See 3GPP TS 29.280 [15] |
| 56 | SRVCC Cause | See 3GPP TS 29.280 [15] | See 3GPP TS 29.280 [15] |
| 52 to 70 | Reserved for Sv interface | See 3GPP TS 29.280 [15] | See 3GPP TS 29.280 [15] |
| 71 | Access Point Name (APN) | Variable Length / 8.6 | Not Applicable |
| 72 | Aggregate Maximum Bit Rate (AMBR) | Fixed Length / 8.7 | 8 |
| 73 | EPS Bearer ID (EBI) | Extendable / 8.8 | 1 |
| 74 | IP Address | Variable Length / 8.9 | Not Applicable |
| 75 | Mobile Equipment Identity (MEI) | Variable Length / 8.10 | Not Applicable |
| 76 | MSISDN | Variable Length / 8.11 | Not Applicable |
| 77 | Indication | Extendable / 8.12 | 2 |
| 78 | Protocol Configuration Options (PCO) | Variable Length / 8.13 | Not Applicable |
| 79 | PDN Address Allocation (PAA) | Variable Length / 8.14 | Not Applicable |
| 80 | Bearer Level Quality of Service (Bearer QoS) | Extendable / 8.15 | 22 |
| 81 | Flow Quality of Service (Flow QoS) | Extendable / 8.16 | 21 |
| 82 | RAT Type | Extendable / 8.17 | 1 |
| 83 | Serving Network | Extendable / 8.18 | 3 |
| 84 | EPS Bearer Level Traffic Flow Template (Bearer TFT) | Variable Length / 8.19 | Not Applicable |
| 85 | Traffic Aggregation Description (TAD) | Variable Length / 8.20 | Not Applicable |
| 86 | User Location Information (ULI) | Extendable / 8.21 | "f+4-4" (See Figure 8.21-1) |
| 87 | Fully Qualified Tunnel Endpoint Identifier (F-TEID) | Extendable / 8.22 | 9/21/25 |
| 88 | TMSI | Variable Length / 8.23 | Not Applicable |
| 89 | Global CN-Id | Variable Length / 8.24 | Not Applicable |
| 90 | S103 PDN Data Forwarding Info (S103PDF) | Variable Length / 8.25 | Not Applicable |
| 91 | S1-U Data Forwarding Info (S1UDF) | Variable Length/ 8.26 | Not Applicable |
| 92 | Delay Value | Extendable / 8.27 | 1 |
| 93 | Bearer Context | Extendable / 8.28 | Not Applicable |
| 94 | Charging ID | Extendable / 8.29 | 4 |
| 95 | Charging Characteristics | Extendable / 8.30 | 2 |
| 96 | Trace Information | Variable Length / 8.31 | Not Applicable |
| 97 | Bearer Flags | Extendable / 8.32 | 1 |
| 98 | Reserved |  |  |
| 99 | PDN Type | Extendable / 8.34 | 1 |
| 100 | Procedure Transaction ID | Extendable / 8.35 | 1 |
| 101 | Reserved |  |  |
| 102 | Reserved |  |  |
| 103 | MM Context (GSM Key and Triplets) | Extendable / 8.38 | "r+1-4" (See Figure 8.38-1) |
| 104 | MM Context (UMTS Key, Used Cipher and Quintuplets) | Extendable / 8.38 | "r+1-4" (See Figure 8.38-2) |
| 105 | MM Context (GSM Key, Used Cipher and Quintuplets) | Extendable / 8.38 | "r+1-4" (See Figure 8.38-3) |
| 106 | MM Context (UMTS Key and Quintuplets) | Extendable / 8.38 | "r+1-4" (See Figure 8.38-4) |
| 107 | MM Context (EPS Security Context, Quadruplets and Quintuplets) | Extendable / 8.38 | "s+64-4" (See Figure 8.38-5) |
| 108 | MM Context (UMTS Key, Quadruplets and Quintuplets) | Extendable / 8.38 | "r+1-4" (See Figure 8.38-6) |
| 109 | PDN Connection | Extendable / 8.39 | Not Applicable |
| 110 | PDU Numbers | Extendable / 8.40 | 9 |
| 111 | P-TMSI | Variable Length / 8.41 | Not Applicable |
| 112 | P-TMSI Signature | Variable Length / 8.42 | Not Applicable |
| 113 | Hop Counter | Extendable / 8.43 | 1 |
| 114 | UE Time Zone | Extendable / 8.44 | 2 |
| 115 | Trace Reference | Fixed Length / 8.45 | 6 |
| 116 | Complete Request Message | Variable Length / 8.46 | Not Applicable |
| 117 | GUTI | Variable Length / 8.47 | Not Applicable |
| 118 | F-Container | Variable Length / 8.48 | Not Applicable |
| 119 | F-Cause | Variable Length / 8.49 | Not Applicable |
| 120 | PLMN ID | Variable Length / 8.50 | Not Applicable |
| 121 | Target Identification | Variable Length / 8.51 | Not Applicable |
| 122 | Reserved |  |  |
| 123 | Packet Flow ID | Variable Length / 8.53 | Not Applicable |
| 124 | RAB Context | Fixed Length / 8.54 | 9 |
| 125 | Source RNC PDCP Context Info | Variable Length / 8.55 | Not Applicable |
| 126 | Port Number | Extendable / 8.56 | 2 |
| 127 | APN Restriction | Extendable / 8.57 | 1 |
| 128 | Selection Mode | Extendable / 8.58 | 1 |
| 129 | Source Identification | Variable Length / 8.59 | Not Applicable |
| 130 | Reserved |  |  |
| 131 | Change Reporting Action | Variable Length / 8.61 | Not Applicable |
| 132 | Fully Qualified PDN Connection Set Identifier (FQ-CSID) | Extendable / 8.62 | "q+1-4" (See Figure 8.62-1) |
| 133 | Channel needed | Variable Length / 8.63 | Not Applicable |
| 134 | eMLPP Priority | Variable Length / 8.64 | Not Applicable |
| 135 | Node Type | Extendable / 8.65 | 1 |
| 136 | Fully Qualified Domain Name (FQDN) | Variable Length / 8.66 | Not Applicable |
| 137 | Transaction Identifier (TI) | Variable Length / 8.68 | Not Applicable |
| 138 | MBMS Session Duration | Extendable / 8.69 | 3 |
| 139 | MBMS Service Area | Variable Length / 8.70 | Not Applicable |
| 140 | MBMS Session Identifier | Extendable / 8.71 | 1 |
| 141 | MBMS Flow Identifier | Extendable / 8.72 | 2 |
| 142 | MBMS IP Multicast Distribution | Extendable / 8.73 | "m+1-4" (See Figure 8.73-1) |
| 143 | MBMS Distribution Acknowledge | Extendable / 8.74 | 1 |
| 144 | RFSP Index | Fixed Length / 8.77 | 2 |
| 145 | User CSG Information (UCI) | Extendable / 8.75 | 8 |
| 146 | CSG Information Reporting Action | Extendable / 8.76 | 1 |
| 147 | CSG ID | Extendable / 8.78 | 4 |
| 148 | CSG Membership Indication (CMI) | Extendable / 8.79 | 1 |
| 149 | Service indicator | Fixed Length / 8.80 | 1 |
| 150 | Detach Type | Fixed Length / 8.81 | 1 |
| 151 | Local Distiguished Name (LDN) | Variable Length / 8.82 | Not Applicable |
| 152 | Node Features | Extendable / 8.83 | 1 |
| 153 | MBMS Time to Data Transfer | Extendable / 8.84 | 1 |
| 154 | Throttling | Extendable / 8.85 | 2 |
| 155 | Allocation/Retention Priority (ARP) | Extendable / 8.86 | 1 |
| 156 | EPC Timer | Extendable / 8.87 | 1 |
| 157 | Signalling Priority Indication | Extendable / 8.88 | 1 |
| 158 | Temporary Mobile Group Identity (TMGI) | Extendable / 8.89 | 6 |
| 159 | Additional MM context for SRVCC | Extendable / 8.90 | "e-4" (See Figure 8.90-1) |
| 160 | Additional flags for SRVCC | Extendable / 8.91 | 1 |
| 161 | Reserved |  |  |
| 162 | MDT Configuration | Extendable / 8.93 | "q-4" (See Figure 8.93-1) |
| 163 | Additional Protocol Configuration Options (APCO) | Extendable / 8.94 | "m-4" (See Figure 8.94-1) |
| 164 | Absolute Time of MBMS Data Transfer | Extendable / 8.95 | 8 |
| 165 | H(e)NB Information Reporting | Extendable / 8.96 | 1 |
| 166 | IPv4 Configuration Parameters (IP4CP) | Extendable / 8.97 | 5 |
| 167 | Change to Report Flags | Extendable / 8.98 | 1 |
| 168 | Action Indication | Extendable / 8.99 | 1 |
| 169 | TWAN Identifier | Extendable / 8.100 | "k+6-4" (See Figure 8.100-1) |
| 170 | ULI Timestamp | Extendable / 8.101 | 4 |
| 171 | MBMS Flags | Extendable / 8.102 | 1 |
| 172 | RAN/NAS Cause | Extendable / 8.103 | "m-4" (See Figure 8.103-1) |
| 173 | CN Operator Selection Entity | Extendable / 8.104 | 1 |
| 174 | Trusted WLAN Mode Indication | Extendable / 8.105 | 1 |
| 175 | Node Number | Extendable / 8.106 | "p-4" (See Figure 8.106-1) |
| 176 | Node Identifier | Extendable / 8.107 | "q-4" (See Figure 8.107-1) |
| 177 | Presence Reporting Area Action | Extendable / 8.108 | "t-4" (See Figure 8.108-1) |
| 178 | Presence Reporting Area Information | Extendable / 8.109 | 4 |
| 179 | TWAN Identifier Timestamp | Extendable / 8.110 | 4 |
| 180 | Overload Control Information | Extendable / 8.111 | Not Applicable |
| 181 | Load Control Information | Extendable / 8.112 | Not Applicable |
| 182 | Metric | Fixed Length / 8.113 | 1 |
| 183 | Sequence Number | Fixed Length / 8.114 | 4 |
| 184 | APN and Relative Capacity | Extendable / 8.115 | "m-4" (See Figure 8.115 |
| 185 | WLAN Offloadability Indication | Extendable / 8.116 | 1 |
| 186 | Paging and Service Information | Extendable / 8.117 | m-4 (See Figure 8.117-1) |
| 187 | Integer Number | Variable / 8.118 | Not Applicable |
| 188 | Millisecond Time Stamp | Extendable / 8.119 | 6 |
| 189 | Monitoring Event Information | Extendable / 8.120 | "k+2-4" (See Figure 8.120-1) |
| 190 | ECGI List | Extendable / 8.121 | "m*7+2" (See Figure 8.121-1) |
| 191 | Remote UE Context | Extendable / 8.122 | Not Applicable |
| 192 | Remote User ID | Extendable / 8.123 | "c-4" (see Figure 8.123-1) |
| 193 | Remote UE IP information | Variable Length / 8.124 | Not Applicable |
| 194 | CIoT Optimizations Support Indication | Extendable / 8.125 | 1 |
| 195 | SCEF PDN Connection | Extendable / 8.126 | Not Applicable |
| 196 | Header Compression Configuration | Extendable / 8.127 | 4 |
| 197 | Extended Protocol Configuration Options (ePCO) | Variable Length / 8.128 | Not Applicable |
| 198 | Serving PLMN Rate Control | Extendable / 8.129 | 4 |
| 199 | Counter | Extendable / 8.130 | 5 |
| 200 | Mapped UE Usage Type | Extendable / 8.131 | 2 |
| 201 | Secondary RAT Usage Data Report | Extendable / 8.132 | 27 |
| 202 | UP Function Selection Indication Flags | Extendable / 8.133 | 1 |
| 203 | Maximum Packet Loss Rate | Extendable / 8.134 | 1 |
| 204 | APN Rate Control Status | Extendable / 8.135 | 20 |
| 205 | Extended Trace Information | Extendable / 8.136 | "r-4" (see figure 8.136-1) |
| 206 | Monitoring Event Extension Information | Extendable / 8.137 | "k-4" (See Figure 8.137-1) |
| 207 | Additional RRM Policy Index | Fixed Length / 8.138 | 4 |
| 208 | V2X Context | Extendable / 8.139 | Not Applicable |
| 209 | PC5 QoS Parameters | Extendable / 8.140 | Not Applicable |
| 210 | Services Authorized | Extendable / 8.141 | 2 |
| 211 | Bit Rate | Extendable / 8.142 | 4 |
| 212 | PC5 QoS Flow | Extendable / 8.143 | 11 |
| 213 | SGi PtP Tunnel Address | Extendable / 8.144 | 1 |
| 214 | PGW Change Info | Extendable / 8.145 | Not Applicable |
| 215 | PGW FQDN | Extendable / 8.146 | m-4 |
| 216 | Group Id | Variable Length / 8.147 | Not Applicable |
| 217 | PSCell ID | Fixed Length / 8.148 | 8 |
| 218 | UP Security Policy | Extendable / 8.149 | 1 |
| 219 | Alternative IMSI | Variable Length / 8.150 | Not Applicable |
| 220 | NF Instance ID | Fixed Length / 8.151 | 36 |
| 221 | Timer in Seconds | Extendable / 8.152 | 3 |
| 222 to 253 | Spare. For future use. |  |  |
| 254 | Special IE type for IE Type Extension | See NOTE 2 | Not Applicable |
| 255 | Private Extension | Variable Length / 8.67 | Not Applicable |
| X | URI | Variable Length / 8.Y | Not Applicable |
| Z to 65535 | Spare. For future use. |  |  |
| NOTE 1: The size of the TLI (Type, Length and Instance) fields, i.e "4" octets, has been subtracted from the number of the fixed octets of the "Fixed Length" and "Extendable" IEs. Hence for some of the "Extendable" IEs, for which the length is defined in terms of variable number of octets, "4" is explicitly subtracted while defining the fixed number of octets. E.g. Length of User Location Information is defined as "f+4" and fixed number of octets for the same is defined as "f+4-4". NOTE 2: The IE Type value 254 indicates that the IE Type shall be further identified by an IE Type Extension field; see clause 8.2.1A. A GTP-C entity which does not support any IE Type encoded with an IE Type Extension field shall ignore an IE received with the IE Type value 254. |  |  |  |

* * * * Next Change * * * *

8.Y	URI

The URI IE type shall be encoded as shown in Figure 8.Y-1.

| . |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = X (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to n+4 | URI |  |  |  |  |  |  |  |  |

Figure 8.Y-1: URI

The URI field in octets 5 to (n+4) shall contain an octet string representing the URI formatted according to IETF RFC 3986 [X].

* * * * End of Changes * * * *