3GPP TSG-CT WG4 Meeting #126	C4-245411

# Orlando, U.S; 18th – 22nd November 2024											was C4-245046

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.008 | CR | 0589 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Time Reference Information Distribution Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_TIME_SUB_EPS |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR implements the stage 2 requirements approved at SA#105 (S2-2409198/SP-241272) on subscription control for reference time distribution in EPS. According to S2-2409198/SP-241272, CT4#125 agreed CR#0862 to TS 29.272 which proposes to add the Time Reference Information Distribution Indication in the Subscription-Data-Flags. Therefore, it is necessary to include this indication to the subscription data in TS 23.008. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes a new clause for Time Reference Information Distribution Indication and adds the indication to Table 5.2A-1 (Overview of data used for PS Network Access Mode (EPS 3GPP access)). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements on supporting subscription control for reference time distribution in EPS cannot be supported.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2.13.xxx (new), 5.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 2.13.xxx	Time-Reference-Information-Distribution-Indication

Time-Reference-Information-Distribution-Indication is defined in 3GPP TS 29.272 [81]. It indicates whether the UE is subscribed to receive time reference information in access stratum.

Time-Reference-Information-Distribution-Indication is permanent subscriber data and conditionally stored in the HSS and MME.

* * * Next Change * * * *

## 5.2A	PS Network Access Mode Storage (EPS)

Table 5.2A-1: Overview of data used for PS Network Access Mode (EPS 3GPP access)

| PARAMETER |  | Clause |  | HSS |  | VLR (see note3) |  | S4-SGSN |  | MME |  | S-GW |  | PDN-GW |  | TYPE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMSI |  | 2.1.1.1 |  | M |  | C |  | C |  | C |  | C |  | C |  | P |  |  |
| Network Access Mode |  | 2.1.1.2 |  | M |  | - |  | - |  | C (see note 1) |  | - |  | - |  | P |  |  |
| IMSI Unauthenticated indicator |  | 2.1.1.3 |  | - |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| International MS ISDN number |  | 2.1.2 |  | C |  | - |  | C |  | C |  | C |  | C |  | P |  |  |
| P-TMSI |  | 2.1.5 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| TLLI |  | 2.1.6 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| Random TLLI |  | 2.1.7 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| IMEI |  | 2.1.9 |  | C |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| External Identifier Set |  | 2.1.10 |  | C |  | - |  | - |  | - |  | - |  | - |  | P |  |  |
| UE radio capability ID |  | 2.1.11 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| IMEISV |  | 2.2.3 |  | C |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| RAND/SRES and Kc |  | 2.3.1 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| RAND, XRES, CK, IK, AUTN |  | 2.3.2 |  | M |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| RAND, XRES, KASME, AUTN |  | 2.3.2 |  | M |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| Ciphering Key Sequence Number |  | 2.3.3 |  | C |  | - |  | M |  | - |  | - |  | - |  | T |  |  |
| Key Set Identifier (KSI) |  | 2.3.4 |  | - |  | - |  | M |  | - |  | - |  | - |  | T |  |  |
| KSIASME |  | 2.3.4 |  | - |  | - |  |  |  | M |  | - |  | - |  | T |  |  |
| Selected Ciphering Algorithm |  | 2.3.5 |  | - |  | - |  | M |  | - |  | - |  | - |  | T |  |  |
| Current Kc |  | 2.3.6 |  | - |  | - |  | M |  | - |  | - |  | - |  | T |  |  |
| P-TMSI Signature |  | 2.3.7 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| Key Status |  | 2.3.9 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| Routing Area Identity |  | 2.4.3 |  | - |  | - |  | M |  | - |  | - |  | - |  | T |  |  |
| IWF number |  | 2.4.8.3 |  | C |  | - |  | - |  | - |  | - |  | - |  | T |  |  |
| RSZI Lists |  | 2.4.11.1 |  | C |  | - |  |  |  | - |  | - |  | - |  | P |  |  |
| Zone Code List |  | 2.4.11.2 |  | - |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| SGSN area restricted Flag |  | 2.4.14 |  | M |  | - |  |  |  | - |  | - |  | - |  | T |  |  |
| RA not allowed flag |  | 2.4.14a |  | - |  | - |  | M |  | - |  | - |  | - |  | T |  |  |
| TA not allowed flag |  | 2.4.14b |  | - |  | - |  |  |  | M |  | - |  | - |  | T |  |  |
| Roaming Restricted in the SGSN due to unsupported feature |  | 2.4.15.3 |  | M |  | - |  | M |  | - |  | - |  | - |  | T |  |  |
| Roaming Restricted in the MME due to unsupported feature |  | 2.4.15.3a |  | M |  | - |  | - |  | M |  | - |  | - |  | T |  |  |
| Cell Global Identity or Service Area ID |  | 2.4.16 |  | - |  | - |  | C |  | C |  | - |  | C |  | T |  |  |
| E-UTRAN Cell Global ID |  | 2.4.16A |  | - |  | C (see note 2) |  | - |  | C |  | - |  | C |  | T |  |  |
| Access Restriction Data |  | 2.4.18 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| Closed Subscriber Group Information |  | 2.4.22 |  | C |  | C |  | C |  | C |  | - |  | - |  | P |  |  |
| IAB-Operation Allowed |  | 2.4.27 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Confirmed by Radio Contact indicator |  | 2.7.4.1 |  | - |  | C |  | - |  | - |  | - |  | - |  | T |  |  |
| Subscriber Data Confirmed by HLR/HSS Indicator |  | 2.7.4.2 |  | - |  | - |  | M |  | M |  | - |  | - |  | T |  |  |
| Location Info Confirmed by HLR/HSS Indicator |  | 2.7.4.3 |  | - |  | - |  | M |  | M |  | - |  | - |  | T |  |  |
| VLR-Reliable Indicator |  | 2.7.4.5 |  | - |  | - |  | C(see note 5) |  | C (see note 2) |  | - |  | - |  | T |  |  |
| UE purged in SGSN flag |  | 2.7.6 |  | M |  | - |  | - |  | - |  | - |  | - |  | T |  |  |
| UE purged in MME flag |  | 2.7.6A |  | M |  | - |  | - |  | - |  | - |  | - |  | T |  |  |
| URRP-MME |  | 2.7.9.1 |  | C |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| URRP-SGSN |  | 2.7.9.2 |  | C |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| Subscriber Status |  | 2.8.1 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| Barring of outgoing calls |  | 2.8.2.1 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| Barring of roaming |  | 2.8.2.3 |  | C |  | - |  | - |  | - |  | - |  | - |  | P |  |  |
| Barring of Packet Oriented Services |  | 2.8.2.8 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| ODB PLMN-specific data |  | 2.8.3 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| PS and SMS Only |  | 2.10.7 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| SMS in SGSN Allowed |  | 2.10.8 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| Trace Activated in SGSN |  | 2.11.7 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| Trace Reference 2 |  | 2.11.9 |  | C |  | C |  | C |  | C |  | C |  | C |  | P |  |  |
| Trace depth |  | 2.11.10 |  | C |  | C |  | C |  | C |  | C |  | C |  | P |  |  |
| List of NE types to trace |  | 2.11.11 |  | C |  | C |  | C |  | C |  | - |  | - |  | P |  |  |
| Triggering events |  | 2.11.12 |  | C |  | C |  | C |  | C |  | C |  | C |  | P |  |  |
| List of interfaces to trace |  | 2.11.13 |  | C |  | C |  | C |  | C |  | C |  | C |  | P |  |  |
| IP address of trace collection entity |  | 2.11.14 |  | C |  | C |  | C |  | C |  | C |  | C |  | P |  |  |
| MDT-Configuration |  | 2.11.15 |  | C |  | C |  | C |  | C |  | - |  | - |  | P |  |  |
| MDT User Consent |  | 2.11.16 |  | C |  | C |  | C |  | C |  | - |  | - |  | P |  |  |
| Access Point Name (APN). |  | 2.13.6 |  | M |  | - |  | M |  | M |  | M |  | M |  | P |  |  |
| MME name |  | 2.13.26 |  | M |  | C |  | - |  | - |  | - |  | - |  | T |  |  |
| VLR name |  | 2.13.27 |  | - |  | - |  | - |  | C (see note 2) |  | - |  | - |  | T |  |  |
| NEAF |  | 2.13.28 |  | - |  | - |  | - |  | C (see note 2) |  | - |  | - |  | T |  |  |
| UE level APN-OI-Replacement |  | 2.13.29 |  | C |  | - |  |  |  | C |  | - |  | - |  | P |  |  |
| Subscribed UE-AMBR |  | 2.13.30 |  | M |  | - |  | M |  | M |  | - |  | - |  | P |  |  |
| Used UE-AMBR |  | 2.13.30A |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| APN-Configuration-Profile |  | 2.13.31 |  | M |  | - |  | M |  | M |  | - |  | - |  | P |  |  |
| Subscribed APN-AMBR |  | 2.13.32 |  | M |  | - |  | M |  | M |  | M |  | - |  | P |  |  |
| Used APN-AMBR |  | 2.13.32A |  | - |  | - |  | C |  | C |  | - |  | C |  | T |  |  |
| Subscribed-RFSP-ID |  | 2.13.33 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| GUTI |  | 2.13.34 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| ME identity (IMEISV) |  | 2.13.35 |  | C |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| Selected NAS Algorithm |  | 2.13.36 |  | - |  | - |  | - |  | M |  | - |  | - |  | T |  |  |
| Selected AS Algorithm |  | 2.13.37 |  | - |  | - |  | - |  | M |  | - |  | - |  | T |  |  |
| Context Identifier |  | 2.13.38 |  | M |  | - |  | M |  | M |  | M |  | M |  | P |  |  |
| PDN Address |  | 2.13.39 |  | C |  | - |  | C |  | C |  | C |  | C |  | P/T (see note4) |  |  |
| VPLMN Address Allowed |  | 2.13.40 |  | M |  | - |  | M |  | M |  | - |  | - |  | P |  |  |
| PDN GW identity |  | 2.13.41 |  | C |  | - |  | C (see note 6) |  | C (see note 6) |  | - |  | - |  | P/T (see note4) |  |  |
| Tracking Area List |  | 2.13.42 |  | - |  | - |  | - |  | M |  | - |  | - |  | T |  |  |
| APN Restriction |  | 2.13.43 |  |  |  | - |  | C |  | C |  | C |  | C |  | P |  |  |
| APN in use |  | 2.13.44 |  | - |  | - |  | M |  | M |  | M |  | M |  | T |  |  |
| TAI |  | 2.13.45 |  | - |  | C (see note 2) |  | - |  | M |  | - |  | - |  | T |  |  |
| E-UTRAN Cell Identity Age |  | 2.13.46 |  | - |  | C (see note 2) |  | - |  | C |  | - |  | - |  | T |  |  |
| MME F-TEID for S11 (control plane) |  | 2.13.47 |  | - |  | - |  | - |  | C |  | C |  | - |  | T |  |  |
| MME UE S1AP ID |  | 2.13.48 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| S-GW F-TEID for S11 (control plane) |  | 2.13.49 |  | - |  | - |  | - |  | C |  | C |  | - |  | T |  |  |
| S4-SGSN F-TEID for S4 (control plane) |  | 2.13.50 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| S4-SGSN F-TEID for S4 (User plane) |  | 2.13.51 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| S-GW F-TEID for S5/S8 (control plane) |  | 2.13.52 |  | - |  | - |  | - |  | C |  | C |  | C |  | T |  |  |
| S-GW F-TEID for S1-U |  | 2.13.53 |  | - |  | - |  | - |  | C |  | C |  | - |  | T |  |  |
| S-GW F-TEID for S5/S8 (user plane) |  | 2.13.54 |  | - |  | - |  | - |  | - |  | C |  | C |  | T |  |  |
| eNodeB Address |  | 2.13.55 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| eNodeB UE S1AP ID |  | 2.13.56 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| eNodeB F-TEID for S1-U |  | 2.13.57 |  | - |  | - |  | - |  | C |  | C |  | - |  | T |  |  |
| E-UTRAN/UTRAN Key Set flag |  | 2.13.58 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| Selected CN operator id |  | 2.13.59 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| UE Radio Access Capability |  | 2.13.60 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| Location Change Report Required |  | 2.13.62 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| UE specific DRX parameters |  | 2.13.63 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| PDN GW F-TEID for S5/S8 (user plane) |  | 2.13.64 |  | - |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| PDN GW F-TEID for S5/S8 (control plane) |  | 2.13.65 |  | - |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| EPS Bearer ID |  | 2.13.66 |  | - |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| EPS Bearer QoS |  | 2.13.67 |  | - |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| EPS Subscribed QoS Profile |  | 2.13.67A |  | M |  | - |  | M |  | M |  | - |  | - |  | P |  |  |
| UL TFT |  | 2.13.68 |  | - |  | - |  | - |  | C |  | C |  | C |  | T |  |  |
| DL TFT |  | 2.13.69 |  | - |  | - |  | - |  | C |  | C |  | C |  | T |  |  |
| Charging Id |  | 2.13.70 |  | - |  | - |  | C |  | - |  | C |  | C |  | T |  |  |
| EPS PDN Connection Charging Characteristics |  | 2.13.71 |  | C |  | - |  | C |  | C |  | C |  | C |  | P |  |  |
| Default bearer |  | 2.13.72 |  | - |  | - |  | - |  | C |  | C |  | C |  | T |  |  |
| URRP-MME |  | 2.13.73 |  | C |  | - |  | - |  | C |  |  |  |  |  | T |  |  |
| RAT Type (Access Type) |  | 2.13.75 |  | C |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| Diameter Server Identity of the HSS |  | 2.13.99 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| SGSN name |  | 2.13.100 |  | M |  | - |  | - |  | - |  | - |  | - |  | T |  |  |
| S-GW F-TEID for S12 |  | 2.13.101 |  | - |  | - |  | - |  | - |  | C |  | - |  | T |  |  |
| RNC F-TEID for S12 |  | 2.13.102 |  | - |  | - |  | C |  | - |  | C |  | - |  | T |  |  |
| MME F-TEID for S3 |  | 2.13.103 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| S4-SGSN F-TEID for S3 |  | 2.13.104 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| PDN GW Allocation Type |  | 2.13.105 |  | M |  | - |  | - |  | M |  | - |  | - |  | P |  |  |
| S-GW F-TEID for S4 (control plane) |  | 2.13.106 |  | - |  | - |  | C |  | - |  | C |  | - |  | T |  |  |
| S-GW F-TEID for S4 (user plane) |  | 2.13.107 |  | - |  | - |  | C |  | - |  | C |  | - |  | T |  |  |
| RFSP-ID in Use |  | 2.13.108 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| APN level APN-OI-Replacement |  | 2.13.109 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| PDN Connection ID |  | 2.13.111 |  | - |  | - |  | - |  | - |  | C |  | C |  | T |  |  |
| MS Network Capability |  | 2.13.112 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| Voice Domain Preference and UE's Usage Setting |  | 2.13.113 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| SIPTO Permission |  | 2.13.114 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| SIPTO Local Network Permission |  | 2.13.114A |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| Subscribed Periodic RAU/TAU Timer |  | 2.13.115 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| MPS CS Priority |  | 2.13.120 |  | - |  | - |  | - |  | C |  | - |  | - |  | P |  |
| MPS EPS Priority |  | 2.13.121 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |
| LIPA Permission |  | 2.13.122 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| LIPA Allowed VPLMN List |  | 2.13.123 |  | C |  | - |  | - |  | - |  | - |  | - |  | P |  |  |
| VPLMN LIPA Allowed |  | 2.13.124 |  | - |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| Relay Node Indicator |  | 2.13.125 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Higher bitrates than 16Mbps flag |  | 2.13.127 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| H(e)NB Local IP Address |  | 2.13.131 |  | - |  | - |  | C |  | C |  | - |  | C |  | T |  |  |
| H(e)NB UDP Port Number |  | 2.13.132 |  | - |  | - |  | C |  | C |  | - |  | C |  | T |  |  |
| MME/S4 SGSN Identifier |  | 2.13.134 |  | - |  | - |  | - |  | - |  | C |  | C |  | T |  |  |
| SGW node name |  | 2.13.135 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| Co-located GGSN-PGW FQDN |  | 2.13.136 |  | - |  | - |  | C (see note 6) |  | C (see note 6) |  | - |  | - |  | T |  |  |
| GERAN Cell Identity Age |  | 2.13.137 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| UTRAN Service Area Identity Age |  | 2.13.138 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| Homogeneous Support of IMS Voice over PS Sessions |  | 2.13.141 |  | C |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| Signalling Priority Indication |  | 2.13.142 |  | - |  | - |  | C |  | C |  | C |  | C |  | T |  |  |
| Restoration Priority |  | 2.13.143 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| Presence Reporting Area Action |  | 2.13.145 |  | - |  | - |  | C |  | C |  | - |  | C |  | T |  |  |
| WLAN offloadability |  | 2.13.146 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| Expected UE Activity Behaviour |  | 2.13.147.2 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| Expected HO Interval |  | 2.13.147.3 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| Active Time Value for PSM |  | 2.13.148 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| Origination Time Stamp |  | 2.13.149 |  | - |  | - |  | - |  | - |  | - |  | C |  | T |  |  |
| DL Data Buffer Expiration Time |  | 2.13.150 |  | - |  | - |  | C |  | C |  | C |  | - |  | T |  |  |
| DL Buffering Suggested Packet Count |  | 2.13.151 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| Notify-on-available-after-DDN-failure flag |  | 2.13.152 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| IMSI-Group Identifier List |  | 2.13.153 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| UE Usage Type |  | 2.13.154 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| Remote UE Contexts |  | 2.13.156 |  | - |  | - |  | - |  | C |  | C |  | C |  | T |  |  |
| Extended idle mode DRX parameters |  | 2.13.157 |  | C |  | - |  | C |  | C |  | - |  | - |  | P/T (see note 7) |  |  |
| Delay Tolerant Connection Indication |  | 2.13.158 |  | - |  | - |  | C |  | C |  | - |  | C |  | P |  |  |
| Pending Network Initiated PDN Connection Signalling Indication |  | 2.13.159 |  | - |  | - |  | C |  | C |  | . |  | - |  | T |  |  |
| UE Radio Capability for Paging information |  | 2.13.160 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| Information on Recommended Cells and ENBs for Paging |  | 2.13.161 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| Paging Attempt Count |  | 2.13.162 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| User Plane Integrity Protection Indicator |  | 2.13.163 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| Non-IP-PDN-Type-Indicator |  | 2.13.164 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Non-IP-Data-Delivery-Mechanism |  | 2.13.165 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| SCEF-ID |  | 2.13.166 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| PDN Type |  | 2.13.167 |  | C |  | - |  | C |  | C |  | C |  | C |  | P |  |  |
| PDN-Connection-Restricted flag |  | 2.13.168 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Preferred-Data-Mode |  | 2.13.169 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| IOV_updates counter |  | 2.13.170 |  | - |  | - |  | C |  | - |  | - |  | - |  | T |  |  |
| Dedicated Core Network Identifier |  | 2.13.172 |  | - |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| Service Gap Time |  | 2.13.174 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Subscribed Additional RRM Policy Index |  | 2.13.175 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Additional RRM Policy Index in use |  | 2.13.176 |  | - |  | - |  | - |  | C |  | - |  | - |  | T |  |  |
| Ethernet-PDN-Type-Indicator |  | 2.13.177 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Time-Reference-Information-Distribution-Indication |  | 2.13.xxx |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Privacy Exception List |  | 2.16.1.1 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| GMLC Numbers |  | 2.16.1.2 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| MO-LR List |  | 2.16.1.3 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| Service Types |  | 2.16.1.4 |  | C |  | - |  | C |  | - |  | - |  | - |  | P |  |  |
| Broadcast Location Assistance Data Types |  | 2.16.7.1 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Subscribed Charging Characteristics |  | 2.19.1 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| ICS Indicator |  | 2.20.1 |  | C |  | C |  | C |  | C |  | - |  | - |  | P |  |  |
| STN-SR |  | 2.21.1 |  | C |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| UE SRVCC Capability |  | 2.21.2 |  | C |  | - |  | C |  | C |  | - |  | - |  | T |  |  |
| Subscribed vSRVCC |  | 2.21.3 |  | C |  | - |  | - |  | C |  | - |  | - |  | P |  |  |
| Additional MSISDN |  | 3.1.16 |  | C |  | - |  | C |  | C |  | - |  | - |  | P |  |  |
| NOTE 1: This parameter is relevant in the MME only when the SGs interface is installed. NOTE 2: Only is applicable if SGs interface is installed. It only indicates EPS related data to be stored and is only relevant to EPS subscribers registered in VLR. NOTE 3: The VLR column is applicable if SGs/Sv interface is installed. It only indicates EPS related data to be stored and is only relevant to EPS subscribers registered in VLR. NOTE 4: If Static IP address allocation provisioned in the subscriber profile in the HSS is chosen, PDN address is permanent data. NOTE 5: Only is applicable if Gs interface is installed. It only indicates EPS related data to be stored and is only relevant to EPS subscribers registered in VLR. NOTE 6: The MME/S4 SGSN stores either the PDN GW identity or the Co-located GGSN-PGW FQDN since they are identical information. NOTE 7: The eDRX cycle length may be stored in HSS as a subscribed parameter, and in that case, it is permanent data. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For special condition of storage see in clause 2. See clause 4 for explanation of M, C, T and P in table 5.2A-1.

Table 5.2A-2: Overview of data used for PS Network Access Mode (EPS non 3GPP access)

| PARAMETER | Clause | HSS | MME | S-GW | PDN-GW | ePDG | 3GPP AAA server | 3GPP AAA server Proxy | TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMSI | 2.1.1.1 | M | C | C | C | C | - | - | P |
| IMSI Unauthenticated indicator | 2.1.1.3 | - | C | C | C | C | - | - | T |
| International MS ISDN number | 2.1.2 | C | C | C | C | C | C | - | P |
| IMEI | 2.1.9 | C | - | - | C | C | C | - | T |
| RAND, XRES, CK, IK, AUTN | 2.3.2 | M | - | - | - | - | M | - | T |
| RAND, XRES, KASME, AUTN | 2.3.2 | M | - | - | - | - | M | - | T |
| Access Network Identity | 2.3.8 | C | - | - | - | - | C | - | T |
| Trace Reference 2 | 2.11.9 | C | - | - | C | - | C | - | P |
| Trace depth | 2.11.10 | C | - | - | C | - | C | - | P |
| List of NE types to trace | 2.11.11 | C | - | - | - | - | C | - | P |
| Triggering events | 2.11.12 | C | - | - | C | - | C | - | P |
| List of interfaces to trace | 2.11.13 | C | - | - | C | - | C | - | P |
| IP address of Trace Collection Entity | 2.11.14 | C | - | - | C | - | C | - | P |
| APN-Configuration-Profile | 2.13.31 | M | - | - | C | C | C | - | T |
| Subscribed APN-AMBR | 2.13.32 | M | - | - | C | C | C | - | P |
| Used APN-AMBR | 2.13.32A | - | - | - | C | - | - | - | T |
| ME Identity (IMEISV) | 2.13.35 | C | - | - | C | C | C | - | T |
| PDN Address | 2.13.39 | C | - | C | C | C | C | - | T/P (see Note) |
| VPLMN Address Allowed | 2.13.40 | M | C | - | - | C | C | - | P |
| PDN GW identity | 2.13.41 | M | C | - | - | C | C | - | P |
| APN in use | 2.13.44 | - | - | - | C | C | - | - | T |
| EPS Bearer ID | 2.13.66 | - | - | - | C | C | - | - | T |
| EPS Bearer QoS | 2.13.67 | - | - | - | C | C | - | - | T |
| EPS PDN Connection Charging Characteristics | 2.13.71 | C | - | - | C | C | C | - | P |
| RAT Type (Access Type) | 2.13.75 | C | - | C | C | C | C | - | T |
| Permanent User Identity | 2.13.79 | M | - | M | M | M | M | - | P |
| Mobility Capabilities | 2.13.80 | - | - | - | M | C | C | - | T |
| MAG IP address | 2.13.81 | - | - | - | - | - | C | - | T |
| Visited Network Identifier | 2.13.82 | C | - | - | C | C | C | - | T |
| EAP payload | 2.13.83 | - | - | - | - | - | C | - | P |
| MIP Subscriber profile | 2.13.86 | M | - | - | M | - | - | - | P |
| Uplink S5 GRE Key | 2.13.87 | - | C | C | C | - | - | - | T |
| Downlink S5 GRE Key | 2.13.88 | - | - | C | C | - | - | - | T |
| Uplink S8 GRE Key | 2.13.89 | - | C | C | C | - | - | - | T |
| Downlink S8 GRE Key | 2.13.90 | - | - | C | C | - | - | - | T |
| S2a GRE Keys | 2.13.91 | - | - | C | C | - | - | - | T |
| S2b GRE Keys | 2.13.92 | - | - | C | C | C | - | - | T |
| Mobile Node Identifier | 2.13.93 | - | - | C | C | - | - | - | T |
| IPv4 Default Router Address | 2.13.94 | - | - | C | C | - | - | - | T |
| Link-local address | 2.13.95 | - | - | C | C | - | - | - | T |
| Non 3GPP User Data | 2.13.96 | C | - | - | - | C | C | - |  |
| 3GPP AAA Server Identity | 2.13.97 | C | - | - | C | C | - | - | T |
| Selected IP mobility mode | 2.13.98 | - | - | - | C | C | C | - | T |
| Diameter Server Identity of HSS | 2.13.99 | - | C | - | - | - | C | - | T |
| Unauthenticated IMSI | 2.13.110 | - | - | C | C | - | - | - | T |
| PDN Connection ID | 2.13.111 | - | - | C | C | C | - | - | T |
| SIPTO Permission | 2.13.114 | C | - | - | - | - | C | - | P |
| ePDG F-TEID for S2b (control plane) | 2.13.116 | - | - | - | C | C | - | - | T |
| ePDG F-TEID for S2b (user plane) | 2.13.117 | - | - | - | C | C | - | - | T |
| PGW F-TEID for S2b (control plane) | 2.13.118 | - | - | - | C | C | - | - | T |
| PGW F-TEID for S2b (user plane) | 2.13.119 | - | - | - | C | C | - | - | T |
| Restricted RAT Types | 2.13.126 | C | - | - | - | - | C | - | P |
| UE Local IP Address | 2.13.129 | - | - | - | - | C | - | - | T |
| UE UDP Port Number | 2.13.130 | - | - | - | - | C | - | - | T |
| Default APN for Trusted WLAN | 2.13.139 | C | - | - | - | - | C | - | P |
| Access Information for Trusted WLAN | 2.13.140 | C | - | - | - | - | C | - | P |
| Origination Time Stamp | 2.13.149 | - | - | - | C | - | C | - | T |
| Emergency Indication | 2.13.155 | - | - | - | - | C | C | C | T |
| Emergency Info | 2.13.171 | C | C | - | - | C | C | - | T |
| Subscribed Charging Characteristics | 2.19.1 | M | - | - | - | C | C | - | P |
| ERP-Authorization | 2.24.2 | C | - | - | - | - | C | - | P |
| ERP Keying Material | 2.24.3 | - | - | - | - | - | C | C | T |
| ERP-Realm | 2.24.4 | - | - | - | - | - | C | C | P |
| Master session Key | 3.11.3. 5 | - | - | - | C | C | C | - | T |
| NOTE: If Static IP address allocation provisioned in the subscriber profile in the HSS is chosen, PDN address is permanent data. |  |  |  |  |  |  |  |  |  |

For special condition of storage see in clause 2. See clause 4 for explanation of M, C, T and P in table 5.2A-2.

Table 5.2.A-3 contains additional parameter to be hold when optimised handover to 3GPP2 is supported.

Table 5.2A-3: Overview of data used for PS Network Access Mode (optimized handover to 3GPP2)

| PARAMETER | Clause | HSS | MME | S-GW | PDN-GW | ePDG | 3GPP AAA server | 3GPP AAA server Proxy | TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Access Restriction Data | 2.4.18 | C | C | - | - | - | - | - | P |
| Barring of Packet Oriented Services | 2.8.2.8 | C | C | - | - | - | - | - | P |
| RAT Type | 2.13.75 | C | - | - | - | - | - | - | T |
| S101 HRPD access node IP address | 2.13.76 | - | C | - | - | - | - | - | T |
| S103 Forwarding Address | 2.13.77 | - | C | C | - | - | - | - | T |
| S103 GRE key(s) | 2.13.78 | - | C | C | - | - | - | - | T |

NOTE:	A UE may be simultaneously attached to EPS and have simultaneously active PDN connections or IP flows via different access systems (3GPP access and a non 3GPP access).

* * * * End of Changes * * * *