3GPP TSG-CT WG4 Meeting #126	C4-245456

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0886 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RTP Header Extension Additional Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, XRM |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RTP Header Extension Additional Information is currently not capturing latest agreements on the optional attribute fields that may be present in RTP Header Extension for PDU Set marking, as defined in Release 18 TS 26.522, V18.1.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Bits NPDSI and NPDS are added to the RTP Header Extension Additional Information. Bits NPDSI and NPDS indicate the presence of the optional attribute “Number of PDUs in the PDU Set [NPDS]” field in the RTP HE for PDU Set marking as specified in TS 26.522. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Lack of support for AF to indicate PCF/NEF correctly the ProtocolDescription with PDU Set marking - Lack of support for correct and complete UPF parsing of PDU Set marking RTP Header Extension. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.241 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 8.2.241	RTP Header Extension Additional Information

The RTP Header Extension Additional Information provides additional information on the RTP Header Extension. It shall be coded as depicted in Figure 8.2.241-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 349 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  |  | NPDSI | PSSAI | FI |  |
|  | 6 | Spare |  |  |  | NPDS | PSSA | Format |  |  |
|  | 7 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.241-1: RTP Extension Header Additional Information

The following flags are coded within Octet 5:

-	Bit 1 – FI (Format Indication): If this bit is set to "1", it indicates that the Format field in octet 6 indicates the RTP header extension format. If this bit is set to "0", the format field in octet 6 shall be coded as all "0" and be ignored by the receiver.

-	Bit 2 – PSSAI (PDU Set Size Active Indication): If this bit is set to "1", it indicates that the PSSA field in octet 6 indicates whether the PDU Set size in bytes is present in the RTP header extension of every RTP packet. If this bit is set to "0", the PSSA field in octet 6 shall be coded as all "0" and be ignored by the receiver.

-	Bit 3 – NPDSI (Number of PDUs in the PDU Set): If this bit is set to "1", it indicates that the NPDS field in octet 6 indicates whether the Number of PDUs in the PDU Set in bytes is present in the RTP header extension of every RTP packet. If this bit is set to "0", the NPDS field in octet 6 shall be coded as all "0" and be ignored by the receiver.

-	Bit 4 to 8 Spare, for future use and set to "0".

The Format field in octet 6 shall be encoded as defined in Table 8.2.241-1.

Table 8.2.241-1: Format field

| Value (Decimal) | Format field |
| --- | --- |
| 1 | 1-byte (short) format is used |
| 2 | 2-byte (long) format is used |
| 0, 3 | Spare, for future use. |

The PDU Set Size Active (PSSA) field in octet 6 shall be encoded as "1" if the PSSAI flag is set to 1 and the PDU Set Size is present in the RTP header extension of every RTP packet, and as "0" otherwise.

The Number of PDUs in the PDU Set (NPDS) field in octet 6 shall be encoded as "1" if the NPDSI flag is set to 1 and the Number of PDUs in the PDU Set is present in the RTP header extension of every RTP packet, and as "0" otherwise.

End of changes