3GPP TSG-CT WG4 Meeting #126	C4-245466

# Orlando, U.S; 18th – 22nd November 2024										Revision of C4-245404

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0893 | rev | 2 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of differentiated QoS handling for multiplexed media flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR 23.501 #5783 enables the support of differentiated QoS handling for multiplexed media flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is introduced to describe the support of traffic identification and differentiated QoS for multiplexed media flows. The SDF Filter IE is extended with (S)RTP Multiplexed Media Information that includes Synchronization Source (SSRC) and Payload Type (PT). A new UPF feature is defined to indicate the support by the UPF of traffic identification and differentiated QoS for multiplexed media flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 5.38.0, 5.38.x (new), 8.2.5, 8.2.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5783 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: - the SDF Filter IE is extended to support encoding multiple (S)RTP Multiplexed Media Information and multiple SSRCs. - correction on spare bits in Figure 8.2.5-1: SDF Filter - use of hard space in one reference Rev. 2: describe how the UPF shall interpret an (S)RTP Multiplexed Media Information not including any SSRC |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[3]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[4]	IETF RFC 768: "User Datagram Protocol".

[5]	IETF RFC 791: "Internet Protocol".

[6]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[7]	3GPP TS 23.203: "Policy and charging control architecture; Stage 2".

[8]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[9]	3GPP TS 29.274: "3GPP Evolved Packet System. Evolved GPRS Tunnelling Protocol for EPS (GTPv2)".

[10]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[11]	3GPP TS 29.213: "Policy and Charging Control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[13]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[14]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[15]	3GPP TS 22.153: "Multimedia Priority Service".

[16]	IETF RFC 4006: "Diameter Credit Control Application".

[17]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[18]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[19]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[20]	3GPP TS 33.107: "3G security; Lawful interception architecture and functions".

[21]	3GPP TS 29.251: "Gw and Gwn reference points for sponsored data connectivity".

[22]	IETF RFC 2474, "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[23]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[24]	3GPP TS 23.007: "Restoration procedures".

[25]	3GPP TS 29.303: "Domain Name System Procedures; Stage 3"

[26]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[27]	IETF RFC 1035: "Domain Names - Implementation and Specification".

[28]	3GPP TS 23.501:"System Architecture for the 5G System"

[29]	3GPP TS 23.502:"Procedures for the 5G System"

[30]	IEEE 802.1Q: "Virtual Bridged Local Area Networks"

[31]	IEEE 802.3: "IEEE Standard for Ethernet"

[32]	IETF RFC 826: "An Ethernet Address Resolution Protocol or Converting Network Protocol Addresses".

[33]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)". .

[34]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[35]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[36]	IETF RFC 4282: "The Network Access Identifier".

[37]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[38]	IETF RFC 3162: "RADIUS and IPv6".

[39]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[40]	3GPP TS 23.527: "5G System; Restoration procedures".

[41]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[42]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[43]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[44]	3GPP TS 23.503:"Policy and Charging Control Framework for the 5G System".

[45]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[46]	3GPP TS 29.512: "Session Management Policy Control Service, Stage 3".

[47]	3GPP TS 33.127: "Security; Lawful Interception (LI) architecture and functions".

[48]	3GPP TS 23.003: "Numbering, addressing and identification".

[49]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3".

[50]	3GPP TS 29.502: "5G System, Session Management Services; Stage 3".

[51]	Void

[52]	IETF RFC 2236: "Internet Group Management Protocol, Version 2".

[53]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[54]	IETF RFC 4604: "Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast".

[55]	IETF RFC 2710: "Multicast Listener Discovery (MLD) for IPv6".

[56]	Void

[57]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[58]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[59]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

[60]	IETF RFC 8803: "0-RTT TCP Convert Protocol".

[61]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[62]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses".

[63]	3GPP TS 24.539: "5G System (5GS); Network to TSN translator (TT) protocol aspects; Stage 3".

[64]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[65]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[66]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[67]	IETF RFC 2661: Layer Two Tunneling Protocol "L2TP"

[68]	IETF RFC 2868: RADIUS Attributes for Tunnel Protocol Support

[69]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[70]	Void

[71]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[72]	3GPP TS 23.247:" Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[73]	3GPP TS 29.564: "5G System; User Plane Function Services; Stage 3".

[74]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[75]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[76]	Void

[77]	Void

[78]	Void

[79]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[80]	3GPP TS 29.585: "5G System (5GS); Session Management Function (SMF) / Centralized User Configuration (CUC) to Access Network Talker Listener (AN-TL) and Core Network Talker Listener (CN-TL) protocol aspects; Stage 3".

[81]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)".

[82]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[83]	IETF RFC 3711: The Secure Real-time Transport Protocol (SRTP"

[84]	IETF RFC 6184: "RTP Payload Format for H.264 Video"

[85]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC)"

[86]	IETF RFC 8285: "A General Mechanism for RTP Header Extensions".

[87]	3GPP TS 32.279: "Charging management; 5G Multicast-broadcast Services charging; Stage 2".

[x1]		IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

[x2]		IETF RFC 5764: "Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[x3]		IETF RFC 7983: "Multiplexing Scheme Updates for Secure Real-time Transport Protocol (SRTP) Extension for Datagram Transport Layer Security (DTLS)".

[x4]		IETF RFC 8872: "Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams".

[x5]		IETF RFC 9143: "Negotiating Media Multiplexing Using the Session Description Protocol (SDP)".

[x6]		IETF RFC 9443: "Multiplexing Scheme Updates for QUIC".

* * * Next Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Associated PDU Session: A PDU Session associated to a multicast session that is used for 5GC Individual MBS traffic delivery method and for signalling related to a user's participation in a multicast session such as join and leave requests.

Associated QoS Flow: A unicast QoS Flow that belongs to the associated PDU Session and is used for 5GC Individual MBS traffic delivery method. The associated QoS Flow is mapped from a multicast QoS Flow in a multicast MBS session.

Common Tunnel EndPoint Identifier (C-TEID): See definition in clause 3.1 of 3GPP TS 29.281 [3]. It is used for the delivery of user plane traffic using multicast transport via a common GTP-U tunnel to NG-RAN nodes over N3mb and/or to UPFs over N19mb.

CP function: A node with a Control Plane function (see 3GPP TS 23.214 [2]) supporting one or more PFCP entities. A Control Plane function, i.e. a Control Plane Node, is identified by the Node ID that is set to either an FQDN or an IP address.

Low Layer Source Specific Multicast address (LLSSM): It is used to deliver user plane traffic using multicast transport to NG-RAN nodes over N3mb and/or to UPFs over N19mb. The destination multicast IP address identifies the multicast group, while the related unicast source IP address identifies the source of the multicast traffic (MB-UPF). The MB-UPF sets the outer header of the DL UP packets with the respective source IP address and destination multicast IP address.

Match Field: a field of the Packet Detection Information of a Packet Detection Rule against which a packet is attempted to be matched.

Matching: comparing the set of header fields of a packet to the match fields of the Packet Detection Information of a Packet Detection Rule.

MBS ingress address: It is used for the delivery of DL MBS UP traffic using unicast transport to the MB-UPF over N6mb or Nmb9 interface. The MBS ingress address identifies the MB-UPF's unicast IP address and port number for receiving DL data.

Multicast transport: the delivery of user plane traffic by IP multicast transport, as opposed to IP unicast transport. Multicast transport is supported over N3mb, N19mb and N6mb interfaces, but is not supported over Nmb9.

Node: Either a CP function or an UP function supporting one or more PFCP entities. A Node is identified by the Node ID, which is set to either an FQDN or an IP address.

PFCP Entity: An endpoint in a CP (or UP) function supporting PFCP, that is identified by the IP address. The IP address of a PFCP entity may or may not be the IP address included in the Node ID.

Source Specific Multicast address (SSM): It is used to deliver user plane traffic using multicast transport. The destination multicast IP address identifies the multicast group, while the related unicast source IP address identifies the source of the multicast data (e.g. AF/AS or MBSTF).

(S)RTP Multiplexed Media Information: The Packet filter information used to identify different media flows that are transported in (S)RTP, when (S)RTP, (S)RTCP and other associated protocols are multiplexed into a single IP traffic flow as specified in IETF RFC 5761 [x1], IETF RFC 5764 [x2], IETF RFC 7983 [x3], IETF RFC 8872 [x4], IETF RFC 9143 [x5] and IETF RFC 9443 [x6].

Unicast transport: the delivery of user plane traffic by IP unicast transport, as opposed to IP multicast transport. For MBS, unicast transport is supported over N3mb, N19mb, N6mb and Nmb9 interfaces.

UP function: A node with a User Plane function (see 3GPP TS 23.214 [2]) supporting one or more PFCP entities. A User Plane function, i.e. a User Plane Node, is identified by the Node ID that is set to either a FQDN or an IP address.

* * * Next Change * * * *

## 5.38	Support for high data rate low latency services, (eXtended Reality) XR and interactive media services

### 5.38.0	General

Stage 2 requirements related to the support for high data rate low latency services, eXtended Reality (XR) and interactive media services are specified in clause 5.37 of 3GPP TS 23.501 [28], which includes the following features requiring protocol enhancement to signalling interactions between the SMF and UPF over N4:

-	Support of ECN marking for L4S as specified in clause 5.37.3 of 3GPP TS 23.501 [28];

-	Support of Network Exposure of 5GS information, including control of reporting the congestion information, Data rate and Round trip delay measurement for QoS flow(s) as specified in clause 5.37.4 of 3GPP TS 23.501 [28];

-	Support of PDU Set based QoS Handling as specified in clause 5.37.5 of 3GPP TS 23.501 [28];

-	Support of N6 Jitter Measurement and End of Data Burst Indication for UE power saving in clause 5.37.8 of 3GPP TS 23.501 [28];

-	Support of traffic identification and differentiated QoS flow mapping for multiplexed media flows in the same transport connection, as specified in clause 5.37.x of 3GPP TS 23.501 [28].

It is optional for both the SMF and the UPF to support above features, hence the SMF may trigger the procedures specified in this clause only when the UPF has indicated its support of the corresponding feature via the UP Function Features IE. (see clause 8.2.25)

* * * Next Change * * * *

### 5.38.X	Traffic identification and differentiated QoS for multiplexed media flows

Stage 2 requirements for traffic identification and differentiated QoS for media flows multiplexed on a same end-to-end transport layer connection are specified in clause 5.37.x of 3GPP TS 23.501 [28].

A UPF may support traffic identification and differentiated QoS for multiplexed media flows. A UPF that supports this capability shall indicate its support of the MUXMF feature in the UP Function IE (see clause 8.2.25).

If the UPF supports the MUXMF feature, the SMF may provision SDF Filters in PDRs which include (S)RTP Multiplexed Media Information to differentiate different media flows that are transported in (S)RTP, when (S)RTP, (S)RTCP and other associated protocols are multiplexed into a single IP traffic flow.

* * * Next Change * * * *

### 8.2.5	SDF Filter

The SDF Filter IE type shall be encoded as shown in Figure 8.2.5-1. It contains an SDF Filter, i.e. a single IP flow packet filter.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 23 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  | SMMI | BID | FL | SPI | TTC | FD |  |
|  | 6 | Spare |  |  |  |  |  |  |  |  |
|  | m to (m+1) | Length of Flow Description |  |  |  |  |  |  |  |  |
|  | (m+2) to p | Flow Description |  |  |  |  |  |  |  |  |
|  | s to (s+1) | ToS Traffic Class |  |  |  |  |  |  |  |  |
|  | t to (t+3) | Security Parameter Index |  |  |  |  |  |  |  |  |
|  | v to (v+2) | Flow Label |  |  |  |  |  |  |  |  |
|  | w to (w+3) | SDF Filter ID |  |  |  |  |  |  |  |  |
|  | a | Number of (S)RTP Multiplexed Media Information |  |  |  |  |  |  |  |  |
|  | b to (b+1) | Length of (S)RTP Multiplexed Media Information 1 |  |  |  |  |  |  |  |  |
|  | (b+2) to c | (S)RTP Multiplexed Media Information 1 |  |  |  |  |  |  |  |  |
|  | … | … |  |  |  |  |  |  |  |  |
|  | … | … |  |  |  |  |  |  |  |  |
|  | d to (d+1) | Length of (S)RTP Multiplexed Media Information n |  |  |  |  |  |  |  |  |
|  | (d+2) to e | (S)RTP Multiplexed Media Information n |  |  |  |  |  |  |  |  |
|  | x to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.5-1: SDF Filter

The following flags are coded within Octet 5:

-	Bit 1 – FD (Flow Description): If this bit is set to "1", then the Length of Flow Description and the Flow Description fields shall be present, otherwise they shall not be present.

-	Bit 2 – TTC (ToS Traffic Class): If this bit is set to "1", then the ToS Traffic Class field shall be present, otherwise the ToS Traffic Class field shall not be present.

-	Bit 3 – SPI (Security Parameter Index): If this bit is set to "1", then the Security Parameter Index field shall be present, otherwise the Security Parameter Index field shall not be present.

-	Bit 4 – FL (Flow Label): If this bit is set to "1", then the Flow Label field shall be present, otherwise the Flow Label field shall not be present.

-	Bit 5 – BID (Bidirectional SDF Filter): If this bit is set to "1", then the SDF Filter ID shall be present, otherwise the SDF Filter ID shall not be present.

-	Bit 6 – SMMI (S(RTP) Multiplexed Media Information): If this bit is set to "1", then the Number of (S)RTP Multiplexed Media Information, the Length of (S)RTP Multiplexed Media Information and the (S)RTP Multiplexed Media Information fields shall be present, otherwise they shall not be present.

-	Bit 7 to 8: Spare, for future use and set to "0".

The Flow Description field, when present, shall be encoded as an OctetString as specified in clause 5.4.2 of 3GPP TS 29.212 [8].

The ToS Traffic Class field, when present, shall be encoded as an OctetString on two octets as specified in clause 5.3.15 of 3GPP TS 29.212 [8]. The ToS/Traffic Class mask field may be set to detect packets with a specific DSCP value (e.g. "0xFC"), or with specific ECN bits (e.g. "0x03") or both (e.g. "0xFF").

The Security Parameter Index field, when present, shall be encoded as an OctetString on four octets and shall contain the IPsec security parameter index (which is a 32-bit field), as specified in clause 5.3.51 of 3GPP TS 29.212 [8].

The Flow Label field, when present, shall be encoded as an OctetString on 3 octets as specified in clause 5.3.52 of 3GPP TS 29.212 [8] and shall contain an IPv6 flow label (which is a 20-bit field). The bits 8 to 5 of the octet "v" shall be spare and set to zero, and the remaining 20 bits shall contain the IPv6 flow label.

An SDF Filter may:

-	be a pattern for matching the IP 5 tuple (source IP address or IPv6 network prefix, destination IP address or IPv6 network prefix, source port number, destination port number, protocol ID of the protocol above IP). In the pattern:

-	a value left unspecified in a filter matches any value of the corresponding information in a packet;

-	an IP address may be combined with a prefix mask;

-	port numbers may be specified as port ranges;

-	the pattern can be extended by the Type of Service (TOS) (IPv4) / Traffic class (IPv6) and Mask;

-	consist of the destination IP address and optional mask, protocol ID of the protocol above IP, the Type of Service (TOS) (IPv4) / Traffic class (IPv6) and Mask and the IPsec Security Parameter Index (SPI);

-	consist of the destination IP address and optional mask, the Type of Service (TOS) (IPv4) / Traffic class (IPv6) and Mask and the Flow Label (IPv6);

NOTE 1:	The details about the IPsec Security Parameter Index (SPI), the Type of Service (TOS) (IPv4) / Traffic class (IPv6) and Mask and the Flow Label (IPv6) are defined in 3GPP TS 23.060 [19] clause 15.3.

-	extend the packet inspection beyond the possibilities described above and look further into the packet. Such service data flow filters need to be predefined in the PGW-U, as specified in clause 5.11 of 3GPP TS 23.214 [2].

NOTE 2:	Such filters may be used to support filtering with respect to a service data flow based on the transport and application protocols used above IP, e.g. for HTTP and WAP. Filtering for further application protocols and services can also be supported.

The SDF Filter ID, when present, shall be encoded as an Unsigned32 binary integer value. It shall uniquely identify an SDF Filter among all the SDF Filters provisioned for a given PFCP Session. The source/destination IP address and port information, in a bidirectional SDF Filter, shall be set as for downlink IP flows. The SDF filter for the opposite direction has the same parameters, but having the source and destination address/port parameters swapped. When being provisioned with a bidirectional SDF filter in a PDR, the UP function shall apply the SDF filter as specified in clause 5.2.1A.2A.

The Number of (S)RTP Multiplexed Media Information field shall indicate the number of (S)RTP Multiplexed Media Information (and related Length of (S)RTP Multiplexed Media Information) included in the SDF Filter IE, as an integer greater or equal to 1.

When present, the (S)RTP Multiplexed Media Information field shall be encoded as defined in Figure 8.2.5-x.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 | Spare |  |  |  |  |  | PTI | SSRCI |  |
|  | a | Number of Synchronization Source (SSRC) |  |  |  |  |  |  |  |  |
|  | b to (b+3) | Synchronization Source (SSRC) 1 |  |  |  |  |  |  |  |  |
|  | … | … |  |  |  |  |  |  |  |  |
|  | (b+4(n-1)) to (b+4(n-1)+3) | Synchronization Source (SSRC) n |  |  |  |  |  |  |  |  |
|  | c | APTI | Payload Type (PT) |  |  |  |  |  |  |  |
|  | c+1 | APTI | Additional Payload Type (PT) |  |  |  |  |  |  |  |
|  | c+2 | … | … |  |  |  |  |  |  |  |
|  | c+3 | APTI | Additional Payload Type (PT) |  |  |  |  |  |  |  |
|  | d | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.5-x: (S)RTP Multiplexed Media Information

The following flags are coded within Octet 1:

-	Bit 1 – SSRCI (SSRC Indicator): If this bit is set to "1", then the Number of Synchronization Source (SSRC) field and the Synchronization Source (SSRC) field shall be present, otherwise these fields shall not be present.

-	Bit 2 – PTI (Payload Type Indicator): If this bit is set to "1", then the Payload Type (PT) field shall be present, otherwise this field shall not be present.

-	Bit 3 to 8 are spare and reserved for future use, shall be set to "0" by the sender and discarded by the receiver.

The Number of Synchronization Source (SSRC) field shall indicate the number of Synchronization Source (SSRC) fields included in the (S)RTP Multiplexed Media Information, as an integer greater or equal to 1.

The Synchronization Source (SSRC) field shall be encoded as a 32-bit identifier (see IETF RFC 8872 [x4]).

The Payload Type field shall encode the (S)RTP payload type, as an integer between (and including) 1 and 127.

The APTI (Additional Payload Type Indicator) field indicates whether an additional payload type is encoded in the following octet. If this flag is set to "1", this indicates that an additional payload type is encoded in the following octet, otherwise no additional payload type is encoded in the following octet.

If the SSRC field is present without any payload type field, any user plane packet with a matching SSRC field shall be considered as matching the (S)RTP Multiplexed Media Information.

If multiple Synchronization Source (SSRC) fields are included, the Payload Types shall apply to all SSRCs; if no payload type field is included, any user plane packet matching one of the SSRCs shall be considered as matching the (S)RTP Multiplexed Media Information.

If the (S)RTP Multiplexed Media Information does not include any SSRC, i.e. if the SSRCI bit is set to 0, any user plane packet matching one Payload Type included in the (S)RTP Multiplexed Media Information shall be considered as matching the (S)RTP Multiplexed Media Information.

Editor's note:	The encoding of the (S)RTP Multiplexed Media Information needs alignment with CT3/CT1 and is FFS.

Editor's note:	Further (S)RTP Multiplexed Media Information for identification of multiplexed RTCP packets is FFS depending on input from SA2 and SA4.

* * * Next Change * * * *

### 8.2.25	UP Function Features

The UP Function Features IE indicates the features supported by the UP function. It is coded as depicted in Figure 8.2.25-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 43 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to 6 | Supported-Features |  |  |  |  |  |  |  |  |
|  | 7 to 8 | Additional Supported-Features 1 |  |  |  |  |  |  |  |  |
|  | 9 to 10 | Additional Supported-Features 2 |  |  |  |  |  |  |  |  |
|  | 11 to 12 | Additional Supported-Features 3 |  |  |  |  |  |  |  |  |
|  | 13 to 14 | Additional Supported-Features 4 |  |  |  |  |  |  |  |  |
|  | 15 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.25-1: UP Function Features

The UP Function Features IE takes the form of a bitmask where each bit set indicates that the corresponding feature is supported. Spare bits shall be ignored by the receiver. The same bitmask is defined for all PFCP interfaces.

The following table specifies the features defined on PFCP interfaces and the interfaces on which they apply.

Table 8.2.25-1: UP Function Features

| Feature Octet / Bit | Feature | Interface | M/O | Description |
| --- | --- | --- | --- | --- |
| 5/1 | BUCP | Sxa, N4 | O (NOTE 2) | Downlink Data Buffering in CP function is supported by the UP function. |
| 5/2 | DDND | Sxa, N4 | O | The buffering parameter 'Downlink Data Notification Delay' is supported by the UP function. |
| 5/3 | DLBD | Sxa, N4 | O | The buffering parameter 'DL Buffering Duration' in PFCP Session Report Response is supported by the UP function. |
| 5/4 | TRST | Sxb, Sxc, N4 | O | Traffic Steering is supported by the UP function. |
| 5/5 | FTUP | Sxa, Sxb, N4 | M | F-TEID allocation / release in the UP function is supported by the UP function. |
| 5/6 | PFDM | Sxb, Sxc, N4 | O | The PFD Management procedure is supported by the UP function. |
| 5/7 | HEEU | Sxb, Sxc, N4 | O | Header Enrichment of Uplink traffic is supported by the UP function. |
| 5/8 | TREU | Sxb, Sxc, N4 | O (NOTE 3) | Traffic Redirection Enforcement in the UP function is supported by the UP function. |
| 6/1 | EMPU | Sxa, Sxb, N4 | M | Sending of End Marker packets supported by the UP function. |
| 6/2 | PDIU | Sxa, Sxb, Sxc, N4, N4mb | O (NOTE 4) | Support of PDI optimised signalling in UP function (see clause 5.2.1A.2). |
| 6/3 | UDBC | Sxb, Sxc, N4 | O | Support of UL/DL Buffering Control |
| 6/4 | QUOAC | Sxb, Sxc, N4 | O | The UP function supports being provisioned with the Quota Action to apply when reaching quotas. |
| 6/5 | TRACE | Sxa, Sxb, Sxc, N4 | O | The UP function supports Trace (see clause 5.15). |
| 6/6 | FRRT | Sxb, N4 | O | The UP function supports Framed Routing (see IETF RFC 2865 [37] and IETF RFC 3162 [38]). |
| 6/7 | PFDE | Sxb, N4 | O | The UP function supports a PFD Contents including a property with multiple values. |
| 6/8 | EPFAR | Sxa, Sxb, Sxc, N4 | O | The UP function supports the Enhanced PFCP Association Release feature (see clause 5.18). |
| 7/1 | DPDRA | Sxb, Sxc, N4 | O | The UP function supports Deferred PDR Activation or Deactivation. |
| 7/2 | ADPDP | Sxa, Sxb, Sxc, N4 | O | The UP function supports the Activation and Deactivation of Pre-defined PDRs (see clause 5.19). |
| 7/3 | UEIP | Sxb, N4 | O (NOTE 5) | The UP function supports allocating UE IP addresses or prefixes (see clause 5.21). |
| 7/4 | SSET | N4, N4mb | O | (MB-)UPF support of PFCP sessions successively controlled by different (MB-)SMFs of a same (MB-)SMF Set (see clause 5.22). |
| 7/5 | MNOP | Sxa, Sxb, Sxc, N4 | O | The UP function supports measurement of number of packets which is instructed with the flag 'Measurement of Number of Packets' in a URR. See also clause 5.2.2.2.1. |
| 7/6 | MTE | N4 | O | UPF supports multiple instances of Traffic Endpoint IDs in a PDI. |
| 7/7 | BUNDL | Sxa, Sxb, Sxc, N4, N4mb | O | PFCP messages bundling (see clause 6.5) is supported by the UP function. |
| 7/8 | GCOM | N4 | O | UPF support of 5G VN Group Communication. (See clause 5.23) |
| 8/1 | MPAS | N4, N4mb | O | (MB-)UPF support for multiple PFCP associations to the (MB-)SMFs in an (MB-)SMF set (see clause 5.22.3). |
| 8/2 | RTTL | N4 | O | UPF supports redundant transmission at transport layer. |
| 8/3 | VTIME | Sxb,N4 | O | UP function support of quota validity time feature. |
| 8/4 | NORP | Sxa, Sxb, Sxc, N4 | O | UP function support of Number of Reports as specified in clause 5.2.2.2. |
| 8/5 | IPTV | N4 | O | UPF support of IPTV service (see clause 5.25) |
| 8/6 | IP6PL | N4 | O | UPF supports: - UE IPv6 address(es) allocation with IPv6 prefix length other than default /64 (including allocating /128 individual IPv6 addresses), as specified in clause 4.6.2.2 of of 3GPP TS 23.316 [57]; and - multiple UE IPv6 addresses allocation using multiple instances of the UE IP Address IE in a same PDI or Traffic Endpoint, or using multiple PDIs or Traffic Endpoints with a different UE IP Address as specified in clause 5.21.1. |
| 8/7 | TSN | N4 | O | Integration of 5GS into a TSN data network is supported by the UPF (see clause 5.26.1). |
| 8/8 | MPTCP | N4 | O | UPF support of MPTCP Proxy functionality (see clause 5.20) |
| 9/1 | ATSSS-LL | N4 | O | UPF support of ATSSS-LL steering functionality (see clause 5.20) |
| 9/2 | QFQM | N4 | O | UPF support of per QoS flow per UE QoS monitoring for packet delay (see clause 5.24.4). |
| 9/3 | GPQM | N4 | O | UPF support of per GTP-U Path QoS monitoring (see clause 5.24.5). |
| 9/4 | MT-EDT | Sxa | O | SGW-U support of reporting the size of DL Data Packets. (see clause 5.2.4.1). |
| 9/5 | CIOT | Sxb, N4 | O | UP function support of CIoT feature, e.g. small data packet rate enforcement. (see 5.4.15) |
| 9/6 | ETHAR | N4 | O | UPF support of Ethernet PDU Session Anchor Relocation (see clause 5.13.6). |
| 9/7 | DDDS | N4 | O | UPF support of reporting the first buffered / first discarded downlink data after buffering / directly dropped downlink data for downlink data delivery status notification. |
| 9/8 | RDS | Sxb, N4 | O | UP function support of Reliable Data Service (see clause 5.29). |
| 10/1 | RTTWP | N4 | O | UPF support of RTT measurements towards the UE Without PMF. |
| 10/2 | QUASF | Sxb, Sxc, N4 | O | The UP function supports being provisioned in a URR with an Exempted Application ID for Quota Action or an Exempted SDF Filter for Quota Action which is to be used when the quota is exhausted. See also clauses 5.2.2.2.1 and 5.2.2.3.1. |
| 10/3 | NSPOC | Sxa, Sxb, N4 | O | UP function supports notifying start of Pause of Charging via user plane. |
| 10/4 | L2TP | Sxb, N4 | O | UP function supports the L2TP feature as described in clause 5.31. |
| 10/5 | UPBER | N4 | O | UP function supports the uplink packets buffering during EAS relocation. |
| 10/6 | RESPS | Sxb, N4 | O | UP function supports Restoration of PFCP Sessions associated with one or more PGW-C/SMF FQ-CSID(s), Group Id(s) or CP IP address(es) (see clause 5.22.4) |
| 10/7 | IPREP | N4 | O | UP function supports IP Address and Port number replacement (see clauses 5.33.3, 5.33.4 and 5.33.6). |
| 10/8 | DNSTS | N4 | O | UP function support DNS Traffic Steering based on FQDN in the DNS Query message (see clause 5.33.4) |
| 11/1 | DRQOS | N4 | O | UP function supports Direct Reporting of QoS monitoring events to Local NEF or AF (see clause 5.33.5). |
| 11/2 | MBSN4 | N4 | O | UPF supports sending MBS multicast session data to associated PDU sessions using 5GC individual delivery. |
| 11/3 | PSUPRM | N4, N4mb | O | UP function supports Per Slice UP Resource Management (see clause 5.35). |
| 11/4 | EPPPI | N4 | O | UP function supports Enhanced Provisioning of Paging Policy Indicator feature as specified in clause 5.36.2. |
| 11/5 | RATP | Sxb, N4 | O | UP function supports Redirection Address Types set to "Port", "IPv4 address and Port", "IPv6 address and Port", or "IPv4 and IPv6 addresses and Port". |
| 11/6 | UPIDP | N4 | O | UP function supports User Plane Inactivity Detection and reporting per PDR feature as specified in clause 5.11.3. |
| 11/7 | AFSFC | N4 | O | UP function supports inserting metadata when Application Function influence on Service Function Chaining is used as specified in clause 5.4.8. |
| 11/8 | MPQUIC | N4 | O | UPF support of MPQUIC Proxy functionality (see clause 5.20). |
| 12/1 | REDSM | N4 | O | UP function supports the Redundant Steering Mode (see clause 5.32.8 of 3GPP TS 23.501 [28] and clause 5.2.7.1) |
| 12/2 | DBDM | Sxa, N4 | O | The buffering parameters 'DL Buffering Duration' and 'DL Buffering Suggested Packet Count' in PFCP Session Modification Request are supported by the UP function. |
| 12/3 | TSCTS | N4 | O | AF requested support of Time Synchronization and/or Time Sensitive Communication is supported by the UP function (see clause 5.26.1). |
| 12/4 | DRTSC | N4 | O | UP function supports Direct Reporting of TSC management information events to TSN AF or TSCTSF (see clause 5.26.3.2). |
| 12/5 | N6JEDB | N4 | O | UPF support of N6 Jitter, DL Periodicity and UL Periodicity Measurement and Reporting, and End of Data Burst marking as specified in clause 5.38.4. |
| 12/6 | QMCON | N4 | O | UPF support of QoS monitoring of Congestion information (see clause 5.39.3.3). |
| 12/7 | DETNET | N4 | O | Deterministic Networking is supported by the UP function (see clause 5.26.1). |
| 12/8 | EML4S | N4 | O | UP function supports ECN Marking for L4S (see clause 5.38.1). |
| 13/1 | PDUSM | N4 | O | UP function supports PDU Set Marking (see clause 5.38.3). |
| 13/2 | CN-TL | N4 | O | UPF support of the TSN Talker and Listener (TL) functionality, i.e. CN-TL. See clause 5.26a. |
| 13/3 | QMDRM | N4 | O | UPF support of QoS monitoring of Data Rate Measurement (see clause 5.39.3.4). |
| 13/4 | EDBNC | N4 | O | UP function supports Extended DL Buffering Notification Control (see clause 5.2.4.1). |
| 13/5 | MT-SDT | N4 | O | UPF support of reporting the size of DL Data Packets per QoS flow (see clause 5.2.4.1). |
| 13/6 | UPSBIES | N4 | O | UPF support of event subscription via Service Based Interface. (See clause 5.40). |
| 13/7 | UMN6IP | N4 | O | UPF supports to allocate a mapped N6 IP address and perform IP replacement as described in clause 5.33.7. |
| 13/8 | UN6TU | N4 | O | UPF supports N6 tunnelling using N6 routing information as described in clause 5.33.7. |
| 14/1 | MBSCH | N4mb | O | MB-UPF support of MBS charging (see clause 5.34.2.5) |
| 14/x | MUXMF | N4 | O | UPF support of Traffic identification and differentiated QoS for multiplexed media flows (see clause 5.38.x). |
| NOTE 1: Features are defined as follows: - Feature Octet / Bit: The octet and bit number within the Supported-Features IE, e.g. "5 / 1". - Feature: A short name that can be used to refer to the octet / bit and to the feature. - Interface: A list of applicable interfaces to the feature. - M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O") for a UP function complying with this release of the specification. A feature defined as mandatory in this release of the specification may not necessarily be defined or mandatory to support in earlier releases (see the respective specifications), therefore a CP function complying with this release of the specification shall be prepared to receive a UP Function Features IE with a mandatory feature bit set to "0". - Description: A clear textual description of the feature. NOTE 2: Downlink data should be buffered preferably in the UP function. Downlink data may be buffered in the CP function when desired, e.g. for UEs using power saving methods. NOTE 3: If the traffic that the UP function can support may be subject to traffic redirection, traffic redirection enforcement in the UP function shall be supported by the UP function. NOTE 4: CP and UP functions complying with this release of the specification should support this feature. NOTE 5: A UPF that supports the SSET or MPAS feature shall support this feature. |  |  |  |  |

* * * End of Changes * * * *