3GPP TSG-CT6 Meeting #120-bis	C6-240626

# Orlando, United States, 19th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0782 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PLI slice info test cases to handle allowed slices with mapping information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple AB Denmark, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R18 |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently there is no Rel 18 test cases to verify PROVIDE LOCAL INFORMATION - slice information in roaming scenarios, when the roaming network has provided allowed slices with mapping information in registration accept IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicability table updated (Section 3.4) to include this new test case for Rel 18 UE. Initial conditions updated to include a new V-PLMN cell specifying allowed s-NNSAI with mapping configuration (section 27.22.4.15.4.1) New expected test sequence (1.26A) added (Section 27.22.4.15.4.2) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case to verify UE behavior to handle PROVIDE LOCAL INFORMATION - slice information in roaming scenarios |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.4.15.4.1, 27.22.4.15.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has dependency on CR 0780 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##first change###

## 3.4	Applicability table

NOTE:	It is possible that the applicability of some tests indicated in table B.1 does not match with the value in the Release column, due to late definition of the test sequences. Tests should be performed without considering the Release column, but only based on the conditions indicated for each release.

Table B.1: Applicability of tests

| Test | Test sequence | Description | from Rel. | up to Rel. (see note 2) | Applicability | Terminal Profile | Network Dependency | Support | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27.22.1 |  | PROFILE DOWNLOAD |  |  |  |  |  |  |  |
|  | 1 | PROFILE DOWNLOAD | R99 |  | M | E.1/1 | No |  |  |
| 27.22.2 |  | Contents of the TERMINAL PROFILE command |  |  |  |  |  |  |  |
|  | 1 | Contents of the TERMINAL PROFILE command | R99 |  | M | E.1/1 | No |  |  |
| 27.22.3 |  | Servicing of Proactive UICC Commands |  |  |  |  |  |  |  |
|  | 1 | Servicing of Proactive UICC Commands | R99 |  | M |  | No |  |  |
| 27.22.4.1 |  | DISPLAY TEXT |  |  |  |  |  |  |  |
|  | 1.1 | Unpacked | R99 |  | C177 | E.1/17 AND E.1/110 | No |  |  |

…

| 27.22.4.15 |  | PROVIDE LOCAL INFORMATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | location information | R99 |  | M | E.1/31 | USS only |  | AER003 |
|  | … | … | … | … | … | … | … |  |  |
|  | 1.24 | Slices Information | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.25 | Slices Information, no served Slice | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.26 | Slices Information, several served Slices | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.26A | Slices Information, several served Slices, allowed slices with same mapping information | Rel-18 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.26B | Slices Information, several served Slices, allowed slices with different mapping information | Rel-18 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.27 | NG-RAN Timing advance | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.28 | PROVIDE LOCAL INFORMATION, NG-RAN Intra-Frequency Measurements | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.29 | PROVIDE LOCAL INFORMATION, NG-RAN Inter-Frequency Measurements | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.30 | PROVIDE LOCAL INFORMATION, CAG information list | Rel-17 |  | C235 | E.1/287 | NG-SS only |  |  |
|  | 1.31 | Satellite NG-RAN PrimaryTiming advance | Rel-17 |  | C236 | E.1/305 | SAT-NG-SS only |  |  |
|  | 1.32 | Access Technology, Satellite NG-RAN | Rel-17 |  | C236 | E.1/72 | SAT-NG-SS only |  |  |

##second changes###

#### 27.22.4.15	PROVIDE LOCAL INFORMATION

##### 27.22.4.15.1	Definition and applicability

See clause 3.2.2.

##### 27.22.4.15.2	Conformance requirement

The ME shall support the PROVIDE LOCAL INFORMATION facility as defined in:

-	TS 31.111 [15] clause 6.4.15.

##### 27.22.4.15.3	Test purpose

…

##### 27.22.4.15.4	Method of tests

27.22.4.15.4.1	Initial conditions

The ME is connected to the USIM Simulator.

For sequences 1.1 to 1.7 and 1.9 to 1.13 the UICC shall contain elementary files coded as defined in the default values for USIM Application Toolkit testing from clause 27.22.2A.

To operate the sequences 1.14 to 1.18 the E-UTRAN/EPC UICC as defined in clause 27.22.2B.1 shall be used.

To operate the sequences 1.22 to 1.29, 1.31 and 1.32 the NG-RAN UICC as defined in clause 27.22.2D.1 shall be used.

Prior to these test sequences the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

To operate the sequence 1.30 the NG-RAN UICC supporting CAG, as defined in clause 27.22.2D.4 shall be used.

Sequences 1.2, 1.4, 1.5, 1.9 and 1.11 do not need any System Simulator connection.

For sequences 1.1, 1.7, 1.12 and 1.13 the ME is connected to a USS, for sequences 1.15, 1.16 and 1.18 the ME is connected to an E-USS only, for sequences 1.14 and 1.17 the ME is connected to an E-USS or NB-SS. In all listed sequences it has to perform the location update procedure or routing area update or combined update procedure.

For sequences 1.10 the ME is connected to an SS, a USS or an E-USS.

For sequences 1.22 to 1.30 the ME is connected to NG-SS. For all sequences but 1.30 it has to perform the Registration procedure.

The browser's cache shall have been cleared before execution of the test sequence. For sequence 1.24, 1.25, 1.26, 1.26A and 1.26B the URSP rules stored in the ME are:

URSP:

Rule Precedence =1

Traffic Descriptor:

- DNN=TestGp.rs

Route Selection Descriptor:

- Precedence=1

- Network Slice Selection, S-NSSAI: 01 01 01 03 (ST: eMBB, SD: 010103)

- SSC Mode Selection: SSC Mode 1

- Access Type preference: 3GPP access

Rule Precedence =2

Traffic Descriptor:

- DNN=Test12.rs

Route Selection Descriptor:

- Precedence=1

- Network Slice Selection, S-NSSAI: 01 01 01 02 (ST: eMBB, SD: 010102)

- SSC Mode Selection: SSC Mode 1

- Access Type preference: 3GPP access

Rule Precedence = <lowest priority>

Traffic Descriptor: *

Route Selection Descriptor:

- Precedence =1

- SSC Mode Selection: SSC Mode 1

- Access Type preference: 3GPP access

The NG-RAN parameters of the system simulator are:

- -	Mobile Country Code (MCC) = 001;

- -	Mobile Network Code (MNC) = 01;

- -	Tracking Area Code (TAC) = 000001;

- -	NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

- -	Allowed S-NSSAIs = 01010103 (SST: eMBB, SD: 010103), 01010102 (SST: eMBB, SD: 010102);

- -	NG-RAN/Satellite NG-RAN Primary Timing Advance = 0;

The NG-RAN parameters of the system simulator for sequence 1.26A is:

- -	Mobile Country Code (MCC) = 001;

- -	Mobile Network Code (MNC) = 02;

- -	Tracking Area Code (TAC) = 000001;

- -	NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

- -	Allowed S-NSSAIs is configured to include mapping information =

'01010103' (SST: eMBB, SD: 010103) '01010103' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010103)

'01010102' (SST: eMBB, SD: 010102) '01010102' (Mapped HPLMN SST: eMBB, mapped HPLMN SD:010102);

- -	NG-RAN/Satellite NG-RAN Primary Timing Advance = 0;

The NG-RAN parameters of the system simulator for sequence 1.26B is:

- -	Mobile Country Code (MCC) = 001;

- -	Mobile Network Code (MNC) = 02;

- -	Tracking Area Code (TAC) = 000001;

- -	NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

- -	Allowed S-NSSAIs is configured to include mapping information:

'01010106' (SST: eMBB, SD: 010106) '01010103' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010103)

'01010104' (SST: eMBB, SD: 010104) '01010102' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010102);

- -	NG-RAN/Satellite NG-RAN Primary Timing Advance = 0;

The E-UTRAN/NB-IoT parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

The UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

The GERAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

-	Timing advance = 0;

-	Neighbour allocations = 561, 565, 568, 569, 573, 575, 577, 581, 582 and 585.

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

-	Timing advance = 0;

-	Neighbour allocations = 561, 565, 568, 569, 573, 575, 577, 581, 582 and 585.

Expected sequence 1.3 and 1.6 shall be used on a USS setting up only a GERAN or PCS 1900 cell and expected sequences 1.7 and 1.12 shall be used on a USS setting up only a UTRAN cell.

Expected sequence 1.12 requires two UTRA cells on the same frequency and 1.13 requires two UTRA cells on different frequencies.

Expected sequences 1.14 and 1.17 shall be used on a E-USS/NB-SS setting up only a E-UTRAN/NB-IoT cell.

Expected sequence 1.22, 1.23, 1.24, 1.25, 1.26, 1.26A, 1.26B and 1.27 shall be used on a NG-SS setting up only a NG-RAN cell.

Expected sequence 1.15 requires two E-UTRA cells on the same frequency and 1.16 requires two E-UTRA cells on different frequencies, with second cell having EARFCN less than maxEARFCN. For both sequences if the USIM request is triggered in the RRC_CONNECTED state, the system simulator shall configure the corresponding frequency measurement for a sufficient period before sending the USIM request.

Expected sequence 1.18 requires two E-UTRAN cells configured in CSG mode.

For sequence 1.18 the default E-UTRAN/EPC UICC is used and the E-USS transmits on two cells with the following parameters:

Network parameters for Cell 1:

-	TAI (MCC/MNC/TAC):	001/01/0001.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	01 (27 bits)

- Home (e)NB Name	Home ONE

Network parameters for Cell 2:

-	TAI (MCC/MNC/TAC):	001/01/0002.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	02 (27 bits)

- Home (e)NB Name	Home TWO

Expected sequences 1.28 and 1.29 require two NG-RAN cells on the same NR band.

For sequences 1.28 and 1.29, the default NG-RAN UICC is used and the two cells with the following parameters:

Network parameters for Cell 1:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	 NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

-	Allowed S-NSSAIs = 01010103 (SST: eMBB, SD: 010103), 01010102 (SST: eMBB, SD: 010102);

-	 NG-RAN/Satellite NG-RAN Primary Timing Advance = 0;

Network parameters for Cell 2:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	 NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0002 (36 bits);

For sequence 1.28, the Cell 1 is the serving cell, and Cell 2 is the intra-frequency neighbour cell.

Note – Cell 1 and Cell 2 are configured with proper cell reference power, as well as SIB1 and SIB2 cellReselection Thresholds and Priority values that ensure ME will not reselect to Cell 2.

For sequence 1.29, the Cell 1 is the serving cell, and Cell 2 is the inter-frequency neighbour cell.

Note – Cell 1 is configured with SIB4 interFreqCarrierFreqList includes dl-CarrierFreq of Cell 2 Downlink NR SSB ARFCN; Cell 1 and Cell 2 are configured with proper cell reference power, as well as SIB1 and SIB4 with the cellReselection Thresholds and Priority values that ensure ME will not reselect to Cell 2.

For sequences 1.28 and 1.29, the ME is connected to Cell 1 and it has performed the Registration procedure.

Expected sequence 1.30 requires two NR CAG cells.

For sequence 1.30, the NG-SS transmits two NR CAG cells with following configuration:

Cell 1:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 083;

-	Tracking Area Code (TAC) = 000001;

Cell 2:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 084;

-	Tracking Area Code (TAC) = 000002;

CAG cell 1 has the following parameters configured for NPN-Identity and corresponding HRNN in SIB1 and SIB 10 respectively:

Entry 1:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 083;

-	CAG ID:	00 00 00 01

-	No CAG human-readable network name broadcasted

Entry 2:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 084;

-	CAG ID:	00 00 00 02

-	CAG human-readable network name: ′CAG-00000002′

Expected sequence 1.31 shall be used on a SAT-NG-SS setting up only a Satellite NG-RAN cell with following configuration:

The Satellite NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

-	NG-RAN/Satellite NG-RAN Primary Timing Advance = 0.

27.22.4.15.4.2	Procedure

#####Third change####

Expected Sequence 1.26A (PROVIDE LOCAL INFORMATION, slice(s) information), several served slices, allowed slices with same mapping information

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER → ME | Internet PDU session using DNN "TestGp.rs" is configured in the terminal. The second PDU session using DNN "Test12.rs" is configured in the terminal | DNN: "TestGp.rs" for internet PDU |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | Internet PDU Session is established successfully. | DNN=TestGp.rs, First Served S-NSSAI = 01010103 |
| 4 | USER ME | Initiate a data call using DNN=”Test12.rs” |  |
| 5 | ME NG-SS | Second PDU Session is established successfully. | DNN=Test12.rs, Second Served S-NSSAI = 01010102 |
| 6 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.24.1 |  |
| 7 | ME UICC | FETCH |  |
| 8 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.24.1 |  |
| 9 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.26A.1 | [Command performed successfully] |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.26A.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"15" slices information

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Slice Information

Slices information (served)

Number of Served S-NSSAIs: 2

First Served S-NSSAI:  '01 01 01 03' (SST: eMBB, SD: 010103)

Second Served S-NSSAI:  '01 01 01 02' (SST: eMBB, SD: 010102)

Allowed Slices information: empty (zero length)

Allowed Slices information with S-NSSAI mapping:

Length of allowed S-NSSAI with mapping information elements: 16

Allowed S-NSSAI with mapping information element 1:  '01010103' (SST: eMBB, SD: 010103) '01010103' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010103)

Allowed S-NSSAI with mapping information element 2:  '01010102' (SST: eMBB, SD: 010102) '01010102' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010102)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 15 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D6 | 09 | 02 | 01 | 01 | 01 | 03 | 01 | 01 | 01 | 02 | 78 |
|  | 00 | 77 | 10 | 01 | 01 | 01 | 03 | 01 | 01 | 01 | 03 | 01 |
|  | 01 | 01 | 01 | 02 | 01 | 01 | 01 | 02 |  |  |  |  |

Expected Sequence 1.26B (PROVIDE LOCAL INFORMATION, slice(s) information), several served slices, allowed slices with different mapping information

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER → ME | Internet PDU session using DNN "TestGp.rs" is configured in the terminal. The second PDU session using DNN "Test12.rs" is configured in the terminal | DNN: "TestGp.rs" for internet PDU |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | Internet PDU Session is established successfully. | DNN=TestGp.rs, First Served S-NSSAI = 01010106 |
| 4 | USER ME | Initiate a data call using DNN=”Test12.rs” |  |
| 5 | ME NG-SS | Second PDU Session is established successfully. | DNN=Test12.rs, Second Served S-NSSAI = 01010104 |
| 6 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.24.1 |  |
| 7 | ME UICC | FETCH |  |
| 8 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.24.1 |  |
| 9 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.26B.1 | [Command performed successfully] |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.26B.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"15" slices information

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Slice Information

Slices information (served)

Number of Served S-NSSAIs: 2

First Served S-NSSAI:  '01 01 01 06' (SST: eMBB, SD: 010106)

Second Served S-NSSAI:  '01 01 01 04' (SST: eMBB, SD: 010104)

Allowed Slices information: empty (zero length)

Allowed Slices information with S-NSSAI mapping:

Length of allowed S-NSSAI with mapping information elements: 16

Allowed S-NSSAI with mapping information element 1:  '01010106' (SST: eMBB, SD: 010106) '01010103' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010103)

Allowed S-NSSAI with mapping information element 2:  '01010104' (SST: eMBB, SD: 010104) '01010102'  (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010102)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 15 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D6 | 09 | 02 | 01 | 01 | 01 | 06 | 01 | 01 | 01 | 04 | 78 |
|  | 00 | 77 | 10 | 01 | 01 | 01 | 06 | 01 | 01 | 01 | 03 | 01 |
|  | 01 | 01 | 01 | 04 | 01 | 01 | 01 | 02 |  |  |  |  |

##end of changes###