3GPP TSG RAN Meeting #104	 RP-24 1773

Melbourne, Australia, September 9-12, 2024

Agenda item:		9.2.2	Study on solutions for Ambient IoT in NR [New RAN1 SI: FS_Ambient_IoT_solutions]

Source:	Nokia

Title:	Rel-19 study on Ambient IoT - Clarifications

Document for:	Discussion

# 1	Introduction

In RAN#103 the Study on solutions for Ambient IoT (Internet of Things) in NR [1] was approved.

In this paper, we propose a way forward and ask for clarifications of the SID [1].

# 2	Discussion

## 2.1	Motivation & proposal

With reference to [1] the general scope of Ambient IoT is:

General Scope:

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope:

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

- X  is to be decided in WGs.

- Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

- For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ.

NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement.

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

- FR1 licensed spectrum in FDD.

- Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

- Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command).

- From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case.

Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum.

The following objectives are set, within the General Scope:

- Evaluation assumptions

- Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1].

  - Clause 5.3: Applicable maximum distance target values(s)

  - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs

  - Clause 5.8: 2D distribution of devices

- Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4]

- Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1]

- Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology.

NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically.

NOTE: strive to minimize evaluation cases in RAN1.

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848.

Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation).

Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects).

## 2.2	Request for clarification

The Ambient IoT study [1] objective includes:

Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects).

Proximity determination was concluded in RAN1#118 [3] to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item.

Solution 1

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission

- then the device is determined as near to the reader based on measurements at the reader side

Solution 2

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader

Even with the agreement above, the “proximity determination” is a source for uncertainty in RAN1. In particular, the purpose of defining proximity determination. In Nokia’s view the purpose of proximity determination is to identify appropriate R2D reader, CW node and D2R reader for successful A-IoT device communication and to enable efficient (non-blind) reconfiguration of those entities by proximity identification of the weak link/s in multi-static configurations. Above agreement is only applicable for mono-static configurations.

Thus, we ask the RAN#105 to define the purpose for the “feasibility and required functionalities for proximity determination”. Proximity should be defined for all essential links for a successful Ambient IoT communication session (R2D, CW to Ambient IioT, D2R) for both mono-static configuration as well as multi-static configurations to enable efficient configuration and reconfigurations of these essential links and nodes.

Proposal 1: Define proximity for all essential links for a successful Ambient IoT communication session (R2D, CW to Ambient IoT, D2R) for both mono-static configuration as well as multi-static configurations to enable efficient configuration and reconfigurations of these essential links and nodes.

# 3	Conclusions

Proposal 1: Define proximity for all essential links for a successful Ambient IoT communication session (R2D, CW to Ambient IoT, D2R) for both mono-statis configuration as well as multi-statis configurations to enable efficient configuration and reconfigurations of these essential links and nodes.

# References

RP-234058: New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

RP-240929 Rel-19 study on Ambient IoT – Clarifications

R1-2406845	FL summary#2 on downlink and uplink channel/signal aspects